| Rapporteur Meeting of Questions 1/16 |  |  | Q1-H05 |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | STUDY GROUP 16 |  |
|  |  | Original: English |  |
| Question(s): | 1 | Geneva, 27-28 August 2008 |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |
| Source: | Polycom |  |  |
| Title: | Proposed H.281 changes for supporting multiple video streams |  |  |
| Purpose: | Proposal |  |  |

# 1.	Introduction

Since the adoption of H.239, H.320 systems have been capable of transmitting two video streams. Systems using H.245 signalling also can transmit multiple video streams. However the H.281 standard for far end camera control does not support camera control for the additional streams.

H.282 (Remote Device Protocol for Multimedia Applications) does provide for stream identification.  However, H.282 can only be used with H.320 systems that also support T.120 services – which are no longer widely supported.   H.282 itself was never widely deployed; in fact we are not aware of any implementations of this standard since its ratification in 1999.

H.281, by contrast, continues to be widely used in H.32x systems.  Therefore we propose that H.281 be extended to support multiple video streams.

## 2.	Discussion

We propose to extend H.281 by incorporating “channelized” forms of each message.  The channelized messages are identical to the existing H.281 messages, except they also include a 16 bit channelID.  We also propose that the existing messages and the existing FECC Capability be constrained to describe the “main video channel” as described in Table 1/H.239.

ChannelID is defined consistently with Section 8.3.2/H.239; carrying the AMC ChannelID values in H.320 systems and the logicalChannelNumber in H.245 systems.  Gateways and MCUs that forward this parameter would convert it (as specified also in Section 8.3.2/H.239).

## FECC Capability

The proposed modifications to H.281 include a modification to the FECC Client Capabilities described in section 6.1.

We propose that bit 6 of the “Number of presets octet” in Figure 8/H.281 signal if the H.281 client is channelized.  It would be be set to 1 if the H.281 client is capable of receiving the messages detailed below.  If bit 6 is set to 0, then the H.281 client is not capable of receiving these new messages.

| 8 | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  | C |  | B | Number of presets (binary encoded) |  |  |  |

FIGURE  1 Modified Number of Presets Octet

## VIDEO CHANNEL CAPABILITY Indication

If all the H.281 receivers are capable of receiving the Video Channel Capability Indication, then each H.281 client should broadcast the Video Channel Capability Indication for each open video channel (other than the main video channel, whose capabilities are signalled in the FECC Capability).  These indication messages should be broadcast whenever the FECC Capability is sent, and whenever a new logical channel is opened.  The proposed C flag in the “Number of Presets Octet” shall not be set to 1 in this message.

| FIGURE 2 VIDEO CHANNEL CAPABILITY Message 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VIDEO CHANNEL CAPABILITY Indication |  |  |  |  |  |  |  | Code value  0  08 |
| ChannelID High Byte |  |  |  |  |  |  |  | Bits 9-16 of ChanneIID |
| ChannelID Low Byte |  |  |  |  |  |  |  | Bits 1-7 of ChannelID |
| Capability Octets (per Section 6.1/H.281) |  |  |  |  |  |  |  | Variable number of octets |

## CHANNELIZED Messages

These messages operate identically to their corresponding non-Channelized Messages in the existing H.281 standard.

| FIGURE 3 CHANNELIZED START ACTION Message 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C START ACTION Request |  |  |  |  |  |  |  | Code value  0  09 |
| ChannelID High Byte |  |  |  |  |  |  |  | Bits 9-16 of ChanneIID |
| ChannelID Low Byte |  |  |  |  |  |  |  | Bits 1-7 of ChannelID |
| P | R  1 L  0 | T | U  1 D  0 | Z | I  1 O  0 | F | I  1 O  0 |  |
| Reserved |  |  |  | Timeout |  |  |  |  |

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C CONTINUE ACTION Request |  |  |  |  |  |  |  | Code value  0  A |
| ChannelID High Byte |  |  |  |  |  |  |  | Bits 9-16 of ChanneIID |
| ChannelID Low Byte |  |  |  |  |  |  |  | Bits 1-7 of ChannelID |
| P | R  1 L  0 | T | U  1 D  0 | Z | I  1 O  0 | F | I  1 O  0 |  |

FIGURE  4 CHANNELIZED CONTINUE ACTION Message

| FIGURE 5 CHANNELIZED STOP ACTION Message 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C STOP ACTION Request |  |  |  |  |  |  |  | Code value  0  B |
| ChannelID High Byte |  |  |  |  |  |  |  | Bits 9-16 of ChanneIID |
| ChannelID Low Byte |  |  |  |  |  |  |  | Bits 1-7 of ChannelID |
| P | R  1 L  0 | T | U  1 D  0 | Z | I  1 O  0 | F | I  1 O  0 |  |

| FIGURE 6 CHANNELIZED SELECT VIDEO SOURCE Request 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SELECT VIDEO SOURCE Request |  |  |  |  |  |  |  | Code value  0  C |
| ChannelID High Byte |  |  |  |  |  |  |  | Bits 9-16 of ChanneIID |
| ChannelID Low Byte |  |  |  |  |  |  |  | Bits 1-7 of ChannelID |
| 4-bit encoded source Number |  |  |  | Reserved |  | M1 | M0 |  |

| FIGURE 7 CHANNELIZED VIDEO SOURCE SWITCHED message 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C VIDEO SOURCE SWITCHED indication |  |  |  |  |  |  |  | Code value  0  D |
| ChannelID High Byte |  |  |  |  |  |  |  | Bits 9-16 of ChanneIID |
| ChannelID Low Byte |  |  |  |  |  |  |  | Bits 1-7 of ChannelID |
| 4-bit encoded source Number |  |  |  | Reserved |  | M1 | M0 |  |

| FIGURE 8 CHANNELIZED STORE AS PRESET Message 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C STORE AS PRESET Request |  |  |  |  |  |  |  | Code value  0  E |
| ChannelID High Byte |  |  |  |  |  |  |  | Bits 9-16 of ChanneIID |
| ChannelID Low Byte |  |  |  |  |  |  |  | Bits 1-7 of ChannelID |
| 4-bit encoded preset Number |  |  |  | Reserved |  |  |  |  |

| FIGURE 9 CHANNELIZED ACTIVE PRESET Message 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C ACTIVATE PRESET Request |  |  |  |  |  |  |  | Code value  0  F |
| ChannelID High Byte |  |  |  |  |  |  |  | Bits 9-16 of ChanneIID |
| ChannelID Low Byte |  |  |  |  |  |  |  | Bits 1-7 of ChannelID |
| 4-bit encoded preset Number |  |  |  | Reserved |  |  |  |  |

# 3.	Conclusion

We propose that the additions above should be incorporated into a revised Recommendation.

__________________