| Rapporteur Meeting of Question 1/16 Geneva, 27 - 28 August 2008 | Q1-H12r1 |
| --- | --- |

| Question(s): | 1 |  | Meeting, date: | 27 - 28 August 2008 |
| --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2/16 |  |
| Source: | Rapporteur Q1/16 |  |  |  |
| Title: | Draft Liaison to IMTC 3G-324M AG on Proposed Annex to H.324 |  |  |  |
| LIAISON STATEMENT |  |  |  |  |
| To: | IMTC 3G-324M Activity Group |  |  |  |
| Approval: | Rapporteur meeting of Question 1/16 (Geneva, 27-28 August 2008) |  |  |  |
| For: | Comment |  |  |  |
| Deadline: | January 2009 |  |  |  |
| Contact: | Patrick Luthi Tandberg Norway |  |  | Tel: +47 67 125 125 Email: patrick.luthi@tandberg.com |
| Contact: | Brody Kenrick Dilithium Networks, Inc USA |  |  | Tel: +1 707 792 3908 Fax: +1 707 795 3758 Email: Brody.Kenrick@dilithiumnetworks.com |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |

ITU-T Question 1 of SG16 would like to request the comments of the experts in the 3G-324M Activity Group on a new proposed annex to H.324 that clarifies the mandatory aspects of H.245 signalling entities and messages in H.324. It is similar to H.323 Annex A.

This new annex was contributed in response to recent questions by the 3G-324M AG on FunctionNotSupported.

In particular, if you could please provide any inputs on any messages in the proposed annex that have what you believe is an incorrect status or any missing messages.

We look forward to your response ahead of our next meeting in January 2009.

Annex M

H.245 messages used by H.324 endpoints

The following rules apply to the use of H.245 messages by H.324 endpoints:

•	An endpoint shall not malfunction or otherwise be adversely affected by receiving H.245 messages that it does not recognize. An endpoint receiving an unrecognized request, response, or command shall return "function not supported". (This is not required for indications.)

•	The following abbreviations are used in Tables M.1 to M.12:

M	Mandatory.

O	Optional.

F	Forbidden to transmit.

•	A message marked as mandatory for the receiving endpoint indicates that the endpoint shall accept the message and take the appropriate action. A message marked as mandatory for the transmitting endpoint indicates that the endpoint shall generate the message under the appropriate circumstances.

An implemented optional request message at a terminal’s transmitting endpoint causes the corresponding response message at the terminal’s receiving endpoint to be mandatory.

Table M.1/H.324 – Master-slave determination messages

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Determination | M | M |
| Determination Acknowledge | M | M |
| Determination Reject | M | M |
| Determination Release | M | M |

Table M.2/H.324 – Terminal capability messages

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Capability Set | M | M |
| Capability Set Acknowledge | M | M |
| Capability Set Reject | M | M |
| Capability Set Release | M | M |

Table M.3/H.324 – Logical channel signalling messages

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Open Logical Channel | M | M |
| Open Logical Channel Acknowledge | M | M |
| Open Logical Channel Reject | M | M |
| Open Logical Channel Confirm | M | M |
| Close Logical Channel | M | M |
| Close Logical Channel Acknowledge | M | M |
| Request Channel Close | M | O |
| Request Channel Close Acknowledge | O | O |
| Request Channel Close Reject | O | M |
| Request Channel Close Release | O | M |

Table M.4/H.324 – Multiplex table signalling messages

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Multiplex Entry Send | M | M |
| Multiplex Entry Send Acknowledge | M | M |
| Multiplex Entry Send Reject | M | M |
| Multiplex Entry Send Release | M | M |

Table M.5/H.324 – Request multiplex table signalling messages

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Request Multiplex Entry | M | O |
| Request Multiplex Entry Acknowledge | O | O |
| Request Multiplex Entry Reject | O | M |
| Request Multiplex Entry Release | O | M |

Table M.6/H.324 – Request mode messages

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Request Mode | M | O |
| Request Mode Acknowledge | O | O |
| Request Mode Reject | O | M |
| Request Mode Release | O | M |

Table M.7/H.324 – Round trip delay messages

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Round Trip Delay Request | M | O |
| Round Trip Delay Response | O | M |

Table M.8/H.323 – Maintenance loop messages

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Maintenance Loop Request | M | O |
| Maintenance Loop Acknowledge | O | O |
| Maintenance Loop Reject | O | M |
| Maintenance Loop Command Off | O | O |

Table M.9/H.324 – Commands

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Send Terminal Capability Set | M | O |
| Encryption | O | O |
| Flow Control | M | O |
| End Session | M | M |
| Miscellaneous Commands |  |  |
| Equalize Delay | O | O |
| Zero Delay | O | O |
| Multipoint Mode Command | O | O |
| Cancel Multipoint Mode Command | O | O |
| Video Freeze Picture | O | O |
| Video Fast Update Picture | M | O |
| Video Fast Update GOB | M | O |
| Video Fast Update MB | M | O |
| Video Temporal Spatial Trade Off | O | O |
| Video Send Sync Every GOB | O | O |
| Video Send Sync Every GOB Cancel | O | O |
| Max H.223 MUXPDU Size | O | O |
| H.223 Multiplex Reconfiguration | O | O |
| Mobile Multilink Reconfiguration | O | O |

Table M.10/H.324 – Indications

| Message | Receiving endpoint status | Transmitting endpoint status |
| --- | --- | --- |
| Function Not Understood | M | M |
| Function Not Supported | M | M |
| Miscellaneous Indication |  |  |
| Logical Channel Active | M | O |
| Logical Channel Inactive | M | O |
| Video Indicate Ready to Activate | O | O |
| Video Temporal Spatial Trade Off | O | O |
| Video Not Decoded MBs | O | O |
| Vendor Indications | O | O |
| Jitter Indication | O | O |
| H.223 Skew Indication | M | O |
| H2250MaximumSkewIndication | F | F |
| New ATM Virtual Channel Indication | F | F |
| User Input | M (for 0-9, *and #) | M (for 0-9, *and #) |
| Flow Control Indication | O | O |
| Mobile Multilink Reconfiguration Indication | O | O |

Non-standard commands, requests, etc. are allowed.