| Rapporteur Meeting of Questions 1/16 |  |  | Q1-I11 |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | STUDY GROUP 16 |  |
|  |  | Original: English |  |
| Question(s): | 1 | Geneva, 24-25 June 2009 |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |
| Source: | Tandberg |  |  |
| Title: | Clarifications to H.239 |  |  |
| Purpose: | Discussion |  |  |

# 1.	Introduction

While reviewing the latest version of H.239, we noticed that the clauses about MCU procedures didn’t contain a definition of master and slave MCUs.

We have experienced that potential implementers sometimes incorrectly understand master and slave MCU designation to be related to the master/slave determination defined in H.245.

It would be useful to add some clarifying text and a reference to H.243 where MCU procedures and master/slave designation are defined in details.

# 2.	Proposed clarifications to H.239

## 2.1	Addition of a reference

The following reference to H.243 should be added to clause 2 (reference):

[Begin Correction]

…

–	ITU-T Recommendation H.243 (2005), Procedures for establishing communication between three or more audiovisual terminals using digital channels up to 1920 kbit/s.

[End Correction]

## 2.2	Addition of clarifying text in 11.3 and 11.4

The following clarifying text should be added to 11.3 and 11.4:

[Begin Correction]

## 11.3	Master MCU procedures

The designation of a master MCU in a cascaded MCU case is defined in clause 5.7/H.243. In a non-cascaded MCU case, the Master MCU procedures defined in this recommendation should be used.

At the start of a conference, the MCU should consider the token to be unowned.

When the MCU receives presentationTokenIndicateOwner from a device that is not the token owner, it should send presentationTokenRequest with a 0 symmetryBreaking value to it, and should then consider the token to be unowned.

[End Correction]

[Begin Correction]

## 11.4	Slave MCU procedures

The designation of a slave MCU in a cascaded MCU case is defined in clause 5.7/H.243.

Slave MCUs shall forward all token messages received from its end-user systems or slave MCUs to the master MCU.

presentationTokenRequest, presentationTokenResponse, and presentationTokenRelease messages received from the master MCU shall be routed to the end-user system based on the terminalLabel parameter value.

presentationTokenIndicateOwner messages from the Master MCU shall be forwarded to all other connected devices.

[End Correction]

# 3.	Conclusion

As proposed in Q1-I07, we recommend that these clarifications be added at the occasion of the next revision or when an amendment is created for H.239.

__________________