| Rapporteur Meeting of Questions 1/16 |  |  | Q1-J10 |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | STUDY GROUP 16 |  |
|  |  | Original: English |  |
| Question(s): | 1 | Research Triangle Park, 1-2 December 2010 |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |
| Source: | Editor |  |  |
| Title: | Draft H.222.0¦ISO/IEC 13818-1 Amendment 5 – Transport of JPEG 2000 Part 1 video |  |  |
| Purpose: | Discussion |  |  |

This draft Amendment 5 contains additional specifications for carrying JPEG 2000 Part 1 video over the transport stream of H.222.0|ISO/IEC 13818-1 (so called MPEG-2 systems). It specifically defines stream_id, stream type, J2K video descriptor and normative constraints for this purpose.

The attached draft is MPEG’s FPDAM (Final Proposed Draft Amendment) version currently under ballot until 3 December 2010. MPEG will finalize the text at its meeting in January 2011, which will be ready for SG16’s Consent in March 2011.

__________________

- INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

- CODING OF MOVING PICTURES AND AUDIO

- ISO/IEC JTC1/SC29/WG11/ N11364

July 2010, Geneva, Switzerland

| Title: | Text of ISO/IEC 13818-1:2007/FPDAM5 - Transport of JPEG 2000 part 1 video (ITU-T Rec. 800 ¦ ISO/IEC 15444-1) over ITU-T Rec H.222.0 ¦ ISO/IEC 13818-1 |
| --- | --- |
| Editors: | S. Narasimhan (Motorola) and Dale Stolitzka (Analog Devices) |
| Source: | Systems |

INFORMATION TECHNOLOGY -

GENERIC CODING OF MOVING PICTURES AND AUDIO: SYSTEMS

Amendment 5: Transport of JPEG 2000 part 1 video (ITU-T Rec. T.800 | ISO/IEC 15444-1) over ITU-T Rec H.222.0 | ISO/IEC 13818-1

ISO/IEC 13818-1:2007/FPDAM 5

International Standard

Summary

This amendment specifies the carriage of JPEG 2000 video bitstreams conforming to one or more profiles defined in ITU-T Rec T.800 | ISO/IEC 15444-1 over ITU-T Rec H.222.0 (05/2006) | ISO/IEC 13818-1:2007. The structure of JPEG 2000 access units as well as syntax and semantics are specified in the JPEG 2000 standards and are also included in Annex S of this specification. Each access unit is self-contained and decodable without reference to any other access unit in the video stream. The carriage of JPEG 2000 video will be limited to transport streams only in this amendment.

INTERNATIONAL STANDARD

ITU-T RECOMMENDATION

INFORMATION TECHNOLOGY -- GENERIC CODING OF MOVING PICTURES AND ASSOCIATED AUDIO INFORMATION: SYSTEMS

Amendment 5: Transport of JPEG 2000 Part 1 (ITU-T Rec T.800 | ISO/IEC 15444-1) video over ITU-T Rec H.222.0 | ISO/IEC 13818-1

# Subclause 1.2.3

In 1.2.3: Additional References, add:

- ITU-T Rec T.800 | ISO/IEC 15444-1, Information technology – JPEG 2000 Image coding system: Core coding system

- ITU-T Rec T.800 (08/2002) | ISO/IEC 15444-1:2004/AMD4,  Information technology – JPEG 2000 Image coding system: Core coding system

# Subclause 2.1

In 2.1: Definitions, add the following definitions:

2.1.89 JPEG 2000 (J2K) video access unit: An access unit defined in ITU-T Rec. T.800 (08/2002) | ISO/IEC 15444-1:2004 Amd.4 which includes all the parameters required to decode the access unit and display the decoded data.

2.1.90 J2K video elementary stream: Video elementary stream consisting of succession of J2K video access units.

2.1.91 J2K video sequence: J2K video elementary stream where all the access units have the same profile/level and video parameters.

2.1.90 J2K still picture (system): Same as a J2K video access unit.

# Subclause 2.4.2.9

Add new subclause 2.4.2.9:

The interpretation, extensions, use and constraints for syntax elements in the adaptation header (2.4.3.4 and 2.4.3.5) for JPEG 2000 part 1 video are defined in S.5.

The interpretation, extensions, use and constraints for syntax elements in the PES header (2.4.3.6 and 2.4.3.7) for JPEG 2000 Part 1 video are defined in S.5.

To define the decoding of J2K video elementary streams carried in a Transport Stream, the T-STD model may need extensions. The T-STD extensions and T-STD parameters for decoding of J2K video elementary streams conforming to one or more profiles defined in ITU-T Rec. T.800 | ISO/IEC 15444-1 are defined in clause S.6.

# Subclause 2.4.3.7

In 2.4.3.7 replace Table 2-18 by the following:

Table 2-18 – Stream_id assignments

| stream_id | Note | stream coding |
| --- | --- | --- |
| 1011 1100 | 1 | program_stream_map |
| 1011 1101 | 2, 9 | private_stream_1 |
| 1011 1110 |  | padding_stream |
| 1011 1111 | 3 | private_stream_2 |
| 110x xxxx |  | ISO/IEC 13818-3 or ISO/IEC 11172-3 or ISO/IEC 13818-7 or ISO/IEC 14496-3 audio stream number x xxxx |
| 1110 xxxx |  | ITU-T Rec. H.262 ¦ ISO/IEC 13818-2 or ISO/IEC 11172-2 or ISO/IEC 14496-2 or ITU-T Rec.H.264 ¦ ISO/IEC 14496-10 video stream number xxxx |
| 1111 0000 | 3 | ECM_stream |
| 1111 0001 | 3 | EMM_stream |
| 1111 0010 | 5 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Annex A or ISO/IEC 13818-6_DSMCC_stream |
| 1111 0011 | 2 | ISO/IEC_13522_stream |
| 1111 0100 | 6 | ITU-T Rec. H.222.1 type A |
| 1111 0101 | 6 | ITU-T Rec. H.222.1 type B |
| 1111 0110 | 6 | ITU-T Rec. H.222.1 type C |
| 1111 0111 | 6 | ITU-T Rec. H.222.1 type D |
| 1111 1000 | 6 | ITU-T Rec. H.222.1 type E |
| 1111 1001 | 7 | ancillary_stream |
| 1111 1010 |  | ISO/IEC14496-1_SL-packetized_stream |
| 1111 1011 |  | ISO/IEC14496-1_FlexMux_stream |
| 1111.1100 |  | metadata stream |
| 1111.1101 | 8 | extended_stream_id |
| 1111 1110 |  | reserved data stream |
| 1111 1111 | 4 | program_stream_directory |
| The notation x means that the values '0' or '1' are both permitted and results in the same stream type. The stream number is given by the values taken by the x’s. NOTE 1 – PES packets of type program_stream_map have unique syntax specified in 2.5.4.1. NOTE 2 – PES packets of type private_stream_1 and ISO/IEC_13552_stream follow the same PES packet syntax as those for ITU-T Rec. H.262 ¦ ISO/IEC 13818-2 video and ISO/IEC 13818-3 audio streams. NOTE 3 – PES packets of type private_stream_2, ECM_stream and EMM_stream are similar to private_stream_1 except no syntax is specified after PES_packet_length field. NOTE 4 – PES packets of type program_stream_directory have a unique syntax specified in 2.5.5. NOTE 5 – PES packets of type DSM-CC_stream have a unique syntax specified in ISO/IEC 13818- 6. NOTE 6 – This stream_id is associated with stream_type 0x09 in Table 2-29. NOTE 7 – This stream_id is only used in PES packets, which carry data from a Program Stream or an ISO/IEC 11172-1 System Stream, in a Transport Stream (refer to 2.4.3.8). NOTE 8 – The use of stream_id 0xFD (extended_stream_id) identifies that this PES packet employs an extended syntax to permit additional stream types to be identified. NOTE 9 – JPEG 2000 video streams (stream_type = 0x21) are carried using the same PES packet syntax as private_stream_1. |  |  |

# Subclause 2.4.4.9

In 2.4.4.9, Semantic definition of fields in Transport Stream program map section, replace Table 2-34 by the following:

Table 2-34 – Stream type assignments

| Value | Description |
| --- | --- |
| 0x00 | ITU-T ¦ ISO/IEC Reserved |
| 0x01 | ISO/IEC 11172-2 Video |
| 0x02 | ITU-T Rec. H.262 ¦ ISO/IEC 13818-2 Video or ISO/IEC 11172-2 constrained parameter video stream |
| 0x03 | ISO/IEC 11172-3 Audio |
| 0x04 | ISO/IEC 13818-3 Audio |
| 0x05 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 private_sections |
| 0x06 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 PES packets containing private data |
| 0x07 | ISO/IEC 13522 MHEG |
| 0x08 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Annex A DSM-CC |
| 0x09 | ITU-T Rec. H.222.1 |
| 0x0A | ISO/IEC 13818-6 type A |
| 0x0B | ISO/IEC 13818-6 type B |
| 0x0C | ISO/IEC 13818-6 type C |
| 0x0D | ISO/IEC 13818-6 type D |
| 0x0E | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 auxiliary |
| 0x0F | ISO/IEC 13818-7 Audio with ADTS transport syntax |
| 0x10 | ISO/IEC 14496-2 Visual |
| 0x11 | ISO/IEC 14496-3 Audio with the LATM transport syntax as defined in ISO/IEC 14496-3 |
| 0x12 | ISO/IEC 14496-1 SL-packetized stream or FlexMux stream carried in PES packets |
| 0x13 | ISO/IEC 14496-1 SL-packetized stream or FlexMux stream carried in ISO/IEC 14496_sections |
| 0x14 | ISO/IEC 13818-6 Synchronized Download Protocol |
| 0x15 | Metadata carried in PES packets |
| 0x16 | Metadata carried in metadata_sections |
| 0x17 | Metadata carried in ISO/IEC 13818-6 Data Carousel |
| 0x18 | Metadata carried in ISO/IEC 13818-6 Object Carousel |
| 0x19 | Metadata carried in ISO/IEC 13818-6 Synchronized Download Protocol |
| 0x1A | IPMP stream (defined in ISO/IEC 13818-11, MPEG-2 IPMP) |
| 0x1B | AVC video stream conforming to one or more profiles defined in Annex A of ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 or AVC video sub-bitstream as defined in 2.1.78 or MVC base view sub-bitstream as defined in 2.1.85 or AVC video sub-bitstream of MVC as defined in 2.1.88 |
| 0x1C | ISO/IEC 14496-3 Audio, without using any additional transport syntax, such as DST, ALS and SLS |
| 0x1D | ISO/IEC 14496-17 Text |
| 0x1E | Auxiliary video stream as defined in ISO/IEC 23002-3 |
| 0x1F | SVC video sub-bitstream of an AVC video stream conforming to one or more profiles defined in Annex G of ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 |
| 0x20 | MVC video sub-bitstream of an AVC video stream conforming to one or more profiles defined in Annex H of ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 |
| 0x21 | Video stream conforming to one or more profiles as defined in ITU-T Rec T.800 ¦ ISO/IEC 15444-1 |
| 0x22-0x7E | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Reserved |
| 0x7F | IPMP stream |
| 0x80-0xFF | User Private |

# Subclause 2.6.1

In 2.6.1, Semantic definition of fields in program and program element descriptors, replace Table 2-45 by:

Table 2-45 - Program and program element descriptors

| descriptor_tag | TS | PS | Identification |
| --- | --- | --- | --- |
| 0 | n/a | n/a | Reserved |
| 1 | n/a | X | Forbidden |
| 2 | X | X | video_stream_descriptor |
| 3 | X | X | audio_stream_descriptor |
| 4 | X | X | hierarchy_descriptor |
| 5 | X | X | registration_descriptor |
| 6 | X | X | data_stream_alignment_descriptor |
| 7 | X | X | target_background_grid_descriptor |
| 8 | X | X | video_window_descriptor |
| 9 | X | X | CA_descriptor |
| 10 | X | X | ISO_639_language_descriptor |
| 11 | X | X | system_clock_descriptor |
| 12 | X | X | multiplex_buffer_utilization_descriptor |
| 13 | X | X | copyright_descriptor |
| 14 | X |  | maximum_bitrate_descriptor |
| 15 | X | X | private_data_indicator_descriptor |
| 16 | X | X | smoothing_buffer_descriptor |
| 17 | X |  | STD_descriptor |
| 18 | X | X | IBP_descriptor |
| 19-26 | X |  | Defined in ISO/IEC 13818-6 |
| 27 | X | X | MPEG-4_video_descriptor |
| 28 | X | X | MPEG-4_audio_descriptor |
| 29 | X | X | IOD_descriptor |
| 30 | X |  | SL_descriptor |
| 31 | X | X | FMC_descriptor |
| 32 | X | X | external_ES_ID_descriptor |
| 33 | X | X | MuxCode_descriptor |
| 34 | X | X | FmxBufferSize_descriptor |
| 35 | X |  | multiplexBuffer_descriptor |
| 36 | X | X | content_labeling_descriptor |
| 37 | X | X | metadata_pointer_descriptor |
| 38 | X | X | metadata_descriptor |
| 39 | X | X | metadata_STD_descriptor |
| 40 | X | X | AVC video descriptor |
| 41 | X | X | IPMP_descriptor (defined in ISO/IEC 13818-11, MPEG-2 IPMP) |
| 42 | X | X | AVC timing and HRD descriptor |
| 43 | X | X | MPEG-2_AAC_audio_descriptor |
| 44 | X | X | FlexMuxTiming_descriptor |
| 45 | X | X | MPEG-4_text_descriptor |
| 46 | X | X | MPEG-4_audio_extension_descriptor |
| 47 | X | X | Auxiliary_video_stream_descriptor |
| 48 | X | X | SVC extension descriptor |
| 49 | X | X | MVC extension descriptor |
| 50 | X | n/a | J2K video descriptor |
| 51-63 | n/a | n/a | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Reserved |
| 64-255 | n/a | n/a | User Private |

# Subclauses 2.6.80 and 2.6.81

After 2.6.79, Semantics of fields in MVC extension descriptor, add new subclauses 2.6.80 and 2.6.81:

2.6.80	J2K video descriptor

For J2K video elementary streams conforming to one or more profiles defined in ISO/IEC 15444-1 the J2K video descriptor provides information that may be present in each J2K access unit as well as for the J2K video sequence. In addition, it provides information to signal J2K still pictures. This descriptor shall be included for each J2K video elementary stream component in the PMT with stream_type equal to 0x21.

Table AMD5-1 – J2K video descriptor

| Syntax | No. Of bits | Mnemonic |
| --- | --- | --- |
| J2K_video_descriptor() { descriptor_tag descriptor_length profile_and_level horizontal_size vertical_size max_bit_rate max_buffer_size DEN_frame_rate NUM_frame_rate color_specification still_mode interlaced_video reserved for (i=0; i<N; i++) { private_data_byte } } | 8 8 16 32 32 32 32 16 16 8 1 1 6 8 | uimsbf uimsbf uimsbf uimsbf uimsbf uimsbf uimsbf uimsbf uimsbf bslbf bslbf bslbf bslbf bslbf |

2.6.81	Semantics of fields in J2K video descriptor

profile_and_level – This field shall be in the range 0x0101-0x04ff and coded as defined in Table A-10 of ISO/IEC 15444-1 and indicates broadcast profile and level values.

horizontal_size – This field shall be coded the same as Xsiz parameter found in the J2K codestream main header, as defined in Annex A of ISO/IEC 15444-1: 2004/Amd.4.

vertical_size -- This field shall be coded the same as Ysiz parameter found in the J2K codestream main header, as defined in Annex A of ISO/IEC 15444-1: 2004/Amd.4.

max_bit_rate - This field may be coded the same as in the j2k_brat box specified in section I.5.3.8.2 of ITU-T Rec. T.800 (08/2002) | ISO/IEC 15444-1:2004 Amd.4 and shall not exceed the Maximum Compressed Bit Rate value for the profile and level specified in Table S-1. This field shall be set appropriately and signaled when profile_and_level = 0x0307, where no maximum bit rate is specified.

max_buffer_size – This field shall not exceed the Maximum buffer size value for the profile and level specified in the j2k_brat box in Table S-1.  When profile_and_level = 0x0307, the max_buffer_size shall be set appropriately and shall not exceed  (max_bit_rate / 1.60E5), where max_bit_rate is expressed in bits/second.

DEN_frame_rate - This field shall be coded the same as frat_denominator field in the j2k_frat box specified in Table S-1 (see Annex S).

NUM_frame_rate - This field shall be coded the same as frat_numerator field in the frat box specified in Table S-1 (see Annex S)..

Note: J2K frame rate is derived from the DEN_frame_rate and NUM_frame_rate values and Table AMD5-2

lists examples of typical broadcast frame rates with associated values of DEN_frame_rate and NUM_frame_rate.

Table AMD5-2 – Example frame rates based on DEN_frame_rate and NUM_frame_rate values

| DEN_frame_rate | NUM_frame_rate | Frame rate ratio (decimal representation) | Frame rate |
| --- | --- | --- | --- |
| 0000 0000 0000 0000 |  |  | forbidden |
| 0000 0011 1110 1001 | 0101 1101 1100 0000 | 24 000 / 1001 | 23,976 |
| 0000 0000 0000 0001 | 0000 0000 0001 1000 | 24 / 1 | 24,0 |
| 0000 0000 0000 0001 | 0000 0000 0001 1001 | 25 / 1 | 25,0 |
| 0000 0011 1110 1001 | 0111 0101 0011 0000 | 30 000 / 1001 | 29,97 |
| 0000 0000 0000 0001 | 0000 0000 0001 1110 | 30 / 1 | 30,0 |
| 0000 0000 0000 0001 | 0000 0000 0011 0010 | 50 / 1 | 50,0 |
| 0000 0011 1110 1001 | 1110 1010 0110 0000 | 60 000 / 1001 | 59,94 |
| 0000 0000 0000 0001 | 0000 0000 0011 1100 | 60 / 1 | 60,00 |

color_specification - This field shall be coded the same as the bcol_colrc 8-bit field of the j2k_bcol box as specified in Table S-1 (see Annex S).

still_mode – This 1-bit field when set to '1' indicates that the J2K video stream may include J2K still pictures. When set to '0', then the associated J2K video stream shall not contain J2K still pictures.

interlaced_video – This 1-bit field indicates whether the J2K video stream contains interlaced video. When this flag is set to '1' the J2K access unit elementary stream header (see Table S-1) shall include the syntax elements Auf2, fiel_box_code, fic and fio. When this flag is set to ‘0’, these syntax elements shall not be present in the J2K access unit elementary stream header.

# Annex S

After Annex R add the following:

Annex S

Carriage of JPEG 2000 part 1 video over MPEG-2 Transport Streams

(This annex forms an integral part of this Recommendation | International Standard)

### S.1 	Introduction

This annex specifies normative constraints for the carriage of JPEG 2000 video in an MPEG-2 transport stream. The parameters specified include mapping of J2K video streams into MPEG-2 transport packets, signaling of J2K streams as well as T-STD parameters for various profiles. Transport of J2K video shall be limited to transport stream only. Program stream support may be added in the future based on application requirements.

### S.2 	J2K video access unit, J2K video elementary stream, J2K video sequence and J2K still picture.

The J2K video access unit contains the elementary stream (elsm) header created in a manner following Annex M of ITU-T Rec. T.800 (08/2002) | ISO/IEC 15444-1:2004 Amd. 4 concatenated with self-contained ITU-T Rec. T.800 | ISO/IEC 15444-1 codestreams .  The (elsm) header contains all video-related parameters necessary to display the decoded codestream(s).  Multiple codestreams may comprise an access unit when, for instance, the access unit is an interlaced frame. The J2K video elementary stream is a progression of J2K access units and the J2K video sequence is a subset of J2K video elementary stream where all the J2K access units have same parameters in the (elsm) header.

The J2K still picture is an MPEG-2 system defined function that supports transmission of J2K video access units at a rate much lower than the frame rate where decoders ‘repeat’ decoded pictures at the display frame rate. This is used in applications such as ‘slide show’ and ‘stills with Music’.

### S.3 	Elementary Stream header (elsm) and mapping to PES packets.

The following table shows each portion of the elsm header as defined in Annex M of ISO/IEC 15444-1 Amd.4 and the concatenation of detailed metadata boxes within the elsm header.

Table S - . J2K Access Unit elementary stream header

| Syntax | No. Of bits | Mnemonic |
| --- | --- | --- |
| j2k_es() { j2k_elsm ‘0x656c 736d’ // j2k_frat frat_box_code ’0x6672 6174’ frat_denominator frat_numerator // j2k_brat brat_box_code = ‘0x6272 6174’ Maxbr Auf1 // If (interlaced_video == 1) { Auf2 } // j2k_fiel // If (interlaced_video == 1) { fiel_box_code = 0x6669 656c fic fio } // j2k_tcod tocd_code = 0x7463 6f64 HH (0-23) MM (0-59) SS (0-59) FF (1-60) // j2k_bcol bcol_code = 0x6263 686c bcol_colcr reserved } | 32 32 16 16 32 32 32 32 32 8 8 32 8 8 8 8 32 8 8 |  |

j2k_frat – a field box defined in Annex M of ITU-T REC. T.800 (08/2002) | ISO/IEC 15444-1 Amd.4 corresponding to the required frat box containing frame rate coding.

j2k_brat – a field box defined in Annex M of ITU-T REC. T.800 (08/2002) | ISO/IEC 15444-1 Amd.4 corresponding to the required brat box containing the maximum instantaneous bit rate of the elementary stream.

j2k_fiel – a field box defined in Annex M of ITU-T REC. T.800 (08/2002) | ISO/IEC 15444-1 Amd.4 corresponding to the optional fiel box containing interleaved field coding. If the j2k_fiel is present, there shall be two contiguous codestreams present.

j2k_tcod – a field box defined in Annex M of ITU-T REC. T.800 (08/2002) | ISO/IEC 15444-1 Amd.4 corresponding to the required tcod box containing time code and frame count information of the j2k access unit.

j2k_bcol – a field box defined in Annex M of ITU-T REC. T.800 (08/2002) | ISO/IEC 15444-1 Amd.4 corresponding to the required bcol box containing broadcast color specification coding.

Figure S-1 shows the structure and mapping of J2K video access unit into PES packets. JPEG 2000 represents each frame as one or two Part 1 (ISO/IEC 15444-1:2004) contiguous codestreams. The codestream main header,included within each contiguous condestream contains all information to decode its image, including the image size and the profile indicator, called a SIZ marker in ITU-T Rec. T.800 | ISO/IEC 15444-1.  In addition to each codestream’s main header, ITU-T Rec. T.800 (08/2002) | ISO/IEC 15444-1:2004/Amd.4 Annex M adds an elementary stream (elsm) header containing video-related information as shown in Table S-1 and the j2k_video_descriptor in section 2.6.80.

Figure S- – Structure and order of JPEG 2000 Access Units

### S.4	J2K transport constraints.

When a J2K video elementary stream conforming to one or more profiles as defined in ITU-T Rec. T.800 | ISO/IEC 15444-1 is transported using MPEG-2 systems the following constraints apply:

- Each J2K access unit shall contain an elementary stream header (elsm) defined by ITU-T Rec. T.800 (08/2002) | ISO/IEC 15444-1:2004 Amd. 4 followed by one or two codestream(s).

- Each J2K elementary stream header shall contain a SIZ marker containing a J2K profile marker equating to the profile_and_level parameter of this standard and conforming to Section 2.6.81.

- The J2K access units shall be ordered in the J2K video elementary stream in a monotonic display order.

- Each J2K access unit shall include a PES header with PTS and each PES packet shall contain exactly one J2K access unit.

- Increment to PTS for successive J2K access units shall be consistent with increments to corresponding J2K_tcod parameters in the elsm header.

- The following constraints apply to the coding of syntax elements in the adaptation header for transport of J2K video elementary stream:

  - Both random_access_indicator and elementary_stream_priority_indicator flags can be set to ‘1’ for each J2K video access unit contained in the transport packet. Applications may limit the signaling of random access based on their use cases. Any J2K video access unit is also a J2K video ‘access point’ required for random access.

  - All other flags should be set appropriately.

- The following constraints apply to the coding of syntax elements in the PES header for transport of J2K video elementary stream:

  - stream_id shall be set to 0xBD (same as Private_stream_1).

  - PES_packet_length shall be set to 0x0000.

  - data_alignment_indicator shall be set to ‘1’. Also see clause S.5 for alignment constraints.

  - PTS_DTS_flags shall be set to ‘10’. This constrains no display re-ordering for J2K video access units.

  - All other flags should be set appropriately.

### S.5 	Interpretations of flags in adaptation and PES headers for J2K video elementary streams.

The interpretation, extensions, use and constraints for the following syntax elements in the adaptation header (2.4.3.4 and 2.4.3.5) and PES header (2.4.3.6 and 2.4.3.7) for JPEG 2000 part 1 video are defined in this clause:

- In the semantics for discontinuity_indicator (see 2.3.4.5), a J2K video elementary stream access point is defined as the first byte of J2K video access unit.

- The elementary_stream_priority_indicator in adaptation header (see 2.4.3.4 and 2.4.3.5) may be set to ‘1’ if the transport packet payload contains the start of a J2K video access unit.

- For J2K video elementary streams when data_alignment_indicator (see 2.4.3.6) is set to ‘1’ the PES packet header shall be immediately followed by the first byte of J2K video access unit. The data_stream_alignment_descriptor is optional and, if included for J2K video, the alignment_type shall be set to 0x00.

For J2K video elementary streams conforming to one or more profiles defined in ITU-T Rec. T.800 |  ISO/IEC 15444-1, if a PTS is present in the PES packet header (see 2.4.3.7), it shall refer to the first byte of J2K video access unit that commences in this PES packet.

### S.6	T-STD extension for J2K video elementary streams.

The T-STD model includes a transport buffer TBn and J2K video elementary stream buffer EBn for decoding of each J2K video elementary stream n. See Figure S-2.

Figure S-2 – T-STD model extensions for J2K Video

The following notation is used to describe the Transport Stream system target decoder and is partially illustrated in Figure 2-1 above.

i		index to bytes in the Transport Stream. The first byte has index 0.

j		is an index to access units in the elementary streams.

k		index to presentation units in the elementary streams.

n		is an index to the elementary streams.

p		is an index to Transport Stream packets in the Transport Stream.

t(i)		indicates the time in seconds at which the i-th byte of the Transport Stream enters the system target decoder. The value t(0) is an arbitrary constant.

PCR(i)	is the time encoded in the PCR field measured in units of the period of the 27 MHz system clock where i is the byte index of the final byte of the program_clock_reference_base field.

An(j)	is the j-th access unit in elementary stream n. An(j) is indexed in decoding order.

tdn(j)	is the decoding time, measured in seconds, in the system target decoder of the j-th access unit in elementary stream n.

Pn(k)	is the k-th presentation unit in elementary stream n. Pn(k) results from decoding An(j). Pn(k) is indexed in presentation order.

tpn(k)	is the presentation time, measured in seconds, in the system target decoder of the k-th presentation unit in elementary stream n.

t			is time measured in seconds.

Fn(t)		is the fullness, measured in bytes, of the system target decoder input buffer for elementary stream n at time t.

Bn			is the main buffer for elementary stream n. It is present only for audio elementary streams.

BSn		is the size of buffer, Bn, measured in bytes.

EBn		is the elementary stream buffer for elementary stream n. It is present only for video elementary streams.

EBSn	is the size of the elementary stream buffer EBn, measured in bytes.

TBn		is the transport buffer for elementary stream n.

TBSn	is the size of TBn, measured in bytes.

Rxn		is the rate at which data are removed from TBn.

TBn and EBn buffer management

The input to buffer TBn and its size TBSn are specified in 2.4.2.3. The rate Rxn between TBn and buffer EBn and the following constraints apply for carriage of J2K video elementary stream:

Size EBSn of buffer EBn – See Table S-1 for the maximum buffer size based on profile and level of J2K video  stream.

Rate Rxn:

when there is no data in TBn then Rxn is equal to zero.

Otherwise:	Rxn = bit_rate

where bit_rate is the bit rate is specified for the profile level in Table S-1. All J2K video payload data bytes enter EBn instantaneously upon leaving TBn.

Removal of J2K access units from EBn

Each J2K video access unit An(j) that is present in EBn is removed instantaneously at time tdn(j). The decoding time tdn(j) is specified by the PTS (as PTS=DTS for J2K video elementary streams) in the PES header for the J2K video access unit.

STD delay

The total delay of any J2K video elementary stream data other than J2K still picture data through the System Target Decoders buffers TBn and EBn shall be constrained by tdn(j) – t(i)  1 second for all j, and all bytes i in J2K video access unit An(j).

The delay of any J2K still picture data through the System Target Decoders buffers TBn and EBn shall be constrained by tdn(j)  t(i)  60 seconds for all j, and all bytes i in J2K video access unit An(j).

Buffer management conditions

Transport streams shall be constructed so that the following conditions for buffer management are satisfied:

	TBn shall not overflow and shall be empty at least once every second.

	EBn shall not overflow or underflow.

Note:  EBn shall not underflow as J2K video elementary streams do not support low delay mode.

### S.6.1	J2K Video elementary stream buffer size

The ITU-T Rec. T.800 (08/2002) Amd. 4 | ISO/IEC 15444-1:2004 Amd. 4 elementary stream supplies parameters suitable for determining a standard decoder model buffer size.  This CODEC uses variable rate compression. Profiles from ITU-T Rec. T.800 | ISO/IEC 15444-1:2004/Amd.4 specifies operating levels that limit the maximum compressed bit rate of the ITU-T Rec. T.800 | ISO/IEC 15444-1 code stream. Moreover within access unit an elementary stream header contains the present maximum bit rate an encoder must support to decode video without overflow. A decoder may scale the buffer size by reading the maximum bit rate expected by a particular sequence of frames from the elsm header. This header information shall not exceed the limit set by the operating level specified in ITU-T Rec. T.800 (08/2002) | ISO/IEC 15444-1:2004/Amd.1.

Table S-1. Operating levels and Maximum Buffer size for JPEG 2000 Broadcast profiles

(From Table A-49 in   ITU-T Rec.T.800 (08/2002) | ISO/IEC 15444-1:2004/Amd.4)

| Levels (Note 1) | Max. compressed Bit Rate (Mbits/sec) | Maximum buffer size for a 60.0Hz frame rate, progressive video (MBytes) |
| --- | --- | --- |
| Level 1 | 200 | 1.25 |
| Level 2 | 200 | 1.25 |
| Level 3 | 200 | 1.25 |
| Level 4 | 400 | 2.5 |
| Level 5 | 800 | 5 |
| Level 6 | 1600 | 10 |
| Level 7 | Unspecified | Unspecified |

Note 1. Levels are specified in the JPEG 2000 codestream main header SIZ marker segment. Refer to Table A-10 of ITU-T Rec.T.800 (08/2002) | ISO/IEC 15444-1:2004/Amd.4.