ITU-T SG15

Working Party 15/1	7/30/2026

Expert's Group For Very Low

Bitrate Visual Telephony

SOURCE:	R. SCHAPHORST, RAPPORTEUR FOR VERY LOW BITRATE VISUAL TELEPHONY (QUESTION 2/15)

TITLE:	DRAFT REPORT OF THE THIRTEENTH LBC MEETING IN DALLAS, TEXAS

APRIL 23-26, 1996

µ1	OVERVIEW	1

2	OPENING SESSION	1

2.1	Twelfth Meeting Report	1

2.2	Ad Hoc Committee Reports	1

2.3	ISO MPEG4	2

2.4		2

2.5		2

3	VIDEO CODER (H.263)	2

3.1	H.263 Revision	2

3.2	H.263 Conformance	2

4	H.263 ENHANCEMENTS (H.263+)	3

4.1	Introduction	3

4.2	Scope, Goals and Schedule	3

4.3	Guidelines and Test Conditions for H.263+ Work	3

4.4	Joint Collaboration	4

4.5	Evaluation of Technical Contributions	4

4.6	Key Technical Areas of Contributions	5

4.7	Conclusions	5

5	ADVANCED VIDEO CODING (H.263L)	5

5.1	H.263L Schedule STG	5

5.2	Proposal & Core Experiment Evaluation Procedure STG	5

5.3	Request for Proposals STG 	6

5.4	Requirements Document STG	6

5.5	H.263+/H.263L Working Relationship	6

5.6	Joint Project (H.263L/MPEG4) Proposal	6

5.7	H.263L Proposals	6

5.8	H.263L Working Model	6

5.9	H.263L Core Experiments	6

5.10	Relevant E-Mail Reflectors and FTP Sites	7

5.11	H.263L Mission (Plan) for Activities to Next LBC Meeting	7

6	SPEECH CODER (G.723.1)	7

6.1	Version 4.1 of the Fixed-Point C Source Code	7

6.2	Voice Activity Detection and Comfort Noise Generation	8

6.3	Interoperable Floating-Point Specification of G.723.1	8

6.4	Mobile Channel Coding for G.723.1	9

7	SYSTEM (H.324)/MULTIPLEX (H.223)	9

7.1	Documents	9

7.2	Mission statement for period between Dallas and London meetings	10

8	COMMUNICATION CONTROL (H.245)	11

9	MOBILE (H.324M)	13

9.1	Multiplex Layer	13

9.2	Adaptation Layers	13

9.3	Error Tracking	14

9.4	Schedule and Target Documents	14

10	H.324 TESTING	14

11	NON-CONVERSATIONAL SERVICES	15

12	CLOSING SESSION	16

12.1	Review/Approval of Work by Sub-Groups	16

12.2	LBC Response to SG15 Regarding Proposed New Questions for 1996-2000 Study Period	16

12.3	Liaison Statements	16

12.4	Ad Hoc Committees	16

12.5	Contributions for the Next Meeting	23

12.6	Future Meeting Schedule	23

1	OVERVIEWtc "_autonumlgl§	OVERVIEW"\l§

The thirteenth meeting of the ITU-T Experts Group for Very Low Bit Rate Visual Telephony was held in Dallas, Texas, April 23-26, 1996.  Excellent arrangements were provided by Texas Instruments, the host organization.  A list of the 114 collaborating experts attending the meeting is included in Annex A, and a list of the 97 technical contributions which were presented at the meeting is provided in Annex B.  An updated list of the experts who are collaborating in this work is included in Annex C.  The meeting was organized in the eleven separate segments listed below, and the report summarizes the work accomplished in each segment.

| SUBJECT | CHAIRPERSON |
| --- | --- |
| 2.0 Opening Session | R. Schaphorst |
| 3.0 Video Coder (H.263) | K. Rijkse |
| 4.0 H.263 Enhancements (H.263+) | G. Sullivan |
| 5.0 Advanced Video Coding (H.263L) | R. Ivy |
| 6.0 Speech Coder (G.723.1) | I. Varga |
| 7.0 System (H.324) / Multiplex (H.223) | D. Lindbergh |
| 8.0 Communication Control (H.245) | W. Welsh |
| 9.0 Mobile (H.324M) | J. Villasenor |
| 10.0 H.324 Testing | C. Gates |
| 11.0 Non-Conversational Services | W. Welsh |
| 12.0 Closing Session | R. Schaphorst |

2	OPENING SESSIONtc "_autonumlgl§	OPENING SESSION"\l§

3	Twelfth Meeting Reporttc "_autonumlgl§	Twelfth Meeting Report"\l 2§

The Report of the Twelfth Meeting of the LBC Experts Group (LBC-96-060) was reviewed.

4	Ad Hoc Committee Reportstc "_autonumlgl§	Ad Hoc Committee Reports"\l 2§

Reports were presented by the Chairpersons of ten Ad Hoc Committees which were appointed at the previous meeting.  The reports which were presented and the corresponding technical contribution are listed below.

| AD HOC COMMITTEE | CHAIRPERSON | DOCUMENT NO. |
| --- | --- | --- |
| Speech (G.723.1) | I. Varga | 005 |
| H.324 Test | C. Gates | 006 |
| Advanced Video (H.263L) | R. Ivy | 009 |
| DTE/DCE Interface | T. Geary | 066 |
| V.8/V.8 bis | I. Sebestyn | 067 |
| Control (H.245) | W. Welsh | 068 |
| Video (H.263) | K. Rijkse | 073 |
| System (H.324) | D. Lindbergh | 072 |
| G.723.1 Mobile Channel Coding | J. Mueller | 070 |
| H.263 Conformance | T. Gardos | 071 |

5	ISO MPEG4tc "_autonumlgl§	ISO MPEG4"\l 2§

Documents 077, 065, and 103 related to the joint activity with the ISO MPEG4 project were reviewed.

6	tc "_autonumlgl§	"\l 2§

A proposal (document 124) for a new structure of Questions for the next ITU Study period was reviewed.

7	tc "_autonumlgl§	"\l 2§

All technical contributions (TD-2) were briefly presented by their author.  The meeting plan outlined in TD-3 was reviewed, modified, and approved.

8	VIDEO CODER (H.263)tc "_autonumlgl§	VIDEO CODER (H.263)"\l§

9	H.263 Revisiontc "_autonumlgl§	H.263 Revision"\l 2§

Some ambiguities in the H.263 text were found and reported in a White Document for the ITU-T Study Group 15 meeting in May.  Recently another defect in the H.263 text was found in Section F.3.  In this section, the description of the calculation of the prediction is only valid for full pixel motion vectors, the possibility of half pixel motion vectors is not taken into account.  A revised proposal for improvement was developed.

The proposed improvements/corrections do not influence the intended operations and are in line with the freely available H.263 software of Telenor.  Only the part of Section F.3 that contains suggested changes are included; unchanged text has been removed.

10	H.263 Conformancetc "_autonumlgl§	H.263 Conformance"\l 2§

No meetings were held, however comments on the H.263 Conformance document were solicitated.  No comments were received which will be taken as tacit approval.  The current version of the document will be made available at the ftp site maintained by PictureTel.  The first version of the document will be called “H263Con1.doc”.

Volunteers to create test bit streams were solicited as well, although no one came forward.  If and when test bit streams become available, they will be distributed by some means.

11	H.263 ENHANCEMENTS (H.263+)tc "_autonumlgl§	H.263 ENHANCEMENTS (H.263+)"\l§

12	Introductiontc "_autonumlgl§	Introduction"\l 2§

The H.263+ development effort is intended for near-term standardization of enhancements of the H.263 video coding algorithm for real-time telecommunication.  The H.263+ group held meetings throughout the April 22-25 LBC Experts Group meeting, and is pleased to report that it made significant progress toward its goal.  The group's activities during this week can be summarized as follows:

a)	We agreed on the scope and goals of our work effort

b)	We defined our workplan schedule

c)	We reviewed 3 contributions providing guidance for the conduct of our efforts

d)	We adopted guidelines and test conditions for use in experiments and proposals

e)	We adopted a simulation test model definition to use as the basis for experiments

f)	We collaborated jointly with the H.263L and Mobile Video groups

g)	We reviewed 12 technical proposals for algorithm and syntax features

h)	We identified 9 key technical areas for core experiment definition

i)	We appointed coordinators for  rapid (within two weeks) definition of core experiments in each identified key technical area

j)	We completed the definition of core experiments in 3 of the 9 technical areas

The following sections describe these activities in greater detail.

13	Scope, Goals and Scheduletc "_autonumlgl§	Scope, Goals and Schedule"\l 2§

The H.263+ group agreed on the scope, goals, and workplan schedule for its activities, and provided a written description of these intentions in document LBC-96-148.

14	Guidelines and Test Conditions for H.263+ Worktc "_autonumlgl§	Guidelines and Test Conditions for H.263+ Work"\l 2§

The group reviewed 3 contributions providing guidance for the conduct of our efforts:

Document		Company		Subject Summary

LBC-96-104		PictureTel		Proposed Goals and Schedule

LBC-96-111		Iterated Systems	Guidelines for Core Experiments

LBC-96-112		Iterated Systems	Display Formats and Video Delay Display

The following guidelines and reference test model were adopted to guide the work of those wishing to make H.263+ contributions:

a)	Email communications for our group will be conducted using the advanced video email reflector provided by Iterated Systems:

sg15_lbc_adv_video@listserv.iterated.com

b)	The reference test model, currently TMN6 as described in LBC-96-141 should be used as a basis for core experiment comparisons

c)	The exchange of software among the members of the group is encouraged for easing the burden on those wishing to make contributions

d)	However, core experimenters should not exchange software to implement the tested features of the experiment (in order to aid work on developing written descriptions of the features by ensuring that descriptions provide sufficient information for unambiguous interpretation)

e)	At least two companies are expected to perform any core experiment prior to the adoption of a tested feature into H.263+

f)	Proposers of core experiments are expected to provide draft text for writing the H.263+ recommendation description of the tested feature

g)	Decoding of bitstreams is expected from core experimenters prior to reporting results (in order to ensure proper testing of the associated syntax features)

h)	Fixed frame rate, fixed QP tests are useful for evaluating compression efficiency performance

i)	The possibility of interactions between optional features of the H.263+ syntax will be considered and evaluated

j)	Additional test condition recommendations (including recommended test sequence data, frame rates, and display conditions) are adopted per the report of the video test conditions special task group LBC-96-138, and further refined as a result of discussions in the H.263L meeting.  These STG conclusions appear in LBC-96-139.

15	Joint Collaborationtc "_autonumlgl§	Joint Collaboration"\l 2§

The H.263+ video group held joint discussions with the H.263L video group in order to coordinate the efforts of the two groups.  In particular, the guidelines and test conditions for the two groups were developed through joint efforts.  The viewing of video demonstrations was conducted jointly, and several technical proposals were presented with evaluation of their

applicability toward the goals and schedules of the two projects.

The H.263+ video group also held joint sessions with the Mobile video group for evaluation of the guidelines and test conditions appropriate for the two groups, for presentation of technical proposals of joint interest, and for viewing of demonstration videos having Mobile video ramifications.

16	Evaluation of Technical Contributionstc "_autonumlgl§	Evaluation of Technical Contributions"\l 2§

The H.263+ video group held presentations of 12 technical contributions:

Document		Company		Subject Summary

LBC-96-084		PictureTel		Modulated Lapped Transform

LBC-96-085		PictureTel		Reduced Delay Syntax

LBC-96-087		Oki			Error Resilient Coding

LBC-96-091		Nokia			Variable Segmentation Video Coding

LBC-96-096		Deutsche Tel.		Loop Filter Evaluation

LBC-96-097		Telenor		Intra Picture Predictive Coding

LBC-96-098		Telenor		Variable Size (8x8 and 4x4) DCT

LBC-96-099		Telenor		Reducing Video Delay

LBC-96-116		Natl. Semic. Corp.	GOB Source Format

LBC-96-117		Natl. Semic. Corp.	Adaptive Arithmetic Coding

LBC-96-118		Natl. Semic. Corp.	Video Frame Size Quadrupler

LBC-96-132		Intel			Error Resilient Syntax

17	Key Technical Areas of Contributionstc "_autonumlgl§	Key Technical Areas of Contributions"\l 2§

The H.263+ video group identified 9 key technical areas in which proposal contributions had been provided.  The group agreed that these areas were worthy of further investigation, and appointed a coordinator to provide a core experiment description for each.  The coordinators volunteered to provide these C.E. descriptions to the group (via email or current meeting contribution) within two weeks, and several of them have in fact already fulfilled this  commitment as noted below:

Technical Feature Area		Related Doc. #s	Coordinator		C.E. Doc. #

Deblocking Filter			96, 98			G. Bjontegaard	LBC-96-146

Intra Picture Predictive Coding	97			G. Bjontegaard	LBC-96-147

GOB-Layer Syntax			85, 99, 116, 132	X. Ran			LBC-96-145

Adaptive Arithmetic Coding		117			X. Ran			via email

Frame Size Quadrupler		118			X. Ran			via email

Variable Segmentation Coding	91, 98			J. Muller		via email

Motion Handling Basis		91			J. Nieweglowsk	via email

Modulated Lapped Transform	84			R. Grinnell		via email

Back-Channel Error Resilience	87			T. Nakai		via email

18	Conclusionstc "_autonumlgl§	Conclusions"\l 2§

The H.263+ video group is very pleased with its progress at this meeting. We wish to thank those who contributed to make this a very productive meeting, and to urge those who can contribute to our efforts to bring forth their ideas in our upcoming continuing activities.  We believe we are on track toward delivering an enhanced video coding recommendation within our scheduled timeframe.

19	ADVANCED VIDEO CODING (H.263L)tc "_autonumlgl§	ADVANCED VIDEO CODING (H.263L)"\l§

The Advanced Video Coding ad hoc held evening special task group (STG) sessions on Tuesday and Wednesday and met in full session Wednesday afternoon, Thursday afternoon, and Friday morning.  The following provide information on the various points of work and discussion.

20	H.263L Schedule STGtc "_autonumlgl§	H.263L Schedule STG"\l 2§

This group, chaired by Alfred Riccomi, met Tuesday evening to review and modify a proposed schedule for H.263L (LBC-96-102).  A schedule was generated (LBC-102R1) and subsequently approved by the ad hoc.  Due to changes in the expected LBC meeting schedule, the meeting schedule and working draft schedule was somewhat modified (LBC-96-102R2) but the overall plan and schedule remained unchanged.

21	Proposal & Core Experiment Evaluation Procedure STGtc "_autonumlgl§	Proposal & Core Experiment Evaluation Procedure STG"\l 2§

This group, chaired by Mike Zeug, met Tuesday evening to review and recommend “common conditions” for proposals and core experiments.  Input documents included LBC-96-101, 111, and 112.  The task group results (LBC-96-138) were presented to the ad hoc.  Subsequent discussion resulted in a set of common conditions for proposals (LBC-96-139) which were approved by the ad hoc.  These conditions were also accepted by H.263+.

22	Request for Proposals STG tc "_autonumlgl§	Request for Proposals STG "\l 2§

This group, chaired by Richard Ivy, met Wednesday evening to review and modify a proposed request for H.263L proposals (LBC-96-101).  An RFP was generated and subsequently approved by the ad hoc (LBC-96-137).

23	Requirements Document STGtc "_autonumlgl§	Requirements Document STG"\l 2§

This group, chaired by Kevin O’Connell, met Wednesday evening to review and finalize the Real-Time Audio/Video Conversational Services Applications Requirements Document (LBC-95-262R2).  The resulting document (LBC-96-136) was subsequently approved by the ad hoc.

24	H.263+/H.263L Working Relationshiptc "_autonumlgl§	H.263+/H.263L Working Relationship"\l 2§

Except for the H.263L need for a different working model, the H.263L ad hoc agreed to the set of 10 guidelines recommended by the H.263+ ad hoc (LBC-96-149).  Due to the large degree of commonality between H.263+ and H.263L, it was agreed that a close working relationship was required and that significant exchange of information was expected.

25	Joint Project (H.263L/MPEG4) Proposaltc "_autonumlgl§	Joint Project (H.263L/MPEG4) Proposal"\l 2§

The joint project and meeting proposal (LBC-96-103) from MPEG was discussed.  Kevin O’Connell presented his view of the MPEG perspective.  Richard Schaphorst proposed a response which was approved (LBC-96-133).

26	H.263L Proposalstc "_autonumlgl§	H.263L Proposals"\l 2§

Four proposals were presented to the ad hoc.

LBC-96-091 - Nokia - VLB Video Codec

LBC-96-115 - U. of Strathclyde - Strathclyde Compression Transform

LBC-96-143 - TI - Robust Object Based Video Coding

A review of a previous proposal from Iterated Systems.

LBC-96-082 - Ericsson - Error Resilient Fixed Length Code Tables

All proposals were well received and each offered interesting benefits to be obtained.  Due in part to a large degree of commonality among the proposals, proposers were encouraged to collaborate as appropriate on the development of experiments to be performed.

27	H.263L Working Modeltc "_autonumlgl§	H.263L Working Model"\l 2§

John Muller coordinated the development of the recommended settings for the MPEG4 VM to be used for H.263L core experiments (LBC-96-154) which were approved by the ad hoc.  A more formal report is contained in LBC-96-155.  R.Ivy will communicate this information to MPEG4, indicate that the H.263+ group will accept CE results for experiments using the MPEG4 CE, and suggest that MPEG4 should accept H.263+ CE results as well.

28	H.263L Core Experimentstc "_autonumlgl§	H.263L Core Experiments"\l 2§

Several core experiment proposals are expected to be developed via e-mail reflector during the two weeks following the Dallas meeting.  These experiments will be performed and the results presented during the MPEG4 meeting in July.  CE’s are expected from TI (Robust Coding), Nokia (advanced motion), and Ericsson (resilient code tables) and others may be developed.  It was noted that several MPEG4 CE’s are of particular interest to the ad hoc including variable block size (ISI), MLT (PictureTel), matching pursuits and 5 error resilience (Mot), and motion segmentation (TI).

29	Relevant E-Mail Reflectors and FTP Sitestc "_autonumlgl§	Relevant E-Mail Reflectors and FTP Sites"\l 2§

LBC Advanced Video Coding AHG

sg15_lbc_adv_video@listserv.iterated.com

sg15_lbc_adv_video-request@listserv.iterated.com

LBC Video

sg15.lbc’video@research.kpn.com

MPEG-4 Requirements AHG

mpeg4_require@listserv.iterated.com

mpeg4_require-request@listserv.iterated.com

MPEG-4 Test Methodologies

mpeg4test@drogo.cselt.stet.it

H.263L Archive FTP Site

ftp://ftp.std.com/vendors/PictureTel/h263L/xxx.doc

30	H.263L Mission (Plan) for Activities to Next LBC Meetingtc "_autonumlgl§	H.263L Mission (Plan) for Activities to Next LBC Meeting"\l 2§

H.263L will attempt to formalize the MPEG4/H.263L joint meeting arrangement.

H.263L will meet with MPEG4 in July (Tempere, Finland)

H.263L will attempt to develop some core experiments within the next two weeks, perform the experiments (2 or more companies preferred), and present results during the July MPEG meeting.

H.263L will solicit proposals to be presented at the July LBC meeting.

31	SPEECH CODER (G.723.1)tc "_autonumlgl§	SPEECH CODER (G.723.1)"\l§

32	Version 4.1 of the Fixed-Point C Source Codetc "_autonumlgl§	Version 4.1 of the Fixed-Point C Source Code"\l 2§

Version 4.1 complies with the main body of the Recommendation G.723.1. France Telecom/CNET presented LBC-96-089 which gives a detailed description of the taming procedure used in version 4.1. The taming procedure solves the instability problem for some high-frequency sine waves in the case of desynchronization between the encoder and decoder due to frame erasures or interoperation between fixed-point and floating-point. Tests on this approach using a real-time hardware implementation always showed a recovery of unstabilities. No difference was found between the version using the taming procedure and the version without the taming procedure for a very large speech data base. The group decided to accept the approach in LBC-96-089. The group also decided that including a documentation on the taming procedure in the software disc belonging to G.723.1 is sufficient and there is no need to request a revision of the Recommendation G.723.1.

Version 4.1 is stable and public. It can be obtained from ITU Sales Organization. The distribution package consists of the C source code and of the test vectors.  Documentation on the taming procedure and on the use of the test vectors is included.

The group decided to close any discussion and work on the main body of Recommendation G.723.1 and on the corresponding version 4.1 of the fixed-point C code.

33	Voice Activity Detection and Comfort Noise Generationtc "_autonumlgl§	Voice Activity Detection and Comfort Noise Generation"\l 2§

LBC-96-127 was discussed by the group. This document describes objective test results on the efficiency of the silence compression scheme described in Draft Annex A. The results show that the efficiency of the silence compression approach is good. More specifically, there is no significant difference between the DTX efficiency of the G.723.1 Annex A and the GSM half rate coder.

A formal subjective CCR (comparison category rating) test was performed by France Telecom/CNET on a preliminary version 5.0 according to the test plan which was agreed by the group at the last meeting. The test results in LBC-96-121 were presented and discussed by the group. Based on the test results, the group accepted the silence compression scheme described in Annex A. Based on informal listening tests, it was noticed that a fine tuning of the parameters of the VAD is still needed for low SNR conditions. This work will be performed by France Telecom/CNET and AudioCodes.

The current version of Draft Annex A (LBC-96-119) was reviewed and some minor editorial changes were introduced. The group agreed that France Telecom/CNET and AudioCodes will prepare the final document during the next two weeks. It will be sent to ITU as a delayed contribution for the SG15 meeting together with the current version 5.0 of the fixed-point C source code. France Telecom/CNET and AudioCodes will prepare the final version 5.0 of the C source code. It will be submitted to ITU in time in order to make its distribution possible by ITU Sales Organization after approval.

The group discussed LBC-96-090 which addresses the problem of transmitting and interpreting messages from the multiplex layer to the speech decoder in DTX mode including the effects of transmission errors (control of speech decoder BFI flag). The group proposed to discuss this document in the multiplex and system group.

34	Interoperable Floating-Point Specification of G.723.1tc "_autonumlgl§	Interoperable Floating-Point Specification of G.723.1"\l 2§

LBC-96-126 was presented by Intel. This document describes the results of the second formal subjective test performed on the version 4.1f of the interoperable floating-point C code. This test was performed by NTT-AT and was founded by Intel. Note that the same software was used in the first and the second formal subjective test performed by France Telecom/CNET in December 1995 and by NTT-AT in March 1996, respectively. Like the first test, the second test also showed satisfactory results and indicated a similar behaviour of the floating-point software. More specifically, LBC-96-126 points out that, for some very few test conditions and for a confidence level of 5\% only, there is a statistically significant difference between the floating-point reference version and the fixed-point version.

Based on the results of the first and second subjective test performed by France Telecom/CNET in December 1995 and by NTT-AT in March 1996, respectively, the group decided to accept the floating-point reference version contributed by Intel to be the interoperable floating-point reference C code belonging to Annex B to G.723.1.

Intel presented LBC-96-125 which describes a test methodology for floating-point speech coder implementations. The group accepted this approach which uses bit exact testing on short (10 frames) input sequences for the encoder and SNR criteria for decoder evaluation.

A set of test vectors and test tools have been designed and exercised on several hardware platforms. Some further work is needed e.g. on a DSP hardware implementation to weed out any remaining fragile vectors in the encoder test and to tune the success threshold. France Telecom/CNET will contribute to this work in order to ensure that the work can be completed in time. The group invites interested organizations to help in this work.

Version 4.1f will be distributed by Intel for evaluation purposes to experts who have signed an agreement. They are asked to contact Mike Keith at mike\_keith@ccm.jf.intel.com. A set of test vectors, test tools and a documentation on their use will also be included in the distribution package. Version 5.0f will be created as soon as version 5.0 of the fixed-point C code becomes available. Version 5.0f will be the final version corresponding to Annex B.

The group decided that the text of Draft Annex B is frozen. The floating-point C code will be submitted to ITU within the next two weeks to support the approval procedure. The final C code version 5.0f will be submitted to ITU in time in order to make its distribution possible by ITU Sales Organization after approval.

35	Mobile Channel Coding for G.723.1tc "_autonumlgl§	Mobile Channel Coding for G.723.1"\l 2§

LBC-96-094 was discussed by the group. It gives a high-level description of the mobile channel coding scheme of Draft Annex C to G.723.1 and outlines the structure of the C code and the test vectors.

The group discussed the results of the formal subjective tests (LBC-96-093 and LBC-96-095). The test was conducted by Deutsche Telekom and was founded by Bosch Telecom. The test evaluated the subjective quality of the G.723.1 coder using the mobile scalable channel coding approach described in Draft Annex C over the GSM channel in comparison with the GSM full rate coder. An ACR testing was performed according to P.80. The test results showed that it is possible to achieve the same subjective quality using the G.723.1 coder protected by the channel coding method of Annex C over the GSM network with the same relative redundancy than using the GSM full rate coder. More specifically, it has been shown that there is no statistically significant difference between the GSM full rate coder and the G.723.1 high-rate coder above 13.2 kbit/s rate for EP1 and EP2 error conditions. Furthermore, there is no significant difference between the GSM full rate coder and the G.723.1 low-rate coder above 11.2 kbit/s rate for EP1 error condition and above 12.1 kbit/s rate for EP2 error condition. Note that the GSM full rate coder transmits at 22.8 kbit/s.

The group discussed LBC-96-083 by NTT DoCoMo. This test was performed on the mobile coding approach of Annex C in Japanese over the PDC network. The results indicate that the Annex C provides widely scalable bit error resilience with Japanese speech and the PDC network. The group also discussed the revision of Draft Annex C (LBC-96-092) and agreed to the very minor changes.

Based on the test results and of the overview of the channel coding method, the group accepted the mobile channel coding approach described in Annex C.

After having closed the discussion on Draft Annex C, some experts of the mobile group requested to restrict the channel codec configuration codeword from 3 bits to 1 bit. This requires a change of the BCH generator polynomial on page 10 of Annex C (LBC-96-092). Since the speech coding group has already closed the discussion of that document, the closing plenary is asked to agree to this change.

The final version of Annex C and the C source code will be submitted to ITU as a delayed contribution for the SG15 meeting. The C code is available to experts for evaluation purposes after signing an agreement with Bosch. They are asked to contact Joerg-Martin Mueller at jmm@bk.bosch.de. The distribution package will consist of the C code, of the test vectors and a documentation on their use. The C code becomes public after approval by ITU.

36	SYSTEM (H.324)/MULTIPLEX (H.223)tc "_autonumlgl§	SYSTEM (H.324)/MULTIPLEX (H.223)"\l§

37	Documentstc "_autonumlgl§	Documents"\l 2§

The following documents were addressed and the indicated actions were taken.

066 - DTE/DCE Report (T. Geary, DTE/DCE Chair)

Reviewed and accepted.  Appendix for H.324 on V.25ter and V.80 (V.IB) will wait on SG14’s resolution of V.8bis issue.

067 - V.8/V.8bis Report (I. Sebesteyn, V.8/V8bis Chair)

Reviewed and accepted.

072 - H.324 System Report (D. Lindbergh, H.324 Chair)

Reviewed and accepted.  Chair asked for multilink contributions again.  We agreed to send a liaison to SG14 on V.34 loopback issue.

078 - Proposed H.324 Changes (D. Lindbergh, H.324 Chair)

1. Agreed. Editorial.

2. Agreed. Error.

3. Not needed, this is obvious.

4. Given to V.8/V.8bis ad-hoc group.

Resolution: Only change is to H.324 sec. 7.3.1, should say “H.324 terminals should signal the H.324 V.8 call function, and shall not signal a V.8 protocol category.”  All else same.  No new Annex.  SG15 must draft liaison to SG14 requesting codepoint.  (Informally request codepoint ASAP.)

5. Agreed. Editorial.

6. Agreed, except that sentences are reversed, and 2nd sentence prefixed with “for example” (see agreed changes below). Editorial.

7. Agreed “complete”, not “whole”.  Editorial.

8. Agreed.  Error.

Must fix second A.2.3 to read A.2.4.

9. Not agreed yet, need more time for further work.  See notes, this is work in progress.

We may work on PICS instead.  Non-normative.

10.  (Clarify in 6.6 and 6.7 that uni-directional audio/video is permitted.) Agreed.

We also agreed to add some clarifying text to H.245 on how to signal caps for uni-directional video.

079 - H.324/H.320 Interworking (PictureTel)

We agreed this should be a work item toward a new “H.DISPATCH” Recommendation.

090 - G.723.1 Mux Procedure (FT)

We concluded this is a local implementation issue, no change to the standard is needed.

128 - Fax in H.324 (Siemens)

This is first draft of SG8 document for T.30 fax over H.324.  For information.

124 - Proposed WP1/15 Questions (PictureTel)

We Reviwed how this relates to the new SGMM proposal, status of that.  Summary of status is that nothing is decided yet but four Study Groups have interest in AVMMS:

SG15 - WP1, WP2 conversational MM (=> new SGMM proposal)

SG 8 - (T.120, T.80...).

SG 13 - “GII”.

SG 14 - MM Platforms (modems, DSVD...)

Schedule for decisions:

SG15 - May/June

TSAG - July

WTSC - October

Issue remains unclear, but 124 addresses only organization of expert’s work in WP1/15.  The content of the contribution re WP1/15 was supported by the experts, and the decision was to include a general suggestion for this kind of Question structure in the Rapporteur’s report to SG15.

38	Mission statement for period between Dallas and London meetingstc "_autonumlgl§	Mission statement for period between Dallas and London meetings"\l 2§

1.	Prepare H.324 Appendix on use of V.25ter Annex A and V.80 (V.IB) with H.324, after SG14 resolution of V.8bis issue.

2.	Solicit contributions on H.324 multilink extensions.

3.	Request H.324 codepoint in V.8 from SG14.

4.	Continue work on non-normative H.324 Appendix listing mandatory/optional/forbidden messages in H.245 (item 9 from LBC-96-078), and/or PICS statement for H.324 if contributions are received.

5.	Start work toward a new “H.DISPATCH” recommendation (as proposed in LBC-96-079).

a)	Continue work toward “H.INTERWORKING” (H.24i).

39	COMMUNICATION CONTROL (H.245)tc "_autonumlgl§	COMMUNICATION CONTROL (H.245)"\l§

There were 11 documents for consideration by the ad-hoc group along with some problems in H.245 which had been raised but which were not documented.

086 - 	This is the defect report for H.245 submitted to SG15.

122 - 	from PictureTel raised some further potential defects in H.245.

114 - 	from Lucent points out some unclear text in H.245 relating to bi-directional signalling.

129 - 	submitted by Stuart Dunstan was able to clarify most of the points raised in 114.

123 -	from Intel expressed concerns with backward compatibility in H.245 if the proposed defect report is implemented. The argument was that these changes will not be incorporated into the document until October at the earliest and until that time implementors would be in great confusion about what the definitive draft of H.245 actually is. It was a concern of those companies planning to release products by October.

Because of these concerns, we initially put on hold those items of the defect report which required syntax changes to be made and dealt only with the items which required editorial changes for clarification. We then discussed the concerns regarding changes in the H.245 document. The two most serious problems with H.245 were concerned with the MasterSlaveDetermination Signalling Entity and the FunctionNotSupported Indication. In Figure 4(v) of H.245 concerned with master-slave signalling, two > signs were mistakenly inserted instead of less than signs. This would mean that the terminal with the smaller statusDeterminationNumber would become the master which is the opposite of what is written in H.324. The functionNotSupported indication will not work in its present form.

The available options were reduced to two:

1. Not to change anything in the current draft and regard it as the first definitive version of H.245 even with the errors it contains. Put all the changes into a Version 2.

2. Identify all the things which are broken in the current draft and create a new version of the document with all the known problems corrected and get this onto the ftp site as quickly as possible together with an implementors guide explaining all the changes that have been made. This version of H.245 would then become the definitive version 1. Also go forward with revision 2 with all the new material using extension markers in order to be backward compatible with version 1.0.

The main problem with the first option was recognised as there being a requirement for all future terminals that use H.245 to support the V1.0 terminals containing the errors. This seemed a poor legacy to leave to the future and the consensus of the group was to go forward with the second option. Consequently, it is our intention to produce an updated version of the defect report for submission as a TD to SG15 along with the corrected Version 1 of H.245 for determination at the May/June meeting of SG15. These documents will also be put onto the ftp site together with an implementors guide referencing them.

120 - This is a liaison from SG14 on revisions to H.245 syntax for V.70. It appears that the modified syntax contained in this was in an annex to the V.75 document decided in SG14 in March. We discovered some errors in this syntax and have corrected these errors before including the syntax in the modified defect report.

Editing work was carried out to produce a modified version of the defect report. All the issues agreed earlier were accounted for in the editing process. It was decided to include a corrected version of the syntax for the V.70 additions as well as corresponding semantics. The edited document was reviewed by the entire group and some further modifications were made. The final modifications and additions to the original defect report 086 are summarised below:

Change of title to indicate that only part of the document refers to technical defects.

Each item has been categorised as an editorial or technical change to the original document or an addition.

In total the breakdown of technical defects, editorial defects and technical additions is as follows:

8 technical defects

21 editorial defects or clarifications

2 technical additions

Item 1 (editorial) - same as 086

Item 2 ( technical) - same as 086

Item 3 (technical) - same as 086

Item 4 (technical) - same as 086

Item 5 (technical) - same as 086

Item 6 (editorial) - same as 086 except ‘decoder’ replaced by ‘receiver’ in two places

Item 7 (technical) - same as 086

Item 8 (editorial) - same as 086

Item 9 (technical) - same as 086

Item 10 (technical) - same as 086

Item 11 (editorial) - same as 086

Item 12 (editorial) - same as 086 except typo ‘contructed’ changed to ‘constructed’.

Item 13 (technical) - same as 086

Item 14 ( editorial) - same as 086

Item 15 (editorial) - same as 086

Item 16 (technical) - same as 086

Item 17 (editorial) - same as 086

Item 18 (editorial addition) - Protocol Identifier changed in Annex A to match change in Item 1.

Item 19 (editorial addition) - Clarification of bi-directional logical channel signalling (Section 8.5.1, Page 95)

Deletion of old text, addition of new text for clarification, correction of typo.

Item 20 (editorial addition) - Error in B-LCSE message names and fields table (Table 32, Page 100)

Direction of message is wrong way round.

Item 21 (editorial addition) - Clarification of Close Logical Channel Procedures

Modification of old text, addition of new text for clarification.

Item 22 (editorial addition) - Addition of definitions for ‘Forward’, ‘Reverse’, ‘Out-going’ and ‘In-coming’ (Section 3, page 3)

Item 23 (editorial addition) - Additonal Timing Diagrams for bi-directional procedures (Appendix II, Section II.9)

These are timing diagrams supplied by Stuart Dunstan in document 129. We included all of these except for the last one which should never occur.

Item 24 (editorial addition) - Missing OpenLogicalChannelConfirm message (Figure 17, Section 8.5.4.1, Page 101)

Item 25 (technical addition) - Non Standard Parameter missing extension markers (Section 6, Page 9)

Item 26 (editorial addition) - Clarification of use of NonStandardIdentifier (Section 7, Page 35)

New text added.

Item 27 (editorial addition) - Add NOTE clarifying use of bi-directional channels (Section 7.3.1, Page 45)

Item 28 (editorial addition)  - Clarification of H223SkewIndication.

Item 29 (technical addition) - Correction of defects in H.245 for use with V.70 DSVD (Sections 6, 7)

The syntax given by SG14 was corrected and modified to fit better within the framework of H.245 without changing the intended functionality.

Work for revision 2 of H.245.

1. Addition of codepoints for MPI allowing very low frame rates for surveillance and similar applications.

2. A new appendix giving suggested procedures for new extensions of H.245.

40	MOBILE (H.324M)tc "_autonumlgl§	MOBILE (H.324M)"\l§

The Mobile Ad Hoc Group reviewed the contributions described in the following documents: 80, 82, 87, 92-95, 105-110, 113, 116, and 130.  Most of these documents concerned techniques or simulation results to explore specific approaches to FEC, ARQ, or design of the mobile multiplexer.  Several contributions described modifications to the H.263 syntax to increase robustness.  Joint meetings were held with the Video, Systems, and Audio groups. The Mobile group has converged on a basic frame work consisting of a multiplex layer using fixed-length packets, adaptation layers supporting FEC and ARQ, and error tracking.

41	Multiplex Layertc "_autonumlgl§	Multiplex Layer"\l 2§

The multiplexer utilizes fixed length packets similar to those described in the first draft of the mobile multiplexer description (LBC-95-276).  Each packet contains a synchronization word and a header, and one or more AL-PDUs. The header is interleaved throughout the packet. At the San Jose meeting it was noted that while fixed-length packets are desirable to provide robustness to poor channels, to support interworking it is necessary to retain the multiplex table format described in H.223and H.245. A solution permitting the H.223 table descriptions to be used in combination with the fixed length packets was proposed in documents108 and 109, and was further refined in discussions within the Mobile group and during a joint meeting with the Systems group.

42	Adaptation Layerstc "_autonumlgl§	Adaptation Layers"\l 2§

Two new adaptation layers were developed based on documents 107 and 110. The first adaptation layer, AL3M, supports video, and provides robustness using a combination of FEC and ARQ. The FEC is obtained using rate-compatible punctured codes (RCPC). Two forms or ARQ are supported. ARQ-I utilizes retransmission of an entire packet; ARQ-II relies on the transmission of additional parity bits if the first transmission contains errors.

Adaptation layer AL1M supports the transmission of data and control messages.  Since these messages are variable length, a bit stuffing procedure is described to allow their transmission using fixed-length multiplex slots.

43	Error Trackingtc "_autonumlgl§	Error Tracking"\l 2§

Error tracking provides a means for the decoder to send back to the encoder information regarding the location of corrupted macroblocks.  This capability can be provided by the addition of several code points to H.245, as described in document 96-80.

44	Schedule and Target Documentstc "_autonumlgl§	Schedule and Target Documents"\l 2§

Opportunities to submit documents for determination will occur at the SG15 meetings in late May 1996 and April 1997. The Mobile group decided that it was preferable to work on a very rapid schedule in order to meet the May meeting as opposed to incurring a delay of almost one year. While there is no intent to change the basic framework of the proposed mobile standard after May 1996, studies of the entire system will need to be performed in the intervening time between May 1996 andApril 1997 to explore parameter choice and to further specify some details of the FEC and ARQ.  Prior to the May 1996 SG15 meeting, the Mobile group will work to perform the following tasks:

1) Edit an annex to H.223 describing the mobile multiplexer and adaptation layers.  Pedro Crespo and Peter Vogel are leading the effort to provide text for the multiplex layer; Ralf Fischer and Phillipe Perrot are drafting text for the adaptation layer.

2) Edit an annex to H.324 describing the modifications needed to support mobile communications

3) Edit an informative appendix (to be provided by Niko Faerber) to H.263 describing error tracking.

4) Provide new H.245 codepoints to the H.245 editor for inclusion in the revised H.245 description. Niko Faerber is leading the work to define codepoints.

During a joint meeting with the Systems group on April 24, it was agreed that several members of the Systems group (Corey Gates of Creative Labs and Gary Corcoran of Lucent) would provide feedback on drafts of the documents listed above. The Mobile group feels that this interaction is critical to the generation of a sufficiently mature specification prior to the May SG15 meeting.

45	H.324 TESTINGtc "_autonumlgl§	H.324 TESTING"\l§

The System Test Ad Hoc Committee met on Tuesday, April 23 in the evening to discuss interoperability tests and future plans.  A few members of the committee gave their opinions on the test event which was held on Monday, April 22.  All of these opinions were favorable.  Document LBC-96-063 contains the report of the test event.

The group discussed document LBC-96-076 from Intel, which contains a test plan for H.245.  This test plan was used during the test event and was felt to contain the test procedures needed to fill in Test 2/D of document LBC-96-041R1.  It was also pointed out that these procedures are being used by a number of implementers to test their H.245 implementations with one another remotely.

A proposal came from the chair to hold another Interoperability Event before the next LBC meeting in July.  No details are available at this time, but it will probably be held at the end of May or the beginning of June.  Location is also to be determined.  Information about this event will be made available as soon as possible.

NOTE: Although it was not discussed at the meeting, it is the intention of the interoperability tests to follow a common version of H.245.  When syntax changes to H.245 are agreed to be made, the implementers should use the changed syntax for the interoperability tests.

46	NON-CONVERSATIONAL SERVICEStc "_autonumlgl§	NON-CONVERSATIONAL SERVICES"\l§

A special evening session of the Non-conversational Services ad-hoc committee was held during the Dallas meeting. Two contributions from KPN were presented by Karel Rijkse.

131 - discussed some aspects of using the H.245 UserInputIndication message for signalling to an audiovisual server.

134 - addressed a concern that if a terminal signals to the remote terminal that it has no receive video or audio capability the remote terminal should not disconnect as the other terminal might be an audiovisual server.

Both contributions are concerned with accessing an audiovisual server using a basic H.324 terminal, that is one which does not have user data capability. In this case the menus have to be displayed using video and the only means the user has to interact with the server is via the UserInputIndication message on the control channel. The author wished to ensure that the use of this message remains as is in H.324, that is, H.324 itself does not specify what a terminal should do with the message, it is left up to a higher layer. Also he wanted to ensure that manufacturers should not implement H.324 terminals so that they disconnected if the remote terminal turned out not to have receive video and audio capability.

After some discussion, it was agreed that no changes to H.324 were necessary to enable the above application to work as proposed. It is important that implementors should implement all the mandatory features of H.324 such as UserInputIndication.

Another concept proposed by KPN was that if a terminal indicated a capability to receive two independent video channels, one channel could carry the normal video and the other video containing the menu and implementors could ensure that both channels would be displayed simultaneously on the screen. This would avoid the user losing sight of the menu if the video was playing and then not knowing what to do next. It would help legibility of the menus if the video used to display these was of the higher resolution. The frame rate could of course be quite low.

In general, there was more interest in this work than at previous meetings. On the one hand, there is a feeling that if audiovisual services are made accessible to H.324 terminals in the near term, this will provide a stimulus for consumers to take them up. On the other hand, it is clear that similar services accessible via the World Wide Web are undergoing a tremendous growth which does not show any signs of abating. It is likely that the majority of accesses to these services will made be via the PSTN and ISDN in the near and medium term and thus H.324 may be able to play a significant role here.

The Chairman encouraged companies who are making efforts in this area to bring their proposals into this arena as soon as is practically possible as it would be in the interests of everyone, both consumers and the companies themselves, to arrive at a unified way to use H.324 for these services.

47	CLOSING SESSIONtc "_autonumlgl§	CLOSING SESSION"\l§

48	Review/Approval of Work by Sub-Groupstc "_autonumlgl§	Review/Approval of Work by Sub-Groups"\l 2§

The Experts reviewed the work of each individual sub-group and agreed with text provided in Sections 3 through 11 respectively.

49	LBC Response to SG15 Regarding Proposed New Questions for 1996-2000 Study Periodtc "_autonumlgl§	LBC Response to SG15 Regarding Proposed New Questions for 1996-2000 Study Period"\l 2§

The LBC group reviewed document 124 on this topic and generally agreed with the proposal.

50	Liaison Statementstc "_autonumlgl§	Liaison Statements"\l 2§

The following liaison documents were reviewed by the LBC Experts Group and approved for distribution.

LBC-96-133		Liaison to MPEG

LBC-96-157		Liaison to SG14 on Loopback

51	Ad Hoc Committeestc "_autonumlgl§	Ad Hoc Committees"\l 2§

Eleven Ad Hoc Committees were established to continue the work between the Dallas and the London meetings. The  mandates and membership in the committees are provided below. Any expert who desires to participate in a committee should contact the chairman directly.  Conversely, if your name is on a list and you do not plan to actively participate, you are invited to advise the committee chairman to withdraw your membership. It is anticipated that work will be primarily performed by means of E-mail.  However, fax and audio conferences may be used at the discretion of the chairman.  Ad Hoc chairmen will submit a report at the London meeting summarizing the work which was accomplished.  In defining our work method, the ITU states that "work should be accomplished through correspondence as much as possible."  The Ad Hoc committees are established to facilitate this work process.

| AD HOC COMMITTEE | CHAIRPERSON |
| --- | --- |
| System/H.324 | D. Lindbergh |
| H.263L | R. Ivy |
| Interworking | D. Lindbergh |
| Speech Coding/G.723.1 | I. Varga |
| System Test | C. Gates |
| Communication/Control/H.245 | W. Welsh |
| Mobile | D. Lappe |
| Non-Conversational Services | W. Welsh |
| V.8/V.8bis | I. Sebestyen |
| H.263 Conformance | T. Gardos |
| H.263+ | G. Sullivan |

SYSTEM/H.324

-	Review and edit Draft Recommendation H.324.

-	Investigate multilink issues for inclusion in H.324.

-	Make adjustments for V.8 bis and DTE/DCE issues as appropriate.

Membership:

D. Lindbergh (Chairperson)		C. Auyeung		F. Azadegan

K. Barraclough			A. Bist			L. Brown

P. Boucheron				M. Buckley		C. Choo

G. Corcoran				P. Crouch		E. deBoer

S. Eckart				V. Eyuboglu		C. Fogg

C. Gates				T. Geary		J. Guichard

S. Gupta				I. Haignere		B. Haskell

F. Henkel				P. Hsieh		C. Hansen

F. Huffman				A. Hsueh		A. Intrater

R. Ivy					J. Jeon			T. Joe

N. Kenyon				M. Kerdranvat		K. Krechmer

G. Landsburg				D. Lappe		F. Lecanu

J. Lee					D. Lindbergh		L. Liu

B. Lozach				J. Magill		M. Motamedi

D. Morley				A. Moyler		M. Nilsson

S. Pan					I. Park			J. Park

P. Perrot				D. Qian		X. Ran

C. Reader				R. Reynolds		P. Riglet

K. Rijkse				R. Schaphorst		J. Schmitt

I. Sebestyen				K. Sim			J. Smolinske

C. Szcaepucha			E. Tanskanen		I. Varga

W. Welsh				F. Whittington		M. Willebeek-LeMair

L. Yeh					M. Yong		M. Zeug

W. Zheng

H.263L

-	H.263L will attempt to formalize the MPEG4/H.263L joint meeting arrangement.

-	H.263L will meet with MPEG4 in July (Tempere, Finland)

-	H.263L will attempt to develop some core experiments within the next two weeks, perform the experiments (2 or more companies preferred), and present results during the July MPEG meeting.

-	H.263L will solicit proposals to be presented at the July LBC meeting.

Membership:

R. Ivy(Chairperson)		H. Ahn 			O. Andersson

B. Andrews			C. Auyeung 			G. Bang

S. Battista			A. Bist				G. Bjoutegaard

L. Brown			S. Cheung			D. Choi

C. Choo 			K. Chu				P. Crouch

S. Eckart			N. Faerber			C. Fan

R. Fischer			M. Frater			R. Fryer

G. Campbell			T. Gardos			R. Grinnell

S. Gupta			I. Haignere			B. Haskell

K. Hibi				H. Hsieh			P. Hsieh

A. Hsueh			F. Huffman			J. Jeon

T. Joe				G. Jung			K. Illgner

J. Jung				T. Kawahara			M. Kerdranvat

G. Kim				J. Kim				K. Kneib

D. Lappe			K. Lillevold			D. Lindbergh

P. List				Y. Machida			Y. Mao

T. Miki				J. Moon			J. Muller

T. Nakai			J. Nieweglowski		M. Nilsson

G. Nitsche			K. O’Connell			K. Oehler

T. Ohya			J. Osterman			S. Othman

D. Qian			X. Ran				R. Reynolds

A. Riccomi			P. Riglet			K. Rijkse

A. Rodriguez			G. Russo			A. Salai

J.C. Schmitt			 I. Sebestyen			G. Sullivan

E. Tanskanen			P. Voois			D. Wang

J. Webb			W. Welsh			M. Willebeek-Lemair

M.Whybray			M. Zeug			C. Zhu

Q. Zhu

INTERWORKING

-	Provide LBC focus for dialog with related ITU organizations/ rapporteurs to insure interworking with H,320, H.323, H.310 systems.

-	One objective is to minimize the complexity and distortion of interworking adapters between H.324 terminals and other ITU terminals.

-	Review the H.324 multipoint situations and initiate action to insure multipoint operation is achieved in a cost-effective and timely manner.

Membership:

D. Lindbergh (Chairperson)	K. Barraclough		P.  Boucheron

L. Brown			M. Buckley			S. Cheung

G. Corcoran			V. Eyuboglu			C. Gates

T. Geary			S. Gupta			I. Haighere

D. Lappe			J. Lee				P. List

J. Park				X. Ran				K. Rijkse

J.C. Schmitt			I. Sebestyen			M. Yong

M. Zeug			W. Zheng

SPEECH CODING/G.723.1

-	Complete work on G.723.1, including Annexes A, B and C.

Membership:

R. Cox (Chairperson)		C. Auyeung			Y. Beery

L. Bialik			P. Boucheron			L. Brown

R. Burns			C. Choo 			K. Chu

C. Cox 			S. Gupta			A. Hsueh

H. Hsieh			P. Hsieh			Y. Huang

T. Joe				S. Johnson			J. Johnston

G. Jung			M. Keith			K. Kneib

C. Lamblin			D. Lappe			S. Lee

L. Liu				B. Lozach			Y. Machida

J. Magill			Y. Mao				H. Oh

T. Ohya			S. Othman			J. Petit

X. Ran				R. Reynolds			R. Schaphorst

I. Sebestyen			E. Sonnic			S. Steinberger

H. Su				M.Tseng			D. Wang

K. Wang			I. Varga			P. Voois

X. Yang			M. Yong			M. Zeug

H.324 TEST

-	Develop a plan for interoperability testing of H.324 terminal testing & verification.

-	Establish procedure for real time testing between implementers via low error rate communication channels.

-	Promote and conduct non-real time and real time tests.

Membership:

C. Gates (Chairperson)	A. Aharoni			C. Auyeung

K. Barraclouch		Y. Beery			A. Bist

G. Bjontegaard		P. Boucheron			L. Brown

K. Chu				G. Corcoran			R. Cox

C. Cox				P. Crouch			V. Eyuboglu

T. Geary			I. Gerson			R. Haermens

I. Haignere			P. Hsieh			F. Huffman

A. Hsueh			J. Hong			Y. Huang

M. Jabri			T. Joe				M. Keith

M. Kerdranvat			K. Kneib			F. Lecanu

K. Lillevold			D. Lindbergh			P. List

J. Magill			Y. Mao				M. Motamedi

K. Oehler			H. Oh				S. Othman

S. Pan				I. Park				J. Park

D. Pascal			J. Petit				C. Porter

X. Ran				R. Reynolds			P. Riglet

K. Rijkse			G. Russo			R. Schaphorst

A. Schiano			I. Sebestyen			K. Sim

C. Szczepucha		E. Tanskanen			M. Tzseng

I. Varga			P. Voois			D. Wang

M. Willebeek-LeMair		B. Woo			X. Yang

M. Yong			M. Zeug			W. Zheng

Q. Zhu

COMMUNICATION/CONTROL/H.245

-	Review and edit Draft Recommendation H.245.

-	Add code points for mobile operation.

-	Review code points for H.324 multipoint operation.

-	Work jointly with the interworking Ad Hoc Committee to address ITU interworking issues.

Membership:

W. Welsh (Chairperson)		K. Barraclough		G. Bjontegaard

P. Boucheron				L. Brown			M. Buckley

F. Burg				G. Corcoran			C. Choo

V. Eyuboglu				C. Gates			T. Geary

C. Hansen				F. Henkel			P. Hsieh

Y. Huang				J. Jeon				T. Joe

M. Kerdranvat				G. Lansburg			J. Lee

D. Lindbergh				L. Liu				J. Magill

T. Miki					D. Morley			M. Motamedi

M. Nilsson 				S. Othman			I. Park

J. Park					R. Reynolds			P. Riglet

K. Rijkse				J. Schmitt			I. Sebestyen

K. Sim					G. Sullivan			C. Szczepucha

E. Tanskanen				I. Varga			P. Voois

M. Yong				M. Zeug			W. Zheng

MOBILE

-	Edit an annex to H.223 describing the mobile multiplexer and adaptation layers.  Pedro Crespo and Peter Vogel are leading the effort to provide text for the multiplex layer; Ralf Fischer and Phillipe Perrot are drafting text for the adaptation layer.

-	Edit an annex to H.324 describing the modifications needed to support mobile communications

-	Edit an informative appendix (to be provided by Niko Faerber) to H.263 describing error tracking.

-	Provide new H.245 codepoints to the H.245 editor for inclusion in the revised H.245 description. Niko Faerber is leading the work to define codepoints.

Membership:

D. Lappe (Chairperson)	H. Ahn				C. Auyeung

G. Bang			L. Bengtsson			A. Bist

G. Bjontegaard		P. Boucheron			L. Brown

S. Cheung			C. Clanton			P. Crespo

P. Crouch			N. Farber			R. Fischer

M. Frater			C. Gates			T. Geary

J. Guichard			S. Gupta			I. Haignere

B. Haskell			K. Hibi				S. Hodur

P. Hsieh			A. Hsueh			K. Illgner

R. Ivy				J. Jeon				G. Jung

T. Kawahara 			G. Kim				J. Kim

K. Kneib			P. List				M. Luomi

Y. Machida			J. Magill			T. Miki

J. Moon			D. Morley			J. Muller

T. Nakai			J. Nieweglowski		M. Nilsson

K. O'Connell			K. Oehler			T. Ohya

S. Othman			S. Pan				J. Park

P. Perrot			D. Qian			X. Ran

C. Reader			R. Reynolds			K. Rijkse

P. Salmon			R. Schaphorst			I. Sebestyen

F. Seytter			A. Silvershatz			J. Smolinske

E. Steinbach			M. Throndson			P. Vogel

D. Wang			J. Webb			B. Welsh

M. Willebeek-LeMair		B. Wimmer			M. Zeug

Y. Zhang			C. Zhu				Q. Zhu

NON-CONVERSATIONAL SERVICES

-	Investigate techniques for adding information access capability to H.324 and H.324M.

-	Define requirements, applications.

Membership:

W. Welsh (Chairperson)	P. Boucheron			L. Brown

M. Buckley			F. Burg			S. Cheung

G. Corcoran			P. Crouch			G. Campbell

V. Eyuboglu			C. Gates			S. Gupta

I. Haignere			P. Hsieh			J. Jeon

K. Kneib			D. Lappe			P. Latra-Rasku

D. Lindbergh			P. List				M. Luomi

J. Magill			M. Motamedi			J. Muller

J. Nieweglowski		K. O’Connell			H. Oh

T. Ohya			S. Othman			J. Park

J. Petit				C. Porter			R. Reynolds

A. Schiano			J.C. Schmitt			I. Sebestyen

I. Varga			F. Whittington			M. Willebeek-LeMair

M. Yong			M. Zeug			W. Zheng

Q. Zhu

V.8/V.8bis

-	Continue liaison with SG14 in V.8/V.8bis area.

-	Prepare recommendations for possible H.324 changes related to v.8/v.8bis.

Membership:

I. Sebestyen (Chairperson)	C. Auyeung			P. Bourcheron

L. Brown			M. Buckley			P. Crouch

C. Gates			T. Geary			C. Hanson

P. Hsieh			R. Ivy				S. Lee

L. Liu				J. Magill			R. Nobakht

J. Park				R. Reynolds			A. Schiano

H. Singh			I. Varga			W. Welsh

M. Whybray			M. Yong			M. Zeug

H.263 CONFORMANCE

-	Develop procedures that aid manufacturers in developing conforming H.263 codecs.  Conformance is achieved when H.263 encoders generate only valid H.263 bit streams as defined in ITU-T Recommendation H.263 and when H.263 decoders produce acceptable pictures for all valid input bit streams.  Although the intent is to develop as thorough a set of tests as possible, in no way can they be considered complete and hence do not guarantee conformance.  Procedures, test videos and bit streams will be developed which will aid manufacturers in quality testing H.263 encoder and decoder implementations.

Membership:

T. Gardos (Chairperson)	H. Ahn				B. Andrews

A. Bist				S. Cheung			C. Clanton

T. Gardos			S. Gupta			I. Haignere

A. Hsueh			R. Ivy				G. Jung

M. Kerdranvat			G. Kim				J. Kim

F. Lecanu			P. List				Y. Machida

T. Markas			J. Muller			T. Nakai

K. O’Connell			S. Othman			K. Rijkse

J. Schmitt			D. Wang			J. Webb

M. Zeug

H.263+

-	To collaborate on the development of enhancements to the H.263 video coding recommendation for near-term standardization:

1.	To develop new proposals for the technical content of these near-term enhancements.

2.	To define, conduct, and discuss core experiments decided upon at the April meeting.

3.	To develop draft text for possible adoption describing syntax and semantics of proposed algorithmic features.

Membership:

G. Sullivan (Chairperson)

Same as H.263L.

52	Contributions for the Next Meetingtc "_autonumlgl§	Contributions for the Next Meeting"\l 2§

The Rapporteur reminded experts that the effectiveness of the meetings will be improved if technical contributions are prepared early and distributed to participants prior to the meeting.  The Rapporteur will distribute to all experts those contributions which are received by June 27, 1996.  After that date, the author should distribute contributions directly to all experts.  In any case, the author should bring 100 copies to the meeting itself.

53	Future Meeting Scheduletc "_autonumlgl§	Future Meeting Schedule"\l 2§

The future meeting schedule listed below was agreed to.

| Location | Date | Host |
| --- | --- | --- |
| London, England | July 15-18, 1996 | British Telecom |
| Atlanta, GA | November 11-14, 1996 | Iterated Systems |
| Europe | February 17-20, 1997 | ? |