| ITU – Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Post-Osaka Contribution for White Document Preparation | Document Filename: Generated: 19 July 2000 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Gary Sullivan Microsoft Corp. One Microsoft Way Redmond, WA 98052 USA | Tel: Fax: Email: | (425) 703-5308 (425) 936-7329 Gary.Sullivan@itu.int |
| Title: | H.263++ Annex U Draft |  |  |
| Purpose: | Proposal |  |  |

Summary

This draft represents a refinement of my draft of 17 July.  Changes were made after consultation with Wiegand, Fogg, Horowitz, and Hannuksela:

- RPBT specified to not be present in a B picture since it serves no purpose there.

- The value definition for NOERPSL was backwards at the beginning of U.4.1.4.

- The former subclause U.4.1.5.3 was moved after the former subclause U.4.1.5.5.

- The values of COD were backwards in the second and third sentences of U.4.2.2.

- The picture reference parameters used for COD='1' skipping in B and EP pictures were clarified.

- The total buffer memory size must be specified in the bitstream if sliding window operation is expected to work without external information.  Indexing past the short-term pictures must cause a reference to a long-term picture, so the decoder must know how many short-term pictures it is expected to keep.  It can determine how many short-term pictures to keep if it knows the total buffer size and subtracts from that the number of pictures that have active long-term indices.  The segmentation of the pictures into sub-picture units also should be known even when the buffer is reset.  To fix this, we removed NOERPSL from the picture level – making the presence of the ERPS layer mandatory at the picture level for all ERPS-mode pictures.  This allowed two RMPNI codes to be shortened (since it alleviated start-code emulation prevention requirements).  We then modified the sub-picture dimension MMCO command to make it a “buffer parameter specification” command with a total size and a reset indicator.

- Modified the description of sliding window operation to allow it to require removal of more than one picture if necessary in the case of sub-picture removal.

- Specified that MRPA shall be "0" if the picture invokes the RPR mode (Annex P) and that the same picture will be specified as the reference picture at both the picture and GOB or slice levels in any such picture and that the buffer must be reset if the picture size ever changes.

- Added a picture reference parameter PRB and an associated emulation prevention bit MEPBB to MVDB for Improved PB frames.

- Added provision for video redundancy coding sync pictures in U.4.1.3.

- Fixed an interaction between MRPA and BTPSM.  Eliminated BTPSM when MRPA is "0" (since can’t reference two backward-prediction pictures unless the macroblock syntax allows it, and the macroblock syntax is controlled by MRPA).  Allow two pictures to be re-mapped in a B picture when MRPA is "0" (so you can choose both one forward and one backward prediction picture).

- Altered the description of picture number counting to specify numbering within enhancement layers.  Otherwise the numbering scheme would not work in scenarios such as layer 2 of Figure O.4.

- Various (mostly editorial) changes to the “decoder process” section.

- Specified some restrictions on the contents of SPRB to prevent nonsense.

Revision marks are included relative to my draft of 17 July.

Things that need work:

- Should we require Annex D to be active whenever Annex U is in use?  (This would eliminate the need for MEPB0, MEPB, MEPB2-4, MEPBB, MEPBFW, and MEPBBW.)  I personally think not, since those bits don’t look too hard to handle to me.  But this seems like a tough judgment call.

- Incorporating Miska’s remarks on a new back-channel format as necessary.

Annex U
Enhanced Reference Picture Selection mode

(This annex forms an integral part of this Recommendation.)

## U.1	Introduction

This annex describes the optional Enhanced Reference Picture Selection (ERPS) mode of this Recommendation. The capability to use this optional mode is negotiated by external means (for example Recommendation H.245). The amount of picture memory accommodated in the decoder for ERPS operation may also be signaled by external means.  The use of this mode shall be indicated by setting the formerly-reserved bit 16 of the optional part of the PLUSPTYPE (OPPTYPE) to "1". The mode provides benefits for both error resilience and coding efficiency by using a memory buffer of reference pictures.

A sub-mode of the ERPS mode is specified for Sub-Picture Removal.  The purpose of Sub-Picture Removal is to reduce the amount of memory required to store multiple reference pictures.  The memory reduction is accomplished by specifying the partitioning of each reference picture into smaller rectangular units called sub-pictures.  The encoder can then indicate to the decoder that specific sub-picture areas of specific reference pictures will not be used as a reference for the prediction of subsequent pictures, thus allowing the memory allocated in the decoder for storing these areas to be used to store data from other reference pictures.  The support for this sub-mode and the allowed fragmentation of the picture memory into minimum picture units (MPUs) for  Sub-Picture Removal as defined herein is also negotiated by external means (for example, Recommendation H.245).

A sub-mode of the ERPS mode is specified for enabling two-picture backward prediction in B pictures.  This sub-mode can enhance performance by providing encoders for B pictures not only with an ability to use multiple references for forward prediction, but also to use more than one reference picture for backward prediction.  The support for this sub-mode is negotiated by external means (for example, Recommendation H.245).

For error resilience, the ERPS mode can use backward channel messages, which are signaled by external means (e.g. Recommendation H.245) sent from a decoder to an encoder to inform the encoder which part of which pictures have been incorrectly decoded. The ERPS mode provides enhanced performance compared to the Reference Picture Selection (RPS) mode defined in Annex N. It shall not be used simultaneously with the RPS mode. (It can be used in such a way as to provide essentially the same functionality as the RPS mode.)

For coding efficiency, motion compensation can be extended to prediction from multiple pictures. The extension of motion compensation to multi-picture prediction is achieved by extending the motion vector by a picture reference parameter that is used to address a macroblock or block prediction region for motion compensation in any of the multiple reference pictures. The picture reference parameter is a variable length code specifying a buffer index. For that, the reference pictures are assembled in a buffering scheme that is controlled by the encoder.

The ERPS mode shall not be used with the Syntax-based Arithmetic Coding mode (see Annex E) or the Data Partitioned Slice mode (see Annex V).

Once activated, the ERPS mode shall not be inactivated in subsequent pictures in the bitstream unless  the initial inactivation occurs in an I or EI picture and any reactivation is also in an I or EI picture and is accompanied by a buffer reset (RESET equal to "1").  If inactivated, the entire contents of the ERPS multi-picture buffer shall be set to “unused” status.

## U.2	Video source coding algorithm

The source coder of this mode is shown in generalized form in Figure U.1. This figure shows a structure that uses a number of picture memories.

The video source coding algorithm can be extended to multi-picture motion compensation. Enhanced coding efficiency may be achieved by allowing reference picture selection on the macroblock level. A picture buffering scheme with relative indexing is employed for efficient addressing of pictures in the multi-picture buffer. The multi-picture buffer control may work in several modes of operation.

For example, a sliding window over time can be accommodated by the buffer control unit. In such a buffering scheme using M picture memories PM0...PMM-1, the most recent preceding (up to M) decoded and reconstructed pictures are stored in the picture memories and can be used as references for decoding. If the number of pictures maximally accommodated by the multi-picture buffer corresponds to M, the motion estimation when coding picture m, if 0£m£M-1, can utilize m pictures. In case m³M, the maximum number of pictures M can be used. Alternatively, an adaptive buffer control can be utilized with a more flexible use of the picture memories than with the simple sliding window scheme.

The operation of the ERPS mode results in the assignment of “unused” status to some pictures or sub-picture areas of pictures that have been sent to the decoder.  Once some picture or area of a picture has been assigned to “unused” status, the bitstream shall not contain any data that causes a reference to the “unused” area for the prediction of subsequent pictures.  By managing the assignment of “unused” status to previous pictures, the encoder can ensure that sufficient memory is available in the decoder to store all data needed for the representation of subsequent pictures.

The source coder may select one or several of the picture memories to suppress temporal error propagation caused by inter-frame coding. The Independent Segment Decoding mode (see Annex R), which treats boundaries of GOBs with non-empty headers or slices as picture boundaries, can be used to avoid spatial error propagation due to motion compensation across the boundaries of the GOBs or slices when this mode is applied to a smaller unit than a picture, such as a GOB or slice. The information to signal which picture is selected for prediction is included in the encoded bitstream.

The strategy used by the encoder to select the picture to be used for prediction is out of the scope of this Recommendation.

FIGURE U.1/H.263

Source coder for Enhanced Reference Picture Selection mode

## U.3	Back-Channel messages

An out-of-band channel, which need not necessarily be reliable, can be used to convey back-channel messages. The syntax of this out-of-band channel (which could be a separate logical channel, for example using Recommendation H.223 or Recommendation H.225.0) should be the one defined in Annex N, except substituting picture numbers as defined herein instead of the TR and TRP fields used in the back-channel messages defined in Annex N.  The “videomux” operation of back-channel messages as defined in Annex N is not supported in the ERPS mode.

## U.4	Syntax

The syntax is altered in the Picture, Group of Blocks (GOB),  and slice layers.  When indicated by a parameter MRPA = "1", the syntax is also altered in the macroblock layer. In the Picture, GOB, and Slice layers, an Enhanced Reference Picture Selection layer (ERPS layer) is inserted. On the macroblock layer, picture reference parameters are inserted under certain conditions to enable multi-picture motion compensation.

### U.4.1	Syntax of the Picture, GOB, and Slice layer

The Enhanced Reference Picture Selection syntax for the PLUS header (otherwise as shown in Figure 8) is shown in Figure U.2. The fields of RPSMF, PN, and the ERPS layer are inserted into the PLUS header.  The fields of TRPI, TRP, BCI, and BCM are not present (since they are only needed for the RPS mode of Annex N, which is not allowed when the ERPS mode is active).

FIGURE U.2/H.263

Structure of PLUS Header for the ERPS mode

The syntax for the GOB layer is shown in Figure U.3. The fields of PNI, PN, NOERPSL, and the ERPS layer are added to the syntax (otherwise defined as in Figure 9).

FIGURE U.3/H.263

Structure of GOB layer for the ERPS mode

When the optional Slice Structured mode (see Annex K) is in use, the syntax of the slice layer is modified in the same way as the GOB layer. The syntax for the Slice layer is shown in Figure U.4.  The slice that immediately follows the picture start code in the bitstream also includes all of the added fields PNI, PN, NOERPSL, and the ERPS layer.

FIGURE U.4/H.263

Structure of Slice layer for the ERPS mode

The ERPS layer is shown in Figure U.5.

FIGURE U.5/H.263

Structure of the ERPS layer

Variable length codes for the ADPN, LPIR, MLIP1, DPN, LPIN, SPTN, PR, PR0, PR2, PR3, PR4, and PRFW fields are given in Table U.1.

Table U.1/H.263

Variable length codes for ADPN, LPIR, MLIP1, DPN, LPIN, SPTN, PR, PR0, PR2, PR3, PR4, and PRFW.

| Absolute position | Number of bits | Codes |
| --- | --- | --- |
| 0 | 1 | 1 |
| "x0"+1 (1:2) | 3 | 0x00 |
| "x1x0"+3 (3:6) | 5 | 0x11x00 |
| "x2x1x0"+7 (7:14) | 7 | 0x21x11x00 |
| "x3x2x1x0"+15 (15:30) | 9 | 0x31x21x11x00 |
| "x4x3x2x1x0"+31 (31:62) | 11 | 0x41x31x21x11x00 |
| "x5x4x3x2x1x0"+63 (63:126) | 13 | 0x51x41x31x21x11x00 |
| "x6x5x4x3x2x1x0"+127 (127:254) | 15 | 0x61x51x41x31x21x11x00 |
| "x7x6x5x4x3x2x1x0"+255 (255:510) | 17 | 0x71x61x51x41x31x21x11x00 |
| "x8x7x6x5x4x3x2x1x0"+511 (511:1022) | 19 | 0x81x71x61x51x41x31x21x11x00 |
| "x9x8x7x6x5x4x3x2x1x0"+1023 (1023:2046) | 21 | 0x91x81x71x61x51x41x31x21x11x00 |
| "x10x9x8x7x6x5x4x3x2x1x0"+2047 (2047:4094) | 23 | 0x101x91x81x71x61x51x41x31x21x11x00 |

#### U.4.1.1	Reference Picture Selection Mode Flags (RPSMF) (3 bits)

A codeword that is present in the PLUS header whenever the ERPS mode is in use (regardless of the value of UFEP). RPSMF shall not be present in the GOB or slice layer.  When present, RPSMF indicates which type of back-channel messages are needed by the encoder.  The values of RPSMF shall be as defined in subclause 5.1.13.

#### U.4.1.2	Picture Number Indicator (PNI) (1 bit)

A codeword that is always present at the GOB or slice layer and not present in the PLUS header. When present, PNI indicates whether or not the following PN field is also present.

"0": PN field is not present.
"1": PN field is present.

#### U.4.1.3	Picture Number (PN) (10 bits)

PN is a fixed length codeword that is always present in the PLUS header when the ERPS mode is in use, and is present at the GOB or slice layer only when indicated by PNI. PN shall be incremented by 1 for each coded and transmitted I or P picture or Improved PB frame, in a 10-bit modulo operation. For EI and EP pictures, PN shall be incremented for each EI or EP picture within the same scalability enhancement layer.  For B pictures, PN shall have the same value as that of the most temporally-recent reference picture in the reference layer of the B picture.  B pictures are not stored in the multi-picture buffer, as they are not used as references for subsequent pictures.

In a usage scenario known as “Video Redundancy Coding”, the ERPS mode may be used by some encoders in a manner in which more than one representation is sent for the pictured scene at the same temporal instant (usually using different reference pictures).  In such a case in with the ERPS mode is in use and in which adjacent pictures in the bitstream have the same temporal reference and the same picture number, the decoder shall regard this occurrence as an indication that redundant copies have been sent of approximately the same pictured scene content, and shall decode and use the first such received picture while discarding the subsequent redundant picture(s).

The PN serves as a unique ID for each picture stored in the picture buffer within 1024 coded frames.  Therefore, a picture shall not be kept in the buffer after more than 1023 coded frames unless it has been assigned a long-term picture index as specified below.

#### U.4.1.4	No Enhanced Reference Picture Selection Layer (NOERPSL) (1 bit)

NOERPSL is a single bit fixed length codeword that is present  at the GOB or slice level whenever the ERPS mode is in use. It is not present in the PLUS header.  The values of NOERPSL shall be as follows:

"0": The ERPS layer is sent

"1": The ERPS layer is not sent

If NOERPSL is "1", all ERPS settings and re-mappings in effect for the picture shall be applied also for the relevant GOB or slice.  ERPS layer information sent at the GOB or slice level does not affect the decoding process of any other GOB or slice.

#### U.4.1.5	Enhanced Reference Picture Selection layer (ERPS) (variable length)

The ERPS layer is always present at the picture level when the ERPS mode is in use, and is present at the GOB or slice level if NOERPSL is "0". It specifies the buffer indexing used to decode the current picture, GOB, or slice and manages the contents of the picture buffer.

##### U.4.1.5.1	Multiple Reference Pictures Active (MRPA) (1 bit)

MRPA is a single-bit fixed length codeword that is present only if the picture coding type indicates a P-picture, an EP-picture, an Improved PB frame, or B-picture. MRPA is the first element in the ERPS layer if present.  MRPA specifies whether the number of active reference frames for forward-prediction or backward-prediction decoding of the current picture, GOB, or slice may be larger than one.   The value of MRPA shall be as follows:

"1": More than one reference picture may be used for forward or backward motion compensation.

"0": Only one reference picture is used for forward or backward motion compensation. In this case, the extensions of the macroblock layer syntax in subclause U.4.2 do not apply.

MRPA may be changed from GOB to GOB or slice to slice so that different GOBs or slices may address different numbers of reference pictures.

MRPA shall be "0" in any picture which invokes the Reference Picture Resampling mode (see Annex P), and the same picture shall be indicated as the reference picture to be used at both the picture and GOB or slice levels in any such picture.

##### U.4.1.5.2	Re-Mapping of Picture Numbers Indicator (RMPNI) (variable length)

RMPNI is a variable length codeword that is present in the ERPS layer if the picture is a P, EP, Improved PB, or B picture. RMPNI indicates whether any default picture numbers are to be re-mapped for motion compensation of the current picture, GOB, or slice – and how the re-mapping of the relative indices into the multi-picture buffer is to be specified if indicated. RMPNI is transmitted using Table U.2.  If RMPNI indicates the presence of an ADPN or LPIR field, an additional RMPNI field immediately follows the ADPN or LPIR field.

A picture reference parameter is a relative index into the ordered set of pictures. The RMPNI, ADPN, and LPIR fields allow the order of that relative indexing into the multi-picture buffer to be temporarily altered from the default order for the decoding of a particular picture, GOB, or slice.  The default order is for the short-term pictures (i.e., pictures which have not been given a long-term index) to precede the long-term pictures in the reference indexing order.  Within the set of short-term pictures, the default order is for the pictures to be ordered starting with the most recent buffered reference picture and proceeding through to the oldest reference picture (i.e., in decreasing order of picture number in the absence of wrapping of the ten-bit picture number field).  Within the set of long-term pictures, the default order is for the pictures to be ordered starting with the picture with the smallest long-term index and proceeding up to the picture with long-term index equal to the most recent value of NLB-1.

The first ADPN or LPIR field that is received (if any) moves a specified picture out of the default order to the relative index of zero.  The second such field moves a specified picture to the relative index of one, etc.  The set of remaining pictures not moved to the front of the relative indexing order in this manner shall retain their default order amongst themselves and shall follow the pictures that have been moved to the front of the buffer in relative indexing order.

If MRPA is "0", no more than one ADPN or LPIR field shall be present in the same ERPS layer unless the current picture is a B picture.  If the current picture is a B picture and MRPA is "0", no more than two ADPN or LPIR fields shall be present in the same ERPS layer.

Any re-mapping of picture numbers specified for some picture shall not affect the decoding process for any other picture.  Any re-mapping of picture numbers specified for some GOB or slice shall not affect the decoding process for any other GOB or slice.  A re-mapping of picture numbers specified for a picture shall only affect the decoding process for any GOB or slice within that picture in two ways:

- If NOERPSL is "1" at the GOB or slice level, then the re-mapping specified at the picture level is also used at the GOB or slice level.

- If the picture is a B picture, the re-mapping specified at the picture level may effect the calculation of the value of TRB and TRD for direct bidirectional prediction.

An RMPNI “end loop” indication is the last element of the ERPS layer for a B picture if MRPA is "0".  In a B picture with MRPA equal to "1", an RMPNI “end loop” indication is followed by BTPSM.  In a non-B picture, an RMPNI “end loop” indication is followed by RPBT.

Table U.2/H.263

RMPNI operations for re-mapping of reference pictures.

| Value | Re-mapping Specified |
| --- | --- |
| '1' | ADPN field is present and corresponds to a negative difference to add to a picture number prediction value |
| '010' | ADPN field is present and corresponds to a positive difference to add to a picture number prediction value |
| '011' | LPIR field is present and specifies the long-term index for a reference picture |
| '001' | End loop for re-mapping of picture relative indexing default order |

##### U.4.1.5.3	Absolute Difference of Picture Numbers (ADPN) (variable length)

ADPN is a variable length codeword that is present only if indicated by RMPNI. ADPN is transmitted using Table U.1, where the index into the table corresponds to ADPN – 1. ADPN represents the absolute difference between the picture number of the currently re-mapped picture and the prediction value for that picture number.  If no previous ADPN fields have been sent within the current ERPS layer, the prediction value shall be the current picture number.  If some previous ADPN field has been sent, the prediction value shall be the picture number of the last picture that was re-mapped using ADPN.

If the picture number prediction is denoted PNP, and the picture number in question is denoted PNQ, the decoder shall determine PNQ from PNP and ADPN in a manner mathematically equivalent to the following:

if (RMPNI == "1") {	// a negative difference

if (PNP – ADPN < 0)

PNQ = PNP – ADPN +1024;

else

PNQ = PNP – ADPN;

}else{			// a positive difference

if (PNP + ADPN > 1023)

PNQ  = PNP + ADPN – 1024;

else

PNQ  = PNP + ADPN;

}

The encoder shall control RMPNI and ADPN such that the decoded value of ADPN shall not be greater than or equal to 1024.

As an example implementation, the encoder may use the following process to determine values of ADPN and RMPNI to specify a re-mapped picture number in question, PNQ:

DELTA = PNQ – PNP;

if (DELTA < 0) {

if (DELTA < –511)

MDELTA = DELTA + 1024;

else

MDELTA = DELTA;

}else{

if(DELTA > 512)

MDELTA = DELTA – 1024;

else

MDELTA = DELTA;

}

ADPN = abs(MDELTA);

where abs() indicates an absolute value operation.  Note that the index into Table U.1 corresponds to the value of ADPN – 1, rather than the value of ADPN itself.

RMPNI would then be determined by the sign of MDELTA.

##### U.4.1.5.4	Long-term Picture Index for Re-mapping (LPIR) (variable length)

LPIR is a variable length codeword that is present only if indicated by RMPNI. LPIR is transmitted using Table U.1. It represents the the long-term picture index to be re-mapped.  The prediction value used by any subsequent ADPN re-mappings is not affected by LPIR.

##### U.4.1.5.5	B-Picture Two-Picture Prediction Sub-Mode (BTPSM) (1 bit)

BTPSM is a single bit fixed length codeword that is present only in a B picture (see Annex O) and only when MRPA is "1".  It follows an RMPNI “end loop” indication when present and is the last element of the ERPS layer for a B picture if present.  It indicates whether the two-picture backward prediction sub-mode is in use for the picture as follows:

"0" : Single-picture backward prediction

"1" : Two-picture backward prediction

BTPSM has an implied value of "0" if not present (when MRPA is "0").

The set of pictures available for use as forward prediction references is the set of pictures in the multi-picture buffer other than the set of backward reference pictures.  The set of backward reference pictures is determined by the value of BTPSM.  If single-picture backward prediction is specified by BTPSM, the first picture in (possibly re-mapped) relative index order is the only backward reference picture.  If two-picture backward prediction is specified by BTPSM, the first two pictures in (possibly re-mapped) relative index order are the two backward reference pictures.  The relative index for forward prediction then becomes a relative index into the set of forward reference pictures.

The contents of the multi-picture buffer are not affected by the presence of a B picture.  The B picture is not stored in the multi-picture buffer and is not used as a reference for the coding of subsequent pictures.

##### U.4.1.5.6	Reference Picture Buffering Type (RPBT) (1 bit)

RPBT is a single bit fixed length codeword that  specifies the buffering type of the currently decoded picture.  It follows an RMPNI “end loop” indication when the picture is not a B picture.  Its values are defined as follows:

"0" : Sliding Window

"1" : Adaptive Memory Control

In the “Sliding Window” buffering type, the current decoded picture shall be added to the buffer with index 0 if it is not a B picture, and any removal of pictures from the buffer is performed automatically in a first-in-first-out fashion among the set of short-term pictures.  In this case, if the buffer is not full, no pictures shall be marked as unused in the buffer.  If the buffer is full, the picture (or pictures as necessary to free the needed amount of memory in the case of  sub-picture removal) with the largest default index (or indices as necessary in the case of sub-picture removed) among the short-term pictures in the buffer shall be marked as unused.  In the sliding window buffering type, no additional information is transmitted to control the removal of data from the buffer.

In the "Adaptive Memory Control" buffering type, the encoder explicitly specifies any addition or removal of data from the multi-picture buffer and any assignment of long-term indices to short-term pictures. The current picture and other pictures may be explicitly marked as unused in the buffer, as specified by the encoder.  This buffering type requires further information that controlled by memory management control operation (MMCO) parameters.

The reference picture buffering type, if present in GOB or slice layers, shall be the same as in the picture layer.  Any MMCO command present in GOB or slice layers shall convey the same operation as in the picture layer.

If the picture is a B picture, RPBT shall not be present and the decoded picture shall not be stored in the multi-picture buffer.  This ensures that a B picture shall not affect the contents of the multi-picture buffer.

Similarly, the B-picture part of an Improved PB frame shall not be stored in the buffer.  All control fields associated with controlling the storage of an Improved PB frame shall be considered to be associated with controlling the storage of only the P-picture part of the Improved PB frame.

##### U.4.1.5.7	Memory Management Control Operation (MMCO) (variable length)

MMCO is a variable length codeword that is present only when RPBT is "1", and may occur multiple times if present.  It specifies a control operation to be applied to manage the multi-picture buffer memory.  The MMCO parameter is followed by data necessary for the operation specified by the value of MMCO, and then an additional MMCO parameter follows – until the MMCO value indicates the end of the list of such operations.  MMCO commands do not affect the buffer contents or the decoding process for the decoding of the current picture – rather, they specify the necessary buffer status for the decoding of subsequent pictures in the bitstream.  The values and control operations associated with MMCO are defined in Table U.3.

All memory operations specified using MMCO shall be specified in the picture layer.  Some or all of the same operations as are specified at the picture layer may be specified at the GOB or slice layer (with the same associated data).  MMCO shall not specify memory operations at the GOB or slice layer that are not also specified with the same associated data at the picture layer.

A buffer size and structure specification MMCO command must precede any use of the “sliding window” reference picture buffering type (RPBT equal to "0") in the bitstream.  A buffer size and structure specification MMCO command with RESET equal to "1" must precede any use of MMCO to indicate marking sub-picture areas of any short-term or long-term pictures as unused.  The sub-picture width and height specified in a buffer size and structure MMCO command shall not differ from the value of these parameters in a prior buffer size and structure MMCO command unless the current picture is an I or EI picture with RESET equal to "1".  The picture height and width shall not change within the bitstream except within a picture containing a buffer size and structure MMCO command with RESET equal to "1".

If a B picture using single-picture backward prediction is present in the bitstream, exactly one temporally subsequent non-B picture in the reference layer of the B picture shall precede the B picture in bitstream order, as specified in subclause O.2. No memory management control operations shall be present within any ERPS layer of these immediately temporally subsequent non-B picture within the reference layer of the B picture which marks any part of that immediately temporally succeeding non-B picture as unused, since that reference layer picture is needed for display until after the decoding of the B picture.

The transmission order constraints specified in subclause O.2 are adjusted as necessary for B pictures using two-picture backward prediction.  If a B picture using two-picture backward prediction is present in the bitstream, exactly two temporally subsequent non-B pictures in the reference layer of the B picture shall precede the B picture in bitstream order.  The other restrictions on the transmission order of the B picture in the bitstream specified in O.2 shall apply, but as adjusted for the use of two temporally subsequent reference layer pictures.  No memory management control operations shall be present within any ERPS layer of these two immediately temporally subsequent two non-B pictures within the reference layer of the B picture which mark any part of these two non-B pictures as unused, since these reference layer pictures are needed for display until after the decoding of the B picture.

Table U.3/H.263

Memory Management Control Operation (MMCO) Values

| Value | Memory Management Control Operation | Associated Data Fields Following |
| --- | --- | --- |
| '1' | End MMCO Loop | None (end of ERPS layer) |
| '001' | Assign a Long-Term Index to a Picture | DPN and LPIN |
| '010' | Mark a Short-Term Picture as Unused | DPN |
| '011' | Mark a Long-Term Picture as Unused | LPIN |
| '00001' | Mark Short-Term Sub-Picture Areas as Unused | DPN and SPRB |
| '00010' | Mark Long-Term Sub-Picture Areas As Unused | LPIN and SPRB |
| '00011' | Specify the Maximum Long-Term Picture Index | MLIP1 |
| '000001' | Specify the Buffer Size and Structure | SPWI, SPHI, SPTN, and RESET |

##### U.4.1.5.8	Difference of Picture Numbers (DPN) (variable length)

DPN is present when indicated by MMCO. DPN follows MMCO if present. DPN is transmitted using codewords in Table U.1 and is used to calculate the PN of a picture for a memory control operation.  It is used in order to assign a long-term index to a picture, mark a short-term picture as unused, or mark sub-picture areas of a short-term picture as unused.  If the current decoded picture number is PNC and the decoded value from Table U.1 is DPN, an operation mathematically equivalent to the following equations shall be used for calculation of PNQ, the specified picture number in question:

if (PNC – DPN < 0)

PNQ = PNC – DPN + 1024;

else

PNQ = PNC – DPN;

Similarly, the encoder may compute the DPN value to encode using the following relation:

if (PNC – PNQ < 0)

DPN = PNC – PNQ + 1024;

else

DPN = PNC – PNQ;

For example, if the decoded value of DPN is zero and MMCO indicates marking a short-term picture as unused, the current decoded picture shall be marked as unused.

##### U.4.1.5.9	Long-term Picture Index (LPIN) (variable length)

LPIN is present when indicated by MMCO.  LPIN is transmitted using codewords in Table U.1 and specifies the long-term picture index of a picture. It follows DPN if the operation is to assign a long-term index to a picture.  It follows MMCO if the operation is to mark a long-term picture as unused or to mark sub-picture areas of a long-term picture as unused.

##### U.4.1.5.10	Sub-Picture Removal Bit-Map (SPRB) (fixed length)

SPRB is a fixed length codeword that contains one bit for each sub-picture and is present when indicated by MMCO.  The number of bits of SPRB data is determined by the most recent value of SPWI and SPHI.  SPRB is used to indicate which sub-picture areas of a buffered picture are to be marked as unused.  SPRB follows DPN if the operation is to mark sub-picture areas of a short-term picture as unused, and follows LPIN if the operation is to mark sub-picture areas of a long-term picture as unused.

Sub-pictures are numbered in raster scan order starting from the upper-left of the frame.  For example, consider a case in which a reference picture, specified by DPN, is partitioned into six sub-pictures. Let “s1 s2 s3 s4 s5 s6” represent six bits of SPRB data. If bit si is "1", then the decoder should mark the ith sub-picture in the indicated reference picture as “unused”. For instance, if the SPRB is '000110' then the fourth and fifth sub-pictures areas are marked as unused.

To prevent start code emulation, SPREPB emulation prevention bits may be inserted within or following the SPRB data as specified in subclause U.4.1.5.11.

If SPRB is present and the specified picture has been previously been affected by a prior SPRB bit-map, the bit-map specified by SPRB shall contain a "1" for any sub-picture area that contained a "1" in the previous SPRB bit-map.  Every SPRB bit-map shall contain at least one bit having the value "0" and at least one bit having the value "1".

##### U.4.1.5.11	Sub-Picture Removal Emulation Prevention Bit (SPREPB) (one bit)

SPREPB is a single-bit fixed length codeword having the value "1" which shall be inserted immediately after any string of 8 consecutive zero bits of SPRB data.

##### U.4.1.5.12	Maximum Long-Term Picture Index Plus 1 (MLIP1) (variable length)

MLIP1 is a variable length codeword that is present if indicated by MMCO.  MLIP1 follows MMCO if present.  MLIP1 is transmitted using codewords in Table U.1.  If present, MLIP1 is use to determine the maximum index allowed for long-term reference pictures (until receipt of another value of MLIP1).  The decoder shall initially assume MLIP1 is "0" until some other value has been received.  Upon receiving an MLIP1 parameter, the decoder shall consider all long-term pictures having indices greater than the decoded value of MLIP1 minus 1 as unused for referencing by the decoding process for subsequent pictures.  For all other stored pictures, no change of status shall be indicated by MLIP1.

##### U.4.1.5.13	Sub-Picture Width Indication (SPWI) (7 bits)

SPWI is a fixed length codeword of 7 bits that is present if indicated by MMCO. SPWI follows MMCO when indicated.  SPWI specifies the width of a sub-picture in units of 16 luminance samples, such that the indicated sub-picture width is 16·(SPWI+1) luminance samples. The current picture has a width in sub-picture units of ceil ( ceil(pw / 16) / (SPWI+1) ) sub-pictures, where pw is the width of the picture,   and "/" indicates floating-point division. For positive numbers, the ceiling function, ceil ( x ), equals x if x is an integer and otherwise ceil ( x ) equals one plus the integer part of x. If a minimum picture unit (MPU) size defining the minimum width and height of a sub-picture has been negotiated by external means (for example, Recommendation H.245), the sub-picture width specified by SPWI shall be an integer multiple of the width of the MPU; otherwise, the sub-picture width specified by SPWI shall be such that SPWI is equal to ceil(pw / 16) - 1.

##### U.4.1.5.14	Sub-Picture Height Indication (SPHI) (7 bits)

SPHI is a fixed length codeword of 7 bits that is present if SPWI is present (as indicated by MMCO). SPHI follows SPWI if present.  SPWI specifies the height of a sub-picture in units of 16 luminance samples, such that the indicated sub-picture height is 16·SPHI. The allowed range of values of SPHI is from 1 to 72.  The current picture has a height of ceil ( ceil(ph / 16) / SPHI ) sub-pictures, where ph is the height of the picture and "/" indicates floating-point division. If a minimum picture unit (MPU) size defining the minimum width and height of a sub-picture has been negotiated by external means (for example, Recommendation H.245), the sub-picture height specified by SPHI shall be an integer multiple of the height of the MPU; otherwise, the sub-picture height specified by SPHI shall be such that SPHI is equal to ceil(pw / 16).

##### U.4.1.5.15	Sub-Picture Total Number (SPTN) (variable length)

SPTN is a variable length codeword that is present if SPWI and SPHI are present (as indicated by MMCO).  SPTN follows SPHI if present.  SPTN is coded using Table U.1, where the index into Table U.1 corresponds to the decoded value of SPTN – 1.  The decoded value of STPN is the total operational size capacity of the multi-picture buffer in units of sub-pictures as specified by SPWI and SPHI.  The memory capacity needed for the decoding of current pictures is not included in SPTN – only the memory capacity needed for storing the reference pictures to use for the prediction of other pictures.  When sub-picture removal is not in use (i.e. when SPWI and SPHI have whole-picture dimensions), the maximum number of active short-term reference pictures (for example, for sliding window operation) is thus given by SPTN minus the number of pictures that have been assigned to long-term indices.

##### U.4.1.5.16	Buffer Reset Indicator (RESET) (1 bit)

RESET is a single bit fixed length codeword that is present if SPWI, SPHI, and SPTN are present (as indicated by MMCO).  RESET follows SPTN if present.  The values of RESET shall be as follows:

"0": The buffer contents are not reset

"1": The buffer contents are reset

If RESET is "1", all pictures in the multi-picture buffer (not the current picture) shall be marked “unused” (including both short-term and long-term pictures).

### U.4.2	Macroblock layer syntax

### U.4.2.1	P-Picture and Improved PB frames Macroblock Syntax

The macroblock layer syntax is modified if the ERPS layer is present for P-pictures and Improved PB frames when the number of selected forward reference pictures may be greater than one, as indicated by MRPA. The field MRPA is signaled in the ERPS layer. The macroblock layer syntax is shown in Figure U.7 when MRPA is "1".  Otherwise, the macroblock syntax format in a P picture is not altered.

FIGURE U.7/H.263

Structure of P-picture and Improved PB frame Macroblock layer for the ERPS mode

#### U.4.2.1.1	Interpretation of COD

If the COD bit is "1", no further information is transmitted for the macroblock. In that case, the decoder shall treat the macroblock as an INTER macroblock with the motion vector for the entire macroblock equal to zero, picture reference parameter equal to zero, and with no coefficient data. If the COD bit is "0", indicating that the macroblock is coded, the syntax of the macroblock layer is depicted in Figure U.7 with the fields PR0, PR, PR2, PR3, and PR4 being included in the syntax. PR0, PR, PR2, PR3, and PR4 each consist of a variable length codeword as given in Table U.1.

### U.4.2.1.2	Picture Reference Parameter 0 (PR0) (variable length)

PR0 is a variable length codeword as specified in Table U.1.  It is present whenever COD is "0".  If PR0 has a decoded value of zero (codeword ‘1’), it indicates that further information will follow for the macroblock.  If decoded as non-zero, it indicates the coding of the macroblock using only a picture reference parameter.

If the field PR0 does not have a decoded value of zero (codeword '1'), no further information is transmitted for this macroblock. In that case the decoder shall treat the macroblock as an INTER macroblock with the motion vector for the whole block equal to zero, the picture reference parameter equal to PR0, and with no coefficient data.

If the field PR0 has a decoded value of zero (codeword '1'), the macroblock is coded. The meaning and usage of the fields MCBPC, CBPB, CBPY, and DQUANT remains unaltered. The field PR is included together with the field MVD for all INTER macroblocks (and in Improved PB frames mode also for INTRA macroblocks). The use of MODB in Improved PB frames is described in subclause U.4.2.1.4.

### U.4.2.1.3	Macroblock Emulation Prevention Bit 1 (MEPB1) (1 bit)

MEPB1 is a single bit having the value "1" that follows PR0 if and only if the following conditions are satisfied:

- COD is "0", and

- the decoded value of PR0 is "1" (codeword '000'), and

- COD follows another macroblock which also has COD = "0" and PR0 = "1" (codeword '000'), and

- the PR0 of the previous macroblock is not followed by an MEPB1 bit.

The purpose of MEPB1 is to prevent start-code emulation.

#### U.4.2.1.4	PR, PR2-4, and PRB (variable length)

PR is the primary picture reference parameter. PR is present whenever MVD is present.  The three codewords PR2-4 are included together with MVD2-4 if indicated by PTYPE and if MCBPC specifies an INTER4V or INTER4V+Q macroblock (a macroblock of type 2 or 5 in Tables 8 and 9). PR2-4 and MVD2-4 are only present when in Advanced Prediction mode (see Annex F) or Deblocking Filter mode (see Annex J).  PRB is only present in an Improved PB frame when MODB indicates that MVDB is present.  PR, PR2-4, and PRB each specify a picture reference relative index into the multi-picture buffer.

PR is used as the picture reference parameter for motion compensation of the entire macroblock if the macroblock is not an INTER4V or INTER4V+Q macroblock. If the macroblock is an INTER4V or INTER4V+Q macroblock, PR is used for motion compensated prediction of the first of the four 88 luminance blocks in the macroblock and for the two chrominance blocks of the macroblock (altered for application to chrominance data as specified in subclause 6.1.1 and Table 18 or subclause F.2 and Table F.1 as appropriate).  PR2-4 are used for motion compensation of the remaining three 88 blocks of luminance data in the macroblock.  If MODB indicates that MVDB is present, PRB is the picture reference parameter for forward prediction of the B part of the Improved PB frame.

In Improved PB frames when MODB indicates BPB bidirectional prediction, the values of TRD and TRB shall be computed as the temporal reference increments based on the temporal reference data of the current picture and that of the most recent previous reference picture, regardless of whether or not the most recent previous reference picture has been re-mapped to a difference relative index order, marked as “unused”, or assigned to a long-term index.  The picture used as the forward reference picture for BPB bidirectional prediction in Improved PB frames shall be the picture specified by PR.

#### U.4.2.1.5	MEPB , MEPB2-4, and MEPBB (1 bit each)

MEPB, MEPB2-4, and MEPBB are each a single bit having the value "1" if present.  Each shall be present if and only if the Unrestricted Motion Vector mode (see Annex D) is not in use and the associated PR, PR2-4, or PRB field is present and has the decoded value "1" (codeword '000').  Their purpose is to prevent start-code emulation.

### U.4.2.2	B- and EP-picture Macroblock Syntax

The macroblock layer syntax for B and EP pictures (see Annex O) is modified in a similar fashion as in P pictures.  The COD bit, if equal to "1", indicates a skipped macroblock as defined in Annex O, using a picture reference parameter of zero for the forward (skipped) prediction in an EP picture and for the forward part of direct (skipped) bidirectional prediction in a B picture and using the first backward prediction picture for the backward part of direct (skipped) bidirectional prediction in a B picture (in the case of two-picture backward prediction, as when BSBBW is present and equal to "0"). If COD is "0", a PR0 parameter is inserted into the syntax and is used in a similar manner as described in U.4.2.1.2.  If PR0 is present and does not have a decoded value of zero (codeword '1'), it indicates that the macroblock is to be predicted with forward INTER prediction using a zero-valued motion vector and a picture reference parameter of PR0.  If PR0 has a decoded value of zero, MBTYPE follows and specifies the macroblock type.  The format of the CBPC, CBPY, and DQUANT fields is unchanged.  The MVDFW and MVDBW fields are encoded in the same manner as when the ERPS mode is not in use, but are each used in conjunction with a picture reference, and possibly an emulation prevention bit.

For a B picture, the backward reference pictures in the multi-picture buffer are defined as follows:

- In the case of single-picture backward prediction, there is only one backward reference picture, which is the first picture in (possibly re-mapped) relative index order, and

- In the case of two-picture backward prediction, there are two backward reference pictures, which are the first two pictures in (possibly re-mapped) relative index order.

The forward reference pictures in the multi-picture buffer are defined as the pictures in the multi-picture buffer other than the backward reference pictures.  The relative indexing for forward prediction is a relative index into the forward reference picture set.

The TRD used for direct bidirectional prediction in a B picture shall be computed as the temporal reference increment between the first forward reference picture in (possibly re-mapped) relative index order and the first backward reference picture in (possibly re-mapped) relative index order (i.e. if two-picture backward prediction is in use, this would be the picture referenced when BSBBW is “0” as described in sub-clause U.4.2.2.3).  The TRB used for direct bidirectional prediction in a B picture shall be computed as the temporal reference increment between the B picture and the first forward reference picture in (possibly re-mapped) relative index order.  The relative index order used in the computation of TRD and TRB shall be that specified by the ERPS layer at the picture level of the B picture syntax (i.e. re-mappings at the GOB or slice level shall not affect the values of TRD and TRB).

FIGURE U.8/H.263

Structure of EP and B picture Macroblock layer for the ERPS mode

##### U.4.2.2.1	Picture Reference for Forward Prediction (PRFW) (variable length)

A picture reference parameter PRFW is present whenever forward motion vector data is present, and is encoded using Table U.1.  This is a relative index into the set of forward reference pictures.

##### U.4.2.2.2	Emulation Prevention Bit for Forward Prediction (MEPBFW) (1 bit)

A single bit having the value "1" shall be inserted after PRFW if and only if PRFW is present and has a decoded value of "1" (codeword '000') and the unrestricted motion vector mode (see Annex D) is not in use.

##### U.4.2.2.3	B-Picture Selection Bit for Backward Prediction (BSBBW) (1 bit)

BSBBW is a single bit that is present only for B pictures and only when two-picture backward prediction is specified for the B picture operation.  The meaning of this bit shall be:

"0" : Prediction from the first backward reference picture in relative index order (in default order, this would be the most recent short-term reference picture if that picture has not been assigned a long-term index or marked as unused)

"1" : Prediction from the second backward reference picture in relative index order (in default order, this would be the second-most recent short-term reference picture if neither of the last two reference pictures has been assigned a long-term index or marked as unused)

##### U.4.2.2.4	Emulation Prevention Bit for Backward Prediction (MEPBBW) (1 bit)

MEPBBW is a single bit having the value "1" that is present only under the following conditions:

- BSBBW is present and equal to "0", and

- The unrestricted motion vector mode (see Annex D) is not in use, and

- BSBBW is preceded by five bits having the value '00000'

## U.5	Decoder Process

The decoder for the ERPS mode stores the reference pictures for inter-frame decoding in a multi-picture buffer.  The decoder may need additional memory capacity to store the multiple decoded pictures (relative to the memory capacity needed without support of the ERPS mode). The decoder replicates the multi-picture buffer of the encoder according to the reference picture buffering type and any memory management control operations specified in the bitstream. The buffering scheme may also be operated when partially erroneous frames are decoded.

Each transmitted picture is assigned a Picture Number (PN) which is stored with the picture in the multi-picture buffer. PN represents a sequential picture counting identifier. PN is constrained, using modulo 1024 arithmetic operation. For the first transmitted picture, PN should be "0". For each and every other transmitted non-B picture, PN shall be increased by 1 (within a given scalability layer, if Annex O is in use).  If the difference (modulo 1024) of the PNs of two consecutively received pictures is not 1, the decoder shall assume the loss of pictures. In such a case, a back-channel message indicating the loss of pictures should be sent to the encoder.

Besides the PN, each picture stored in the multi-picture buffer has an associated index, called the default index. When a picture is first added to the multi-picture buffer it is given default index 0. The indices of pictures in the multi-picture buffer are modified when pictures are added to or removed from the multi-picture buffer.

The pictures stored in the multi-picture buffers can also be divided into two categories: long-term pictures and short-term pictures.  A long-term picture can stay in the multi-picture buffer for a long time (more than 1023 coded picture intervals). A picture is initially stored into the buffer as a short-term picture and can be changed to a long-term picture by assigning it a long-term index according to information in the bitstream. The PN is the unique ID for all short-term pictures in the multi-picture buffers. When a short-term picture is changed to a long-term picture, it is also assigned a long-term picture index (LPIN).  A long-term picture index is assigned to a picture by associating its PN to an LPIN.  Once a long-term picture index has been assigned to a picture, the only subsequent use of the long-term picture’s PN within the bitstream would be in a repetition of the long-term index assignment.  The PNs of the long-term pictures are unique within 1024 transmitted frames. Therefore, the PN of a long-term picture shall not be used for assignment of a long-term index after 1023 transmitted frames.  LPIN becomes the unique ID for the life of a long-term picture.

PN (for a short-term picture) or LPIN (for a long-term picture) can be used to re-map the pictures into re-mapped indices for efficient reference picture addressing.

U.5.1	Decoder Process for Short/Long-term Picture Management

The decoder may have both long-term pictures and short-term pictures in its multi-picture buffer. The MLIP1 field is used to indicate the maximum long-term picture index allowed in the buffer. The decoder shall initially assume that no long-term pictures are in use, i.e. MLIP1 is initially "0". Upon receiving an MLIP1 parameter, a new MLIP1 shall take effect until another value of MLIP1 is received.  Upon receiving a new MLIP1 parameter in the bitstream, the decoder shall examine the multi-picture buffer, and any long-term pictures with associated long-term indices  greater than or equal to MLIP1 shall be marked “unused”.  The frequency of transmitting MLIP1 is out of the scope of this Recommendation. However, the encoder should send an MLIP1 parameter upon receiving an error message, such as an Intra request message.

A short-term picture can be changed to a long-term picture by using an MMCO command, followed by DPN and LPIN. The short-term picture number is derived from DPN while the long-term picture index is derived from LPIN. Upon receiving such fields, the decoder shall change the short-term picture with a PN indicated by DPN to a long-term picture and shall assign the long-term index indicated by LPIN. If a long-term picture with the same long-term index is already existing, the previously-existing long-term picture shall be marked “unused”. An encoder shall not assign a long-term index greater than MLIP1-1 to any picture.  If LPIN is greater than MLIP1 – 1, this condition shall be treated as a error. For error resilience, the encoder may send the same long-term index assignment operation or MLIP1 specification message repeatedly. If the picture specified in a long-term assignment operation is already associated with the required LPIN, no action shall be taken by the decoder. An encoder shall not assign the same picture to more than one long term index value.  If the picture specified in a long-term index assignment operation is already associated with a different long-term index, this condition shall be treated as an error.  An encoder shall only change a short-term picture to a long-term picture within 1023 transmitted pictures. In other words, a short-term picture shall not stay in the short-term buffer for more than 1023 frames. An encoder shall not assign a long-term index to a short-term picture that has been marked as “unused” by the decoding process prior to the first such assignment message in the bitstream.  An encoder shall not assign a long-term index to a picture number that has not been sent.

### U.5.2	Decoder Process for Reference Picture Buffer Mapping

The decoder employs indices when referencing a picture for motion compensation on the macroblock layer using the fields PR0, PR, PR2, PR3, PR4, PRB, PRFW, and BSBBW. In pictures other than B pictures, these indices are the default relativeindices of pictures in the multi-picture buffer when the fields ADPN and LPIR are not present in the current picture, GOB, or slice layer as applicable, and are re-mapped indices when these fields are present.  In B pictures, the first one or two pictures (depending on BTPSM) in relative index order are used for backward prediction, and the forward picture reference parameters specify a relative index into the remaining pictures for use in forward prediction.

The index of pictures in the multi-picture buffer can be re-mapped onto newly specified indices by transmitting the RMPNI, ADPN, and LPIR fields. RMPNI indicates whether ADPN or LPIR is present. If ADPN is present, RMPNI specifies the sign of the difference to be added to a picture number prediction value. The ADPN values correspond to the absolute difference between the PN of the current picture and a prediction of the PN value. The first transmitted ADPN is computed as the difference between the PN of the current picture and the PN of the picture that is being re-mapped. The next transmitted ADPN field represents the difference between the PN of the previous picture that was re-mapped using ADPN and the picture that is being re-mapped. The process continues until all necessary re-mapping is complete. The presence of re-mappings specified using LPIR does not affect the prediction value for subsequent re-mappings using ADPN.  If RMPNI indicates the presence of an LPIR field, the re-mapped picture corresponds to a long-term picture with a long-term index of LPIR.   If any pictures are not re-mapped to a specific order by RMPNI, these remaining pictures shall follow after any pictures having a re-mapped order in the indexing scheme, following the default order amongst these non-re-mapped pictures.

If the decoder detects a missing picture, it may invoke some concealment process, and may insert an error-concealed picture into the multi-picture buffer. Missing pictures can be identified if one or several picture numbers are missing or if a picture not stored in the multi-picture buffer is indicated in a transmitted ADPN  or LPIR. Concealment may be conducted by copying the closest temporally preceding picture that is available in the multi-picture buffer into the position of the missing picture. The temporal order of the short-term pictures in the multi-picture buffer is given by their PN field. In addition or instead, the decoder may send a forced INTRA update signal to the encoder by external means (for example, using Recommendation H.245), or the decoder may use external means or back-channel messages (for example, using Recommendation H.245) to indicate the loss of pictures to the encoder. A concealed picture may be inserted into the multi-picture buffer when using the "Sliding Window" buffering type. If a missing picture is detected when decoding a GOB or Slice layer, the concealment may be applied to the picture as if the missing picture had been detected at the picture layer.

### U.5.3	Decoder Process for Sub-Picture Removal

Sub-picture removal may be used to reduce the amount of memory required to save multiple reference pictures. In sub-picture removal, each reference picture is partitioned into smaller equal-sized sub-pictures. The memory reduction is accomplished by marking undesired sub-pictures as “unused”. The strategy used by the encoder to decide which of the sub-pictures to mark as “unused” is outside the scope of this document. The encoder signals to the decoder the size of the sub-pictures and which of the sub-pictures to mark as “unused” using the enhanced reference picture selection (ERPS) layer.  The encoder shall not send information in the bitstream which causes any samples reference sub-pictures that it has marked as “unused” to be used in the prediction of subsequent pictures.

The sub-picture removal capability is negotiated by external means (for example, using Recommendation H.245). In addition, the decoder signals, also by external means, the minimum partition unit (MPU) which is described in terms of a minimum width and height (in units of 16 luminance samples) of a sub-picture and the total amount of memory it has available for its multi-picture buffer. Memory management is facilitated by the simple partition rules described below.

Each reference picture is partitioned into rectangular sub-pictures of equal size. The encoder specifies the sub-picture size which shall be an integer multiple of the MPU. The width and height of the sub-picture shall be integer multiples of the minimum MPU width and height negotiated externally. The upper-left-hand corner of the first sub-picture is coincident with the upper-left-hand corner of the reference picture. Consequently, the entire partition may be described by specifying the width and height of a sub-picture. Sub-pictures may extend beyond the right and bottom boundaries of the reference picture if the picture size is not an integer multiple of the sub-picture size. When a sub-picture that extends past the reference picture boundary is saved, the decoder may enough memory to save the entire sub-picture and not just the memory necessary to save the portion of the reference picture that lies within that sub-picture. A decoder designed such that each sub-picture occupies the same amount of memory will prevent the possibility of memory fragmentation.

An example method designed to access referenced samples when sub-picture removal is in use is described briefly as follows. One important element in any reference picture access technique is a mechanism to identify where the samples in each sub-picture are stored in memory. If there are R reference pictures and each picture is partitioned into S sub-pictures then there are a total of K = R·S sub-pictures. For example, the sub-picture in the upper-left hand corner of the first reference picture number can be considered sub-picture number 0, and the sub-picture to the right of it can be considered sub-picture number 1 and so on in raster scan order progressing from reference picture 1 to R until all K sub-pictures have a label. The total buffer capacity is SPTN sub-picture memory buffers, and SPTN is typically less than K.  We define a K-element array, subPicMem[K], such that t = subPicMem[k] corresponds to the sub-picture memory area that contains the samples in the kth sub-picture.  For example, a case can be considered in which R = 5 reference pictures each have S = 12 sub-pictures.  Then the samples for the 6th sub-picture in reference picture 3 would be found in sub-picture memory area  t = subPicMem[k] where k = 3·S+6 = 42.

When referencing samples in one block of luminance or chrominance data, it is necessary to acquire nm samples, where n and m may take values of 8 or 9 to accommodate half-integer motion compensation. Since the samples in one block may lie in up to four different sub-pictures, four separate cases must be considered.  In all cases, the first step is finding the location in memory that contains the upper-left hand sample (U) of the block to be referenced.  The sub-picture containing U can be identified by dividing the horizontal or vertical location of U by the sub-picture width or height.  If U lies in sub-picture k, then that sample will be located in the subPicMem[k] sub-picture memory area.  Next, if both the sample m–1 samples to the right of U (i.e. the upper-right-hand corner of the block) and the sample n–1 samples down from U (i.e. the lower-left-hand corner of the block) lie in sub-picture k, this can be considered case number one. If the sample n–1 samples down from U lies within k, but the sample m–1 samples to the right of U does not, this can be considered case two.  If the sample m–1 samples to the right of U lies within k, but the sample n–1 down does not, this can be considered case three. Otherwise, when both the sample m–1 samples to the right of U and the one n–1 samples down lie outside of sub-picture k, this can be considered case four.

In case number one, all samples in the reference block are contained within the kth sub-picture.  In this case, all relevant nm samples may be found in sub-picture memory area subPicMem[k] and it is a simple task to access them. In case two, the samples that lie in the kth sub-picture can be obtained from sub-picture memory area subPicMem[k] and the remaining samples can be obtained from sub-picture kr, the sub-picture to the right of k.  In case three, the samples that lie in the kth sub-picture can be obtained from memory area subPicMem[k], and the remaining samples can be obtained from sub-picture kd, the sub-picture below k.  In case four, the samples that lie in the kth sub-picture can be obtained from sub-picture memory area subPicMem[k] and the remaining samples lie in sub-pictures kr, kd, and krd where kr and kd are defined above and krd is the sub-picture to the right and below k.

### U.5.4	Decoder Process for Multi-Picture Motion Compensation

Multi-picture motion compensation is applied if the MRPA field indicates the use of more than one reference picture. For multi-picture motion compensation, the decoder chooses a reference picture as indicated using the fields PR0, PR, PR2, PR3, PR4, PRB, PRFW, PRBW, and BSBBW on the macroblock layer. Once, the reference picture is specified, the decoding process for motion compensation proceeds as described in subclause 6.1.

In case four motion vectors per macroblock are used and the MRPA field indicates the use of more than one reference picture, the motion vector and picture reference index for both chrominance blocks is determined  from the first of the four motion vectors.

### U.5.5	Decoder Process for Reference Picture Buffering

The buffering of the currently decoded picture can be specified using the reference picture buffering type (RPBT) for non-B pictures. The buffering may follow a first-in, first-out ("Sliding Window") mode. Alternatively, the buffering may follow a customized adaptive buffering ("Adaptive Memory Control") operation that is specified by the encoder in the forward channel.  B pictures do not affect buffer contents.

The "Sliding Window" buffering type operates as follows. First, the decoder determines whether the picture can be stored into “unused” buffer capacity. If there is no insufficient unused buffer capacity, the  short term picture with the largest default index (i.e. the oldest short-term picture in the buffer) shall be marked as unused. This process is repeated if necessary (in the case of sub-picture removal) until sufficient memory capacity is freed to hold the current decoded picture.  The current picture is stored in the buffer and assigned a default relative buffer index of zero.  The default relative index of all other short-term pictures is incremented by one.  The default relative index of all long-term pictures is incremented by one minus the number of short-term pictures removed.

In the "Adaptive Memory Control" buffering type, specified pictures or sub-picture areas may be removed from the multi-picture buffer explicitly.  The currently decoded picture may be inserted with default index 0 in the buffer or may be marked as “unused” by the encoder. The buffering operates as follows. First, the MMCO commands are processed, and then the current picture is added to the buffer if necessary.

- If MMCO indicates that a picture other than the current picture is to be marked as “unused” in the multi-picture buffer and if that picture has not already been marked as “unused”, the specified picture is marked as “unused” in the multi-picture buffer and the default relative index of all subsequent pictures in default order is decremented by one.

- If MMCO indicates that sub-picture areas of some picture are to be marked as “unused” in the multi-picture buffer, the specified sub-picture areas are marked as “unused” and the default relative index order of the pictures shall not be affected.

- If MMCO indicates a maximum long-term index using MLIP1, all long-term pictures having long-term indices greater than or equal to MLIP1 shall be marked as “unused” and the default relative index order of the remaining pictures shall not be affected.

- If MMCO indicates the assignment of a long-term index to a short-term picture and if the specified long-term index has not already been assigned to the specified short-term picture, the specified picture shall be marked in the buffer as a long-term picture with the specified long-term index.  Any existing prior picture with the same long-term index shall be marked as “unused”.  All short-term pictures that were subsequent to the specified picture in default relative index order shall have their associated default relative index decremented by one.  All long-term pictures having a long-term index less than the specified long-term index shall have their associated default relative index decremented by one.

- If MMCO indicates a reset of the buffer contents by using RESET equal to "1", all pictures in the buffer shall be marked as “unused”.

Finally, if no MMCO command has assigned a long-term index to the current picture and no MMCO command has specified marking the current picture as “unused”, the current picture shall be added to the multi-picture buffer with default relative index 0 and all other default relative indices shall be incremented by one.