Annex V
Data Partitioned Slice Mode

(This annex forms an integral part of this Recommendation.)

## V.1		Introduction

This annex describes the optional data-partitioned slice (DPS) mode of H.263. The capability of this mode is signaled by external means (for example Recommendation H.245). The use of this mode shall be indicated by setting the formerly-reserved bit 17 of the optional part of the PLUSPTYPE (OPPTYPE) to ‘1’.  This mode uses the header structure defined in Annex K.

Data partitioning provides robustness in error prone environments. This is accomplished using a rearrangement of the H.263 syntax to enable early detection of and recovery from errors that have been introduced during transmission.

## V.2		Structure of data partitioning

When data partitioning is used, the data is arranged as a video picture segment, as defined in Section R.2.  The MB’s in the segment are rearranged so that the header information for all the MB’s in the segment are transmitted together, followed by the MV’s for all the MB’s in the segment, and then by the DCT coefficients for all the MB’s in the segment.  The segment header uses the same syntax as described in Section K.2.  The header, MV, and DCT partitions are separated by markers, allowing for resynchronization at the end of the partition in which an error occurred. Each segment shall contain the data for an integer number of MB’s. When this mode is in use the syntax shown in Figure V.1 shall be used.

Annex K Header

| SSTUF | SSC | SEPB1 | SSBI | MBA | SEPB2 | SQUANT | SWI | SEPB3 | GFID | Macroblock Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| HD | HM | MVD | LMVV | MVM | Coeff Data |
| --- | --- | --- | --- | --- | --- |

FIGURE V.1/H.263

- Data Partitioning Syntax

Note that when this annex is not active, the MV and DCT data are transmitted in an interleaved fashion for all the MB’s in a video picture segment, in which case an error normally results in the loss of all information for the remaining MB’s in the packet.

### V.2.1	Header Data  (HD) (Variable length)

The Header Data field contains the COD and MCBPC information for all the MB's in the packets, plus the MODB data in case of PB-frames or Improved PB-frames.  A reversible variable length code (RVLC) is used to combine the COD and the MCBPC for all the MB’s in the packet. This code is shown in Tables V.1 through V.5/H.263. If Annex O is in use, the COD is only combined with the MB TYPE to form the RVLC for B and EP pictures using tables V.3 and V.4, and the CBPC is coded with codewords in Table O.4.  If COD=0 and Annex G or Annex M is in use, the codeword for the COD+MCBPC shall be immediately followed by the reversible variable-length encoded data corresponding to the MODB field of the macroblock. Table V.6 shall be used for PB-frames, Table V.7 shall be used for Improved PB-frames.

### V.2.2	Header Marker (HM) (9 bits)

A codeword of 9 bits. Its value is  1010 0010 1. The HM terminates the header partition. When reversed decoding is used by a decoder, the decoder searches for this marker. This value cannot occur naturally in the HD field.

### V.2.3	Motion Vector Data Layer(Variable length)

V.2.3.1	Motion Vector Difference Coding

For the motion vectors, the RVLC codewords shown in Table D.3/H.263 are used to encode the difference between the motion vector and the motion vector prediction. Note that this annex only uses the entropy coding from Annex D, but not the other aspects of it unless Annex D is also in use.

V.2.3.2	Prediction of Motion Vector Values

The first motion vector in the packet is coded using a predictor value of 0 for both horizontal and vertical components, and the MV’s for the subsequent coded MB’s are coded predictively using the MV difference (MVD).  This differs from the method otherwise used for coding the MV’s in which the MV’s following a skipped or INTRA MB are coded using a predictor value of 0 for both horizontal and vertical components.

Forward Direction: MVi = MVi-1 + MVDi=MVi-1 + (MVi- MVi-1)

Backward Direction: MVi-1 = MVi – MVDi =MVi - (MVi- MVi-1).

(MVi and MVDi are the ith MV and MV Difference in the packet respectively)

The motion vector information for the last motion vector in the packet is coded  in this manner and is also coded again in the LMVV field as described below in V.2.4. This allows the decoder to independently decode the sequence of MV’s using two different prediction paths: 1) in the forward direction, starting from the beginning of the motion data of the packet, and 2) in the backward direction, from the end of the motion data in a packet. This provides robustness for better error detection and concealment.

NOTE: When the DPS mode is not in use, motion vectors are predictively coded, with the prediction of the current motion vector being the median value of 3 motion vectors of neighboring locations as described in Section 6.1.1. Because packets in this annex are formed in a way such that the number of MB’s coded in each packet is variable, using the median predictive coding method (which involves motion vectors on different rows of the frame) would prevent reversible decoding of the motion vectors in a slice.  When the DPS mode is in use, a single prediction thread is formed for the MV’s in the whole packet.  This is shown in Figure V.2.

- FIGURE V.2/H.263

Single Thread Motion Vector Prediction

In case of B pictures or EP pictures (Annex O), MVDFW and MVDBW may be present as indicated by the MBTYPE codeword in tables V.3 and V.4.  MVDFW is predictively encoded using the same single prediction thread as described above and MVDBW (when present in B pictures) shall be encoded as specified in O.4.6. MVDFW and MVDBW shall be coded with the codewords from Table D.3/H.263.

In case of PB-frames (Annex G) and Improved PB-frames (Annex M), the MVDB data shall be encoded as specified in corresponding annexes and shall be coded using the codewords from Table D.3/H.263.

NOTE – If the backward decoding mode is engaged in a B frame (Annex O) or in Improved PB-frames (Annex M), MVDB and MVDBW should be discarded by the decoder as the Motion Vector data for the backward prediction may not be recovered properly across the packet boundaries.

V.2.3.3	Start-Code Emulation Prevention in Motion Vector Difference Coding

The MVD start-code-emulation avoidance method is changed from the method described in Section D.2 of Annex D, in order to facilitate independent parsing in the backward direction. A MVD=0 (codeword “1”) shall be inserted between any two consecutive MVD’s that are both equal to 1 (codeword “000”). This differs from Annex D, in which the bit is only inserted when two consecutive MVD=1 form a pair (i.e. when the first MVD is the horizontal component, and the second is the vertical component).   If Annex D and Annex V are both in use, this Annex V method of start-code-emulation avoidance method shall be used instead of the method described in Section D.2.

V.2.4	Last Motion Vector Value (LMVV) (Variable length)

The LMVV field contains the last MV in the packet. It is coded using a predictor value of 0 for both the horizontal and vertical components. If there are no motion vectors or only one motion vector in the packet, LMVV shall not be present. (This use of a fixed zero-valued predictor enables the use of reversible decoding.)

### V.2.5	Motion Vector Marker (MVM) (10 bits)

A codeword of 10 bits having the value ‘0000 0000 01’. The MVM terminates the motion vector partition. When reverse decoding is used in a decoder, the decoder searches for this marker. The Motion Vector Marker (MVM) shall not be included in the packet if the packet does not contain Motion Vector Data (if all the macroblocks in the packet are intra-coded or with COD's equal to 1).

### V.2.6	Coefficient Data Layer (Variable length)

The DCT data layer contains INTRA_MODE (if present), CBPB (if present), CBPC (if present), CBPY, DQUANT (if present), and DCT coefficients coded as specified in Sections I.2, 5.3.4, O.4.3, 5.3.5, 5.3.6, and 5.4.2, respectively. The syntax diagram of DCT Data is illustrated in Figure V.3.  The presence of CBPC is indicated in tables V.3 and V.4.

FIGURE V.3/H.263

Coefficient Data syntax

## V.3		Interaction with Other Optional Modes

The DPS mode acts effectively as a sub-mode of the Slice Structured mode of Annex K, and uses its outer picture and slice header structures.  The SS mode shall therefore be indicated as being in use whenever the DPS mode is in use. Both of the other sub-modes of the Slice Structured mode (the Arbitrary Slice Ordering and Rectangular Slice sub-modes) may be used in conjunction with the DPS mode.

The Syntax-Based Arithmetic Coding mode of  Annex E shall not be used with this annex, as it does not allow for reversible decoding.

Annex H Forward Error Correction should not be used with this annex, as it can result in the bitstream being disrupted in undesirable places.  However, the use of Annex H with the DPS mode is not forbidden, as the FEC defined in Annex H is required in some existing standard system designs.

The Temporal, SNR, and Spatial Scalability (TSSS) mode of Annex O may be used in conjunction with the DPS mode.  When the TSSS and DPS modes are used together, the codewords provided in Tables V.3, V.4, and V.5 shall be used instead of those defined in Annex O.

Annex U shall not be used with this Annex.

TABLE V.1/H.263

COD + MCBPC RVLC table for INTRA MB’s

| MB type | CBPC (56) | Codeword (for combined COD+MCBPC) | Number of Bits |
| --- | --- | --- | --- |
| 3 (INTRA) | 00 | 1 | 1 |
| 3 | 01 | 010 | 3 |
| 3 | 10 | 0110 | 4 |
| 3 | 11 | 01110 | 5 |
| 4 (INTRA+Q) | 00 | 00100 | 5 |
| 4 | 01 | 011110 | 6 |
| 4 | 10 | 001100 | 6 |
| 4 | 11 | 0111110 | 7 |
| stuffing |  | 0011100 | 7 |

TABLE V.2/H.263

COD + MCBPC RVLC Table for INTER MB’s

| MB type | CBPC (56) | Codeword (for combined COD+MCBPC) | Number of Bits |
| --- | --- | --- | --- |
| skipped |  | 1 | 1 |
| 0 (INTER) | 00 | 010 | 3 |
| 0 | 10 | 00100 | 5 |
| 0 | 01 | 011110 | 6 |
| 0 | 11 | 0011100 | 7 |
| 1 (INTER + Q) | 00 | 01110 | 5 |
| 1 | 10 | 00011000 | 8 |
| 1 | 01 | 011111110 | 9 |
| 1 | 11 | 01111111110 | 11 |
| 2 (INTER4V) | 00 | 0110 | 4 |
| 2 | 10 | 01111110 | 8 |
| 2 | 01 | 00111100 | 8 |
| 2 | 11 | 000010000 | 9 |
| 3 (INTRA) | 00 | 001100 | 6 |
| 3 | 11 | 0001000 | 7 |
| 3 | 10 | 001111100 | 9 |
| 3 | 01 | 000111000 | 9 |
| 4 (INTRA + Q) | 00 | 0111110 | 7 |
| 4 | 11 | 0011111100 | 10 |
| 4 | 10 | 0001111000 | 10 |
| 4 | 01 | 0000110000 | 10 |
| 5 (INTER4V + Q) | 00 | 00111111100 | 11 |
| 5 | 01 | 00011111000 | 11 |
| 5 | 10 | 00001110000 | 11 |
| 5 | 11 | 00000100000 | 11 |
| stuffing |  | 0111111110 | 10 |

TABLE V.3/H.263

MBTYPE RVLC codes for B MB’s

| Index | Prediction Type | MVDFW | MVDBW | CBPC + CBPY | DQUANT | MBTYPE | Bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| — | Direct (skipped) |  |  |  |  | 1 (COD=1) | 1 |
| 0 | Direct |  |  | X |  | 010 | 3 |
| 1 | Direct + Q |  |  | X | X | 001100 | 6 |
| 2 | Forward (no texture) | X |  |  |  | 00100 | 5 |
| 3 | Forward | X |  | X |  | 011110 | 6 |
| 4 | Forward + Q | X |  | X | X | 01111110 | 8 |
| 5 | Backward (no texture) |  | X |  |  | 0110 | 4 |
| 6 | Backward |  | X | X |  | 01110 | 5 |
| 7 | Backward + Q |  | X | X | X | 00111100 | 8 |
| 8 | Bi-Dir (no texture) | X | X |  |  | 0011100 | 7 |
| 9 | Bi-Dir | X | X | X |  | 0001000 | 7 |
| 10 | Bi-Dir + Q | X | X | X | X | 0111110 | 7 |
| 11 | INTRA |  |  | X |  | 00011000 | 8 |
| 12 | INTRA + Q |  |  | X | X | 011111110 | 9 |
| 13 | Stuffing |  |  |  |  | 001111100 | 9 |

TABLE V.4/H.263

MBTYPE RVLC Table for EP MB’s

| Index | Prediction Type | MVDFW | MVDBW | CBPC + CBPY | DQUANT | MBTYPE | Bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| — | Forward (skipped) |  |  |  |  | 1 (COD=1) | 1 |
| 0 | Forward | X |  | X |  | 010 | 3 |
| 1 | Forward + Q | X |  | X | X | 0110 | 4 |
| 2 | Upward (no texture) |  |  |  |  | 01110 | 5 |
| 3 | Upward |  |  | X |  | 00100 | 5 |
| 4 | Upward + Q |  |  | X | X | 011110 | 6 |
| 5 | Bi-Dir (no texture) |  |  |  |  | 001100 | 6 |
| 6 | Bi-Dir | X |  | X |  | 0111110 | 7 |
| 7 | Bi-Dir + Q | X |  | X | X | 0011100 | 7 |
| 8 | INTRA |  |  | X |  | 0001000 | 7 |
| 9 | INTRA + Q |  |  | X | X | 01111110 | 8 |
| 10 | Stuffing |  |  |  |  | 00111100 | 8 |

TABLE V.5/H.263

COD + MCBPC RVLC Table for EI MB’s

| Prediction type | QCBP (56) | Codeword (for combined COD+MCBPC) | Number of Bits |
| --- | --- | --- | --- |
| Upward (skipped) |  | 1 | 1 |
| 0 (Upward) | 00 | 010 | 3 |
| 0 | 01 | 0110 | 4 |
| 0 | 10 | 01110 | 5 |
| 0 | 11 | 00100 | 5 |
| 1 (Upward + Q) | 00 | 011110 | 6 |
| 1 | 01 | 001100 | 6 |
| 1 | 10 | 0111110 | 7 |
| 1 | 11 | 0011100 | 7 |
| 2 (INTRA) | 00 | 0001000 | 7 |
| 2 | 01 | 01111110 | 8 |
| 2 | 10 | 00111100 | 8 |
| 2 | 11 | 00011000 | 8 |
| 3 (INTRA + Q) | 00 | 011111110 | 9 |
| 3 | 01 | 001111100 | 9 |
| 3 | 10 | 000111000 | 9 |
| 3 | 11 | 000010000 | 9 |
| Stuffing |  | 0111111110 | 10 |

TABLE V.6/H.263

RVLC Table for MODB

| Index | CBPB | MVDB | Number of bits | Code |
| --- | --- | --- | --- | --- |
| 0 |  |  | 3 | 010 |
| 1 |  | x | 4 | 0110 |
| 2 | x | x | 5 | 01110 |

Note: “x” means that the item is present in the macroblock

TABLE V.7/H.263

RVLC Table for MODB for Improved PB-frames mode

| Index | CBPB | MVDB | Number of bits | Code | Coding Mode |
| --- | --- | --- | --- | --- | --- |
| 0 |  |  | 3 | 010 | Bi-directional prediction |
| 1 | x |  | 4 | 0110 | Bi-directional prediction |
| 2 |  | x | 5 | 01110 | Forward prediction |
| 3 | x | x | 5 | 00100 | Forward prediction |
| 4 |  |  | 6 | 011110 | Backward prediction |
| 5 | x |  | 6 | 001100 | Backward prediction |

Note — The symbol “x” in the table above indicates that the associated syntax element is present.