ITU Telecommunication Standardization Sector		Document Q15-J-11

Study Group 16

Q.11-15/16 Rapporteur Meeting

Osaka, Japan, 15-19 May 2000		Date: May 8, 2000

SOURCE*: 	Polycom

TITLE:	Demonstration of H.263++ Annex U Performance

###### PURPOSE:	Information

______________________________________________________________________________

# 1. Summary

This document contains a description of a real-time demonstration of H.263++ Annex U encoding to be presented at the ITU-T SG16 May 2000 meeting in Osaka, Japan.  The demonstration, described in Section 2, will highlight subjective quality improvements provided by Annex U by presenting video encoded with and without Annex U enabled.  In addition, objective rate-distortion results comparing encoder performance with and without Annex U is presented in Section 3.

# 2. Real-time Demonstration

This section contains a brief description of the hardware, the video codec and the Annex U implementation that will be used in the real-time demonstration of H.263++ Annex U.

## 2.1 Hardware

The video is acquired using a Sony D-30 CCD camera, encoded and decoded using a Polycom Viewstation FX (NTSC version) operating in near end loop mode.  In near end loop mode, bits from the video encoder are fed directly to the local decoder’s input buffer.  A Tadpole-RDI Ultrabook running Solaris 7 executes a TCL script that enables ‘on-the-fly’ control of certain encoder functions via a LAN connection.  The S- or composite video output is displayed on an NTSC monitor or LCD projector.

## 2.2 Video Codec

The acquired interlaced video signal is digitized and the even field from every other input video frame is resampled to CIF format. The 15 frame per second CIF formatted frames are encoded to an average of 108.8 Kbps of video bandwidth using an H.263 encoder with Annexes F, I, J and T enabled.  Annex U will be alternatively enabled and disabled.  Details regarding the implementation of Annex U are discussed in the next section.   A TCL script on the Ultrabook allows for on-the-fly activation and deactivation of the supported annexes.  If there is interest, diagnostic modes will be demonstrated.  One diagnostic shows which macroblocks are coded using extended reference frames (i.e., all reference frames excluding the most recent).

## 2.3 Annex U Implementation Details

The Annex U implementation to be presented was developed so as to be consistent with the Annex U draft proposed in Geneva to the macroblock level.  None of the Annex U specific syntax at the GOB/slice or at the picture level was implemented.  Instead, a FIFO reference memory buffer with 5 reference frames similar to the RPB sliding window mode described in the Annex U draft with NPRA equal to four.  Finally, bit 16 of the PLUSPTYPE in the picture header was used to signal Annex U.

# 3. Objective Performance

Below we present the results of two experiments in which the rate-distortion performance of a Viewstation FX encoder using H.263 with Annexes F, I, J and T (No Annex U) was compared with an encoder using H.263 with Annexes F, I, J, T and U (Annex U) where Annex U used a total of 5 reference frames.

Two sequences, Woman and MJH are presented.  Woman depicts a woman talking and gesticulating in front of a detailed background while MJH depicts the author talking in front of a non-detailed background.  The sequences were captured using a Sony D-30 camera whose S-Video output was fed into a Sony media converter where the signal was digitized and sent via firewire to a Power Mac G4.  The digitized sequences were then edited and played back through the media converter which output an S-video signal into a Viewstation FX.  The Viewstation then encoded the video sequences in real-time.  The encoding mode and quantization parameters were controlled using a TCL script. Experimental results for the average PSNR versus the average bit rate for the luminance component averaged over 30 frames are presented.  The video sequences were encoded at 10 frames per second.   Results for the sequence Woman are presented in Table 1 and Figure 1.  Results for MJH are shown in Table 2 and Figure 2.

Figure 1. Rate-Distortion Results for the Sequence Woman

# Figure 2. Rate-Distortion Results for the Sequence MJH.

Table 1. PSNR Data for the Sequence Woman.

|  | QP | Bits/Sec | Y_PSNR (dB) |
| --- | --- | --- | --- |
| Annex U | 28 | 65293 | 22.71 |
| Annex U | 19 | 73867 | 23.06 |
| Annex U | 13 | 86981 | 23.34 |
| Annex U | 10 | 111600 | 23.75 |
| Annex U | 8 | 136107 | 24.00 |
| Annex U | 7 | 175936 | 24.34 |
| No Annex U | 28 | 67869 | 22.46 |
| No Annex U | 19 | 76917 | 22.75 |
| No Annex U | 13 | 92568 | 23.06 |
| No Annex U | 10 | 114979 | 23.41 |
| No Annex U | 8 | 142389 | 23.64 |
| No Annex U | 7 | 183443 | 23.88 |

Table 2. PSNR Data for the Sequence MJH.

|  | QP | Bits/Sec | Y_PSNR (dB) |
| --- | --- | --- | --- |
| Annex U | 25 | 48352 | 24.37 |
| Annex U | 15 | 55181 | 24.84 |
| Annex U | 10 | 77741 | 25.65 |
| Annex U | 7 | 111965 | 26.57 |
| Annex U | 5 | 158237 | 27.71 |
| No Annex U | 25 | 50717 | 23.72 |
| No Annex U | 15 | 61475 | 24.09 |
| No Annex U | 10 | 83659 | 24.60 |
| No Annex U | 7 | 120611 | 25.61 |
| No Annex U | 5 | 159061 | 26.61 |

* Contact:	Michael Horowitz	Phone: +1 512 372 7091

Polycom Inc., USA	E-mail: mhorowitz@austin.polycom.com