| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Tenth meeting: Osaka, 16-18 May, 2000 | Document Q15-J-32 Filename: q15j32.doc Generated: 10 May ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Qunshan Gu PictureTel Corp. 100 Minuteman Road Andover, MA 01810 USA | Tel: Fax: Email: | 978-292-4293 978-292-3309 qgu@pictel.com |
| Title: | Modified Integer IDCT Source Code |  |  |
| Purpose: | Information |  |  |

_____________________________

# Background

This document contains the modified integer IDCT source code to be determined at the ITU-T Osaka meeting.

2. 	Source Code

/*****************************************************************************

*

*                           FIXED-POINT IDCT

*

* Fixed-point fast, separable idct

* Storage precision: 16 bits signed

* Internal calculation precision: 32 bits signed

* Input range: 12 bits signed, stored in 16 bits

* Output range: [-256, +255]

* All operations are signed

*

*****************************************************************************/

/*

* Includes

*/

#include <stdlib.h>

#include <stdio.h>

/*

* Typdefs

*/

typedef short int REGISTER;

/*

* Global constants (16 bits signed)

*/

const REGISTER cpo8   = 0x539f;     /* 16384*cos(pi/8)    */

const REGISTER spo8   = 0x4546;     /* 32768*sin(pi/8)    */

const REGISTER cpo16  = 0x7d8a;     /* 32768*cos(pi/16)   */

const REGISTER spo16  = 0x18f9;     /* 32768*sin(pi/16)   */

const REGISTER c3po16 = 0x6a6e;     /* 32768*cos(3*pi/16) */

const REGISTER s3po16 = 0x471d;     /* 32768*sin(3*pi/16) */

const REGISTER OoR2   = 0x5a82;     /* 16384*sqrt(1/2)    */

/*

* Function declarations

*/

void Transpose(REGISTER block[64]);

void HalfSwap(REGISTER block[64]);

void Swap(REGISTER block[64]);

void Scale(REGISTER block[64], signed char sh);

void Round(REGISTER block[64], signed char sh,

const REGISTER min, const REGISTER max);

REGISTER Multiply(const REGISTER a, REGISTER x, signed char sh);

void Rotate(REGISTER *x, REGISTER *y,

signed char sha, signed char shb,

const REGISTER a, const REGISTER b,

int inv);

void Butterfly(REGISTER column[8], char pass);

void IDCT(REGISTER block[64]);

/*

* Transpose():

*      Transpose a block

* Input:

*      REGISTER block[64]

* Output:

*      block

* Return value:

*      none

*/

void Transpose(REGISTER block[64])

{

int i, j;

REGISTER temp;

for (i=0; i<8; i++) {

for (j=0; j<i; j++) {

temp = block[8*i+j];

block[8*i+j] = block[8*j+i];

block[8*j+i] = temp;

}

}

return;

}

/*

* HalfSwap():

*      One-dimensional swap

* Input:

*      REGISTER block[64]

* Output:

*      block

* Return value:

*      none

*/

void HalfSwap(REGISTER block[64])

{

int i;

REGISTER temp;

for (i=0; i<8; i++) {

temp = block[8+i];

block[8+i] = block[32+i];

block[32+i] = temp;

temp = block[24+i];

block[24+i] = block[48+i];

block[48+i] = temp;

temp = block[40+i];

block[40+i] = block[56+i];

block[56+i] = temp;

}

return;

}

/*

* Swap():

*      Swap and transpose a block

* Input:

*      REGISTER block[64]

* Output:

*      block

* Return value:

*      none

*/

void Swap(REGISTER block[64])

{

HalfSwap(block);

Transpose(block);

HalfSwap(block);

}

/*

* Scale():

*      Scale a block

* Input:

*      REGISTER block[64]

*      signed char sh

* Output:

*      block

* Return value:

*      none

*/

void Scale(REGISTER block[64], signed char sh)

{

int i;

if (sh>0) {

for (i=0; i<64; i++)

block[i] >>= sh;

}

else {

for (i=0; i<64; i++)

block[i] <<= -sh;

}

}

/*

* Round():

*      Performs the final rounding of an 8x8 block

* Input:

*      REGISTER block[64]

*      signed char sh

*      const REGISTER min

*      const REGISTER max

* Output:

*      block

* Return value:

*      none

*/

void Round(REGISTER block[64], signed char sh,

const REGISTER min, const REGISTER max)

{

int i;

REGISTER round_val;

round_val = (1 << (sh-1));

for (i=0; i<64; i++) {

block[i] >>= sh;

block[i] = (block[i]<min) ? min : ((block[i]>max) ? max : block[i]);

}

return;

}

/*

* Multiply():

*      Multiply by a constant with shift

* Input:

*      const REGISTER a

*      REGISTER x

*      signed char sh

* Output:

*      none

* Return value:

*      REGISTER, the result of the multiply

*/

REGISTER Multiply(const REGISTER a, REGISTER x, signed char sh)

{

long int tmp;

REGISTER reg_out;

long int round_val;

/* multiply */

tmp = (long int)a * (long int)x;

/* shift */

if (sh > 0)

tmp >>= sh;

else

tmp <<= -sh;

round_val = 0x00008000L;

/* rounding and saturating */

if (tmp) {

if (tmp <= 0x7FFFFFFF - round_val)

tmp = tmp + round_val;

else

tmp = 0x7FFFFFFF;

}

reg_out = (REGISTER)((signed)tmp >>16);

return(reg_out);

}

/*

* Rotate():

*      Perform rotate operation on two registers

* Input:

*      REGISTER *x        pointer to the 1st register

*      REGISTER *y        pointer to the 2nd register

*      signed char sha    shift associated with factor a

*      signed char shb    shift associated with factor b

*      const REGISTER a   factor a

*      const REGISTER b   factor b

*      int inv            1 for inverse dct, 0 for forward dct

* Output:

*      *x, *y

* Return value:

*      none

*/

void Rotate(REGISTER *x, REGISTER *y,

signed char sha, signed char shb,

const REGISTER a, const REGISTER b,

int inv)

{

long int tmplxa, tmplya, tmplxb, tmplyb;

long int tmpl1, tmpl2;

long int round_val;

round_val = 0x8000L;

/*

* intermediate calculation

*/

tmplxa = (long int)(*x) * (long int)a;

if (sha > 0)

tmplxa >>= sha;

else

tmplxa <<= -sha;

tmplya = (long int)(*y) * (long int)a;

if (sha > 0)

tmplya >>= sha;

else

tmplya <<= -sha;

tmplxb = (long int)(*x) * (long int)b;

if (shb > 0)

tmplxb >>= shb;

else

tmplxb <<= -shb;

tmplyb = (long int)(*y) * (long int)b;

if (shb > 0)

tmplyb >>= shb;

else

tmplyb <<= -shb;

/*

* rounding and rotation

*/

if (inv) {

tmplxa += round_val;

tmplxb += round_val;

tmpl1 = tmplxb - tmplya;

tmpl2 = tmplxa + tmplyb;

}

else {

tmplya += round_val;

tmplyb += round_val;

tmpl1 = tmplxb + tmplya;

tmpl2 = -tmplxa + tmplyb;

}

/*

* final rounding

*/

*x = (REGISTER) ((signed)tmpl1 >>16);

*y = (REGISTER) ((signed)tmpl2 >>16);

return;

}

/*

* Butterfly():

*      Perform 1D IDCT on a column

* Input:

*      REGISTER column[8]

*      char pass

* Output:

*      column

* Return value:

*      none

*/

void Butterfly(REGISTER column[8], char pass)

{

int i;

REGISTER shadow_column[8];

/*

* For readability, we use a shadow column

* that contains the state of column at the

* preceding stage of the butterfly.

*/

/*

* Initialization

*/

for (i=0; i<8; i++)

shadow_column[i] = column[i];

/*

* First Phase

*/

Rotate(column+2, column+6, (signed char)(pass-2), (signed char)(pass-1), cpo8, spo8, 1);

Rotate(column+1, column+7, (signed char)(pass-1), (signed char)(pass-1), cpo16, spo16, 1);

Rotate(column+3, column+5, (signed char)(pass-1), (signed char)(pass-1), c3po16, s3po16, 1);

if (pass) {

int a, tmp=column[4], b=column[0];

a = tmp+b;

b = -tmp+b;

column[0] = a >> 1;                             /* Round toward minus infinity */

column[4] = b >> 1;

}

else {

column[0] = shadow_column[0] + shadow_column[4];

column[4] = shadow_column[0] - shadow_column[4];

}

for (i=0; i<8; i++)

shadow_column[i] = column[i];

/*

* Second Phase

*/

column[1] = shadow_column[1] - shadow_column[3];

column[3] = shadow_column[1] + shadow_column[3];

column[7] = shadow_column[7] - shadow_column[5];

column[5] = shadow_column[7] + shadow_column[5];

column[0] = shadow_column[0] + shadow_column[6];

column[6] = shadow_column[0] - shadow_column[6];

column[4] = shadow_column[4] + shadow_column[2];

column[2] = shadow_column[4] - shadow_column[2];

for (i=0; i<8; i++)

shadow_column[i] = column[i];

/*

* Third Phase

*/

column[7] = shadow_column[7] - shadow_column[3];

column[3] = shadow_column[7] + shadow_column[3];

column[1] = Multiply(OoR2, shadow_column[1], -2);

column[5] = Multiply(OoR2, shadow_column[5], -2);

for (i=0; i<8; i++)

shadow_column[i] = column[i];

/*

* Fourth Phase

*/

column[4] = shadow_column[4] + shadow_column[3];

column[3] = shadow_column[4] - shadow_column[3];

column[2] = shadow_column[2] + shadow_column[7];

column[7] = shadow_column[2] - shadow_column[7];

column[0] = shadow_column[0] + shadow_column[5];

column[5] = shadow_column[0] - shadow_column[5];

column[6] = shadow_column[6] + shadow_column[1];

column[1] = shadow_column[6] - shadow_column[1];

return;

}

/*

* IDCT():

*      Perform 2D IDCT on a block

* Input:

*      REGISTER block[64]

* Output:

*      block

* Return value:

*      none

*/

void IDCT(REGISTER block[64])

{

int i;

REGISTER  round_val;

round_val = (1 << 6) - (block[0] & 1);

Scale(block, -4);

if (block[0] <= 0x7FFF - round_val)

block[0] += round_val;

else

block[0] = 0x7FFF;

for (i=0; i<8; i++)

Butterfly(block+8*i, 0);

Transpose(block);

for (i=0; i<8; i++)

Butterfly(block+8*i, 1);

Round(block, 6, -256, 255);

Swap(block);

}