| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Tenth Meeting: Osaka, 16-18 May, 2000 | Document Q15-J-38 Filename: q15j38.doc Generated: 9 May. ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Rickard Sjöberg Ericsson Radio Systems AB KI/ERA/T/VV SE-164 80 Stockholm, Sweden | Tel: Fax: Email: | +46 8 757 2373 +46 8 757 5550 rickard.sjoberg@era.ericsson.se |
| Title: | Some Evaluations of H26L |  |  |
| Purpose: | Information |  |  |

_____________________________

## Summary

Three experiments on the Nokia MVC has been made. The first one compares Nokia MVC still image coding with the upcoming JPEG2000 standard. The second is a test where TML2 has been run with the first intra picture being coded by MVC. The third is a speed test of the MVC decoding software.

## Conclusions

- The MVC still image mode generates as most 15% more bits than JPEG2000, and that is for pictures of size 2048x2048. MVC seems to perform much better than JPEG2000 for small images.

- The second test gives further evidence that MVC is superior to TML2 in INTER compression.

- The third test shows that the MVC decoder software is pretty fast, Silent QCIF can be decoded at 100 fps on a 500MHz PentiumIII.

# Test descriptions

## MVC vs JPEG2000

The JPEG2000 software being used is VM7.0 with its default behavior. Only black and white images with 8 bits per pixel has been tested. The Nokia MVC codec was modified to only compress the luminance part of the pictures and to use 8 bits instead of 7 for width and height, giving the possibility to code picture sizes up to 4096x4096. These images were tested:

| Picture | Size | Description |
| --- | --- | --- |
| Woman | 2048x2560 | Portrait of woman |
| Areal2 | 2048x2048 | Areal view of buildings in desert |
| Gold | 720x576 | Old houses |
| Hotel | 720x576 | Outside picture of hotel with overlay text |
| Foreman CIF | 352x288 | <Well-known> |
| Foreman QCIF | 176x144 | <Well-known> |

JPEG2000 has a picture header of approx. 96 bytes that gives MVC an advantage at low bitrates and small picture sizes. The percentage picture header bits for JPEG2000 at 0.5 bpp encodings is given in the table below.

| Picture | Percent JPEG2000 header bits at 0.5 bpp |
| --- | --- |
| Woman | 0.03% |
| Areal2 | 0.04% |
| Gold | 0.19% |
| Hotel | 0.19% |
| Foreman CIF | 1.52% |
| Foreman QCIF | 6.06% |

For Foreman QCIF, the JPEG2000 result at 0.5 bpp would be 6 percent better if the headers could be omitted. But from the graph in the result section below it is clear that MVC is still 30% more efficient at that PSNR quality.

## Sharp TML2 with MVC INTRA image

In this test, MVC was used to code the first INTRA image in each test sequence and this image was then used in TML2 for the rest of the images in that test sequence. The number of bits used for this first INTRA image was passed to TML2 for the computation of bitrate.

This experiment gives further evidence that MVC INTER coding is superior to TML2 in the PSNR sense. Note that there is a large PSNR benefit for sequences with low motion when a high PSNR start image is used. This may already be known by the Q.15 group.

It should be noted that the tested combination of MVC and TML2 sometimes spend more bits on the first INTRA image than TML2 alone.

## MVC decoder speed

MVC anchor bitstreams with QP 10 has been generated and the decoding speed has been measured on a 500MHz Pentium III. The default decoder parameters has been used so no reconstructed sequences were written on file.

| Sequence | QP | Frames decoded | Bitrate (kbps) | Time | Decoder speed |
| --- | --- | --- | --- | --- | --- |
| Container QCIF | 10 | 100 | 15 | 0.98 s | 102 fps |
| News QCIF | 10 | 100 | 28 | 1.01 s | 99 fps |
| Foreman QCIF | 10 | 100 | 45 | 1.49 s | 67 fps |
| Silent QCIF | 10 | 150 | 35 | 1.48 s | 101 fps |
| Paris CIF | 10 | 150 | 192 | 6.32 s | 24 fps |
| Mobile CIF | 10 | 300 | 996 | 20.22 s | 15 fps |
| Tempete CIF | 10 | 260 | 679 | 15.92 s | 16 fps |

# Results

## MVC vs JPEG2000

| Areal2 |  | Woman |  | Gold |  | Hotel |  | Foreman CIF |  | Foreman QCIF |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bpp | PSNR | bpp | PSNR | bpp | PSNR | bpp | PSNR | bpp | PSNR | bpp | PSNR |
| 1.994 | 38.12 | 1.998 | 44.06 | 1.999 | 43.17 | 1.998 | 43.55 | 1.997 | 47.44 | 1.988 | 45.25 |
| 1.499 | 35.91 | 1.499 | 41.59 | 1.494 | 40.44 | 1.498 | 41.14 | 1.485 | 44.5 | 1.490 | 42.16 |
| 1.200 | 34.31 | 1.199 | 39.74 | 1.200 | 38.81 | 1.187 | 39.48 | 1.195 | 42.6 | 1.198 | 39.75 |
| 0.996 | 33.26 | 1.000 | 38.46 | 0.999 | 37.79 | 0.999 | 38.48 | 0.999 | 41.06 | 0.974 | 37.84 |
| 0.798 | 32.34 | 0.800 | 36.82 | 0.798 | 36.48 | 0.798 | 37.11 | 0.800 | 39.47 | 0.800 | 35.9 |
| 0.600 | 31.27 | 0.599 | 34.8 | 0.600 | 35.04 | 0.600 | 35.24 | 0.599 | 37.54 | 0.600 | 33.46 |
| 0.400 | 29.93 | 0.399 | 32.29 | 0.400 | 33.38 | 0.399 | 32.84 | 0.393 | 35.04 | 0.393 | 30.27 |
| 0.299 | 29.05 | 0.299 | 30.81 | 0.299 | 32.29 | 0.300 | 31.26 | 0.292 | 33.63 | 0.300 | 28.92 |
| 0.200 | 27.9 | 0.200 | 29.05 | 0.199 | 30.91 | 0.200 | 29.31 | 0.199 | 31.55 | 0.199 | 26.85 |
| 0.150 | 27.01 | 0.150 | 27.94 | 0.149 | 30.04 | 0.149 | 27.98 | 0.150 | 30.16 | 0.146 | 25.24 |
| 0.099 | 25.91 | 0.100 | 26.77 | 0.099 | 28.89 | 0.098 | 26.26 | 0.100 | 28.38 | 0.098 | 23.12 |

JPEG2000 results

|  | Areal2 |  | Woman |  | Gold |  | Hotel |  | Foreman CIF |  | Foreman QCIF |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MVC qp | bpp | PSNR | bpp | PSNR | bpp | PSNR | bpp | PSNR | bpp | PSNR | bpp | PSNR |
| 2 | 3.598 | 44.76 | 2.277 | 44.18 | 2.438 | 44.3 | 2.249 | 43.83 | 1.689 | 45.02 | 2.058 | 45.12 |
| 3 | 2.815 | 40.73 | 1.598 | 41.17 | 1.704 | 40.86 | 1.466 | 40.68 | 1.168 | 42.16 | 1.480 | 42.00 |
| 4 | 2.219 | 37.84 | 1.268 | 39.42 | 1.265 | 38.71 | 1.109 | 39.05 | 0.878 | 40.31 | 1.154 | 40.01 |
| 5 | 1.701 | 35.74 | 1.071 | 38.09 | 0.949 | 37.17 | 0.900 | 37.86 | 0.682 | 38.86 | 0.923 | 38.39 |
| 6 | 1.367 | 34.38 | 0.930 | 37.03 | 0.768 | 36.14 | 0.771 | 36.98 | 0.568 | 37.85 | 0.777 | 37.36 |
| 8 | 0.924 | 32.42 | 0.714 | 35.29 | 0.540 | 34.56 | 0.584 | 35.48 | 0.416 | 36.30 | 0.610 | 35.68 |
| 10 | 0.675 | 31.19 | 0.577 | 33.91 | 0.406 | 33.43 | 0.474 | 34.33 | 0.330 | 35.23 | 0.496 | 34.71 |
| 13 | 0.474 | 30.00 | 0.441 | 32.31 | 0.286 | 32.17 | 0.367 | 33.00 | 0.252 | 34.04 | 0.388 | 33.18 |
| 17 | 0.338 | 28.97 | 0.327 | 30.71 | 0.202 | 31.02 | 0.285 | 31.62 | 0.200 | 32.80 | 0.299 | 31.65 |
| 20 | 0.277 | 28.40 | 0.278 | 29.83 | 0.165 | 30.41 | 0.248 | 30.85 | 0.169 | 32.11 | 0.257 | 30.74 |
| 25 | 0.210 | 27.63 | 0.212 | 28.59 | 0.126 | 29.57 | 0.193 | 29.71 | 0.136 | 31.19 | 0.198 | 29.54 |
| 30 | 0.170 | 27.01 | 0.171 | 27.68 | 0.106 | 28.97 | 0.161 | 28.79 | 0.118 | 30.27 | 0.168 | 28.79 |

MVC results

## Sharp TML2 with MVC INTRA image

|  |  | Container 10fps |  | News 10fps |  | Foreman 10fps |  | Silent 15fps |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MVC QP | QP | PSNRY | Bitrate | PSNRY | Bitrate | PSNRY | Bitrate | PSNRY | Bitrate |
| 4 | 13 | 38.15 | 53.56 | 39.071 | 75.61 | 37.804 | 134.68 | 38.249 | 98.063 |
| 5 | 16 | 36.21 | 32.45 | 37.058 | 53.46 | 35.682 | 92.72 | 36.269 | 67.979 |
| 7 | 19 | 34.19 | 19.92 | 34.776 | 36.86 | 33.587 | 63.36 | 34.024 | 46.372 |
| 10 | 22 | 32.11 | 12.61 | 32.413 | 25.22 | 31.553 | 43.26 | 31.962 | 30.805 |
| 15 | 25 | 29.98 | 8.359 | 30.355 | 17.59 | 29.774 | 30.96 | 29.984 | 20.618 |
| 20 | 28 | 28.09 | 5.344 | 28.388 | 11.59 | 27.947 | 21.80 | 28.261 | 13.473 |

QCIF test sequences

|  |  | Paris 15fps |  | Mobile 30fps |  | Tempete 30fps |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MVC QP | QP | PSNRY | Bitrate | PSNRY | Bitrate | PSNRY | Bitrate |
| 4 | 13 | 37.97 | 543.28 | Missing | Missing | 37.05 | 2555.50 |
| 5 | 16 | 35.86 | 380.71 | 33.77 | 2343.98 | 34.67 | 1689.24 |
| 7 | 19 | 33.55 | 259.13 | 31.16 | 1525.53 | 32.26 | 1047.69 |
| 10 | 22 | 31.25 | 169.30 | 28.64 | 935.09 | 29.97 | 614.44 |
| 15 | 25 | 29.01 | 111.17 | 26.35 | 572.28 | 27.96 | 368.01 |
| 20 | 28 | 27.04 | 69.97 | 24.09 | 339.73 | 25.99 | 218.20 |

CIF test sequences

-end-