| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Tenth meeting: Osaka, 16-18 May, 2000 | Document Q15-J-42 Filename: q15j42.doc Generated: 4 May 2000 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Jani Lainema, Marta Karczewicz Nokia Research Center 6000 Connection Drive Irving, TX 75039, U.S.A. | Tel: Fax: Email: | +1 972 894 4045 +1 972 894 4589 jani.lainema@nokia.com |
| Title: | MVC with long-term reference frame memory |  |  |
| Purpose: | Information |  |  |

_____________________________

# Introduction

This document provides information about the implementation of long-term reference frame memory in Nokia's MVC codec. Bitstream syntax is described in document Q15-J-19, simulation results can be found in accompanying spreadsheet q15j42.xls and source code is available as part of the MVC library (document Q15-J-20).

The main aspects of the current implementation are

- Multiple reference frames can be switched on and off on frame basis.

- Reference frame information is transmitted on macroblock layer if multiple reference frames are switched on.

- Temporal order of reference frames is assumed in the frame buffer

- Motion parameter prediction is allowed only between macroblocks/blocks with same reference frame.

# Bitstream elements

There are two new elements in the picture layer syntax. NRF (Number of Reference Frames) is included to the I picture headers and signals the size of the reference frame buffer to the decoder. NRF is coded using reversible VLC (see Q15-J-19 for details). MRFI (Multiple Reference Frame Indicator) is a single bit in the P picture header indicating whether multiple reference frames are to be used for the current picture.

The actual reference frame to be used for the macroblock is signalled in the macroblock layer right after the macroblock mode and DQUANT information. Signalling is again done using the same reversible VLC that was used for indicating the size of the reference frame buffer.

# Encoding algorithm

Motion estimation and mode selection with multiple reference frames follows closely the standard MVC algorithm with one reference frame (see Figure 1 and document Q15-J-46). The main difference is that Full Motion Search and consecutive Coefficient Removal is applied to all the reference pictures. MVC codec considers neighboring blocks as candidates for motion coefficient prediction only if they have the same reference picture as the current block. Only the best prediction candidate (in MSE sense) is selected for Refinement Motion Search for each reference picture. Thus, a maximum of two Refinement Motion Searches can take place for 16x16 blocks and the first 8x8 block in the macroblock. Other 8x8 blocks may undergo as many Refinement Motion Searches as there are reference pictures in the long-term memory. As a result approximated complexity when using 5 reference frames is 2 to 3 times the complexity of the codec operating with one reference frame.

Figure 1: MVC motion estimation and mode selection for an image block

# Simulation results

Simulation results with 1 and 5 reference frames for both MVC and TML-2 case are given in q15j42.xls (attached below). It can be seen that the improvements obtained by using multiple reference frames in MVC are very close to the improvements TML-2 achieves with the same number of reference frames.