| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Tenth meeting: Osaka, 16-18 May, 2000 | Document Q15-J-45 Filename: q15j45.doc Generated: 8 May ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Ting-Yu Yang, Kyminh Liang, Chien-Min Huang and Kris Huber Sorenson Technologies, Inc. 1011 West 400 North Logan, UT 84321, USA | Tel: Fax: Email: | (435) 792-1116 (435) 792-1101 chien-min@sorensontech.com tingyu@sorensontech.com |
| Title: | Temporal Scalability in H.26L |  |  |
| Purpose: | Information |  |  |

_____________________________

# Introduction

In this document, we present the results of an investigation of B frames in H.26L.  A B frame is a frame which is predicted from the previous and next decoded I/P frames.  The main purpose of B frames is to provide the functionality of temporal scalability, since a B frame is disposable without affecting the quality of other decoded frames.  In addition, it can result in improved coding efficiency by allowing the bi-directional prediction of B-frame macroblock information.

# Implementation

## We implemented the B frame in our own software with similar features to those currently defined in H.26L.  The motion search, for both P and B frames, was performed at 1-, 1/2-, and 1/3-pixel resolution and thus the accuracy of motion vector in a macroblock was 1, 1/2, or 1/3.  Only the16x16 block size was allowed in the B frames, with five different modes supported:  direct, forward, backward, bi-directional and the intra prediction modes.

## For this experiment, rate control was implemented to achieve the target bit rates for the combination of P and B frames.  For encoding B frames, we used a fixed quantizer for the entire frame.  The fixed quantizer was determined based on the average value of quantization parameter used to encode the previous and next P frames.  However, there was an exception to this rule for the direct mode.  The quantizer used for direct-mode macroblocks was generated by scaling the quantizer used in the corresponding macroblocks of the following P frame.

# Experiment Results

## We now present the results of our experiments using most of the standard ITU test sequences in QCIF formats at different bit rates ranging from 24 kbits/s to 240 kbits/s. In this experiment, 300 frames of each sequence were encoded over the range of bit rates.  About two–thirds of each target bit rate was used for I and P frames; the remaining one-third was used for B frames.  In the figures below, “no B” means that B frames were not used, “1 B” means that one B frame was inserted between each I/P pair, and “2 B” means that two B frames were inserted between each I/P pair.