| ITU – Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eleventh Meeting: Portland, Oregon, USA, 22-25 August, 2000 | Document Q15-K-18 Filename: q15k18.doc Generated: 10 Aug ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Stockhammer Institute for Communications Eng. Munich University of Technology D-80290 Munich Germany Jürgen Pandel, Gero Bäse, Sebastian Purreiter Siemens AG, ZT IK 2 D-81730 Munich Germany | Tel: Fax: Email: Tel: Fax: Email: | +49 89 289 23474 +49 89 289 23490 stockhammer@ei.tum.de +49 89 636 53193 +49 89 636 52393 Gero.X.Baese@mchp.siemens.de |
| Title: | Data Partitioning for H.324/M – First Results |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

In this proposal we present results if H.26L is transmitted via error prone channels. We distinguish three different packetization modes, one conventional and two data partitioning modes. Additionally, we introduce prioritized transmission by applying unequal error protection to different partitions. We present objective and subjective results and show the benefits of conventional data partitioning and progressive data partitioning. We propose that data partitioning will be introduced in H.26L.

# Data Partitioning

## Conventional Mode: H26LMODE

The conventional H.26L stream is packetized frame-wise. The H.26L stream is mapped frame-wise into packets, i.e. at each PSYNC-element a new packet is started. The pseudo-mux splits the long AL-SDU in several smaller AL-SDU*. If an AL-SDU* is lost the correct earlier part of the AL-SDU is delivered to the data departitioner and all correct macroblocks are reconstructed. The missing macroblocks are copied from the previous frame. In this mode the entire frame is one partition and we refer to it as partition 1.

## Simple Data Partitioning: FRAMEMODE

In this mode several syntax elements in one frame are grouped into one partition. The priority of the partition is determined by the dependency of elements which are shown in Figure 1. The dependencies between the syntax elements show, that erroneous or missing syntax elements only effect decoding of syntax elements which are in the current branch away from the root of the dependency tree. Therefore, the impact on the decoded image quality of syntax elements closer to the root of the graph have higher importance for picture reconstruction and should therefore be assigned to higher priority classes. In this mode the following four partitions are defined:

Partition 1: Psync, Ptype, MBtype, REFframe

Partition 2: IntraPredMode, MVD

Partition 3: CBP, LUM_DC, CHR_DC

Partition 4: LUM_AC, CHR_AC

Figure 1 Dependencies of syntax elements in TML4

## Data Partitioning with Progressive Partitions: PROGFRAMEMODE

This mode is similar to the FRAMEMODE. However, several improvement which use the accessibility of partly correct packets. For the AC-transform coefficients a progressive ordering is introduced. For each macroblock  the runlength-level coded AC transform coefficients are reordered. First the first run-length-level coded transform coefficients of all blocks in one frame are transmitted followed by the second level-run-length coded coefficient of all blocks and so on. The sub-band reordering is not complete because of the characteristic of the combined level-run-length code of the coefficients as this would require additional signaling to indicate non existent subband coefficients in blocks.

With the following proposed reordering no additional signaling is necessary: The coefficients with the highest energy compaction in each block are transmitted first, followed by coefficients of less compaction. The EOB marker bits indicate the end of coefficients in the respective block. Further processing will omit these completed blocks. This method does not require any additional signaling, nor changes the coding of coefficients. This results in an efficient progressive coding which is simple to apply on the encoded video stream. For inter and 4x4 intra coding the reordering starts with the second  run-length-level coded transform coefficient in a block as the first is assigned to the DC-partition. An example for only three blocks is illustrated in Figure2.

Figure 2 Progressive reordering of runlength level coded subband coefficients

Additionally, to the FRAMEMODE, DC and AC luminance coefficients of the Intra and Inter macroblocks are in now split in separate partitions. The different partitions contain the following elements:
Partition 1: Psync, Ptype, MBtype, REFframe

Partition 2: IntraPredMode, MVD

Partition 3: CBP_Intra, LUM_DC_Intra, CHR_DC_Intra

Partition 4: LUM_AC_Intra, CHR_AC_Intra

Partition 5: CBP_Inter, LUM_DC_Inter, CHR_DC_Inter

Partition 6: LUM_AC_Inter, CHR_AC_Inter

# Test Conditions

The test conditions used in this proposal are according to document Q-15-I-60 and Q-15-K-16. The combination of error sequences and video sequences used in this document are shown in Table 1.

Table 1 Combination of error and video sequences

|  | 3-64 | 4-64 | 5-64 | 6-64 | 7-128 | 8-384 |
| --- | --- | --- | --- | --- | --- | --- |
| Foreman QCIF | x | x | x | x | x |  |
| Silent QCIF | x | x | x | x | x |  |
| Paris CIF |  |  |  |  |  | x |

Additionally, the conventional H.26L packet mode (PM), the framed partitioning mode (FM) and the progressive framed (PF) partitioning mode have been used for packetization and data partitioning. For transport the pseudo-mux software emulating H.223 Annex B and D presented in document Q-15-K-15 and Q-15-K-16 has been used. As this offers the possibility of different priorization for different partitions we will specify different error protection modes for H.223 Annex D. We’d like to mention that these protection schemes are not optimized as this is out of scope of this proposal. It just shows the potential of data partitioning and error protection. Therefore, the presented results can be viewed as lower bounds to the performance. The mapping of partitions to logical channels and the applied error protection capability for these resulting partitions are provided in Table 2. Denote that pm0, fm0, and pf0  is without any forward error correction. Pm1,  fm3 and pf3 is with equal error protection, whereas fm1 and pf1 is a weak unequal error protection and fm2 and pf2 is a strong unequal error protection.

Table 2 Mapping of partitions to logical channels and 
error protection capability of assigned to logical channel

| Partition | Logical channel | pm0 | pm1 | fm0 | fm1 | fm2 | fm3 | pf0 | pf1 | pf2 | pf3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 16 | 0 | 32 | 64 | 16 | 0 | 32 | 64 | 16 |
| 2 | 1 | - | - | 0 | 16 | 32 | 16 | 0 | 16 | 32 | 16 |
| 3 | 2 | - | - | 0 | 8 | 16 | 16 | 0 | 8 | 16 | 16 |
| 4 | 3 | - | - | 0 | 0 | 0 | 16 | 0 | 4 | 8 | 16 |
| 5 | 4 | - | - | - | - | - | - | 0 | 4 | 8 | 16 |
| 6 | 5 | - | - | - | - | - | - | 0 | 0 | 0 | 16 |

For each video sequence at least 40 runs with different starting points for the error protection file have been carried out. The PSNR values have been averaged over all runs.

# Results

The results for these experiments are listed in the Appendix. The first column ‘mode’ shows the error protection mode used according to Table 2. The second column ‘quant’ shows the quantization parameter applied throughout the entire sequence. The quantization parameter was selected such that the total bit rate including multiplexing overhead, forward error correction, audio and video did not exceed the channel bandwidth available for each error pattern. The third column ‘sequence’ shows the video sequence used. Columns 4 presents the overall bit rate including the overhead of the multiplexing protocol including forward error protection, audio is not included. Column 5 shows the total video bitrate. Obviously, this is almost equal to column 4 if no error protection is applied. It significantly differs, i.e. is lower if forward error protection is applied. Column 6-8 show the PSNR for the luminance and the two chrominance components, Column is weighted average (4:1:1) of the three component PSNRs. This will be our main criteria for comparing different protection schemes. Finally, column 10 shows the average number of images per sequence which could be decoded. An image can not be decoded if the highest priority partition is lost. The resulting numbers can be compared.

In the following we provide a graphical representation of the results. For each error pattern and each sequence the following diagrams present the gain/loss of a certain coding scheme compared to the unprotected mode. This serves to obtain a better comparison between different partitioning and priorization modes and to draw appropriate conclusions.

For all sequences and all error patterns we see that we always obtain at least one mode which outperforms the unprotected mode pm0. For some cases the equal error protection combined with the H.26L packet mode already provides good performance, especially for low error rates. For higher error rates we immediately observe the benefits of data partitioning and unequal error protection. Usually, the progressive mode gives some additional benefits compared to the conventional data partitioning mode.

Figure 3 PSNR gains in dB for different modes compared to pm0 (28.1 dB foreman, 27.0 dB silent) for error pattern 3-64 (bit error rate 5x10-4)

Figure 4 PSNR gains in dB for different modes compared to pm0 (33.6 dB foreman, 34.4 dB silent) for error pattern 4-64 (bit error rate 1.6x10-4)

Figure 5 PSNR gains in dB for different modes compared to pm0 (14.3 dB foreman, 10.8 dB silent) for error pattern 5-64 (bit error rate 1x10-3)

Figure 6 PSNR gains in dB for different modes compared to pm0 (26.3 dB foreman, 25.0 dB silent) for error pattern 6-64 (bit error rate 1.3x10-4)

Figure 7 PSNR gains in dB for different modes compared to pm0 (11.3 dB foreman, 29.7 dB paris) for error pattern 7-128 (bit error rate 5x10-4) and 8-364 (bit error rate 4.7x10-4)

# Conclusions and Proposals

From the results we have presented some conclusions can be drawn immediately.

- The unprotected mode does not provide satisfying video quality as soon as the error rate increases to more than 10-4.

- In almost all cases equal error protection results in a significant gain which does not necessarily need a data partitioning mode.

- However, for almost all scenarios data partitioning and unequal error protection outperforms the conventional H.26L mode.

- If progressive coding of partitions is supported the gain given due to prioritization and data partitioning can be amplified significantly.

- An adaptation of the error protection rates (rate allocation) is necessary to obtain good or optimized performance. This rate allocation was not considered in this contribution. Hence, the gains presented here compared to the unprotected mode can be viewed as a lower bound.

Therefore, we propose

- To adopt data partitioning as a mean of prioritizing and packetization in H.26L,

- To use progressive partitions if possible,

- To use network specific prioritization/unequal error protection as this increases the quality,

- That the methods /algorithms of prioritization/forward error correction as well as rate allocation algorithms are out of scope of H.26L,

- That H.26L should provide data packets requesting different priorities.

# Appendix

Table 3 Error Pattern 3-64

| mode | quant | sequence | video+overhead | videobr | psnr-y | psnr-u | psnr-v | psnr | img |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fm 0 | 21 | foreman | 52656.0 | 50013 | 26.078 | 37.023 | 38.241 | 29.929 | 74.333 |
| fm 1 | 23 | foreman | 52286.0 | 40094 | 27.953 | 37.442 | 38.568 | 31.304 | 74.650 |
| fm 2 | 23 | foreman | 50251.0 | 40094 | 26.411 | 36.939 | 38.312 | 30.149 | 74.850 |
| fm 3 | 24 | foreman | 52987.0 | 34753 | 28.703 | 36.726 | 38.045 | 31.597 | 74.941 |
| pf 0 | 21 | foreman | 53263.0 | 50056 | 25.106 | 37.113 | 38.406 | 29.324 | 74.120 |
| pf 1 | 23 | foreman | 55706.0 | 40154 | 27.414 | 36.281 | 38.024 | 30.660 | 74.600 |
| pf 2 | 22 | foreman | 55370.0 | 44752 | 28.505 | 37.829 | 39.107 | 31.826 | 74.720 |
| pf 3 | 24 | foreman | 54452.0 | 34804 | 28.301 | 37.248 | 38.336 | 31.465 | 74.800 |
| pm 0 | 21 | foreman | 51806.0 | 49937 | 24.405 | 35.062 | 35.966 | 28.108 | 72.917 |
| pm 1 | 22 | foreman | 54038.0 | 44630 | 27.003 | 34.315 | 35.377 | 29.617 | 73.680 |
| fm 0 | 18 | silent | 54443.0 | 51339 | 21.998 | 36.091 | 37.426 | 26.919 | 98.778 |
| fm 1 | 21 | silent | 48747.0 | 34827 | 30.334 | 36.532 | 38.157 | 32.671 | 99.800 |
| fm 2 | 20 | silent | 51936.0 | 39693 | 29.253 | 37.255 | 38.638 | 32.151 | 99.765 |
| fm 3 | 22 | silent | 52952.0 | 31012 | 31.209 | 37.116 | 38.447 | 33.400 | 99.737 |
| pf 0 | 18 | silent | 55116.0 | 51398 | 22.129 | 35.484 | 37.339 | 26.890 | 98.720 |
| pf 1 | 21 | silent | 53247.0 | 34879 | 31.310 | 37.153 | 38.488 | 33.480 | 99.600 |
| pf 2 | 20 | silent | 52239.0 | 39752 | 29.904 | 37.329 | 38.728 | 32.612 | 99.720 |
| pf 3 | 22 | silent | 54919.0 | 31072 | 29.348 | 36.558 | 38.186 | 32.023 | 99.917 |
| pm 0 | 18 | silent | 53183.0 | 51231 | 22.496 | 35.468 | 36.588 | 27.006 | 97.750 |
| pm 1 | 19 | silent | 54613.0 | 44821 | 27.635 | 33.244 | 34.127 | 29.652 | 98.680 |

Table 4 Error Pattern 4-64

| mode | quant | sequence | video+overhead | videobr | psnr-y | psnr-u | psnr-v | psnr | img |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fm 0 | 21 | foreman | 52656.0 | 50013 | 31.667 | 38.106 | 39.479 | 34.042 | 74.810 |
| fm 1 | 23 | foreman | 52286.0 | 40094 | 30.357 | 37.312 | 38.678 | 32.903 | 75.000 |
| fm 2 | 23 | foreman | 50251.0 | 40094 | 30.053 | 37.540 | 38.631 | 32.730 | 74.864 |
| fm 3 | 24 | foreman | 52987.0 | 34753 | 30.444 | 37.389 | 38.547 | 32.952 | 74.913 |
| pf 0 | 21 | foreman | 53263.0 | 50056 | 30.469 | 38.002 | 39.342 | 33.203 | 74.680 |
| pf 1 | 23 | foreman | 55706.0 | 40154 | 30.330 | 37.792 | 39.031 | 33.024 | 74.800 |
| pf 2 | 22 | foreman | 55370.0 | 44752 | 30.313 | 38.137 | 39.528 | 33.153 | 74.800 |
| pf 3 | 24 | foreman | 54452.0 | 34804 | 30.078 | 37.353 | 38.472 | 32.689 | 74.920 |
| pm 0 | 21 | foreman | 51806.0 | 49937 | 31.086 | 38.010 | 39.315 | 33.611 | 74.360 |
| pm 1 | 22 | foreman | 54038.0 | 44630 | 31.793 | 38.215 | 39.585 | 34.162 | 74.680 |
| fm 0 | 18 | silent | 54443.0 | 51339 | 23.072 | 37.091 | 38.187 | 27.928 | 99.727 |
| fm 1 | 21 | silent | 48747.0 | 34827 | 32.090 | 37.230 | 38.540 | 34.021 | 99.826 |
| fm 2 | 20 | silent | 51936.0 | 39693 | 32.867 | 37.752 | 39.008 | 34.704 | 99.810 |
| fm 3 | 22 | silent | 52952.0 | 31012 | 32.059 | 37.291 | 38.571 | 34.016 | 99.833 |
| pf 0 | 18 | silent | 55116.0 | 51398 | 22.819 | 37.010 | 38.202 | 27.748 | 99.600 |
| pf 1 | 21 | silent | 53247.0 | 34879 | 32.351 | 37.258 | 38.559 | 34.204 | 99.720 |
| pf 2 | 20 | silent | 52239.0 | 39752 | 33.143 | 37.734 | 39.012 | 34.886 | 99.800 |
| pf 3 | 22 | silent | 54919.0 | 31072 | 32.080 | 37.301 | 38.566 | 34.031 | 99.840 |
| pm 0 | 19 | silent | 46568.0 | 44821 | 32.672 | 37.081 | 38.305 | 34.345 | 99.320 |
| pm 1 | 19 | silent | 54613.0 | 44821 | 33.498 | 38.326 | 39.648 | 35.328 | 99.680 |

Table 5 Error Pattern 5-64

| mode | quant | sequence | video+overhd | videobr | psnr-y | psnr-u | psnr-v | psnr | img |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fm 0 | 21 | foreman | 52656.0 | 50013 | 12.096 | 26.890 | 29.280 | 17.426 | 73.200 |
| fm 1 | 23 | foreman | 52286.0 | 40094 | 30.126 | 37.640 | 38.807 | 32.825 | 74.864 |
| fm 2 | 23 | foreman | 50251.0 | 40094 | 24.471 | 36.801 | 38.078 | 28.794 | 75.000 |
| fm 3 | 24 | foreman | 52987.0 | 34753 | 29.416 | 37.166 | 38.148 | 32.163 | 75.000 |
| pf 0 | 21 | foreman | 53263.0 | 50056 | 13.653 | 30.714 | 32.936 | 19.710 | 72.760 |
| pf 1 | 23 | foreman | 55706.0 | 40154 | 29.068 | 37.249 | 38.610 | 32.022 | 74.800 |
| pf 2 | 22 | foreman | 55370.0 | 44752 | 23.534 | 36.604 | 38.166 | 28.151 | 74.720 |
| pf 3 | 24 | foreman | 54452.0 | 34804 | 26.408 | 37.188 | 38.257 | 30.180 | 74.960 |
| pm 0 | 21 | foreman | 51806.0 | 49937 | 11.423 | 20.163 | 19.692 | 14.258 | 60.583 |
| pm 1 | 22 | foreman | 54038.0 | 44630 | 25.122 | 34.301 | 35.240 | 28.338 | 72.880 |
| fm 0 | 18 | silent | 54443.0 | 51339 | 13.651 | 25.389 | 31.461 | 18.576 | 97.000 |
| fm 1 | 21 | silent | 48747.0 | 34827 | 31.715 | 37.108 | 38.447 | 33.736 | 99.762 |
| fm 2 | 20 | silent | 51936.0 | 39693 | 15.582 | 28.482 | 32.351 | 20.527 | 100.000 |
| fm 3 | 22 | silent | 52952.0 | 31012 | 29.028 | 35.591 | 37.801 | 31.584 | 99.875 |
| pf 0 | 18 | silent | 55116.0 | 51398 | 15.565 | 28.318 | 33.352 | 20.655 | 97.560 |
| pf 1 | 21 | silent | 53247.0 | 34879 | 31.087 | 37.091 | 38.439 | 33.313 | 99.840 |
| pf 2 | 20 | silent | 52239.0 | 39752 | 24.933 | 35.765 | 37.838 | 28.889 | 99.920 |
| pf 3 | 22 | silent | 54919.0 | 31072 | 29.977 | 36.898 | 38.304 | 32.519 | 99.880 |
| pm 0 | 18 | silent | 53183.0 | 51231 | 9.904 | 13.034 | 12.248 | 10.816 | 80.120 |
| pm 1 | 19 | silent | 54613.0 | 44821 | 29.427 | 34.463 | 35.420 | 31.265 | 98.200 |

Table 6 Error Pattern 6-64

| mode | quant | sequence | video+overhd | videobr | psnr-y | psnr-u | psnr-v | psnr | img |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fm 0 | 21 | foreman | 52656.0 | 50013 | 24.634 | 36.885 | 38.428 | 28.975 | 74.500 |
| fm 1 | 23 | foreman | 52286.0 | 40094 | 31.715 | 37.832 | 39.085 | 33.963 | 75.000 |
| fm 2 | 23 | foreman | 50251.0 | 40094 | 31.430 | 37.746 | 39.011 | 33.746 | 75.000 |
| fm 3 | 24 | foreman | 52987.0 | 34753 | 30.875 | 37.444 | 38.604 | 33.258 | 75.000 |
| pf 0 | 21 | foreman | 53263.0 | 50056 | 23.574 | 36.211 | 37.305 | 27.969 | 74.640 |
| pf 1 | 23 | foreman | 55706.0 | 40154 | 32.042 | 37.833 | 39.105 | 34.185 | 75.000 |
| pf 2 | 22 | foreman | 55370.0 | 44752 | 31.234 | 38.217 | 39.648 | 33.800 | 75.000 |
| pf 3 | 24 | foreman | 54452.0 | 34804 | 31.255 | 37.435 | 38.591 | 33.508 | 75.000 |
| pm 0 | 21 | foreman | 51806.0 | 49937 | 21.937 | 34.709 | 35.539 | 26.333 | 70.800 |
| pm 1 | 22 | foreman | 54038.0 | 44630 | 32.045 | 38.279 | 39.701 | 34.360 | 74.840 |
| fm 0 | 18 | silent | 54443.0 | 51339 | 22.427 | 36.896 | 38.054 | 27.443 | 99.727 |
| fm 1 | 21 | silent | 48747.0 | 34827 | 32.867 | 37.326 | 38.648 | 34.574 | 99.958 |
| fm 2 | 20 | silent | 51936.0 | 39693 | 32.771 | 37.699 | 38.980 | 34.627 | 99.941 |
| fm 3 | 22 | silent | 52952.0 | 31012 | 32.246 | 37.320 | 38.591 | 34.149 | 100.000 |
| pf 0 | 18 | silent | 55116.0 | 51398 | 21.026 | 35.365 | 37.325 | 26.132 | 99.640 |
| pf 1 | 21 | silent | 53247.0 | 34879 | 32.978 | 37.337 | 38.657 | 34.651 | 100.000 |
| pf 2 | 20 | silent | 52239.0 | 39752 | 33.267 | 37.775 | 39.048 | 34.982 | 100.000 |
| pf 3 | 22 | silent | 54919.0 | 31072 | 31.201 | 36.877 | 38.441 | 33.354 | 100.000 |
| pm 0 | 18 | silent | 53183.0 | 51231 | 20.662 | 33.284 | 33.984 | 24.986 | 94.960 |
| pm 1 | 19 | silent | 54613.0 | 44821 | 33.682 | 38.347 | 39.682 | 35.460 | 99.760 |

Table 7 Error Pattern 7-128

| mode | quant | sequence | video+overhd | videobr | psnr-y | psnr-u | psnr-v | psnr | img |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fm 0 | 18 | foreman | 114278.0 | 108812 | 12.292 | 25.215 | 27.268 | 16.942 | 146.500 |
| fm 1 | 20 | foreman | 108915.0 | 83507 | 29.721 | 38.250 | 39.713 | 32.808 | 149.889 |
| fm 2 | 19 | foreman | 116231.0 | 94695 | 22.556 | 37.595 | 38.788 | 27.768 | 150.000 |
| fm 3 | 21 | foreman | 110905.0 | 73401 | 29.671 | 37.606 | 38.735 | 32.504 | 149.800 |
| pf 0 | 18 | foreman | 115469.0 | 108884 | 14.559 | 32.155 | 33.474 | 20.644 | 145.167 |
| pf 1 | 20 | foreman | 113923.0 | 83587 | 30.754 | 38.025 | 39.640 | 33.447 | 149.560 |
| pf 2 | 19 | foreman | 116069.0 | 94783 | 24.862 | 37.239 | 38.628 | 29.219 | 149.880 |
| pf 3 | 21 | foreman | 112288.0 | 73478 | 27.836 | 37.404 | 38.737 | 31.247 | 149.920 |
| pm 0 | 18 | foreman | 112618.0 | 108656 | 9.399 | 15.527 | 14.780 | 11.318 | 125.136 |
| pm 1 | 19 | foreman | 114289.0 | 94545 | 26.701 | 35.314 | 36.355 | 29.746 | 147.040 |

Table 8 Error Pattern 8-384

| mode | quant | sequence | video+overhd | videobr | psnr-y | psnr-u | psnr-v | psnr | img |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fm 0 | 18 | paris | 371032.0 | 357835 | 26.185 | 34.020 | 35.589 | 29.058 | 147.880 |
| fm 1 | 20 | paris | 335237.0 | 276101 | 28.669 | 36.382 | 36.835 | 31.315 | 148.875 |
| fm 2 | 19 | paris | 351251.0 | 313952 | 28.755 | 34.961 | 36.602 | 31.097 | 149.105 |
| fm 3 | 20 | paris | 349829.0 | 276101 | 29.110 | 34.646 | 36.288 | 31.229 | 149.320 |
| pf 0 | 18 | paris | 372305.0 | 357937 | 24.716 | 32.657 | 34.303 | 27.637 | 147.500 |
| pf 1 | 20 | paris | 341193.0 | 276201 | 28.591 | 36.038 | 36.758 | 31.194 | 148.980 |
| pf 2 | 19 | paris | 350430.0 | 314059 | 29.902 | 35.671 | 37.062 | 32.057 | 149.000 |
| pf 3 | 20 | paris | 345980.0 | 276201 | 27.297 | 35.410 | 36.416 | 30.169 | 149.400 |
| pm 0 | 18 | paris | 369486.0 | 357697 | 27.060 | 34.892 | 35.037 | 29.695 | 145.561 |
| pm 1 | 19 | paris | 372944.0 | 247936 | 30.325 | 36.872 | 37.092 | 32.544 | 148.304 |