| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eleventh Meeting: Portland, Oregon, USA, 22-25 August, 2000 | Document Q15-K-21 Filename: q15k21.doc Generated: 08 Aug ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wedi Institut für Theor. Nachrichtentechnik und Informationsverarbeitung, University of Hannover Applestr. 9A D-30167 Hannover, Germany | Tel: Fax: Email: | +49 511 762 5304 +49 511 762 5333 wedi@tnt.uni-hannover.de |
| Title: | 1/8-pel motion vector resolution for H.26L |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

In document Q15j14 and Q15j15 motion compensated prediction with 1/8-pel motion vector (MV) resolution was investigated in combination with 1/4-pel, Adaptive Motion Accuracy (AMA) and different filter combinations. For some sequences and data rates a gain (up to 1.2 dB) and for some sequences a small loss was reported for 1/8-pel MV resolution compared to 1/4-pel MV resolution. Since no clear solution was presented whether to use 1/8-pel or 1/4-pel or a combination of both, this contribution should bring clarification.

In this contribution the possible gain of 1/8-pel MV resolutions compared to 1/4-pel is investigated in detail. Based on these investigations a solution is presented whether to use 1/8-pel or 1/4-pel MV resolution.

# Interpolation for motion compensated prediction

## Interpolation process

In order to interpolate an image signal on subpel positions, interpolation filters are used. The interpolation is equivalent to an upsampling of the frame. The following Figure 1 shows the interpolation process for motion vector resolution of 1/8-pel. This is the same process that was already presented in Q15i35 and Q15i14.

Figure 1: Interpolation process for 1/8-pel MV resolution

In case of 1/8-pel MV resolution, the frame has to be upsampled by a factor of 8 (8:1 resolution). This is done by applying Filter 1 that produces the 2:1 resolution, followed by Filter 2 that produces the 4:1 resolution and Filter 3 that produces the final 8:1 resolution. Since three different filters might be used for this interpolation, there are several possible filter combinations.

## Interpolation filters

Applied Interpolation filters:

In Table 1 the applied interpolation filters are shown. In this table the Filter length, the impulse response and a short description of the filters are given.

| Filter length | Impulse response | Description |
| --- | --- | --- |
| 2-tap | (16,16)/32 | Bilinear Filter |
| 6-tap | (1,-5,20,20,-5,1)/32 | Telenor Filter (COM-16 D.361) |
| 8-tap | (-1,3,-6,20,20,-6,3,-1)/32 | Wiener Filter (Q15i35) |

Table 1: Applied interpolation filters

The first filter in Table 1 is the bilinear filter (2-tap). The 6-tap filter was presented by Telenor in document COM-16 D.361. The 8-tap Wiener Filter was presented in Q15i35. For horizontal and vertical filtering the same filters are used. More information concerning interpolation filters can be found in Q15j14.

Abbreviations:

In order to specify the applied filter combinations for the interpolation process, abbreviations are used. In Figure 2 the meaning of the abbreviations is illustrated for one example. In the abbreviations the filters are represented by their filter length (e.g. bilinear: 2-tap  abbreviation: (2)).

Figure 2: Example of an abbreviation for a filter combination in the 1/8-pel interpolation scheme. In this abbreviation the filter length represents the applied filter.

# Motion Estimation

In a motion compensated prediction scheme the motion vector candidate and the block-size pattern (e.g. 16x16, four 8x8, etc.) have to be estimated for each macroblock. This is done by the following 5 steps:

- Make a full search to find the best 1/1-pel vector.

- Check the 8 1/2-pel positions around the best 1/1-pel vector in order to find the best 1/2-pel vector.

- Check the 8 1/4-pel positions around the best 1/2-pel vector in order to find the best 1/4-pel vector.

- Check the 8 1/8-pel positions around the best 1/4-pel vector in order to find the best 1/8-pel vector.

- Select the motion vector accuracy and block-size pattern, which produces the lowest rate-distortion cost.

# Experimental Results

Table 2 shows the test-sequences recommended in Q15i62. Since the test-sequence Forest is not available for the group, the test-sequence Flower Garden is used instead.

| Test-sequence | Res. | Frame rate | No.of frames |
| --- | --- | --- | --- |
| Mobile & Calendar | CIF | 30 | 300 |
| Flower Garden | CIF | 30 | 250 |
| Tempete | CIF | 30 | 260 |
| Paris | CIF | 15 | 300 |
| Silent Voice | QCIF | 15 | 300 |
| Container | QCIF | 10 | 300 |
| News | QCIF | 10 | 300 |
| Foreman | QCIF | 10 | 300 |

Table 2: Applied test-sequences.

For each sequence one rate-distortion plot is given in the Appendix. Each rate-distortion plot contains the following curves:

- TML-4: 1/4-pel with 6-tap and bilinear interpolation filter (62)

- 1/8-pel with 6-tap and bilinear interpolation filter (662)

- 1/8-pel with 8-tap and bilinear interpolation filter (882)

# Conclusion and Proposal

Conclusion for QCIF sequences:

Small loss for 1/8-pel compared to 1/4-pel MV resolutions:

| QCIF test-sequence | Loss |
| --- | --- |
| Silent Voice | 0.3 - 0.4 dB |
| Container | 0.0 - 0.5 dB |
| News | 0.0 - 0.5 dB |
| Foreman | 0.0 - 0.4 dB |

Table 3: Results of 1/8-pel MV resolution compared to
TML4 with 1/4-pel MV resolution.

Conclusion for CIF sequences:

A gain up to 1.2 dB PSNR for 1/8-pel compared to 1/4-pel MV resolutions is obtained:

| CIF test-sequence | Gain |
| --- | --- |
| Mobile & Calendar | 0.6 - 1.2 dB |
| Flower Garden | 0.2 - 0.9 dB |
| Tempete | 0.0 - 0.3 dB |
| Paris | 0.0 dB |

Table 4: Results of 1/8-pel MV resolution compared to
TML4 with 1/4-pel MV resolution.

Proposal:

- Use 1/4-pel MV resolution for QCIF sequences.

- Use 1/8-pel MV resolution for    CIF sequences.

# References

D.361:	G. Bjontegaard, “Motion Compensation with 1/4 pixel accuracy”, ITU-T SG16/Q15, Geneva, Feb. 00.

Q15i62:	G. Bjontegaard, “Recommended Simulation Conditions for H.26L”, ITU-T SG16/Q15, doc. Q15-I-62, Red Bank, Oct. 99.

Q15i35:	T. Wedi, “Motion and Aliasing Compensated prediction for H.26L”, ITU-T SG16/Q15, doc. Q15-I-35, Red Bank, Oct. 99.

Q15j15:	T. Wedi, “Results of core experiment on Adaptive Motion Accuracy (AMA) with 1/2, 1/4 and 1/8-pel accuracy”, ITU-T SG16/Q15, doc. Q15-I-15, Red Bank, Oct. 99.

Q15j14:	T. Wedi, “Interpolation filters for motion compensated prediction with 1/4 and 1/8-pel accuracy ”, ITU-T SG16/Q15, doc. Q15-I-14, Red Bank, Oct. 99.

Appendix