| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eleventh Meeting: Portland, Oregon, USA, 22-25 August, 2000 | Document Q15-K-25 Filename: q15k25.doc Generated: 16 Aug ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Mathias Wien Institut für Elektrische Nachrichtentechnik RWTH Aachen 52074 Aachen, Germany Till Halbach Department of Telecommunications NTNU 7491 Trondheim, Norway | Tel: Fax: Email: Tel: Fax: Email: | +49-241-80 7681 +49-241-8888 196 wien@ient.rwth-aachen.de +47-73 59 44 88 +47-73 59 26 40 halbach@tele.ntnu.no |
| Title: | # Results of H.26L Core Experiment on Adaptive Block Transforms |  |  |
| Purpose: | Information |  |  |

_____________________________

# Introduction

In this document, simulation results are given for the proposed Adaptive Block Transforms. A detailed description of the new coding elements that were added to the TML-4 is given in document Q15K24.

# Simulation Conditions

The test sequences HALL_MONITOR, CONTAINER, NEWS, FOREMAN, PARIS, SILENT, TEMPETE, and MOBILE were used for the simulations. The rate/distortion results for all sequences were measured with the quantization parameters

QP = [10, 15, 20, 25, 30] ;

The ABT coder employes scan blocks of size 16 and the non-optimized TML VLC for encoding of the ABT inter-coefficients. Since no deblocking filter is implemented for ABT, the deblocking filter in the TML was switched off. A R/D-quantization scheme like the one used in the TML is not yet implemented for ABT. Nevertheless, R/D-quantization was used in the TML. For single coefficient elimination, the thresholds from the TML were used in both coders. For ABT a threshold0, as described in document Q15K24, was added. The effect of this threshold is shown in Table 2 for the sequences FOREMAN, NEWS, and SILENT.

For ABT the performance of two different 8x8 transform matrices was evaluated: ABT_T17 uses the 8x8 matrix proposed in Q15K24, ABT_T13 uses the 8x8 matrix proposed in Q15I39.

The results for ABT_T17, ABT_T13, and TML are given in Table 1 and in the R/D plots are given at the end of this paper.

| Hall_monitor @ 10Hz, QCIF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ABT_T17 | Rate | 103781 | 48049 | 24919 | 12400 | 7689 |
|  | PSNR | 40,94 | 37,68 | 34,08 | 30,56 | 26,97 |
| ABT_T13 | Rate | 104942 | 48412 | 24900 | 12839 | 7889 |
|  | PSNR | 41,05 | 37,74 | 34,18 | 30,62 | 27,01 |
| TML | Rate | 121763 | 54438 | 25947 | 13237 | 7364 |
|  | PSNR | 41,75 | 38,15 | 34,46 | 30,75 | 26,96 |
| Container @ 10Hz, QCIF |  |  |  |  |  |  |
| ABT_T17 | Rate | 83591 | 34057 | 17100 | 9263 | 5986 |
|  | PSNR | 39,85 | 36,53 | 33,38 | 30,09 | 26,63 |
| ABT_T13 | Rate | 88124 | 35879 | 17536 | 9508 | 6083 |
|  | PSNR | 40 | 36,72 | 33,48 | 30,21 | 26,67 |
| TML | Rate | 90821 | 40524 | 17858 | 9250 | 5145 |
|  | PSNR | 41,02 | 37,25 | 33,78 | 30,39 | 26,71 |
| ## News @ 10Hz, QCIF |  |  |  |  |  |  |
| ABT_T17 | Rate | 110983 | 61549 | 36480 | 20409 | 12492 |
|  | PSNR | 41,01 | 37,29 | 33,77 | 30,19 | 26,67 |
| ABT_T13 | Rate | 112571 | 63114 | 37094 | 21096 | 12781 |
|  | PSNR | 41,11 | 37,47 | 33,88 | 30,3 | 26,73 |
| TML | Rate | 116535 | 66316 | 36992 | 20400 | 11065 |
|  | PSNR | 41,96 | 38,02 | 34,19 | 30,47 | 26,77 |
| ## Foreman @ 10Hz, QCIF |  |  |  |  |  |  |
| ABT_T17 | Rate | 184558 | 97699 | 55962 | 30188 | 19034 |
|  | PSNR | 39,94 | 36,45 | 33,21 | 29,96 | 26,88 |
| ABT_T13 | Rate | 188258 | 100961 | 57491 | 30896 | 19181 |
|  | PSNR | 40,09 | 36,64 | 33,4 | 30,13 | 26,93 |
| TML | Rate | 203003 | 108620 | 60017 | 34551 | 20211 |
|  | PSNR | 41,23 | 37,41 | 33,95 | 30,59 | 27,28 |
| ## Paris @ 15Hz, CIF |  |  |  |  |  |  |
| ABT_T17 | Rate | 678442 | 355693 | 194534 | 105354 | 63753 |
|  | PSNR | 39,97 | 36,22 | 32,53 | 28,86 | 25,45 |
| TML | Rate | 743207 | 396960 | 208526 | 105616 | 52716 |
|  | PSNR | 40,94 | 36,96 | 33,07 | 29,31 | 25,53 |
| Silent @ 15Hz, QCIF |  |  |  |  |  |  |
| ABT_T17 | Rate | 136461 | 73143 | 40580 | 19799 | 10676 |
|  | PSNR | 40,28 | 36,48 | 33,03 | 29,79 | 26,61 |
| ABT_T13 | Rate | 143406 | 77566 | 42309 | 20810 | 11078 |
|  | PSNR | 40,35 | 36,57 | 33,07 | 29,81 | 26,57 |
| TML | Rate | 146941 | 80417 | 43798 | 23787 | 12356 |
|  | PSNR | 40,96 | 36,99 | 33,43 | 30,16 | 26,7 |
| ## Tempete @ 30Hz, CIF |  |  |  |  |  |  |
| ABT_T17 | Rate | 3386311 | 1661351 | 776681 | 364406 | 196239 |
|  | PSNR | 39,16 | 35,35 | 31,83 | 28,55 | 25,48 |
| ABT_T13 | Rate | 3428724 | 1712227 | 801214 | 376375 | 197309 |
|  | PSNR | 39,27 | 35,55 | 32,02 | 28,74 | 25,52 |
| TML | Rate | 3707498 | 1912301 | 896281 | 417615 | 210458 |
|  | PSNR | 40,66 | 36,59 | 32,78 | 29,33 | 25,9 |
| ## Mobile @ 30HZ, CIF |  |  |  |  |  |  |
| ABT_T17 | Rate | 4185280 | 2137792 | 1036306 | 521378 | 308377 |
|  | PSNR | 38,55 | 34,48 | 30,66 | 27,11 | 23,85 |
| ABT_T13 | Rate | 4220929 | 2189712 | 1064514 | 537561 | 310697 |
|  | PSNR | 38,66 | 34,69 | 30,89 | 27,38 | 23,98 |
| TML | Rate | 4447371 | 2373236 | 1160703 | 558797 | 296932 |
|  | PSNR | 40,02 | 35,73 | 31,65 | 27,94 | 24,31 |

Table 1: Rate/Distortion results for the test sequences. The rate is given in bit/s, the PSNR is in dB.

| ## Foreman |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TRS0=Off | Rate | 188048 | 99060 | 56700 | 30316 | 19312 |
|  | PSNR | 40,07 | 36,52 | 33,27 | 29,98 | 26,93 |
| TRS0=0 | Rate | 184558 | 97699 | 55962 | 30188 | 19034 |
|  | PSNR | 39,94 | 36,45 | 33,21 | 29,96 | 26,88 |
| ## News |  |  |  |  |  |  |
| TRS0=Off | Rate | 112268 | 62292 | 36700 | 20586 | 12784 |
|  | PSNR | 41,08 | 37,36 | 33,83 | 30,22 | 26,74 |
| TRS0=0 | Rate | 110983 | 61549 | 36480 | 20409 | 12492 |
|  | PSNR | 41,01 | 37,29 | 33,77 | 30,19 | 26,67 |
| ## Silent |  |  |  |  |  |  |
| TRS0=Off | Rate | 137777 | 74027 | 40769 | 20055 | 10817 |
|  | PSNR | 40,37 | 36,53 | 33,05 | 29,82 | 26,65 |
| TRS0=0 | Rate | 136461 | 73143 | 40580 | 19799 | 10676 |
|  | PSNR | 40,28 | 36,48 | 33,03 | 29,79 | 26,61 |

Table 2: Comparison of single coefficient elimination, using no threshold0 and using threshold0=0.

# Summary

The rate/distortion performance of ABT is evaluated in this document. The results reveal equal or slightly better performance compared to the TML, especially at low bit rates. At high rates, the suboptimum coding method leads to a performance loss. Since a non-optimized VLC is used for the ABT-coefficients we expect a remarkable gain when employing an improved coding scheme.

We propose to continue the Core Experiment on Adaptive Block Transforms. Our future work will be focused on the design of the entropy coder. R/D-quantization, the application of ABT to chroma, and the design of a deblocking filter will also be future work topics.