| ## ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eleventh Meeting: Portland, Oregon, USA, 22-25 August, 2000 | Document Q15-K-26 Filename: q15k26.doc Generated: 17 Aug ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Claudia Mayer, Mathias Wien Institut für Elektrische Nachrichtentechnik RWTH Aachen 52074 Aachen, Germany | Tel: Fax: Email: | +49-241-80 7677 +49-241-8888 196 mayer@ient.rwth-aachen.de wien@ient.rwth-aachen.de |
| Title: | ## Results of H.26L Core Experiment on Fine Granularity Scalability |  |  |
| Purpose: | Information |  |  |

_____________________________

# Introduction

In this document, results of the core experiment on granular quality scalability are reported. The core experiment description is given in document Q15-J-70.

# Testing Conditions

The HHI FGS software 1.02 provided in Q15K11 was used for the simulations. For both, FGS and Simulcast, the following encoder parameters were used:

- Rate control enabled

- Hadamard transform ON

- Skip = 2

- Max ME search range = 16

- Number of  reference frames = 5

- Postfilter OFF

- Error robustness OFF

- All MB modes ON

- INTRA period = 1fps

- For FGS, the additional parameters were:

- YUV weight factors = 8:1:1

- INTRA multiplier = 8 (Parameter 26 in FGS control file)

- Adjust INTRA multiplier ON

### Test Sequences

The simulations were performed on the QCIF sequences FOREMAN, NEWS, CONTAINER, and HALL_MONITOR.

### Scenarios

- 2 Layer 36+60 kbit/s, 10 fps, INTRA-Update 1fps

- 3 Layer 24+36+48 kbit/s, 10 fps, INTRA-Update 1fps

### Simulations

The FGS coder is compared to simulcast scenarios using TML-4. The base layer of the FGS coder is the same as the first layer of the simulcast coder. Two kinds of tests were performed:

- Comparison of the performance using a channel with time-varying bandwidth. The varying bandwidth is modelled by a ramp-function. The PSNR is measured at the decoder side, depending on the currently available bandwidth. Two simulcast scenarios are used:

- 2-Layer Simulcast 36+60 kbit/s, 10 fps, INTRA-Update 1fps

- 3-Layer Simulcast 24+36+48 kbit/s, 10 fps, INTRA-Update 1fps

- The results are given in form of rate plots and distortion plots over time.

- Comparison of the performance using channels with fixed bandwidth. The results are depicted in Rate/Distortion plots at the end of this document. The used simulcast scenarios are

- 2-Layer Simulcast 36+60 kbit/s, 10 fps, INTRA-Update 1fps

- 3-Layer Simulcast 24+36+48 kbit/s, 10 fps, INTRA-Update 1fps

# Conclusion

The proposed FGS scheme shows good performance in comparison to the simulcast scenarios. Especially in the test with time-varying channel bandwidth the advantage of FGS to smoothely adapt to the available bandwidth can be observed.