| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eleventh Meeting: Portland, Oregon, USA, 22-25 August, 2000 | Document Q15-K-41 Filename: q15k41.doc Generated: 16 Aug ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Per Fröjdh and Rickard Sjöberg Ericsson Radio Systems AB Ericsson Reseach SE-164 80 Stockholm, Sweden | Tel: Fax: Email: | +46 8 404 8188 +46 8 757 5550 per.frojdh@era.ericsson.se |
| Title: | Block Orders for H.26L Intra Prediction |  |  |
| Purpose: | Proposal |  |  |

_____________________________

### 1 Introduction

Two simple ways of improving Intra prediction in H.26L include:

- extending the area from which predictions are possible,

- optimizing the ordering of 4x4 blocks within a macroblock.

The first point is already included in MVC Intra prediction and is straightforward to carry over to TML. However, the gain from using extended prediction modes is further increased by adapting the order in which the blocks are coded. With an option of two block orders for each macroblock to be coded, the additional gain is of the same order as that of extending the predictions.

The main aspects of altering the block scan orders are:

- more data available for prediction,

- the additional gain is increased as the prediction modes are improved,

- no need to code the choice of scan order, as it can be predicted,

- low complexity and easy to implement,

- can also be used for motion vector or other types of predictions.

### 2 Extended Prediction

When making Intra predictions for a 4x4 block, TML presently uses blocks above and/or to the left for all 6 modes. These blocks are marked U, L and UL in Figure 1.

U

L

C

UR

UL

DL

Figure 1. Current 4x4 block C to be coded. It may be predicted from blocks L (left), UL (up left) and U (up). Extended prediction includes blocks DL (down left) and UR (up right).

However, two of the diagonal prediction modes include pixels that should be extrapolated from UR and DL, but are instead assigned values from pixels in U and L, respectively. This is illustrated in Figure 2 for mode 1 of TML-4.

C

U

L

UR

UL

DL

Mode 1

Figure 2. Only the shaded area of C can be directly predicted from U with the diagonal prediction mode 1. Extended prediction including UR predicts the rest of C.

For extended prediction we include UR for mode 1 and similarly DL for mode 5.

### 3 Block Scan Orders

The TML block scan order for 4x4 blocks in Intra prediction is depicted in Figure 3. Each 4x4 block has a number of adjacent, previously coded blocks that may be used for prediction. For situations where prediction from blocks above is better than from those to the left, it is more advantageous to scan the blocks row by row. This way the number of blocks to predict from is maximized. Similarly, if the prediction is better from the left than from above, the best choice is to scan the blocks column by column (see Figure 3).

0

1

4

5

2

3

6

7

8

9

12

13

10

11

14

15

TML order

0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

Row by row

0

4

8

12

1

5

9

13

2

6

10

14

3

7

11

15

Column by column

Figure 3. Different block orders of 4x4 luma blocks within a macroblock.

### 4 Results

We have extended the predicton modes 1 and 5 of TML-4 as described above. Furthermore, we have implemented the two new block orders row by row and column by column. The choice of block order for each macroblock is based upon its left and top neighbors. If the left macroblock is available for prediction, the blocks are scanned column by column. On the other hand, if the top macroblock is available for prediction, the blocks are scanned row by row. If both cases apply, we use a majority decision based on the prediction modes of the neighboring 4x4 blocks.

The enclosed curves for PSNR versus bit usage are calculated for the first picture of each sequence. They compare TML-4 with extended prediction and with the combination of extended prediction and adapted block orders. The gain may seem modest: 0.25 to 0.4dB (5% bits saved) for Foreman QCIF and less for other sequences. However, this is to a large extent due to the limited number of extended prediction modes. Note that we have only included a minor extension to the existing modes of TML-4.

As Intra prediction for TML is further improved (for instance by implementing modes from MVC that includes extended prediction) the extra gain from optimizing block orders will be further increased.