| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eleventh Meeting: Portland, Oregon, USA, 22-24 August, 2000 | Document Q15-K-51 (revision 0) Filename: q15k51r0.doc Generated: 24 Aug ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Gary Sullivan Microsoft Corporation One Microsoft Way Redmond, WA 98052 USA | Tel: Fax: Email: | +1 (425) 703-5308 +1 (425) 936-7329 Gary.Sullivan@itu.int |
| Title: | Annex X Draft Reflecting Portland Meeting Result |  |  |
| Purpose: | Draft |  |  |

_____________________________

Annex X

Profiles and Levels Definition

## X.1	Introduction

With the variety of optional modes available in this Recommendation, it is crucial that several preferred mode combinations for operation be defined, so that option-enhanced terminals will have a high probability of connecting to each other using some syntax better than the "baseline." This annex contains a list of preferred feature combinations, which are structured into "profiles" of support.  It also defines some groupings of maximum performance parameters as "levels" of support for these profiles.  The primary objectives of this annex are

- to provide a simple means of describing or negotiating the capabilities of a decoder (by specifying profile and level parameters), and

- to encourage common enhancement features to be supported in decoders for achieving maximal interoperability, and

- to describe feature sets chosen as particularly appropriate for addressing certain key applications.

## X.2	Profiles of preferred mode support

The profiles of support are defined by the set of features supported in the decoder for each profile.  Decoder support for a given profile implies support for all valid subset combinations of the constituent modes of that profile. This requirement exists so that the limitations placed upon an encoder's choice of mode combinations are minimized. This is in keeping with the primary objective of this annex, which is to describe which optional modes should be supported at the decoder to address key applications, rather than to enforce a particular small set of mode combinations upon the encoder.

### X.2.1	The Baseline Profile (Profile 0)

The Baseline Profile, designated as Profile 0, is defined herein to provide a profile designation for the minimal “baseline” capability of this Recommenation (without support of any optional modes).  No optional modes of operation are indicated by support of the Baseline Profile.  Only the baseline design is included.

### X.2.2	H.320 Basic Coding Efficiency Version 2 Backward-Compatibility Profile (Profile 1)

The H.320 Basic Coding Efficiency Version 2 Backward Compatibility Profile, designated as Profile 1, is defined herein to provide compatibility with a feature set adopted into the Recommendation H.242 capability exchange mechanism for use by Recommendation H.320 circuit-switched terminal systems. It provides basic enhanced coding efficiency and simple enhanced functionality within the feature set available in the second version of this Recommendation (which did not include Annexes U, V, and W).  This profile of support is composed of the baseline design plus the following modes:

1)	Advanced INTRA Coding (Annex I) – Use of this mode improves the coding efficiency for INTRA macroblocks (whether within INTRA pictures or predictively-coded pictures). The additional computational requirements of this mode are minimal at both the encoder and decoder (as low as a maximum of 8 additions/subtractions per 8 × 8 block in the decoding process plus the use of a different but very similar VLC table in order to obtain a significant improvement in coding efficiency). For these reasons, Advanced INTRA Coding is included in this basic package of support.

2)	Deblocking Filter (Annex J) – Because of the significant subjective quality improvement that may be realized with a deblocking filter, these filters are widely in use as a method of post-processing in video communication terminals. Annex J represents the preferred mode of operation for a deblocking filter because it places the filter within the coding loop. This placement eases the implementation of the filter (by reducing the required memory) and somewhat improves the coding performance over a post-processing implementation. As with the Advanced Prediction mode, this mode also includes the four-motion-vector-per-macroblock feature and picture boundary extrapolation for motion compensation, both of which can further improve coding efficiency. The computational requirements of the deblocking filter are several hundred operations per coded macroblock, but memory accesses and computational dependencies are uncomplicated. This last point is what makes the Deblocking Filter preferable to Advanced Prediction for some implementations. Also, the benefits of Advanced Prediction are not as substantial when the Deblocking Filter is used as well. Thus, the Deblocking Filter is included in this basic package of support.

3)	Full Frame Freeze Supplemental Enhancement Information (Annex L subclause L.4) – The full-frame freeze is very simple to implement, requiring only that the decoder be able to stop the transfer of new pictures from its output buffer to the video display. This capability is useful for preventing the display of low-fidelity pictures while the encoder is building up a higher fidelity picture.

4)	Modified Quantization (Annex T) – This mode includes an extended DCT coefficient range, modified DQUANT syntax, and a modified step size for chrominance. The first two features allow for more flexibility at the encoder and may actually decrease the encoder's computational load (by eliminating the need re-encode macroblocks when coefficient level saturation occurs). The third feature noticeably improves chrominance fidelity, typically with little added bit-rate cost and with virtually no increase in computation. At the decoder, the only significant added computational burden is the ability to parse some new bitstream symbols.

### X.2.3	Version 1 Backward-Compatibility Profile (Profile 2)

The Version 1 Backward-Compatibility Profile, designated as Profile 2, is defined herein to provide enhanced coding efficiency performance within the feature set available in the first version of H.263 (which did not include Supplemental Enhancement Information or any of the optional features which use PLUSPTYPE).  This profile of support is composed of the baseline design plus the following single mode:

1)	Advanced Prediction (Annex F) – From a coding efficiency standpoint, this mode is the most important of the modes available in the first version (Version 1) of this Recommendation. It includes overlapped block motion compensation, the four-motion-vector-per-macroblock feature, and it allows for motion vectors to point outside of the picture boundaries. The use of Advanced Prediction results in significant improvements in both subjective and objective performance. It does, however, require an appreciable increase in computation and introduce complicating data dependencies in the order of processing at the decoder.  However, since implementations of this Recommendation that were designed prior to the adoption of the other modes in this list might have implemented Advanced Prediction by itself, Advanced Prediction-only operation is recommended for maximal quality with backward compatibility to version 1 decoders.

### X.2.4	Version 2 Interactive and Streaming Wireless Profile (Profile 3)

The Version 2 Interactive and Streaming Wireless Profile, designated as Profile 3, is defined herein to provide enhanced coding efficiency performance and enhanced error resilience for delivery to wireless devices within the feature set available in the second version of this Recommendation (which did not include Annexes U, V, and W).  This profile of support is composed of the baseline design plus the following modes:

1)	Advanced INTRA Coding (Annex I) – See subclause X.2.2 item 1.

2)	Deblocking Filter (Annex J) – See subclause X.2.2 item 2.

3)	Slice Structured Mode (Annex K) with Arbitrary Slice Ordering (ASO) Submode – The Slice Structured mode is included here due to its enhanced ability to provide resynchronization points within the video bitstream for recovery from erroneous or lost data. The Arbitrary Slice Ordering (ASO) submode of the Slice Structured mode is also included in order to provide compatibility for cross-network communication with packet-based network systems. Support for the Rectangular Slice (RS) submode of the Slice Structured mode is not included in this profile, in order to limit the complexity requirements of the decoder.  The additional computational burden imposed by the Slice Structured mode is minimal, limited primarily to bitstream generation and parsing.

4)	Modified Quantization (Annex T) – See subclause X.2.2 item 4.

### X.2.5	Version 3 Interactive and Streaming Wireless Profile (Profile 4)

The Version 3 Interactive and Streaming Wireless Profile, designated as Profile 4, is defined herein to provide enhanced coding efficiency performance and enhanced error resilience for delivery to wireless devices, while taking advantage of the enhanced features of the third version of this Recommendation.  This profile of support is composed of the baseline design, plus the following additional features as follows:

1)	Profile 3 – This feature set provides several enhancements useful for support of wireless video transmission.

2)	Previous Picture Header Repetition Supplemental Enhancement Information (Annex W subclause W.6.3.8) – This feature allows the decoder to receive and recover the header information from a previous picture in case of data loss or corruption.

3)	Data Partitioned Slice Mode (Annex V) with Arbitrary Slice Ordering (ASO) Submode – This feature enhances error resilience performance by separating motion vector data from DCT coefficient data within slices, and protects the motion vector information (the most important part of the detailed macroblock data) by using reversible variable-length coding.  Support of the Arbitrary Slice Ordering (ASO) submode is included so that this profile will always indicate enhanced performance relative to that available in Profile 3.

### X.2.6	Conversational High Compression Profile (Profile 5)

The Conversational High Compression Profile, designated as Profile 5, is defined herein to provide enhanced coding efficiency performance without adding the delay associated with the use of B pictures and without adding error resilience features.  This profile of support is composed of the baseline design, plus the following additional features as follows:

1)	Profile 1 – This feature set provides several enhancements useful for enhanced coding efficiency.

2)	Profile 2 – This profile adds the Advanced Prediction mode (Annex F), which provides a further enhancement of coding efficiency performance and backward-compatibility with implementations of the first version of this Recommendation.

3)	Unrestricted Motion Vectors with UUI = "1" (Annex D) – Annex D has two primary features:

a)	picture boundary extrapolation; and

b)	longer motion vector support.

The first of these features is already supported by the inclusion of Annex J in Profile 1. The longer motion vector support can provide a significant improvement in coding efficiency, especially for large picture sizes, rapid motion, camera movement, and low picture rates. When used with PLUSPTYPE present, this mode also allows for longer motion vector differences, which can significantly simplify encoder operation. The longer motion vectors do present a potential problem for the decoder in terms of memory access, but frame-size-dependent limits on the maximum motion vector size prevent this problem from becoming an appreciable obstacle to implementation.

4)	Enhanced Reference Picture Selection (Annex U) – This mode adds a significant gain in compression efficiency performance due to the ability to use multiple prior pictures as reference data for macroblock-level prediction of the subsequent pictures.  The Sub-Picture Removal submode (Annex U subclause U.4.3) of the Enhanced Reference Picture Selection mode is not included in Profile 5.

### X.2.7	Conversational Internet Profile (Profile 6)

The Conversational Internet Profile, designated as Profile 6, is defined herein to provide enhanced coding efficiency performance without adding the delay associated with the use of B pictures, but adding some error resilience suitable for use on Internet Protocol (IP) networks (which use packet-based data protocols with relatively large packets and which exhibit data losses rather than data corruption).  This profile of support is composed of the baseline design, plus the following additional features as follows:

1)	Profile 5 – This feature set provides several enhancements useful for enhanced coding efficiency.

2)	Slice Structured Mode (Annex K) with Arbitrary Slice Ordering (ASO) Submode – The Slice Structured mode is included here due to its enhanced ability to provide resynchronization points within the video bitstream for recovery from lost data packets. The Arbitrary Slice Ordering (ASO) submode of the Slice Structured mode is also included in order to allow for interleaved packetization for motion-compensated error concealment and for out-of-sequence data reception. Support for the Rectangular Slice (RS) submode of the Slice Structured mode is not included in this profile, in order to limit the complexity requirements of the decoder.  The additional computational burden imposed by the Slice Structured mode is minimal, limited primarily to bitstream generation and parsing.

### X.2.8	Conversational Plus Interlace Profile (Profile 7)

The Conversational Plus Interlace Profile, designated as Profile 7, is defined herein to provide enhanced coding efficiency performance for low-delay applications with support of interlaced video sources.  This profile of support is composed of the baseline design, plus the following additional features as follows:

1)	Profile 5 – This feature set provides several enhancements useful for enhancing coding efficiency without adding delay.

2)	Interlaced Field Indications (Annex W subclause W.6.3.11) – This feature allows video to be sent in an interlaced source picture format for compatibility with existing camera designs.

### X.2.9	High Latency Profile (Profile 8)

The High Latency Profile, designated as Profile 8, is defined herein to provide enhanced coding efficiency performance for applications without critical delay constraints.  This profile of support is composed of the baseline design, plus the following additional features as follows:

1)	Profile 6 – This feature set provides several enhancements useful for enhanced coding efficiency and robustness to data losses.

2)	B Pictures (Temporal Scalability, Annex O subclause O.1.1) – This feature consists of B pictures, which are pictures allowing bidirectional temporal prediction.  The addition of B pictures enhances coding efficiency performance, but at some cost in added processing power and encoding and decoding delay.  The two-picture backward prediction sub-mode for B pictures in Enhanced Reference Picture Selection mode (Annex U subclause U.3.1.5.5) is not supported in Profile 8.

## X.3	Picture formats and picture clock frequencies

To ensure a high quality level of interoperability, encoders and decoders supporting a large standard picture format (QCIF, CIF, 4CIF, 16CIF) should support all smaller standard picture formats.  This is a requirement of all decoders conforming to the profiles and levels defined in this annex. (As specified elsewhere in this Recommendation, decoders shall support sub-QCIF and QCIF, and encoders shall support sub-QCIF or QCIF.)  For example, a decoder conforming to a profile and level defined in this annex which is capable of decoding 4CIF pictures shall also support the decoding of CIF pictures.

Decoders should be capable of operation with a smaller picture format at maximum frame rates no lower than the maximum frame rate for which it is capable of operation with a larger standard picture format. This is a requirement of all decoders conforming to the profiles and levels defined in this annex. For example, a decoder conforming to a profile and level in defined in this annex which is capable of decoding 4CIF pictures at 25 pictures per second shall also be able to decode CIF, QCIF and SQCIF pictures at least at 25 pictures per second.

Encoders and decoders supporting custom picture formats and/or custom picture clock frequencies are recommended to follow the guidelines below:

1)	An encoder or decoder that supports a custom picture format should support all standard picture formats equal or smaller in both height and width than those of any supported custom picture format. For example, an encoder supporting a custom picture format of 256 × 256 should support QCIF and SQCIF picture encoding.

2)	An encoder or decoder that supports a custom picture clock frequency for a standard picture format should support a higher or equal frame rate (in pictures per second) at the standard picture clock frequency of 30 Hz [actually 30*(1000/1001) Hz] for the same picture format. For example, if a decoder supports QCIF pictures at 12.5 pictures per second with a 25 Hz picture clock, it should at least be able to decode QCIF pictures at 15 pictures per second at 30 Hz. If a custom picture format is used, all supported smaller or equal standard picture formats should be capable of operating at frame rates for the standard picture clock frequency no lower than the maximum frame rate of the custom picture format.

## X.4	Levels of Performance Capability

The following three levels of performance capability are encouraged for decoder implementation:

1)	Level 0 – Support of sub-QCIF and QCIF resolution decoding, capable of operation with a bit rate up to 64 000 bits per second and with a picture decoding rate up to 15 000 / 1001 pictures per second.  The Hypothetical Reference Decoder is capable of operation with parameter BPPmaxKb up to 64 and with parameter B up to 72·128 bits, where and 2 for Profile 4. For Annex U operation in Profile 4, the decoder shall be capable of operation using up to five reference pictures of buffer memory (i.e., SPTN equal to 5).

2)	Level 1 – Support of sub-QCIF, QCIF, and CIF resolution decoding, capable of operation with a bit rate up to 2·(64 000) bits per second, with a picture decoding rate up to 15 000 / 1001 pictures per second.   The Hypothetical Reference Decoder is capable of operation with parameter BPPmaxKb up to 256·XHRD and with parameter B up to 2· 72·128·XHRD bits, where XHRD is defined as 1 for Profiles 0 through 3 and 2 for Profile 4. For Annex U operation in Profile 4, the decoder shall be capable of operation using up to five reference pictures of buffer memory (i.e., SPTN equal to 5).

3)	Level 2 – Support of sub-QCIF, QCIF, and CIF resolution decoding, capable of operation with a bit rate up to 30·(64 000) bits per second, with a picture decoding rate up to 30 000 / 1001 pictures per second.   The Hypothetical Reference Decoder is capable of operation with parameter BPPmaxKb up to 256·XHRD and with parameter B up to 30·72·128·XHRD bits, where XHRD is defined as 1 for Profiles 0 through 3 and 2 for Profile 4. For Annex U operation in Profile 4, the decoder shall be capable of operation using up to five reference pictures of buffer memory (i.e., SPTN equal to 5).

The profiles and levels are summarized in Table X.1 and Table X.2.  The minimum picture interval as specified in Table X.2 is the minimum difference in time between any pair of consecutive pictures in the bitstream.  Support of any level implies support of all lower levels.

Table X.1/H.263 – Summary of Profiles

| #### Annex/Subpart Below for Profile listed at Right | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C; Continuous Presence Multipoint and Video Mux |  |  |  |  |  |  |  |  |  |
| D.1; Motion vectors over picture boundaries |  | X | X | X | X | X | X | X | X |
| D.2 with UUI='1' or UUI not present; Extension of the motion vector range |  |  |  |  |  | X | X | X | X |
| D.2 with UUI='01'; Unlimited extension of the motion vector range |  |  |  |  |  |  |  |  |  |
| E; Syntax-based Arithmetic Coding |  |  |  |  |  |  |  |  |  |
| F.2; Four motion vectors per macroblock |  | X | X | X | X | X | X | X | X |
| F.3; Overlapped block motion compensation |  |  | X |  |  |  |  |  |  |
| G; PB-Frames |  |  |  |  |  |  |  |  |  |
| H; Forward Error Correction (use may be imposed at system level as in H.320) |  |  |  |  |  |  |  |  |  |
| I; Advanced Intra Coding |  | X |  | X | X | X | X | X | X |
| J; Deblocking Filter |  | X |  | X | X | X | X | X | X |
| K without submodes; Slice Structured Coding – without submodes |  |  |  | X | X |  | X |  | X |
| K with ASO; Slice Structured Coding – with Arbitrary Slice Ordering submode |  |  |  | X | X |  | X |  | X |
| K with RS; Slice Structured Coding – with Rectangular Slice submode |  |  |  |  |  |  |  |  |  |
| L.4; Supplemental Enhancement Full picture freeze |  | X |  |  |  | X | X | X | X |
| L; Supplemental Enhancement – Other SEI features |  |  |  |  |  |  |  |  |  |
| M; Improved PB-Frames |  |  |  |  |  |  |  |  |  |
| N; Reference Picture Selection (and sub-modes) |  |  |  |  |  |  |  |  |  |
| O.1.1 Temporal (B Pictures); Temporal, SNR, and Spatial Scalability – B pictures for Temporal Scalability |  |  |  |  |  |  |  |  | X |
| O SNR and Spatial; Temporal, SNR, and Spatial Scalability – EI and EP pictures for SNR and Spatial Scalability |  |  |  |  |  |  |  |  |  |
| P; Reference Picture Resampling (and subset features such as implicit factor of four resampling) |  |  |  |  |  |  |  |  |  |
| Q; Reduced Resolution Update |  |  |  |  |  |  |  |  |  |
| R; Independent Segment Decoding |  |  |  |  |  |  |  |  |  |
| S; Alternative Inter VLC |  |  |  |  |  |  |  |  |  |
| T; Modified Quantization |  | X |  | X | X | X | X | X | X |
| U without submodes; Enhanced Reference Picture Selection – Without submodes |  |  |  |  |  | X | X | X | X |
| U with SPR; Enhanced Reference Picture Selection – With Sub-Picture Removal submode |  |  |  |  |  |  |  |  |  |
| U with BTPSM; Enhanced Reference Picture Selection – With B-Picture Two-Picture submode |  |  |  |  |  |  |  |  |  |
| V; Data Partitioned Slices |  |  |  |  | X |  |  |  |  |
| W.6.3.8; Additional SEI Specification – Prior Picture Header Repetition |  |  |  |  | X |  |  |  |  |
| W.6.3.11; Additional SEI Specification – Interlaced Field Indications |  |  |  |  |  |  |  | X |  |
| W; Additional SEI Specification – Other SEI features |  |  |  |  |  |  |  |  |  |

Table X.2/H.263 – Levels of Operation

| Parameter Below for Level listed to Right | #### 10 | #### 20 | 30 | 40 | 50 | 60 | 70 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| #### Max picture format | QCIF (176144) | CIF (352288) | CIF (352288) | CIF (352288) | CIF (352288) support of CPFMT | CPFMT: 720288 support of CPFMT | CPFMT: 720576 support of CPFMT |
| Min picture interval | 2002/ (30 000) sec | 2002/ (30 000) sec for CIF 1001/ (30 000) sec for QCIF | 1001/ (30 000) sec | 1001/ (30 000) sec | 1/50 sec at CIF, 1001/ (60 000) sec at 352240 or lower support of CPCFC | 1/50 sec at 720288, 1001/ (60 000) sec at 720240 or lower support of CPCFC | 1/50 sec at 720576, 1001/ (60 000) sec at 720480 or lower support of CPCFC |
| Max bit rate in 64 000 bits/s units | 1 | 2 | 6 | 32 | 64 | 64 | 256 |
| Max ERPS reference pictures (Annex U) | 5: Prof. 5-7 10: Prof. 8 | 5: Prof. 5-7 10: Prof. 8 | 5: Prof. 5-7 10: Prof. 8 | 5: Prof. 5-7 10: Prof. 8 | 5: Prof. 5-7 10: Prof. 8 | 5: Prof. 5-7 10: Prof. 8 | 5: Prof. 5-7 10: Prof. 8 |
| Max BPPmaxKb in 1 024 bit units | 128: Prof. 5-8 | 512: Prof. 5-8 | 512: Prof. 5-8 | 512: Prof. 5-8 | 512: Prof. 5-8 | 512: Prof. 5-8 | 512: Prof. 5-8 |
| Max HRD B in 16 384 bit units | 1: Prof. 5-8 | 2: Prof. 5-8 | 6: Prof. 5-8 | 32: Prof. 5-8 | 64: Prof. 5-8 | 64: Prof. 5-8 | 256: Prof. 5-8 |

NOTES:

- In profiles for which a maximum number of reference picture buffers is not specified in Table X.2, no support for multiple reference picture buffering is required.

- In profiles for which a maximum BPPmaxKb and HRD B are not specified Table X.2, the minimum value specified in Table 1 applies for the specified maximum bit rate and resolution.