| ### ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Eleventh Meeting: Portland, Oregon, USA, 22-25 August, 2000 | Document Q15-K-57 Filename: q15k57.doc Generated: 29 Aug ’00 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Mathias Wien Institut für Elektrische Nachrichtentechnik RWTH Aachen 52074 Aachen, Germany | Tel: Fax: Email: | +49-241-80 7681 +49-241-8888 196 wien@ient.rwth-aachen.de |
| Title: | ## Description of H.26L Core Experiment on Adaptive Block Transforms |  |  |
| Purpose: | Information |  |  |

_____________________________

Partipicants: 	Mathias Wien ( wien@ient.rwth-aachen.de )

Institut für Elektrische Nachrichtentechnik

RWTH Aachen, Germany

Louis Kerofsky, Shijun Sun ( lkerofsky@sharplabs.com, ssun@sharplabs.com )

Sharp Labs of America

Camas, WA, USA

#### 1. Background

In document Q15J41, a theoretical performance gain of up to 1.5 dB for Adaptive Block Transforms (ABT) was shown. In document Q15K24, integer transform matrices of size 8x8 and 16x16 and the corresponding quantization tables were defined. These are given below. The purpose of this core experiment is to find an entropy coding method that matches the statistics of the ABT coefficients.

1.1  Adaptive Block Transforms

In the TML seven tilings of a macroblock with a corresponding set of motion vectors are used for motion compensation. Each of the occuring subblocks shall be coded with a transform of the size corresponding to the block size used for motion compensation. For the transformation of the 16x16, 16x8, 8x16, 8x8, 8x4, 4x8, and 4x4 blocks, horizontal and vertical transform matrices of size 4x4, 8x8, and 16x16 are needed.

1	2	3	4	5	6	7

Figure 1: Macroblock modes in TML-4

The blocks are transformed by

Cnxm = Tm x Bnxm  x  TnT,	(1)

where Bnxm denotes a block with n pixels m rows. Tn and Tm are the horizontal and vertical transform matrices  of size nxn and mxm, respectively. Cnxm denotes the transformed nxm block. The transform matrices are not normalized. Normalization is performed during the quantization and dequantization procedures.

##### 1.2  Motion Estimation

The variable block sizes and transforms of ABT have to be considered during motion estimation. Therefore, the SATD is calculated for blocks of the size given by the macroblock mode. Hadamard transforms of size 4x4, 8x8, and 16x16 are employed. Besides the SATD, the motion estimation is unchanged from the TML.

1.3  Integer Transform Matrices

The transform matrices proposed here are integer approximations of the DCT of size 8x8 and 16x16. In the following the integer transforms are denoted by ICT for Integer Cosine Transform.

The zero basis functions of the 8x8 and 16x16 ICT matrices are equal to 17. The ICT matrices are:

T8 = [

17  17  17  17  17  17  17  17

24  20  12   6  -6 -12 –20 -24

23   7  -7 -23 -23  -7   7  23

20  -6 -24 –12  12  24   6 -20

17 -17 –17  17  17 -17 –17  17

12 -24   6  20 -20  -6  24 -12

7 –23  23  -7  -7  23 -23   7

6 -12  20 –24  24 -20  12  -6 ];

T16 = [

17  17  17  17  17  17  17  17  17  17  17  17  17  17  17  17

25  21  25  21   9  -3   9  -3   3  -9   3  -9 -21 -25 -21 -25

29  13  11  -5   5 -11 -13 -29 -29 -13 -11   5  -5  11  13  29

9  -3   9  -3 -25 -21 -25 -21  21  25  21  25   3  -9   3  -9

23   7  -7 -23 -23  -7   7  23  23   7  -7 -23 -23  -7   7  23

25  21 -25 -21  -9   3   9  -3   3  -9  -3   9  21  25 -21 -25

11  -5 -29 -13  13  29   5 -11 -11   5  29  13 -13 -29  -5  11

9  -3  -9   3  25  21 -25 -21  21  25 -21 -25  -3   9   3  -9

17 -17 -17  17  17 -17 -17  17  17 -17 -17  17  17 -17 -17  17

21 -25 -21  25  -3  -9   3   9  -9  -3   9   3 -25  21  25 -21

13 -29   5  11 -11  -5  29 -13 -13  29  -5 -11  11   5 -29  13

-3  -9   3   9 -21  25  21 -25  25 -21 -25  21  -9  -3   9   3

7 -23  23  -7  -7  23 -23   7   7 -23  23  -7  -7  23 -23   7

21 -25  21 -25   3   9   3   9  -9  -3  -9  -3  25 -21  25 -21

-5 -11  13 -29  29 -13  11   5   5  11 -13  29 -29  13 -11  -5

-3  -9  -3  -9  21 -25  21 -25  25 -21  25 -21   9   3   9   3 ];

The 4x4 ICT matrix is the transform proposed in the TML. The norms of all possible nxm-transforms are listed below. The different norms are considered during quantization/normalization.

| Block Size | Norm N²nxm |
| --- | --- |
| 16x16 | 16 * 17^2 |
| 16x8 | 8 * sqrt(2) * 17^2 |
| 8x16 | 8 * sqrt(2) * 17^2 |
| 8x8 | 8 * 17^2 |
| 8x4 | 4 * sqrt(2) * 13 * 17 |
| 4x8 | 4 * sqrt(2) * 13 * 17 |
| 4x4 | 4 * 13^2 |

1.4  Quantization Tables

The quantization is performed according to the TML. Here, five quantization tables are needed, since the non-square blocks 16x8/8x16 and 8x4/4x8 can use the same tables, respectively. The number of quantization tables could be further reduced if a higher arithmetic precision is used and/or if a 4x4 transform of norm 4 * 17^2 would be applied.

The quantization tables for the new block sizes are listed below. They are designed to have the same normalized quantization step size as the TML quantizers. Therefore, the quantization table for 4x4 blocks can be taken from the TML. The values in the quantization tables follow the condition

Anxm * Bnxm * N²nxm = 2^40.

QUANTIZATION TABLES

| ### QP | 16x16 |  | 16x8 = 8x16 |  | 8x8 |  | 8x4 = 4x8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | A | B | A | B | A | B |
| 0 | 91 | 565 | 128 | 803 | 181 | 1136 | 335 | 2100 |
| 1 | 81 | 635 | 114 | 902 | 162 | 1270 | 299 | 2353 |
| 2 | 72 | 714 | 102 | 1008 | 144 | 1428 | 266 | 2645 |
| 3 | 64 | 803 | 91 | 1130 | 128 | 1607 | 237 | 2968 |
| 4 | 57 | 902 | 81 | 1270 | 114 | 1804 | 211 | 3334 |
| 5 | 51 | 1008 | 72 | 1428 | 102 | 2017 | 188 | 3742 |
| 6 | 45 | 1143 | 64 | 1607 | 91 | 2260 | 168 | 4188 |
| 7 | 40 | 1286 | 57 | 1804 | 81 | 2539 | 149 | 4721 |
| 8 | 36 | 1428 | 51 | 2017 | 72 | 2857 | 133 | 5289 |
| 9 | 32 | 1607 | 45 | 2286 | 64 | 3214 | 118 | 5962 |
| 10 | 29 | 1773 | 40 | 2571 | 57 | 3609 | 105 | 6700 |
| 11 | 25 | 2057 | 36 | 2857 | 51 | 4033 | 94 | 7484 |
| 12 | 23 | 2236 | 32 | 3214 | 45 | 4571 | 84 | 8375 |
| 13 | 20 | 2571 | 29 | 3546 | 40 | 5142 | 75 | 9380 |
| 14 | 18 | 2857 | 25 | 4114 | 36 | 5714 | 67 | 10500 |
| 15 | 16 | 3214 | 23 | 4472 | 32 | 6428 | 59 | 11924 |
| 16 | 14 | 3673 | 20 | 5142 | 29 | 7093 | 53 | 13274 |
| 17 | 13 | 3956 | 18 | 5714 | 25 | 8228 | 47 | 14968 |
| 18 | 11 | 4675 | 16 | 6428 | 23 | 8943 | 42 | 16750 |
| 19 | 10 | 5142 | 14 | 7346 | 20 | 10285 | 37 | 19014 |
| 20 | 9 | 5714 | 13 | 7911 | 18 | 11428 | 34 | 20691 |
| 21 | 8 | 6428 | 11 | 9350 | 16 | 12856 | 30 | 23450 |
| 22 | 7 | 7346 | 10 | 10285 | 14 | 14693 | 26 | 27058 |
| 23 | 6 | 8571 | 9 | 11428 | 13 | 15823 | 24 | 29313 |
| 24 | 6 | 8571 | 8 | 12856 | 11 | 18700 | 21 | 33500 |
| 25 | 5 | 10285 | 7 | 14693 | 10 | 20570 | 19 | 37026 |
| 26 | 5 | 10285 | 6 | 17141 | 9 | 22855 | 17 | 41382 |
| 27 | 4 | 12856 | 6 | 17141 | 8 | 25712 | 15 | 46900 |
| 28 | 4 | 12856 | 5 | 20570 | 7 | 29385 | 13 | 54116 |
| 29 | 3 | 17141 | 5 | 20570 | 6 | 34283 | 12 | 58625 |
| 30 | 3 | 17141 | 4 | 25712 | 6 | 34283 | 10 | 70350 |
| 31 | 2 | 25712 | 4 | 25712 | 5 | 41139 | 9 | 78167 |

1.5  Scanning Method

To deal with the heterogenous block sizes in ABT, the concept of the well-known zig-zag scan is modified to allow for the application of CBP and the TML VLC scan. In H.26L the CBP concerns a set of 64 coefficients in 4 transformed blocks. To apply this CBP in MBs with 16x8, 8x16 or 16x16 transforms, these blocks are first scanned using the zig-zag scan and then ordered into groups of 64 coefficients. Figure 2 shows examples for the 8x8 transform (b), the 16x16 transform (c) and the 16x8 transform (d). The starting points and the direction of the scans are marked inside the blocks. In Q15K24, these resulting subblocks are divided into scans of 16 coefficients (scan blocks), to make the TML VLC applicable. As the entropy coding stage is the main subject of this experiment, this may be changed if necessary.

Figure 1 : Scan of a 8x8 block. Examples for the blocks of a MB affected by CBP: 8x8 transform, 16x16 transform, 16x8 transform.

###### 2 Experiment Description

The core experiment focuses on the entropy coding stage of the encoder. In Q15K24/25, scan blocks of 16 coefficients were applied in order to make the TML VLC applicable. The match between the statistics of the coded signal and the optimal frequency distribution given by the VLC was improved using single coefficient elimination. However, the TML VLC does not match the statistics of the ABT coefficients.

In this core experiment the following topics shall be studied:

- Investigation of the scan block size

- Investigation of the scanning method

- Application of one or more additional VLCs, e.g. like the one proposed in Q15K45

2.1  Simulation Conditions

Tests shall be performed on the defined CIF and QCIF test sequences over the whole range of quantization parameters.

#### References

Q15J72	Gisle Bjontegaard, ‘H.26L Test Model Long Term Number 4 (TML-4)’, ITU-T Q15/SG16,

Document Q15-J-72, Osaka, May 2000

Q15K24	Mathias Wien, Claudia Mayer, and Jens-Rainer Ohm, ´ Integer Transforms for H.26L using

Adaptive Block Transforms´, ITU-T Q15/SG16, Document Q15-K-24, Portland, August 2000

Q15K25	Mathias Wien, and Till Halbach, ´ Results of H.26L Core Experiment on Adaptive Block Transforms´

ITU-T Q15/SG16, Document Q15-K-25, Portland, August 2000

Q15K45	Louis Kerofsky, ´ Entropy coding of transform coefficients´, ITU-T Q15/SG16, Document Q15-K-45,

Portland, August 2000