ITU-Telecommunication Standardization Sector	Temporary Document 83-E (P) Rev.1

STUDY GROUP 16

Geneva, 13-17 November 2000

Questions: 15,19,20,21,22, 24/16

| SOURCE: | Chairman, WP 3/16 (Mr. Simão F. Campos-Neto) |
| --- | --- |
| TITLE: | WP 3/16 Meeting Plenary report |

________

Contents

1.	Introduction	3

2.	Organization of the work	3

2.1	Documentation	3

2.2	Division of the work	3

3.	Results	4

3.1	General WP 3/16 issues	4

3.1.1	Documentation	4

3.1.2	Plenary Results	5

3.2	Question 15/16: Advanced Video Coding	6

3.2.1	Documentation	7

3.2.2	Summary	8

3.2.3	Results	9

3.2.4	Liaison Statements	11

3.2.5	Future work	11

3.3	Question 19/16: Extension to existing ITU-T speech coding standards at bit rates below 16 kbit/s	12

3.3.1	Documentation	12

3.3.2	Summary	13

3.3.3	Results	13

3.3.4	Liaison Statements	14

3.3.5	Future work	14

3.4	Question 20/16: Audio and wideband coding in public telecommunication networks	14

3.4.1	Documentation	14

3.4.2	Summary	16

3.4.3	Results	16

3.4.4	Liaison Statements	18

3.4.5	Future work	18

3.5	Question 21/16: Encoding of speech signals at bit rates around 4-kbit/s	18

3.5.1	Documentation	18

3.5.2	Summary	19

3.5.3	Results	19

3.5.4	Future Work	21

3.6	Question 22/16: Software and Hardware Tools for Standardization of Speech and Audio Coding Algorithms	21

3.6.1	Summary	21

3.6.2	Documentation	22

3.6.3	Results	22

3.6.4	Liaison Statements	24

3.6.5	Future work	25

3.7	Question 24/16: Variable Rate Coding of Speech Signals	25

3.7.1	Summary	25

3.7.2	Documentation	25

3.7.3	Results	25

3.7.4	Liaison Statements	27

3.7.5	Future work	27

4.	Summary of Liaison Statements produced by WP 3/16	27

5.	Summary of future Rapporteur meetings for WP 3/16	28

Annex WP3.A Liaison Statements received and sent by WP 3/16	29

Annex Q15.A  Terms of Reference for Advanced Video Coding Activities	30

ANNEX Q20.A Terms of Reference for the ITU-T Wideband (7 kHz) Speech Coding Algorithm around 16 kbit/s	32

Annex Q21.A Summary of 4-kbit/s codec floating-point selection results	46

Annex Q21.B Proposed schedule for ITU-T 4-kbit/s Speech Coding Algorithm	48

Annex Q21.C Key Requirements and Objectives	50

Annex Q22.A UGST Action Points as of November/2000	51

# Introduction

Working party 3/16 (Signal Processing) held its meeting on 13-17 November 2000 in Geneva, under the chairmanship of Mr. Simão F. Campos Neto (LMGT/USA).

The questions currently allocated to WP 3/16 (and respective Rapporteurs) are:

| Q.15/16: | Advanced Video Coding (Mr. Gary Sullivan, PictureTel/USA) |
| --- | --- |
| Q.19/16: | Extension to existing ITU-T speech coding standards at bit rates below 16 kbit/s (Mr. Shinji Hayashi, NTT/Japan) |
| Q.20/16: | Audio and wideband coding in public telecommunication networks (Mr. Rosario D. de Iacovo, CSELT/Italy) |
| Q.21/16: | Encoding of speech signals at bit rates around 4 kbit/s (Mr. Paul Barrett, BT/UK) |
| Q.22/16: | Software and hardware tools for signal processing standardization activities (Simão F. Campos-Neto, LMGT/USA) |
| Q.24/16 | Variable rate coding of speech signals (Vacant) |

During this meeting, the focus of the work was:

- To review the documentation for Recommendations for Decision (H.263 Annexes U, V, and W, H.262 Corrigendum 1 and Amendment 1, G.722.1 Annex B, and G.191 Annex A);

- To review contributions and complete texts for Consent (H.263 Annex X and Corrigenda to G.7xx);

- To review and coordinate activities at the technical level with other Study Groups and organizations outside the ITU

- To progress the regular work within each of the questions; and

- To review the proposed draft text of the questions for the next Study Period.

# Organization of the work

## Documentation

Detailed description of the documentation considered under each question is found in Section 3 of this report.

## Division of the work

WP 3/16 adopted the agenda and work schedule in TD (3/16) 7. The discussion of Q.15/16 was held in parallel with the speech coding-related questions (Q.19-22&24/16).

# Results

## General WP 3/16 issues

### Documentation

#### Input Documents

White Contributions

| Number | Question | Source | Subject |
| --- | --- | --- | --- |
| W16-135 | 15 | Rap. for Q.15/16 | Corrigendum 1 to 2nd. edition of H.262 ¦ ISO/IEC 13818-2 (A.K.A Corrigendum 3 of H.262 ¦ ISO/IEC3818-2) for correction of reserved extension data handling |
| W16-136 | 15 | Rap. for Q.15/16 | Amendment 1 to 2nd edition of Recommendation H.262 ¦ ISO/IEC 13818-2 (A.K.A Amendment 7 of H.262 ¦ ISO/IEC3818-2) for video elementary stream content description data |
| W16-140 | 15 | Rap. for Q.15/16 | Draft for "H.263++" Annexes U, V and W to Rec. H.263 |
| W16-121 | 20 | Rap. Q.20 | Draft new Annex B to ITU-T Rec..722.1 - Floating point version of G.772.1 |
| W16-133 | 20 | Rap. Q.20 | Proposed Corrigendum to G.722.1 |
| W16-139 | 22 | Rap., Q.22/16 | Draft revised G.191 Annex A for Decision |

Meeting Reports

| Number | Subject |
| --- | --- |
| R71 | Part I (General) of the Report of Working Party 3/16 (Signal processing) |
| R72 | Part II.A (Determined Recommendations) of the Report of Working Party 3/16 (Signal processing) |
| R73 | Part II.B (Corrigendum to G.722.1) of the Report of Working Party 3/16 (Signal processing) |

Delayed Contributions: None.

TSB Circular/Collective Letters

| Number | Subject |
| --- | --- |
| Circ. 282 | Meeting of Study Group 16 with a view to approving two corrigenda, one amendment, nine draft revised and 25 draft new Recommendations in accordance with the provisions of Resolution 1, Section 8, of the WTSC (Geneva, 1996) |

Temporary Documents (Plenary)

| Number | Question | Source | Subject |
| --- | --- | --- | --- |
| TD(P)011 | WP3 | ITU-R WP 6M | Liaison Statement to ITU-T SG 16 on Multimedia studies - potential overlap |
| TD(P)013 | WP3 | WTSA 2000 | Questions assigned to ITU-T Study Group 16 (Multimedia Services, Systems and Terminals) |
| TD(P)016 | WP3 | Coordination Meeting | Text according to Resolution 1 |

Temporary Documents (General)

| Number | Question | Source | Subject |
| --- | --- | --- | --- |
| TD(G)001 | WP3 | TSAG | Liaison Statement on update on Electronic Document Handling |
| TD(G)007 | WP3 | ITU-R | Note to the Director, TSB |

Temporary Documents (WP 3/16)

| Number | Question | Source | Subject |
| --- | --- | --- | --- |
| TD(3)03 | WP3 | ITU-R SG8 | ITU-R Recommendation M.1079 - Performance and Quality of Service Requirements for International Mobile Telecommunications-2000 (IMT-2000) |
| TD(3)12 | WP3 | Chair | Opening list of documents for WP3/16 |

### Plenary Results

Working Party 3/16 reviewed and accepted the agenda and the work plan for the week in TD 11 (3/16). WP 3/16 reviewed general documents in its opening Plenary and the progress reports for each of the interim activities. WP 3/16 noted general Liaison Statements that did not need a reply.

Rapporteur meeting activities were reported and approved for Q.15/16 on their interim meetings (Osaka, Japan, 16-18 May 2000; and Portland, OR, USA, 22-24 August 2000). Questions 20 and 21 also reported the activities from their joint meeting in Geneva, 8-10 November 2000, which were also approved. Summaries of the Rapporteur meetings are provided in the respective sections of this report. It was noted in the report of Q.20 and Q.21 meetings that the deliverables of the host and listening laboratories participating in their respective exercises were fully met.

The meeting acknowledged the excellent work of Mr.S.Hayashi (NTT/Japan), Rapporteur for Q.19/16, who is retiring from ITU activities at this meeting.

#### Recommendations for Decision

| Recommendation | Final text found in |
| --- | --- |
| Draft Corrigendum to ITU-T Recommendation G.722.1 | COM 16-133 |
| Draft for corrigendum 1 to second edition of common-text Recommendation H.262 ¦ ISO/IEC 13818-2 (a.k.a. Corrigendum 3 of H.262 ¦ ISO/IEC 13818-2) for correction of reserved extension data handling | COM 16-135 |
| Draft new Annex U to ITU-T Recommendation H.263 – Enhanced reference picture selection | COM16-140+TD(P)51 |
| Draft new Annex V to ITU-T Recommendation H.263 – Data partitioned slice (DPS) | COM16-140+TD(P)51 |
| Draft new Annex W to ITU-T Recommendation H.263 – Additional Supplemental Enhancement Information | COM16-140+TD(P)51 & TD(P)19 [A.5 material] |
| Draft for amendment 1 to second edition of common-text Recommendation H.262 ¦ ISO/IEC 13818-2 (a.k.a. Amendment 7 of H.262 ¦ ISO/IEC 13818-2) for video elementary stream content description data | TD(P)71 |
| Draft revised G.191 Annex A– List of software tools available | COM 16-139 |
| Draft new Annex B to ITU-T Recommendation G.722.1 (Floating point version of G.722.1) | TD(P)43 |

All texts were approved with the proposed changes (if any) for forwarding for Decision by SG 16 on its session on Friday 17, afternoon.

#### Recommendations for Consent (AAP)

The following texts were proposed by WP 3/16 to be Consented under the AAP at this meeting of SG 16:

| Subject | Availability | Report Part |
| --- | --- | --- |
| H.263 Annex X | TD(P)53 | Part II.B |
| Proposed Corrigenda to G.729 Annexes | TD(P)42 | Part II.B |

#### Appendices and Implementer’s Guides

No Appendices or Implementer’s guides were produced by WP3/16 at this meeting.

#### Relationship with other bodies

A consolidated list of liaison statements sent and received by WP 3/16 is found in Annex WP3.A.

Liaison statements were received from SG 15, ITU-R SG8, and ISO/MPEG [TD (3/16) 4,5,6], ITU-R SG8 [TD (3) 3], and ITU-T SG 15 [TD (3/16) 1,2]. Specific responses were drafted by the questions from which action was required. The LS from TSAG [TD (G) 1] and ITU-R SG8 were noted.

The texts for the LS prepared by WP3 are found in TD (P) 84.

#### ISO/IEC MPEG Calls for Evidence and for Proposals

In this SG 16 meeting, a Coordination Meeting between MPEG and SG 16 took place. Among other matters, future cooperation between the two bodies was discussed. Of direct interest to WP3/16 was the fact that MPEG is issuing a Call for Proposals for new video technologies and Call for Evidence for new audio coding schemes in the in the January 2001 timeframe, and Call for Proposals for new audio coding schemes in the July 2001 timeframe. SG 16 has been invited to contribute with its current status of the H.26L work, and this issue is addressed in the section with the report of Q.15/16. It was also discussed the possibility that the results of the ongoing wideband standardization work could be provided to the MPEG Audio Call for Proposals in July 2001. This idea needs to be investigated further, and some issues are identified in the Q.20/16 Section of this report.

## Question 15/16:	Advanced Video Coding

Working Party 3/16 addressed Question 15/16 under the chairmanship of Mr. Gary Sullivan (Microsoft/USA) and Mr. Thomas Wiegand (Heinrich Hertz Institute/Germany). The group conducted work on adding new material to Recommendation H.263, amending and correcting the common text Recommendation H.262 | ISO/IEC 13818-2, and furthering the work on the future video coding standard development project known as H.26L.

The group thanks Mr. Keiichi Hibi for his valuable service as Associate Rapporteur for Advanced Video Coding during the 1997-2000 study period.  We regret that he has chosen not to continue in this post.

The group is pleased to note that Mr. Thomas Wiegand of the Heinrich Hertz Institute has volunteered to aid in the future work of the ITU-T Advanced Video Coding Question, and recommends that he be designated as Associate Rapporteur of the Question to recognize his new leadership role.

### Documentation

#### Input Documents

White Contributions

| Number | Source | Title |
| --- | --- | --- |
| COM 16-120-E | Q.14/16 Rapp. | Draft H.245v7 for Decision |
| COM 16-135-E | Q.15/16 Rapp. | Draft Corrigendum 1 to 2nd Edition of Common-Text Recommendation H.262¦ ISO/IEC 13818-2 for Correction of Reserved Extension Data Handling for Approval |
| COM 16-136-E | Q.15/16 Rapp. | Draft Amendment 1 to 2nd Edition of Common-Text Recommendation H.262¦ ISO/IEC 13818-2 for Addition of Video Elementary Stream Content Description Data for Decision |
| COM 16-140-E | Q.15/16 Rapp. | Draft H.263 Annexes U, V, and W for Decision |

Delayed Contributions

| Number | Source | Title |
| --- | --- | --- |
| COM-16 D.35 | PictureTel | Proposed H.245 changes for H.263 Annexes U and W.6.3.12 |
| COM-16 D.36 | PictureTel | Proposed H.242 changes for H.263 Annexes U and W.6.3.12 |
| COM-16 D.37 | PictureTel | Proposal for additional profile for Draft H.263 Annex X |
| COM-16 D.49 | Microsoft | Corrections needed to COM 16-140 Draft Annexes U, V, and W for H.263 |
| COM-16 D.50 | Microsoft | Proposal for Header Content in H.26L |

Temporary Documents

| Number | Source | Title |
| --- | --- | --- |
| TD (P) 19 | Q.15/16 Rapp. | Information on reference to ISOC/IETF RFC 2396 in H.263 Annex W |
| TD (G) 08 | Q.11/16 Rapp. | Draft Implementers Guide to the H.320 Series |
| TD (3/16) 03 | ITU-R | Revision to Rec. ITU-R M.1079 – Performance and QoS Requirements for IMT-2000 |
| TD (3/16) 04 | ISO/IEC JTC1/SC29/WG11 | Liaison Statement to ITU-T SG 16 Regarding Video Coding Activities and IDCT Specification |
| TD (3/16) 05 | ISO/IEC JTC1/SC29/WG11 | Liaison Statement Reply to ITU-T Study Group 16 on Future Video Coding Work |
| TD (3/16) 06 | ISO/IEC JTC1/SC29/WG11 | Liaison Statement to ITU-T Study Group 16 on ISO/IEC 13818 (MPEG-2 Systems and Video) Common Text and MPEG-4 on IP |
| TD (3/16) 11 | WP3 Chair | Agenda and schedule for Working Party 3/16 |
| TD (3/16) 12 | WP3 Chair | List of documents for Working Party 3/16 |
| TD (3/16) 16 | Q.15/16 Rapp. | Q.15/16 Meeting agenda and status report |
| TD (3/16) 17 | Q.15/16 Rapp. | Meeting Report of the Tenth Meeting (Meeting J) of the ITU-T Q.15/16 Advanced Video Coding Experts Group – Osaka, Japan, 16-18 May, 2000 |
| TD (3/16) 18 | Q.15/16 Rapp. | Meeting Report of the Eleventh Meeting (Meeting K) of the ITU-T Q.15/16 Advanced Video Coding Experts Group – Portland, Oregon, USA, 22-24 August, 2000 |
| TD (3/16) 22 | Q.15/16 Rapp. | Information on usage of H.263 profiles in 3GPP environments |

#### Output Documents

| TD (P) 51 | Q.15/16 Rapp. | Corrections needed to COM 16-140 Draft H.263 Annexes U, V, and W |
| --- | --- | --- |
| TD (P) 53 | Q.15/16 Rapp. | Draft H.263 Annex X for Consent |

### Summary

The primary results of the meeting in regard to video coding activities are as follows:

- Draft Annexes U, V, and W to Recommendation H.263 were forwarded for decision with minor modifications.

- Draft Annex X to Recommendation H.263 was forwarded for consent.

- Coordination work was conducted to ensure that the new Annexes U, V, and W had proper support in the H.320, H.323, and H.324 suites.

- Additional future work is planned on developing an implementers guide or corrigendum to Recommendation H.263.

- Additional future work is planned on developing a test model encoding appendix to Recommendation H.263.

- The first amendment to the 2000 edition of the common text Recommendation H.262 | ISO/IEC 13818-2 was forwarded for decision with modifications requested in ISO/IEC liaison activity.

- The first corrigendum to the 2000 edition of the common text Recommendation H.262 | ISO/IEC 13818-2 was forwarded for approval without alteration.

- Work was furthered on the future video coding standardization project known as H.26L

- A plan was adopted to provide demonstrations of the capabilities of the draft H.26L design in an upcoming test to be conducted by ISO/IEC JTC1/SC29/WG11.

### Results

The group adopted the following agenda for the advanced video coding activities of the meeting:

- Opening

- Agenda approval, meeting schedule, document assignment review [TD (3) 11, TD (3) 12, TD (3) 16]

- Status review and report of interim and ad-hoc activities [TD (3) 17, 18]

- Patent policy remarks (http://www.itu.int/ITU-Databases/TSBPatent/)

- Incoming Liaison Statements (3GPP, MPEG, etc.) [TD (3) 04, TD (3) 05, TD (3) 06] and information regarding other organizations [TD (3) 22] and coordination with work in other Questions (e.g., Q.13 and H.323 Annex I and MPEG-4 ES RTP TD (3) 6)  TD (P) 11 TD (3) 3, SG9]

- Maintenance and coordination issues for current recommendations

- Actions in regard to Rec. H.263:

  - Reviewing the draft text of Annexes U, V, and W to Rec. H.263 for Decision [COM16-140], including consideration of the changes proposed by Microsoft [D.49] and the referencing requirements of A.5 [TD (P) 19]

  - Reviewing the draft text of Annex X to Rec. H.263 for Consent [Q15-K-51r3], including consideration of the changes proposed by PictureTel [D.37] and information regarding the progress of work in 3GPP/3GPP2 [TD (3) 22]

  - Coordinating with Q.11-14 regarding the system support and capability exchange requirements for Rec. H.263 [COM16-120, D.35, D.36, TD (G) 8]

  - Progressing the work toward adopting a test model encoding description as Appendix II.

  - Consideration of the approval of an H.263 Implementers Guide or Corrigendum [Q15-K-47, Q15-K-49]

- Actions in regard to Rec. H.262 | ISO/IEC 13818-2 in coordination with ISO/IEC JTC1/SC29/WG11:

  - Reviewing the draft text of Amendment 1 (Content Description Data) to the 2000 edition of Rec. H.262 | ISO/IEC 13818-2 for Decision [COM16-136], including consideration of the modifications made by ISO/IEC JTC1/SC29/WG11 as conveyed in [TD (3) 06]

  - Reviewing the draft text of Corrigendum 1 (Reserved Extension Syntax Correction) to the 2000 edition of Rec. H.262 | ISO/IEC 13818-2 for Approval [COM16-135], including consideration of the remarks made by ISO/IEC JTC1/SC29/WG11 as conveyed in [TD (3) 06]

- Progressing the work on the H.26L future video coding standard development effort, including consideration of the proposal from Microsoft [D.50]

- Consideration of the inclusion of ITU-T video coding technology in the testing to be performed by ISO/IEC JTC1/SC29/WG11 [TD (3) 05 (with attachment)]

- Outgoing liaison statements [MPEG, SG9, 3GPP/3GPP2, IETF]

- All other business necessary for Q.15 consideration

- Workplan review and future meeting plans (Q.15 Germany 9-12 January 2001, ~March/April 2001)

#### Status Report and Report of Interim Activities

The status and priorities and reports of interim activities at the Osaka and Portland meetings of Q.15 were approved.

#### H.263 Annexes U, V, and W for Decision

The draft text COM 16-140 of H.263 Annexes U, V, and W was reviewed.  In response to delayed contribution D.49, the group agreed on making some minor changes to the draft and produced TD (P) 51 to specify those changes.  The group recommended that these annexes be put forward for decision by SG 16 as COM 16-140 as amended by TD (P) 51.

#### H.263 Annex X

The draft text of H.263 Annex X was discussed.  A delayed contribution D.37 was received that proposed adding a new profile to the annex.  Although the group had a generally favorable view of the concept of adopting an additional profile similar in complexity and performance to the proposed profile, there was not an agreement in the group to add the specific profile that was requested.  The proposal was therefore not adopted.  In consultation with the H.245 editor, some H.245 generic capabilities syntax was drafted for inclusion in Annex X.  A draft text TD (P) 53 was produced and is forwarded to WP3 with a recommendation to forward the text for consent.

#### Coordination on H.263 with Systems Support Requirements

The H.245 [COM 16-120] and H.320/H.242 [TD (G) 08] designs for support of H.263 Annexes U, V, and W were reviewed, along with the proposals to change these designs [D.35, D.36].  The group believes that these discussions resulted in proper support of the new H.263 annexes in the output documents produced by Questions 11 to 14.

#### Test Model Appendix for H.263

The group discussed its plan to adopt an Appendix to H.263 containing a test model reference encoding description.  No particular action was taken in regard to this planned appendix at this meeting.  The plan remains in place to adopt such an Appendix at the next meeting of SG 16.

#### Implementers Guide for H.263

The group discussed the status of work on drafting an implementers guide for H.263.  As the drafting work on this implementers guide has not yet reached sufficient maturity, the group plans to continue work toward adopting this work at the next meeting of SG 16.

#### Common Text H.262|ISO/IEC 13818-2 Amendment and Corrigendum

The group reviewed the status of the draft corrigendum [COM 16-135] and the draft amendment [COM 16-136] to the common text Recommendation H.262 | ISO/IEC 13818-2.  A liaison statement [TD (3) 06] was received that contained changes proposed for adoption into the draft amendment.  The group approved this work and requests that:

- The corrigendum be adopted as given in COM 16-135, and

- The amendment be adopted as given in COM 16-136 plus the modifications provided in the liaison statement TD (3) 06.

#### Progress of work on H.26L

A delayed contribution D.50 was provided that suggested several header-level changes proposed for H.26L:

- Color space flexibility

- Number of components flexibility

- Spatial subsampling of components flexibility

- Spatial sampling grid alignment flexibility

- Number of bits per sample flexibility

- Display rectangles

- User data support

- H.263 Annex L supplemental enhancement information

- Hypothetical reference decoder / video buffering verifier

- Time tag representation

The group agreed with the spirit and goals of this contribution.  It was noted that a number of these items required significant further work to specify the precise impact on syntax (some more than others) and that we must ensure that any specific syntax for these features is carefully designed.  There was some questioning of how to carry the type of information described in the document as “sequence header level” data.  There was also some questioning of the proposed manner of representing timing information.

#### Testing to be performed by ISO/IEC JTC1/SC29/WG11

ITU-T SG16 was invited to participate in upcoming tests of video coding technology performance to be conducted by ISO/IEC JTC1/SC29/WG11 [TD (3) 05].  This invitation was warmly received and the group requests that SG16 send a liaison statement to accept this invitation.  The group plans to provide a demonstration of the performance of the draft H.26L design for evaluation in these upcoming tests.

Pending the conclusions that could be reached as a result of these tests, we look forward to discussing possible joint efforts with ISO/IEC JTC1/SC29/WG11 for future video coding standardization activities.

### Liaison Statements

Four liaison statements have been drafted including

- Proposed Liaison Statement to ISO/IEC JTC1/SC29/WG11 on Video Coding Activities,

- Proposed communication to IETF AVT and MMUSIC Working Groups on
Video Coding Activities,

- Proposed communication to 3GPP TSG-S4 and S2 and 3GPP2 TSG-C on Video Codec Support for Wireless Multimedia Terminals,

- Proposed communication to IMTC.

### Future work

The basic schedules for our future work are outlined in Annex Q15.A.

The primary means of document distribution within the Q15/16 Video Coding Experts Group is electronic, and our documents can be found on an ftp site that is maintained at PictureTel:

ftp://standard.pictel.com/video-site           or

http://standard.pictel.com/ftp/video-site

This information is also linked to PictureTel’s broader standards web site which is maintained by Patrick Luthi of PictureTel:

http://standard.pictel.com/

The group conducts its email conversations over an email reflector that is graciously maintained by PictureTel through a majordomo software system:

itu-adv-video@standard.pictel.com

Requests for subscriptions and “unsubscriptions” for this email reflector should be sent to the majordomo automated list manager address:

itu-adv-video-request@standard.pictel.com

The address of the email reflector maintained within the TIES system (tsg16q15@ties.itu.int) is included in the addresses for email to the itu-adv-video list as well, to ensure that those subscribing to the ITU-maintained list receive all group communcations.

We plan two interim Rapporteur’s group meetings between the November ’00 and May/June ’00 meetings of SG 16.  These plans are shown below.  The group plans to also continue the ad-hoc activities created at the Portland meeting and described in the report of that meeting.

Siemens AG has offered to host the first of the two interim meetings in Eibsee, Germany, and the host organization plan for the second meeting is not yet complete.  Judging by the level of interest in our prior work (between 40 and 50 participants at each meeting), we are expecting attendance at the two interim meetings of 40-50 people (generally widely scattered in geographical representation).

## Question 19/16:	Extension to existing ITU-T speech coding standards at bit rates below 16 kbit/s

Working Party 3/16 addressed Question 19/16 under the chairmanship of Mr. Shinji Hayashi. The major activities of the question in this meeting were:

(1)	Certification procedure and test vectors for current speech coding recommendations, especially floating-point implementation,

(2)	External reset in G.728 and G.729 Annexes with DTX function,

(3)	Maintenance of existing speech coding Recommendations, especially G.729.

(4)	Liaison to other Sectors, SGs.

### Documentation

#### Input Documents

White Contributions/Reports: None

Delayed Contributions:

| Number | Source | Title |
| --- | --- | --- |
| COM16-D71-E | Conexant | Investigation of discrepancy between the description and C-code of the VAD in G.729 Annex B. |
| COM16-D79-E | NTT. | Proposal of editorial corrections to ITU-T G.729 Annexes C+, F, G, and I |

Temporary Documents

| Number | Source | ##### Title |
| --- | --- | --- |
| TD (3/16) 01 | SG15 | LS: Synchronous reset of G.728 and G.729 CODECs in CME |
| TD (3/16) 02 | SG15 | LS re: Approval of Appendix II to G.711, Annexes to G.728 and G.729 |
| TD (3/16) 19 | Rapporteur | Existing Implementer’s guides for G.7xx speech coders |

#### Output Documents

| Number | Source | Title |
| --- | --- | --- |
| TD (P) 16 | Rapporteur | Proposed corrigenda to G.729 Annexes B, C+, F, G, and I |
| TD (3/16) 27 | Rapporteur | Chairman’s report of Rapporteur’s meeting of Q.19/16 |

### Summary

The group reviewed the proposed corrections to the Annexes B, C+, F, G, and I and agreed to issue a corrigendum.

The group showed preference of external resetting of G.728 in CME under DTX operation, and will study the quality of G.729 under the same condition.

### Results

The group adopted the following agenda:

- Opening

- Document assignment

- Certification procedure and test vectors for current speech coding recommendations, especially floating-point implementation.

- External reset in G.728 and G.729 Annexes under DTX operation,

- Maintenance of existing speech coding Recommendations, especially G.729.

- Liaison to other Sectors, SGs.

#### Test vectors for various implementation of Recommendation

The floating-point implementation of G.729 lacks the test vectors and extensive verification procedures. No contributions were received at this meeting and the Rapporteur encourages participants to contribute on this issue.

#### Maintenance of existing speech coding Recommendations, especially G.729

COM16-D71 investigates the discrepancy between the textual description of G.729 Annex B and the C-code of G.729 Annex B. It experimentally proved no major quality effects with this discrepancy and the group preferred correction of the textual description to solve the discrepancy.

COM16-D79 presents editorial corrections to G.729 Annexes C+, F, G, and I. The group recognized necessity of purely editorial corrections of the description.

TD19(3/16) shows the summary of the algorithm changes of the speech coding Recommendations published in Implementers Guide. The meeting recognized necessity of issuing formal Corrigenda to officially incorporate all identified corrections, so that a correct set of C-source code and associated text will be available from the ITU. An editorial group was set up and prepared the text of the corrigendum, which is found in TD(P) 42.

#### External reset in G.728 and G.729 Annexes under DTX operation

TD (3/16) 1 presents the request by Q.6/15 to provide them with guidance on synchronous reset capability into circuit multiplication equipment (CMEs) using G.729 and G.728 codecs.

Q.19/16 experts agreed that it would be advisable to support a synchronous reset capability in CME using G.728. It was noted however that G.729 is known to quickly converge after synchronism loss between coder and decoder (such as would happen due to the use of DTX). On the other hand, the experts agreed that the use of synchronous reset under DTX operation in CME is not likely to introduce quality degradation. Experts present at the SG16 meeting manifested intention to submit experimental results to SG15 addressing the effect of synchronous reset on the speech quality for G.729.

### Liaison Statements

ITU-T SG16 replied to SG15 about the external reset of speech codec in DTX operation of CME (see Section 3.3.3.3 above).

### Future work

Next experts meeting will take place during the next meeting of SG16 scheduled for May 2001. The major issues to be addressed will be following:

- Certification procedure and test vectors for current speech coding recommendations, especially floating-point implementation.

- Maintenance of existing speech coding Recommendations

## Question 20/16: Audio and wideband coding in public telecommunication networks

Working Party 3/16 addressed Question 20/16 under the chairmanship of Mr. Rosario Drogo De Iacovo (CSELT S.p.A./Italy). Objectives of the question for this meeting were to review the Qualification test results for the wideband speech coding algorithm at around 16 kbit/s and to plan for the Selection phase.

### Documentation

#### Input Documents

White Contributions

| Number | Source | Title |
| --- | --- | --- |
| COM16-121 | Rap. Q.20 | Draft new Annex B to ITU-T Recommendation.722.1 - Floating point version of G.772.1 |
| COM16-133 | Rap. Q.20 | Proposed Corrigendum to G.722.1 |
| COM16-142 | PictureTel | Results of the interoperability test between the fixed-point G.722.1nd the proposed floating-point version |

Delayed Contributions

| Number | Source | Title |
| --- | --- | --- |
| D.019 | Motorola UK | Codec description and IPR Statement for the Motorola 16 kHz wideband codec candidate |
| D.020 | Motorola UK | WP Experiment 1 Qualification Test results for the Motorola candidate |
| D.021 | Motorola UK | WP Experiment 2 Qualification Test results for the Motorola candidate |
| D.032 | PictureTel | Summary of G.722.1 Interoperability Tests |
| D.033 | PictureTel | Corrections to G.722.1 Annex B floating point source code |
| D.063 | Matsushita | High-level description for proposed wideband (7kHz) codec algorithm |
| D.064 | Matsushita | WB Experiment 1 Qualification test results for candidate codec F (Matsushita codec) |
| D.065 | Matsushita | WB Experiment 2 qualification test results for candidate codec F (Matsushita's codec) |
| D.067 | Lucent | Comments for consideration during the drafting of the selection test plan for "a Wideband Speech Coding Algorithm around 16 kb/s" |
| D.068 | Lucent | High level description and qualification test results of the Lucent Technologies candidate wideband coding algorithm at 12.8 to 24 kbit/sec |
| D.069 | Lucent | Intellectual property statement relating to the Lucent Technologies candidate wideband speech codec algorithm at bit rates around 16 kbit/sec |
| D.076 | Nokia | Communication with ITU-T Q.20/16 "Audio & Wideband Coding" |
| D.077 | Nokia | BER/RBER/FER mapping used in 3GPP/GSM AMR wideband codec selection tests |
| D.078 | Nokia | High level description of the 3GPP/GSM AMR-WB codec recommended for selection in 3GPP |
| D.080 | NTT | High level description of NTT's wideband coder around 16 kbit/s |
| D.081 | NTT | WB experiment 1 qualifications test results for NTT's wideband speech coder |
| D.082 | NTT | WB experiment 2 qualification test results for NTT's wideband speech coder |
| D.084 | Texas Instruments | Experiment 1 test results of Texas Instrument's 16kbit/s wideband speech codec |
| D.085 | Texas Instruments | Experiment 2 test results of Texas Instrument's 16 kbit/s wideband speech codec |
| D.086 | Texas Instruments | High level descriptions of TI's wideband coder |
| D.087 | Texas Instruments | IPR statement for TI's wideband coder |
| D.088 | Texas Instruments | High level description of TI/Nokia/Ericsson 4kb/s coder |
| D.090 | Canada | WB Experiment 1 qualification test results for VoiceAge's candidate codec |
| D.091 | Canada | WB Experiment 2 qualifications test results for VoiceAge's candidate codec |
| D.092 | Canada | High level description of VoiceAge's candidate codef for ITU-T WB speech coding standard |

Temporary Documents

| Number | Source | Title |
| --- | --- | --- |
| TD(3/16)07 | ###### Editor | Editorial correction to draft new Annex B to ITU-T Rec. G.722.1 |
| TD(3/16)08 | Rap.Q.20 | Additional results of the interoperability test between the fixed-point and the corrected floating point version of G.722.1 |
| TD(3/16)15 | Rap.Q.20 | Q.20/16 Rapporteur’s meeting report (Geneva, 8-10 November, 2000) |

#### Output Documents

| Number | Source | Title |
| --- | --- | --- |
| TD (3/16) 30 | Rapporteurs Q.20/16, Q.21/16 | LS to Q.22/12 (SQEG) – Speech coding issues |
| TD(P)84 | Rapporteur Q.20/16 | Communication to ETSI: information on wideband speech coding activity |
| TD (3/16) 32 | Rapporteur Q.20/16 | Q.20/16 meeting report |
| TD (P) 43 | Editor | Corrections to draft Annex B to ITU-T Rec. G.722.1 |

### Summary

The time schedule for wideband coding at around 16 kbit/s was reviewed aiming to a Consent during a possible WP3/16 meeting in July 2001.

A Liaison Statement to SG12 and a communication to ETSI/3GPP SA4 were drafted.

The Corrigendum to G.722.1 and the new Annex B to G.722.1 (floating-point version) were submitted to SG16 plenary for Decision. The agreed changes to the G.722.1 Annex B (editorial and related to the C-source code) are summarized in TD(P)43.

### Results

The group adopted the following example agenda and document allocation:

- Review and approval of the agenda

- Allocation of documents

- 16 kbit/s wideband coding: revision of the time schedule	[TD (3/16) 15]

- 16 kbit/s wideband coding: LSs and communications

- 16 kbit/s wideband coding: additional issues

- G.722.1 matters: Corrigendum G.722.1	[COM16-133]

- G.722.1 matters: Annex B to G.722.1	[COM16-121,142; D.32,33; TD(3/16)7,8]

- Future work

- AOB

The Delayed Contributions not allocated to the above-mentioned agenda items were considered for information since they was already addressed during the previous Rapporteur’s meeting (8-10 Nov., 2000).

#### WB coding around 16 kbit/s: revision of the time schedule

The draft time schedule contained in TD(3/16)15 was revised according to the next planned WP3 meeting in July 2001 (Consent of Recommendation). The updated time schedule is contained in Annex Q20.A to this report.

#### WB coding around 16 kbit/s: LSs and communications

It was agreed to send a Liaison Statement to SG12 and a communication to 3GPP S4. The content of those documents is reported in a separate contribution.

#### WB coding around 16 kbit/s: additional issues

The following issues were briefly introduced for further discussion within the Ad-Hoc group via e-mail.

List of additional issues to be addressed within the Ad-Hoc group:

- definition of the kind of Global Analysis to be performed

- definition of a completely blind procedure in evaluating the Selection Test results

- definition and generation of appropriate background noise material

- issues related to the use of soundcards for playback in the listening sessions

Other issues, already agreed for discussion within the Ad-Hoc group:

- drafting of the Selection test plan

- definition of the processing stages for the host laboratory function

- guidelines for production of demonstration material (random BER and music conditions)

- computation of the codec frequency response using white noise

Ad-Hoc group for drafting the Selection test documents:

| Name | E-mail |
| --- | --- |
| Simão F. Campos-Neto (LMGT) | simao.campos@labs.comsat.com |
| Rosario Drogo De Iacovo (CSELT S.p.A.) | rosario.drogodeiacovo@cselt.it |
| Jari Hagqvist (NOKIA) | jari.hagqvist.nokia.com |
| Sean Ramprashad (LUCENT) | ramprash@research.bell-labs.com |
| Alan D. Sharpley (DYNASTAT) | sharpley@dynastat.com |
| John D. Tardelli (ARCON) | jdt@arcon.com |
| Vishu Viswanathan (TI) | v-viswanathan@ti.com |

The Rapporteur encouraged experts to volunteer for this Ad-Hoc group.

#### ISO/MPEG Audio Call for Proposals

In a coordination meeting held between ITU-T SG16 and ISO/MPEG leaderships, the possibility that the current WB16k work could be provided in the Call for Proposals that will be considered by MPEG in its 23-27 July 2001. This possibility would imply, among other issues, that the Q.20 Rapporteur’s meeting currently planned for that week may need to be anticipated by one week. The following topics were also addressed:

- Impact on the planned schedule for ITU-T wideband coding. It was noticed that the WP3/16 meting planned to evaluate the ITU-T wideband selection test results and the MPEG meeting planned to evaluate the proposed algorithms are overlapping (23-27 July, 2001). Fortunately, the ITU-T wideband schedule allows for a moderate tightening and it was felt possible to move the July meeting one week before the planned date without any serious impact on the overall plan. This would allow, in case of a successful ITU-T selection process, the said participation.

- Kind of information to be submitted following the Call for Proposals. No detailed information was available at the meeting and it was pointed out that the matter could be solved through exchanging of e-mails between the two experts’ groups.

- Agreement at WP3/16 level to submit the ITU-T selected wideband algorithm. It was agreed that such participation would be useful with the goal of developing harmonized solutions.

- Agreement at candidate proponent level to submit its wideband algorithm. This issue, with its implications, was not addressed in detail and was left for further considerations during the next SG16 meeting.

As a general conclusion, the meeting agreed the principle that the ITU-T wideband selected algorithm should participate in the ISO/IEC JTC1/SC29/WG11 audio tests and that more detailed information should be exchanged between the two groups in order to evaluate all possible impacts in current timescale assumptions.

#### G.722.1 matters: Corrigendum to G.722.1

White Contribution COM16-133 contains the corrigendum the group agreed to submit to SG16 plenary for Decision.

#### G.722.1 matters: Annex B to G.722.1

The editor (P.Luthi, PictureTel/USA) presented all the documentation allocated to this agenda item and the group agreed to submit the new Annex B of G.722.1 to SG16 plenary for Decision. The agreed changes (editorial and related to the C-source code) contained in TD(3/16)7, TD(3/16)8 and D.33 were summarized in TD(P)43. The new C-code of G.722.1 Annex B (Release 1.1) will be made available to TSB.

### Liaison Statements

A liaison statement to SQEG/SG12 was drafted to ask support in developing an appropriate Selection Test Plan for the wideband coding algorithm around 16 kbit/s and also consider, during the SG12 meeting in February 2001, the identification of organizations willing to perform the host laboratory, cross-checking laboratory, listening laboratory and global analysis functions with the associated cost.

### Future work

A Q20/16 Rapporteur’s meeting, followed by a WP3/16 meeting, is provisionally planned for July 23-27, 2001. Since this meeting will happen after the next SG 16 meeting, it will be re-confirmed in the next SG 16 meeting.

Between now and the next SG 16 meeting, the work will continue by correspondence as planned in the time schedule for the wideband coding algorithm at around 16 kbit/s.

## Question 21/16:	Encoding of speech signals at bit rates around 4-kbit/s

The meeting addressed Question 21/16 under the chairmanship of Mr. Barrett (BT, UK). The current objective of this question is to select an algorithm that is likely to meet the terms of reference for the future ITU-T 4-kbit/s speech coding standard (G.4k). The specific objective of this meeting was to begin planning the fixed-point selection phase.

### Documentation

#### Input Documents

Delayed Contributions:

| Number | Source | Title |
| --- | --- | --- |
| COM16-D.23 | ###### LMGT | LMGT ITU-T 4 kbit/s Selection Host Laboratory Report |
| COM16-D.24 | LMGT | ITU-T 4 kbit/s Selection Host Laboratory Crosscheck Procedures |
| COM16-D.25 | LMGT | ITU-T 4 kbit/s Selection Experiments 1A and 3B - LMGT Results |
| COM16-D.26 | LMGT, Samsung | High Level Description of LMGT-SAMSUNG 4 kb/s Coder |
| COM16-D.70 | ACDFMN | High level description of the 4 kbit/s algorithm submitted by AT&T, Conexant, Deutsche Telekom, France Telecom, Matsushita, and NTT |
| COM16-D.74 | ACDFMN | Notice of audio demonstration of 2.4 and 6.4 kbit/s extensions and DTX-enabled version of ACDFMN’s 4-kbit/s speech coding candidate |
| COM16-D.88 | Ericsson, Nokia, TI | High Level Description of TI/Nokia/Ericsson 4 kb/s Coder |
| COM16-D.89 | FT | France Télécom R&D Results for the ITU-T 4 kbit/s Selection Phase – Experiments 1 & 3b |

Temporary Documents

| Number | Source | Title |
| --- | --- | --- |
| TD 26(3/16) | ###### Rapporteur | Draft liaison statement to Q.22/12 (SQEG) |
| TD 23(3/16) | Rapporteur | Draft agenda and objectives for Q.21/16 |
| TD 24(3/16) | Rapporteur | Q.21/16 Meeting Report (Geneva, 7-10 November, 2000) |
| TD 25(3/16) | Rapporteur | Draft of 4kbit/s Selection Phase results |

#### Output Documents

| Number | Source | Title |
| --- | --- | --- |
| TD 31(3/16) | ###### Q.21/16 | Liaisons to Q.22/12 |

### Summary

The objective of this meeting was to review the results of the floating-point selection phase of the ITU-T 4-kbit/s speech coding work. The meeting agreed to begin a fixed-point selection phase in August 2001 with two of the three candidates presented at this meeting. The target for selection and consent of the fixed-point recommendation is October 2001.

### Results

The group adopted the following agenda:

1.	Opening of the meeting

2.	Approval of the agenda	 [ TD 23(3/16) ]

3.	Allocation of documents

4.	Last meeting report and interim activities	 [ TD 24(3/16), 25(3/16) ]

D.22, D.23, D.24, D.25, D.26, D.70, D.74, D.88, D.89 ]

5.	Liaisons from other groups

6.	Fixed-point selection phase matters

7.	Review of schedule	[ TD 24 (3/16) ]

8.	Approval of liaisons statements	[ TD 26(3/16) ]

- AOB

Delayed contributions D.22 to D.89 were addressed during the Rapporteur meeting held 7 - 10 November, and were therefore only provided for information at this meeting.

#### Last meeting report and review of interim activities

The main interim activity of Q.21/16 was the floating-point selection phase. The results of this selection phase were reviewed at a Rapporteur meeting held in Geneva, 7 - 10 November 2000. The main decisions made at that meeting are summarised as follows:

#### Review of selection phase activities

The meeting agreed that the two host laboratories and the six listening laboratories had fulfilled the terms of their memoranda of understanding (MoU) with the ITU.

#### Review of candidate algorithms

Candidate algorithms were presented by the following three consortia:

- AT&T, Conexant, Deutsche Telekom, France Telecom, Matsushita, and NTT;

- Texas Instruments Incorporated, Nokia Corporation, and Ericsson Radio Systems AB;

- COMSAT Corporation (now part of Lockheed-Martin Global Telecommunications, LMGT) and SAMSUNG.

All eleven consortia members verbally declared that they would abide by Option 2.2 of the ITU patent policy.

#### Review of results

The meeting reviewed the results of the float-point selection phase, which are summarized in Annex Q21.A. Candidates A and B will proceed to a fixed-point selection phase starting in August 2001. Candidate C will not be considered any further. The new deadline of August 2001 is intended to allow codec developers time to both implement their algorithms in fixed-point arithmetic and to effect performance improvements.

Q.21/16 agreed that an independent party should perform a detailed global analysis of the fixed-point selection phase. The calculations and presentation of results for the global analysis will be agreed and frozen at the first SG16 meeting in 2001. The global analysis will be performed on blind-mapped data, and will be funded by the candidate proponents. The Rapporteur invited contribution on this topic for the next SG16 meeting.

As an aid to the codec proponents, the meeting identified which requirements and objectives are seen as particularly important in the development of a 4-kbit/s speech coding standard. This list is provided in Annex Q21.C.

#### Liaisons from other groups

There were no incoming liaison statements.

#### Fixed-point selection phase matters

The meeting agreed to follow the following double-blind procedure for the fixed-point selection phase. The candidate executables will be delivered to the Q.21/16 Rapporteur on 1 August 2001. The Rapporteur will forward the two executables to the host laboratories using a blind mapping (Codecs 1 and 2). After processing the speech material for the selection phase experiments, the host laboratory will forward the processed speech material to the listening laboratories using a second blind mapping (Codecs A and B). The same mapping will be used for all experiments. The global analysis laboratory will perform its analysis on the double-blind data and forward the results to the Rapporteur. After receiving the global analysis, the Rapporteur will request the second mapping from the host laboratories, and will upload the global analysis information with the first and second mapping labels to the Q.21/16 informal ftp site prior to the deadline for submissions to the selection meeting. The selection meeting will therefore discuss the codecs under their consortium names rather than under blind labels.

Ericsson Radio Systems stated that in their opinion, by not requesting a 2.4kbit/s extension for the fixed-point selection phase, the ITU-T is missing the opportunity to fill a market need in a timely manner. The Rapporteur reminded the meeting that this subject had been addressed during the Rapporteur meeting on 8 - 10 November, and that the proposal to include the 2.4-kbit/s extension in the selection phase was not accepted because it represented a significant change to the existing work-plan of Q.21/16.

#### Review of schedule

The meeting reviewed the revised schedule and list of deliverables attached in Annex Q21.B. Due to the lack of a future meeting calendar, this schedule and the use of the alternative approval procedure (AAP) are provisional, and will be reviewed at the next SG16 meeting.

#### Approval of liaisons

The meeting approved a liaison statement to Q.22/12 (SQEG) requesting the following actions: preparation of a test plan for the fixed-point selection phase; the identification of organisations to perform the host laboratory, listening laboratory, and global analysis functions; guidance on the preparation of the global analysis.

The meeting agreed to include an item in the liaison to Q.22/12 (SQEG) requesting that those laboratories who participate in both ITU-T 4-kbit/s selection phases use different talkers in the fixed-point selection phase to those used in the floating-point selection phase; and that the noise material has not been used in any preceding standardization exercise.

The liaison statement also includes feedback from the floating-point selection phase, which suggests several refinements to the subjective testing procedures that were identified during the Rapporteur meeting on 8 - 10 November.

### Future Work

No Q.21/16 Rapporteur meetings are planned prior to the next SG16 meeting.

## Question 22/16:	Software and Hardware Tools for Standardization of Speech and Audio Coding Algorithms

### Summary

Currently the question is dealing with the maintenance and future enhancements of the STL96. At the last SG 16 meeting, the group agreed to officially issue a new release of the ITU-T Software Tool Library (STL2000), reflected as a revision to G.191 Annex A.

Release 1 of the STL2000 was made available at the February meeting both via the TSB as well as in the IFA for SG16. Some minor bugs were corrected and identified in two subsequent releases (R2 and R3). R3 was delivered to the ITU on July 13, together with the final text for G.191 Annex A (see COM16-139E). The draft revised G.191 Annex A is proposed for Decision at this meeting. The group unanimously agreed that G.191 should be forwarded for approval (Decision) by SG 16.

### Documentation

| Group | Number | Subject: |
| --- | --- | --- |
| Reference: |  |  |
|  | COM-16-1 (97-2000) | [Text of questions] |
|  | TD(P)13 | New Questions |
|  | COM-16 71R | [Report, WP 3/16 meeting, February 2000] |
| Action: | Software tools: |  |
|  | W139 | G.191 Annex A for Decision |
|  | TD10(3/16) | Info on 3GPP/SA4 activities regarding basic operators |

### Results

The following agenda was adopted:

- Opening

- Approval of the agenda

- Review progress on Software Tools

- G.191 Annex A (STL2000) for approval

- Future activities under the question: maintenance and software tools

- Outgoing Liaison Statements (SG12, 3GPP, 3GPP2)

- AOB

- Closing of the meeting

#### Progress on Software Tools

The following are the changes from the STL2000R1 to STL2000R3.

General. Updated pre-defined symbol for compilation under Win32/gcc (from __CYGWIN32__ to __CYGWIN__) for modules G711, G722, BASOP, FIR, UTL, and UNSUP. Compilation scripts were updated and tested for automatic binary generation and testing from the STL2000 root directory (covering modules EID, FIR, G711, G722, G726, G727, IIR, IS54, MNRU, RPELTP, SV56, UNSUP, and UTL) for several platforms/compilers (HPUX/gcc; Win32/cl; CygWin/gcc; Sol7-Sparc/gcc).

Basic operators. Work by correspondence confirmed inclusion of shiftless 32-bit multiplication operators L_mult0(), L_mac0(), and L_msu0(). No discussions was held on the revision of the weights for the 32-bit operators.

EID. No functional changes performed in the module core (eid.c). Added a global variable PORTABILITY_TEST_OPERATION to eid.c that does not affect the module operation but allows the demonstration programs to print a warning to the user that the C code has been compiled with the portability test operation (which causes the module to produce systematic error patterns for portability purposes) and modified the demo programs to use this feature (eiddemo.c, eid8k.c, and gen-patt.c). Identified execution problems of gen-patt.c and ep-stats.c with Win32/cl and Win32/gcc while the same code works properly in Unix systems. It seems that both Win32 compilers share a buggy malloc run time library. The problem has not been corrected.

G711. Corrected g711demo.c for the calculation of the number of blocks to process under Win32/cl and Win32/gcc by including <math.h>. No changes performed in the module core (g711.c).

G722. The ZIP file with the restricted set of test vectors was renamed from g722-unx.zip to tst-g722.zip, for consistency with other modules. Changed makefiles accordingly. Found a bug in g722demo.c (not solved), whereby options N1 and N2 are ignored by the code and the whole file is processed. No changes performed in the module core (g722.c).

G726. Corrected bug in vbr-g726.c when the user does not specify the bit-rate. The array for bit rates was not initialized and caused a segmentation fault, and now it defaults to 32 kbit/s operation. Also fixed the incorrect calculation of the total number of blocks to process when the block size is not a multiple of the file size. No changes performed in the module core (g711.c).

G727. Editorial corrections were done in the C code self-documentation (comments). No changes performed in the module core (g727.c).

MNRU. In mnrudemo.c, changed memory allocation of floating point buffers in[] and out[] from static to dynamic, to prevent memory invasion when block sizes larger than 256 are specified. No changes performed in the module core (mnru.c).

FIR. In fir-irs.c, replaced the RXIRS filter coefficients (8 and 16 kHz) with a new set that more closely follows the Modified-IRS receive mask and generated new test vectors and updated the relevant makefiles. Removed compilation warnings observed with Win32/cl in modules fir-{dsm,irs,pso,tia}.c. Updated filter.c to display online help for option -delay. Also in filter.c, disabled down-sampling with FIR filters when block size is 1 (i.e. sample-based operation) because of a bug in fir_downsampling_kernel() [in fir-lib.c] that causes the output file to be one sample long. Until the function is corrected, this operation will be disabled in this program. In the makefiles, purged test files that had the same CRC32: test022.ref & test023.ref, test009.ref & test011.ref, test014.ref & test016.ref, test004.ref & test006.ref, test020.ref & test021.ref, test008.ref & test010.ref, test012.ref & test013.ref, and test005.ref & test007.ref.

SV56. Changed memory allocation in sv56demo.c and actlevel.c from static to dynamic, command line added options (-blk, -start, -end, and -n) options, updated makefiles and revised the read-me files. No changes performed in the module core (sv-p56.c).

UTL. Added a new informational symbol definition in ugstdemo.h, labeled COMPILER, which contains a text string describing the compiler used in a given built. No changes performed in the module core (ugst-utl.c).

Unsupported tools. A minor bug was corrected in sb.c for the temporary filename (-over) when compiled with Win32/cl and added an option for conditional byte-swapping when overwriting a file (depending on whether the system is little or big-endian). Changed memory allocation in sine.c for data buffer from static to dynamic, to prevent memory invasion when block sizes larger than 256 are used. Corrected bug in oper.c that made incorrect calculation on total number of blocks to process when the block size is not a multiple of the file size. Added two new unsupported tools: astrip.c, concat.c to the UNSUP directory with test vectors. Updated makefiles and test vectors, as well as the top-level makefiles/batchs for automatic compilation of the STL.

After review of the changes, the group unanimously agreed to forward G.191 Annex A for approval (Decision) by SG16.

#### Other working items

Basic Operators. A set of 40-bit operators was circulated to the reflector before the meeting but no feedback was received. It was mentioned at the meeting that 3GPP-2 is likely to use the 40-bit operators in their upcoming SMV variable bit rate coding standard. The group also reviewed information from 3GPP, that they only foresee usage of the 32 bit operators. Q.22 agreed to inform both groups of the approval of the STL2000, including the 32-bit basic operator set, and on the on-going work on the basic operators.

The group agreed to continue the work on the 40-bit basic operators.

STL User’s Manual. Q.22 agreed that the Manual should be maintenance. The Rapporteur provided a draft version of the STL2000 User’s Manual, which is also available in the SG 16 IFA. The group discussed ways to publish the Manual, but no firm conclusion was reached. Options included: current means (an attachment to the C code distribution), an Annex, or an Appendix (either in paper or as a one-page paper with electronic attachment). The current text is available as a postscript/PDF file, since the source of the document is typeset with LaTeX, and not MS Word. The possibility of converting the text into MS Word was noted. The Rapporteur will consult with the TSB trying to identify the best publication means. In the meantime, the Rapporteur invited interested parties to review chapters of the Manual and provide feedback. Discussions towards a final version of the Manual will continue via the WP3 reflector (wp3audio@ctd.comsat.com).

New work items. The group also agreed to try to identify a frequency-response tool, which would have direct use in the Q.20/16 work. At least a framework of the tool should be discussed and agreed to in the reflector discussions by mid February 2001. Other possible relevant work items include: G.728 implementation, linear interface to G.726 and G.727 (i.e. Annex A to G.726 and G.727), and G.711 packet loss concealment (G.711 Appendix I).

#### Action points

Annex Q22.A contains the revised list of action items for Q.22/16. The group reviewed the list in COM 16R-71 eliminating a number of work items that were obsolete. The remaining action items will be progressed by correspondence.

### Liaison Statements

No Liaison Statements were received. A joint LS was produced for SG 12, notifying of the Decision of the STL2000, and a communication was prepared to ETSI/3GPP and TIA TR45.5/3GPP-2 with information on the status of basic operators in the STL.

### Future work

Q.22/16 will continue its work by correspondence and is planned to meet again in the next meeting of SG 16.

## Question 24/16:	Variable Rate Coding of Speech Signals

### Summary

Currently the question is refining its terms of reference. The current terms of reference can be found in COM16R-71E (report from the Feb 2000 SG 16 meeting). This session of Q.24 was chaired by the acting Rapporteur (Mr.S.F.Campos-Neto, LMGT/USA) on November 14.

Several contributions were reviewed and three different approaches were discussed. The group could not agree at this meeting which line would be more appropriate for the intended applications, and decided that three threads of discussions should be held over the email reflector (vbr@ctd.comsat.com) trying to generate initially three sets of terms of reference, that would be harmonized as a next step. The group will formally review the results of the discussions in the next SG16 meeting.

### Documentation

#### Input Documents

| No. | Source | Title | Question |
| --- | --- | --- | --- |
| D066 | Ericsson | Communication to ITU-T Q.24/16 on "Parameters for variable bit rate voice codec operation and information on channel models" | 24 |
| D072 | Conexant, Lucent | Technical merits of VBR coding system and SMV | 24 |
| D073 | Qualcomm | CDMA2000 and IS95 system capacity benefits derived from a VBR coding scheme | 24 |
| TD(3)03 | ITU-R | ITU-R Recommendation M.1079 - Performance and Quality of Service Requirements for International Mobile Telecommunications-2000 (IMT-2000) | WP3 |
| TD(3)09 | TR45.5 | Discussion on VBR requirements | 24 |

#### Output Documents

Communications: Communication to 3GPP/3GPP2.

### Results

The group adopted the following agenda:

- Opening of the meeting

- Approval of the agenda

- Documentation

- Review of documents

- Review of terms of reference

- Outgoing documents

- Continuation of the work

- AOB

- Closing of the meeting

#### Discussion

Document D066 (Ericsson, “Communication to ITU-T Q.24/16”) was presented and conveyed an answer to communication sent to 3GPP early in the year. The document explained several VBR aspects of the existing and soon-to-be-adopted AMR system, which in summary can be seen as a mix of VAD/DTX source-controlled system with multi-rate options that are network-controlled.

The paper D072 (Conexant, Lucent, “Technical merits of VBR coding system and SMV”) and Corrigendum described the VBR solution soon-to-be-adopted in 3GPP2. In summary, it is a scheme that supports different bit-rates which are somewhat specialized for a range of portions of the speech signal (voiced segments, transitions, silence, etc) and is controlled by a front-end signal classification module (Rate Determination Algorithm). Different modes can be network-controlled, in order to achieve different average bit rates (ABR).

D073 (Qualcomm, “CDMA2000 and IS95 system capacity benefits derived from a VBR coding scheme”) shows the gain in capacity that can be achieved from lower average bit rates in IS95 and CDMA2000 systems.

TD(3)09 (TR45.5 [via SG 16 Chairman], “Discussion on VBR requirements”) discusses and proposes a number of issues that in their view are relevant for the refinement of the Terms of Reference in Q.24.

Table 3 in page 21 of TD(3)03 (ITU-R, “ITU-R Rec. M.1079 - Performance and QoS Requirements for International Mobile Telecommunications-2000 (IMT-2000)”) was briefly reviewed by the group and contains reference values for bit rates, delay, delay jitter, and channel errors for IMT-2000 applications. These values should be taken into account in the elaboration of the Terms of Reference for the question.

As part of the discussions, it was brought to the attention of the delegates an early contribution discussed in 1999 on embedded VBR schemes.

After the presentation of the documents and exchange of points of view, the group recognized merits in the three technological approaches discussed:

- Multirate/VAD VBR approach (MVV)

- Specialized Codec VBR approach (SCV)

- Embedded VBR approach (EV)

However, the group could not agree at this time which of the three approaches would be more promising towards the intended applications. It was clarified in the discussions that the objective of this Question is not to standardize a codec for use in the first wave of Third-Generation (3G) systems, but for the next wave of systems as well as packet-based systems where gains may be obtained from variable packet sizes.

As a result the group adopted the Rapporteur’s suggestions to pursue a three-pronged approach in which three ad-hoc groups will develop three sets of terms of reference (ToR), one for each of the technologies above, to be conducted over the VBR e-mail reflector (vbr@ctd.comsat.com) and moderated by three different members. The MVV discussions will be moderated by Mr. Paul Barrett <paul.a.barrett@bt.com>; the SCV discussions will be moderated by Mr.Huan-Yu Su <Huan-Yu.Su@conexant.com>, and the EV discussions will be moderated by Mr. Sean Ramprashad <ramprash@research.bell-labs.com>. The discussions will be based on past and present contributions, as well as on new material that will be circulated through the reflector. The conclusions of the work will be submitted to the next SG 16 meeting as TDs, when the three different ToRs will be compared and a common set of ToRs will be derived. It is the ultimate objective however that a single standard would be identified as a result of this question. Strong consideration will be given to application areas not already covered by existing ITU coding standards.

### Liaison Statements

One Liaison Statement was prepared by Q.24 ETSI/3GPP SA4 and TIA TR 45.5 informing on the progress of the question and inviting interested experts to contribute to the discussions over the reflector. In the SG 16 Plenary it was decided that this text will also be sent as a LS to the newly created Special Study Group on IMT-2000. Interested parties should send an email to <simao.campos@labs.comsat.com> requesting subscription to this VBR reflector.

### Future work

No Rapporteur’s meetings are planned. The work is expected to progress via the email reflector (vbr@ctd.comsat.com) and the group will review the outcome of the discussions at the next SG 16 meeting.

# Summary of Liaison Statements produced by WP 3/16

Annex WP3.A of this report contains the summary of all Liaison statements considered and produced by WP 3/16 during this meeting of SG 16. The agreed texts of the Liaison Statements are available in TD (P) 84.

# Summary of future Rapporteur meetings for WP 3/16

| Tentative Dates | Tentative Host/Place | Q | Detailed agenda items |
| --- | --- | --- | --- |
| 9-12 January 2001 | Siemens / Eibsee, Germany | 15/16 | - Further the work toward H.26L long-term coding standard - Generate the material to be used for H.26L performance demonstration in tests by ISO/IEC JTC1/SC29/WG11 - Discuss comments received during the comment period for H.263 Annex X (provided the comment period has expired) - Develop implementers guide or corrigendum to H.263 - Develop test model appendix to H.263 - Coordination of video coding needs for Q.11-14/16 standards - Study of H.263 capabilities and limits - Particular study of video coding for packet network and mobile applications - Coordination of relationship with video coding technology in ISO/IEC JTC1 SC29/ WG11 (MPEG) - Consideration of video coding needs for non-conversational services - Consideration of video coding needs for users of sign-language and lip-reading - Maintenance of existing ITU video coding standards (H.120, H.261, H.262, H.263) - Other business as necessary for Q.15 |
| - March / April 2001 | - TBD | - 15/16 | - Continuation of agenda items shown above |

- Annex WP3.A
Liaison Statements received and sent by WP 3/16

| Received |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number | Question | Source | Subject | Action |
| TD(G)001 | WP3 | TSAG | Liaison Statement on update on Electronic Document Handling | Noted |
| TD(3)01 | 19 | SG 15 | Liaison Statement to SG16 on synchronous reset of G.728 and G.729 codecs in CME | Replied |
| TD(3)02 | 19 | SG15 | Reply to LS on the approval of Appendix II to G.711, Annexes to G.728 and G.729 | Noted |
| TD(3)03 | WP3 | ITU-R SG8 | ITU-R Recommendation M.1079 - Performance and Quality of Service Requirements for International Mobile Telecommunications-2000 (IMT-2000) | Noted |
| TD(3)04 | 15 | ISO/IEC | Liaison Statement to ITU-T SG16 regarding video coding activities and IDCT specification | Replied |
| TD(3)05 | 15 | ISO/IEC | Liaison Statement reply to ITU-T Study Group 16 on future video coding work | Replied |
| TD(3)06 | 15 | ISO | Liaison Statement to ITU-T Study Group 16 on ISO/IEC 13818 (MPEG-2 Systems and Video) Common Text and MPEG-4 on IP | Replied |

| Sent |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number | Question | Destination | Subject | Purpose |
|  | 15 | ISO/MPEG | Reply to LS to ISO/IEC JTC1/SC29/ WG11 on Video Coding Activities | Reply |
|  | 15 | IETF AVT and MMusic | Communication IETF AVT and MMusic on Video Coding Activities | Information |
|  | 15 | ETSI/3GPP TIA TR45.5 | Communication to 3GPP TSG-S4 and S2 and 3GPP2 TSG-C on Video Codec Support for Wireless Multimedia Terminals | Reply |
| TD(P)84 | 15, 11 | IMTC | Communication to IMTC | Information |
|  | 19 | SG15 | Reply to LS on on Synchronous Reset of G.728 and G.729 codecs in CME | Info & reply |
|  | 20,21,22 | Q.22/12 (SQEG) | Forthcoming testing activities and STL2000 | Action (Feb 2001) |
|  | 20 | ETSI/3GPP TIA TR45.5 | Information on wideband speech coding activity | Info |
|  | 22 | ETSI/3GPP TIA TR45.5 | Basic operators | Info |
|  | 24 | SSG IMT2k ETSI/3GPP TIA TR45.5 | Progress on variable rate voice coding | Info |

- Annex Q15.A 
Terms of Reference for Advanced Video Coding Activities

- Q15.A.1  Scope

- The goal of this Question is to produce advanced moving image coding methods appropriate for conversational audio/visual services.  However, moving image coding should also consider applications for non-conversational services.  This Question will focus on advanced techniques leading to significant quality and performance improvements.  Video coding standardization needs will be investigated for multimedia systems without regard to transport type (Internet, LAN, Mobile, N-ISDN, B-ISDN, GSTN, etc.).  The Question will also include maintenance issues regarding existing video coding standards.

- Q15.A.2  Current Projects

- The primary project under way is the development of a new “H.26L” video coding standard.  H.26L will investigate new video coding algorithms with the objective of achieving a significant improvement in performance relative to the best available version of prior standards.  The target date for decision of a new H.26L recommendation is around the middle of the year 2002.  The workplan schedule for the H.26L project is shown below.

- Also under way are some activities for Recommendation H.263.  A new Annex X containing profile and level definitions has reached SG 16 consent status and needs to have work completed in response to any comments that may be provided.  An appendix is under development to describe a test model reference encoding method for H.263.  A corrigendum or implementers guide is also planned.

- The activities for this Question will continue the work on both H.263++ and H.26L.  Both of these activities relate closely to work being conducted within the MPEG-4 project of ISO/IEC JTC1 WG11/SC29, and should be coordinated closely with that effort, including liaison communication.

- Q15.A.3  Study Items

- Advanced coding methods in order to achieve the following objectives:

- improvements in compression efficiency;

- reduction of real-time delay;

- improvements in picture quality;

- robust operation in error prone environments (e.g., non-guaranteed-bandwidth packet networks or mobile wireless communication);

- organization of the compressed data format to support packetization and streaming;

- temporal and spatial alignment of streams of varying coding;

- reduction of complexity;

- additional features such as object coding and multi-view operation.

- Maintenance of existing H series video coding Recommendations, including H.120, H.261, H.262|ISO/IEC 13818-2, and H.263.

- Variable Bit Rate coding.

- Video coding needs for those using sign language and lip reading communication.

- Video bit rate reduction for compressed-digital to compressed-digital processing.

- The impact of colorimetry, video quality assessment, and quality control requirements on video codec development.

- Impact of downloaded video algorithm coding.

Q15.A.4  Methods of Work

Work will be conducted via correspondence to the maximal possible extent, especially using email and ftp communication (per the SG16 EDH policy) and also possibly fax/or and written letters. Rapporteur’s group meetings will be conducted in the interim between meetings of Working Party 3 of Study Group 16 to further the work toward the above goals.  Liaison communication should also be conducted with ISO/IEC JTC1 SG11/SC29 as authorized by the rapporteur experts group during this period.

H.26L Workplan

| Meeting | Approx Date | Type | Milestone |
| --- | --- | --- | --- |
| SG16-1 | 17 March ‘97 | Study Group |  |
| Q15-A | 24 Jun ‘97 | Experts |  |
| Q15-B | 8 Sep ‘97 | Experts |  |
| Q15-C | 2 Dec ‘97 | Experts |  |
| SG16-2 | 26 Jan ‘98 | SG16 | Issue Call for Proposals |
| Q15-D | 21 Apr ‘98 | Experts |  |
| Q15-E | 21 Jul ‘98 | Experts |  |
| SG16-3 | 14 Sep ‘98 | Study Group |  |
| Q15-F | 3 Nov ‘98 | Experts | First Formal Evaluations |
| Q15-G | 16 Feb ‘99 | Experts |  |
| SG16-4 | 17 May ‘99 | Study Group |  |
| Q15-H | Aug ‘99 | Experts | First Formal Draft Adoptions |
| SG16-5 | Sep ‘99 | Study Group |  |
| Q15-I | Oct ‘99 | Experts |  |
| SG16-6 | Feb ‘00 | Study Group |  |
| Q15-J | May ‘00 | Experts |  |
| SG16-7 | Jun ‘00 | Study Group | - |
| Q15-K | Aug ‘00 | Experts |  |
| WTSA | Sep ‘00 | WTSA | - |
| SG16-1N | Nov ‘00 | Study Group |  |
|  | Jan ‘01 | Experts | Final Major Feature Adoptions |
|  | Apr ‘01 | Experts |  |
|  | May ‘01 | Study Group |  |
|  | Oct ‘01 | Experts | Bug-Checking |
|  | Nov ‘01 | Study Group |  |
|  | Jan ‘02 | Experts |  |
|  | May ‘02 | Study Group | Consent |

- ANNEX Q20.A
Terms of Reference for the ITU-T Wideband (7 kHz) Speech Coding Algorithm around 16 kbit/s

- Background

The following general guidelines are considered relevant for this wideband activity:

- Input and output audio signals should have a bandwidth of 7 kHz at a sampling rate of 16 kHz.

- Primary signals of interest are clean speech and speech in background noise.  Music performance objectives set  at  higher bit-rates (24 kbit/s).

- High speech quality with the objective of equivalence to G.722 at 56/64 kbit/sec.

- 16 kbit/s is the main bit-rate. It is required the ability of the candidate to scale in bit rate to lower bit-rates (less then 16 kbit/s) and up to 24 kbit/s with no fundamental changes in either the technology or the algorithm used.

- Robustness to frame erasures and random bit errors.

- Low algorithmic delay (frame size of 20ms or integer sub-multiples)

- Applications

In the following the applications foreseen for a wideband (7 kHz bandwidth) speech coder around 16 kbit/s are listed:

1. Voice over IP (VoIP) and Internet Applications

| Features: | Wideband transmission over the internet High quality speech for IP video-conferencing Robust under background noise conditions Robust to frame erasures Scalable bit rate (e.g. 12...24 kbit/s) Main focus on speech, good music performance at higher bit rates desirable |
| --- | --- |

2. Mobile Communications

| Features: | High-quality speech for third generation services Mainly to be used under relatively small residual channel error conditions Robust under background noise conditions Robust to random bit errors Robust to frame erasures Scalable bit rate (e.g. 12...24 kbit/s) Main focus on speech |
| --- | --- |

3. PSTN applications

| Features: | High-quality audio-conferencing Business applications (point-to-point, multi-point-links) Robust under background noise conditions Fixed bit rate sufficient Main focus on speech |
| --- | --- |

4. ISDN wideband telephony

| Features: | High-quality audio conferencing for multi-point applications Robust to background noise Fixed bit rate sufficient Main focus on speech |
| --- | --- |

5. ISDN video-telephony and video-conferencing

| Features: | Enabling high-quality speech using only one ISDN channel Improve video quality using two or more ISDN channels Robust under background noise conditions Fixed bit rate sufficient Main focus on speech |
| --- | --- |

The experts group consider that VoIP, Internet Applications and Mobile Communications (listed in item 1 and 2 of the application list) are the primary applications. The Terms of Reference reflect that consideration.

- Performance requirements & objectives

The performance requirements and objectives for a wideband (7 kHz) speech coding algorithm around 16 kbit/s are shown in Table Q20.1.

Table Q20.1 – (Part 1/7)

Performance requirements and objectives for a wideband (7 kHz) speech coding algorithm around 16 kbit/s (Note 1)

| Parameter | Requirement | Objective |
| --- | --- | --- |
| Bit-rate(s) | - not exceeding 13 kbit/s - not exceeding 16 kbit/s - not exceeding 20 kbit/s - not exceeding 24 kbit/s (Note 2) | Scalable on finer increments of bit-rate. The number of bit per frame should be an integer multiple of 8. |
| Speech (single speaker) in error-free condition at input signal nominal level -26 dB with respect to the overload point (Note 3): |  |  |
| 1) at 13 kbit/s (Note 4) | No requirement | Not worse than ITU-T Rec. G.722 at 48 kbit/s |
| 2) at 16 kbit/s | Better than ITU-T Rec. G.722 at 48 kbit/s | Not worse than ITU-T Rec. G.722 at 56 kbit/s |
| 3) at 20 kbit/s (Note 4) | No requirement | Better than the 16 kbit/s under the same condition |
| 4) at 24 kbit/s | Not worse than ITU-T Rec. G.722 at 56 kbit/s | Not worse than ITU-T Rec. G.722 at 64 kbit/s |
| Narrowband speech (single speaker) in error-free condition at input signal nominal level -26 dB with respect to the overload point: (Note 5) 1) at 13 kbit/s | No requirement |  |
| 2) at 16 kbit/s | Not worse than ITU-T Rec. G.726 at 32 kbit/s (Note 6) |  |
| 3) at 20 kbit/s | No requirement |  |
| 4) at 24 kbit/s | Not worse than ITU-T Rec. G.726 at 32 kbit/s (Note 6) |  |

Table Q20.1 – (Part 2/7)

- Performance requirements and objectives for a wideband (7 kHz) speech coding algorithm around 16 kbit/s

| Speech (single speaker) in error condition at input signal nominal level –26 dB with respect to the overload point: |  |  |
| --- | --- | --- |
| - Robustness to random bit errors (BER=10-3) | No requirement | (Note 7) |
| - Detected frame erasures (3% Random) (Note 8, 9) |  |  |
| 1) at 13 kbit/s | No requirement | No more than 10% additional degradation, in terms of PoW (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 48 kbit/s under error free condition |
| 2) at 16 kbit/s | No more than 10% additional degradation, in terms of PoW (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 48 kbit/s under error free condition | No more than 10% additional degradation, in terms of PoW (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 56 kbit/s under error free condition |
| 3) at 20 kbit/s | No requirement | Better than the 16 kbit/s under the same condition |
| 4) at 24 kbit/s | No more than 10% additional degradation, in terms of PoW (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 56 kbit/s under error free condition | No more than 10% additional degradation, in terms of PoW (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 64 kbit/s under error free condition |
| - Detected double-frame erasures (3% random) |  |  |
| 1) at 13 kbit/s | No requirement | No more than 10% additional degradation, in terms of PoW (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 48 kbit/s under error free condition |
| 2) at 16 kbit/s | No requirement | No more than 10% additional degradation, in terms of PoW (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 56 kbit/s under error free condition |
| 3) at 20 kbit/s | No requirement | Better than the 16 kbit/s under the same condition |
| 4) at 24 kbit/s | No requirement | No more than 10% additional degradation, in terms of PoW (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 64 kbit/s under error free condition |

Table Q20.1 – (Part 3/7)

Performance requirements and objectives for a wideband (7 kHz) speech coding algorithm around 16 kbit/s

| Speech (single speaker) quality dependency on the input signal level between –36 dB and –16 dB with respect to the overload point: |  |  |
| --- | --- | --- |
| Low level input speech (-36 dBov) 1) at 13 kbit/s | No requirement | Not worse than ITU-T Rec. G.722 at 48 kbit/s with –36 dBov input level |
| 2) at 16 kbit/s | Better than ITU-T Rec. G.722 at 48 kbit/s with –36 dBov input level | Not worse than ITU-T Rec. G.722 at 56 kbit/s with –36 dBov input level |
| 3) at 20 kbit/s | No requirement | Better than the 16 kbit/s under the same condition |
| 4) at 24 kbit/s | Not worse than ITU-T Rec. G.722 at 56 kbit/s with –36 dBov input level | Not worse than ITU-T Rec. G.722 at 64 kbit/s with –36 dBov input level |
| High level input speech (-16 dBov) 1) at 13 kbit/s | No requirement | Not worse than ITU-T Rec. G.722 at 48 kbit/s with –26 dBov nominal input level |
| 2) at 16 kbit/s | Better than ITU-T Rec. G.722 at 48 kbit/s with –26 dBov nominal input level | Not worse than ITU-T Rec. G.722 at 56 kbit/s with –26 dBov nominal input level |
| 3) at 20 kbit/s | No requirement | Better than the 16 kbit/s under the same condition |
| 4) at 24 kbit/s | Not worse than ITU-T Rec. G.722 at 56 kbit/s with –26 dBov nominal input level | Not worse than ITU-T Rec. G.722 at 64 kbit/s with –26 dBov nominal input level |
| Speech (single speaker) quality dependency on speakers at input signal nominal level (Note 10): |  |  |
| 1) at 13 kbit/s | No requirement | Not worse than ITU-T Rec. G.722 at 48 kbit/s |
| 2) at 16 kbit/s | Better than ITU-T Rec. G.722 at 48 kbit/s | Not worse than ITU-T Rec. G.722 at 56 kbit/s |
| 3) at 20 kbit/s | No requirement | Better than the 16 kbit/s under the same condition |
| 4) at 24 kbit/s | Not worse than ITU-T Rec. G.722 at 56 kbit/s. | Not worse than ITU-T Rec. G.722 at 64 kbit/s |

Table Q20.1 – (Part 4/7)

Performance requirements and objectives for a wideband (7 kHz) speech coding algorithm around 16 kbit/s

| Music in error-free condition at input signal nominal level –26 dB with respect to the overload point: |  |  |
| --- | --- | --- |
| 1) at 13 kbit/s | No requirement. | No requirement. |
| 2) at 16 kbit/s | No requirement. | No requirement. |
| 3) at 20 kbit/s | No requirement | No requirement |
| 4) at 24 kbit/s | No requirement | Not worse than ITU-T Rec. G.722 at 48 kbit/s (Note 11) |
| Performance in the presence of the following background noises for 1, 2, asynchronous tandem encodings (foreground signal is single speaker): - office noise (SNR=15dB) - babble noise (SNR=20dB) - car noise (SNR=15dB) - interfering talker (SNR=15dB) - performance in the presence of reverberant speech conditions. For further study (Note 12) |  |  |
| 1) at 13 kbit/s | No requirement | No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 48 kbit/s |
| 2) at 16 kbit/s | No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 48 kbit/s | No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 56 kbit/s |
| 3) at 20 kbit/s | No requirement | Better than the 16 kbit/s under the same condition |
| 4) at 24 kbit/s | No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 56 kbit/s | No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 64 kbit/s |

Table Q20.1 – (Part 5/7)

Performance requirements and objectives for a wideband (7 kHz) speech coding algorithm around 16 kbit/s

| Tandeming capability for asynchronous tandeming (Note 13): |  |  |
| --- | --- | --- |
| 1) at 13 kbit/s | No requirement. | No requirement. |
| 2) at 16 kbit/s | Not worse than ITU-T Rec. G.722 at 48 kbit/s under the same tandem condition (2 tandems) | Not worse than ITU-T Rec. G.722 at 56 kbit/s under the same tandem condition (2 tandems) |
| 3) at 20 kbit/s | No requirement. | No requirement. |
| 4) at 24 kbit/s | Not worse than ITU-T Rec. G.722 at 56 kbit/s under the same tandem condition (2 tandems) | Not worse than ITU-T Rec. G.722 at 64 kbit/s under the same tandem condition (2 tandems) |
| Transcoding with ITU-T G.722 or other standards | For further study | For further study |
| One-way coder/decoder delay (Note 14, 15): |  |  |
| - frame size | 20 ms (or integer sub-multiples of 20ms) | 10 ms (or integer sub-multiples of 10 ms) |
| - total codec delay (algorithmic delay+processing delay) |  50 ms |  25 ms |
| Encoder-decoder synchronization (Note 16) | Provided externally |  |
| Capability to transmit voice-band data (Note 17) | Not needed | For further study |
| Capability to transmit information and signaling tones (Note 18) | DTMF |  |

Table Q20.1 – (Part 6/7)

Performance requirements and objectives for a wideband (7 kHz) speech coding algorithm around 16 kbit/s

| Effect of switching signal sources to the codec (Note 19) | Capability required |  |
| --- | --- | --- |
| Effect of switching between bit-rates (Note 20) | Capability required | No annoying effects |
| Convergence time (Note 21) |  |  |
| 1) at 13 kbit/s | For further study | For further study |
| 2) at 16 kbit/s | For further study | For further study |
| 3) at 20 kbit/s | For further study | For further study |
| 4) at 24 kbit/s | For further study | For further study |
| Idle channel noise (Note 22) |  |  |
| - unweighted | Less than –66 dBm0 |  |
| - weighted | For further study |  |
| - single frequency | For further study |  |
| Sampling rate (Note 23) | 16 kHz |  |
| Timing requirements (Note 24) | Equivalent to those contained in ITU-T Rec. G.722 as preliminary indication |  |
| Jitter tolerance (Note 25) | The network interface to the codec is assumed to be compliant to the jitter limits appropriate to digital equipments as specified in ITU-T Rec. G.823, G.824 and I.430 (Note 26) |  |
| A/D and D/A converter accuracy | Testing performed with 14 bit linear PCM. 15 and 16 bit linear PCM are for further study. |  |
| Overload point of the A/D and D/A converters | As for ITU-T Rec. G.722 as preliminary indication (+9dBm0 ±0.3dBm0) |  |

Table Q20.1 – (Part 7/7)

Performance requirements and objectives for a wideband (7 kHz) speech coding algorithm around 16 kbit/s

| Attenuation/frequency response of encoder and decoder analog circuitry | As for ITU-T Rec. G.722 as preliminary indication |  |
| --- | --- | --- |
| Nominal frequency range at 13, 16, 20 and 24 kbit/s (Note 27) | 50 to 7000 Hz |  |
| Digital transport compatibility | ITU-T Rec. H.221, H.320, H.323, H.324 |  |
| Variable bit-rate compatibility | Capability of switching bit-rate at frame boundaries required. The frame size should be equal for the different bit-rates. | For further study |
| Interoperability |  | Interoperability with 3G and 2G systems (Note 28) |
| Complexity | As low as possible (Note 29). Encoder and decoder in 1 DSP (single CPU) |  |
| Memory | As low as possible in terms of RAM used | For further study |
| Specification description and implementation | Bit-exact 16 bit fixed-point modular ANSI-C code (Note 30) electronic format, using the ITU-T 32-bit basic operators | Interoperable floating-point specification to follow after fixed-point specification |

Notes to Table Q20.1

| 1. | The requirements and objectives refer to the distortion introduced between the defined input PCM interface of the coder and the defined output PCM interface of the decoder, unless otherwise specified. |
| --- | --- |
| 2. | Hereafter in the Terms of Reference, the wording “at x kbit/s” should be intended as “not exceeding x kbit/s”, as applicable to the bit-rates of the candidate algorithm. |
| 3. | When all requirements are met, the objectives will be used as one of the criteria to discriminate between candidates. |
| 4. | That bit-rate is not tested in the Qualification test. In the Selection test the objectives will be considered and tested. |
| 5. | In some possible scenarios the wideband speech coder may have to code a signal coming from a narrowband source. One such application is the wideband teleconferencing scenario where some of the callers participate in the call using narrowband terminal equipment while others use the wideband option. It should therefore be possible for the wideband speech coder to encode a narrowband signal with high quality.The input characteristic of the narrowband speech is for further study. |
| 6. | The requirement for narrowband input speech is not tested in the Qualification test, where the candidate proponents are requested to provide only demonstration material. This requirement will be tested in the Selection test. |
| 7. | The candidate proponents are requested to provide demonstration material for the Qualification and Selection test at 16 and 24 kbit/s operation modes with the entire bit-stream subjected to a random BER=10-3. Additional material at different levels of BER will be provided for information. More specific guidelines will be agreed and inserted in the Selection Plan documents. |
| 8. | Detection of frame erasures will be indicated to the decoder by external signals. If the test codec has a frame size other than 20 ms, a 20 ms frame boundary will be used for FER testing. Hence, multiples of 20 ms will be erased in the event of frame erasures. |
| 9. | The evaluation of speech quality in detected random frame erasure conditions will consider a requirement in terms of percentages of PoW (Poor or Worse) allocated in addition to the PoW percentage obtained from the reference codec. The additional degradation is an absolute increase of 10% (or 5%). In other words, the %(P+B) is first determined for the reference codec and an absolute value of 10 (or 5) is added to produce the increased reference degradation. The new 'absolute' criterion, introduced to avoid using the MOS concept (subject to relative shifts depending on the context of the experiment, the instructions to the team, the listeners, etc.), needs that the limits at 95% confidence level of PoW are calculated, following a defined computation rule. |
| 10. | Further studies are required to establish an appropriate methodology to test speaker dependency. |
| 11. | The candidate proponents are requested to provide demonstration material for the Selection Test at 24 kbit/s operation mode. More specific guidelines will be agreed and inserted in the Selection Plan documents. |
| 12. | Further studies are required to identify realistic reverberant conditions to be tested (room geometry, distance between microphone and talker(s), reverberation time vs. frequency, etc.). The requirements and objectives related to this type of signal are to be defined. |
| 13. | A primary application requiring tandemed codecs is in multipoint control units (MCU), where signals are also mixed from several codecs before recoding and transmission. Test conditions with 3 tandems will be considered in the Characterization phase. |
| 14. | Algorithmic delay includes the frame size, that is the block size of acquired input signal, plus any other delays inherent in the algorithm (look ahead). Processing delay is the time taken by encoder to process samples and by decoder to decode the incoming bit stream. Total codec delay is the sum of algorithmic delay and processing delay. It is assumed that the processing delay is equal to the frame size. The channel transmission delay assumes that transmission occurs on a serial channel matched to the bit rate of the codec, which typically is equal to the frame size. The total system delay is the sum of algorithmic delay, processing delay, channel transmission delay and such other delays caused by the test equipment and interfaces connected to these. The total system delay can be measured. The test system delay consists of the delay caused by the test equipment and the interface between the wideband speech encoder/decoder and the test equipment. The test system delay can be measured by passing PCM data directly through the system, bypassing only wideband speech encoder and decoder. The total codec delay can be calculated by subtracting the test system and channel transmission delay from the total system delay. |
| 15. | The size of frame acquired by the encoder should be 20ms or a sub-multiple of 20ms, as a requirement. For example, xz = 20ms, where x is an integer and z is the frame size in ms. Maintaining 10ms boundaries provides easy compatibility with ITU-T Rec. H.221. |
| 16. | Bit stream alignment between encoder and decoder. |
| 17. | ITU-T Rec. H.221 provides for data transmission in ISDN applications. In other applications, including DCME, PCME and ATM networks, separate transmission facilities will be provided for voice-band data. |
| 18. | The actual distortion requirements and objectives for the tones to be transmitted are for further study. |
| 19. | In a voice activated MCU where the broadcaster is selected according to voice signal levels, there should be no annoying artifacts caused by switching signal source to the codec. |
| 20. | Switching between operating bit-rates shall not introduce subjectively noticeable effects. |
| 21. | The convergence time should be as small as possible in order not to miss any significant parts of words. The convergence time of a speech codec is the amount of time it takes for the state in the decoder to approximate the state in the encoder, when the decoder has been reset to its initial state during the middle of a talk spurt. This definition readily suggests a methodology for measuring convergence time. Begin with a test signal of speech. It is encoded and output bit stream is saved. The entire bit stream is decoded and the output is uninterrupted decoded speech. In this encoding and decoding, the states of the encoder and decoder are initialized to be equal. Next, remove the beginning of the bit stream. Decode the remainder of the bit stream, having re-initialized the decoder first. Receive a decoded output signal which will evolve toward the true output signal decoded earlier. Then measure the sequence of individual segment signal-to-noise ratios (SNR). The convergence time will be the time beyond which the segment SNR always stays above the pre-determined threshold. |
| 22. | Idle channel noise measured with an encoder and decoder connected back-to-back as described in ITU-T Rec. G.722. Three cases are considered: - unweighted (unweighted noise power measured in the frequency range 50 to 7000 Hz) - weighted (weighted noise power measured in the frequency range 50 to 7000 Hz) - single frequency (the level of any single frequency, in particular 8000 Hz, the sampling frequency and its multiples, measured selectively). |
| 23. | Nominal sampling rate of the analog-to-digital and digital-to-analog converters. |
| 24. | Accuracy of the analog-to-digital and digital-to-analog converters clocks. |
| 25. | Jitter tolerance of the incoming bit-stream (peak-to-peak variation) that avoids introduced errors at the decoder. |
| 26. | In order to ensure that any equipment can be connected to any recommended hierarchical interface within a network, it is necessary to arrange that the input ports of all equipment are capable of accommodating levels of jitter up to the maximum network limit defined in ITU-T Rec. G.823 “The control of jitter and wander within digital networks which are based on 2048 kbit/s hierarchy” and ITU-T Rec. G.824 “The control of jitter and wander within digital networks which are based on the 1544 kbit/s hierarchy”. For equipments connected to an ISDN Basic Rate Access, the NT jitter characteristics contained in ITU-T Rec. I.430 are also relevant. |
| 27. | The nominal 3 dB bandwidth measured with an encoder and decoder connected back-to-back as described in ITU-T Rec. G.722. The measurement should be performed by injection of white noise to the codec in order to compute its transfer function between input and output. |
| 28. | If there is one or more than one bit-rate in common with the source bit-rates for wideband coding defined in 3G (including at least 3GPP and 3GPP2) and 2G systems, the objective is to provide bit-stream compatibility at those bit-rates, i.e. the bit-stream produced by this wideband encoder should be correctly decoded by the 3G and 2G wideband decoder and vice-versa. |
| 29. | When all requirements are met the complexity and memory figures will be used as one of the criteria to discriminate between candidates. |
| 30. | Modular means a software implementation made in accordance to the guidelines given in ITU-T Software Tools Library User Manual. |

- Schedule for wideband coding around 16 kbit/s

A timetable for the development of a wideband speech coding algorithm around 16 kbit/s is reported in the following:

Note: the draft time schedule is split in two parts: Technical Activity and Contractual Activity.

| Date | Technical Activity |
| --- | --- |
| Study Group 16 meeting Nov. 13-17, 2000 | At the Study Group meeting in November 13-17, 2000 : - A clear statement is made indicating whether or not the Host Laboratory Cross-Checking and Listening Laboratory have been judged compliant in meeting their deliverables according to the contents of the Memoranda of Understanding. - Evaluation of the test results of the Qualification Phase. - Define an Ad Hoc group for drafting the Selection Plan documents. (editor of each individual experiment and document is appointed within the Ad Hoc group) |
| 20 November 2000 | Start drafting the Wideband Selection Plan documents within the Ad Hoc group. |
| [t.b.d. 22 November 200] | Qualification Test processed speech material available to Candidate Proponents (each candidate will have access to its own processed material) |
| 20 December 2000 | - Draft Wideband Selection Plan documents available on the e-mail reflectors (tsg12q20@itu.int and tsg16q20@itu.int) and on IFA. - Call for host/cross-checking/listening/global analysis labs issued by the Q.20/16 Rapporteur via e-mail. |
| 19-23 February 2001 (SG12 meeting) | Final version of the Wideband Selection Test Plan approved. |
| 9 March 2001 | - The Host Laboratory receives the source material from the Listening Laboratory. - The Host Laboratory generates the Noise Files and the Error Files. |
| 12 March 2001 | - Preliminary submission of executable code to the Host Laboratory. - The Cross-checking Lab receives a subset of source material for cross-checking activities. |
| 28 March 2001 | The Host Laboratory receives the fixed-point executable code, with test vectors, from each Candidate Proponent. |
| 30 March 2001 | The Cross-checking Lab receives the necessary executables for cross-checking activities. |
| 14 May 2001 | The Listening Laboratory will receive, with blind mapping, the processed (already cross-checked) material for the Candidate Algorithms. |
| 20 June 2001 | Deliverables (raw data and test results) are sent via e-mail by the Listening Laboratories to the Q.20/16 Rapporteur (Mr Rosario Drogo De Iacovo, CSELT/ITALY; e-mail: rosario.drogodeiacovo@cselt.it), to the WP3/16 Chairman (Mr Simão F. Campos Neto, COMSAT/USA; e-mail: simao.campos@labs.comsat.com) and to the appointed contacts of the Candidate Proponents. |
| 22 June 2001 | A notification (e-mail) is sent by the Q.20/16 Rapporteur informing the group, through the ITU-T e-mail reflector (e-mail: tsg16q20@itu.int) that the deliverables (raw data and test results) have been received from Listening Laboratories. |
| 3 July 2001 | Submission of deliverables as Rapporteur’s meeting documents: - Each Listening Laboratory submits a report (in the form of a technical contribution) with test results. The test results will be presented according to the format described in the ITU-T Wideband Selection Test Plan. - Global analysis is performed and submitted as a report by the Global Analysis Laboratory. - Host Laboratory makes available demonstration material (random bit errors and music conditions) to the Candidate Proponents with a blind mapping different from the one used for the Listening Laboratories. Additional contractual deliverables (e.g. computation of the nominal frequency range) are also submitted as a report. |
| 16 July 2001 | Submission of deliverables as Rapporteur’s meeting documents: - Each Candidate Proponent submits the High Level description of its Candidate Algorithm. |
| WP3/16 meeting July 23-27, 2001 [July 16-20, 2001] | - Evaluation of the results of the Selection Phase. - Evaluation of the algorithm complexity. - Consent of Recommendation and C-source code. - Draft version of the Characterization Test Plan. - The selected candidate proponent submits the Detailed description of the algorithm |
|  | Final version of the Characterization Test Plan available on SG16 IFA. |
|  | Start the organization of the Characterization Phase (based on MoUs and NDA as done in the Qualification Phase). |
|  | Start of Host Laboratory sessions. |
|  | End of Host Laboratory sessions. |
|  | Start of Listening Laboratory sessions. |
|  | End of Listening Laboratory sessions. |
|  | Each Listening Laboratory submits a report (in the form of a technical contribution) with test results. Global analysis performed. |
| Study Group 16 meeting February 2002 | - Evaluation of the results of the Characterization Phase. - Completion of the algorithm implementation verification test procedures. - Decision of Recommendation. |

| Date | Contractual Activity |
| --- | --- |
| 17 November 2000 | Letter Of Intent (LOI) available on SG16 Informal FTP Area (IFA). (Note: The LOI will contain the intention to submit a candidate algorithm to the ITU-T Selection process and the intention to follow item 2.2 of the ITU-T patent policy) |
| end Nov. 2000 | Draft version (without price and work allocation) of MoU (A), MoU (B) and MoU (C) and NDA available on IFA. (Note: MoU(A) will be signed between each candidate proponent and the ITU-T, i.e. one per candidate. MoU(B) will be signed between the host lab and the ITU-T. MoU(C) will be signed between the listening lab and the ITU-T. The NDA requires the host and cross-checking laboratories to keep the candidate executables in confidence. Global Analysis Laboratory work may require a separate MoU) |
| 1 December 2000 | LOI received by ITU-T TSB from Candidate Proponents. |
| 4 December 2000 | Information on the LOI received is made available on the e-mail reflector (tsg16q20@itu.int) |
| 15 January 2001 | If a Candidate Proponent whishes to withdraw its ITU-T Wideband Candidate Algorithm it must inform the TSB (Fabio.Bigi@itu.int and Thierry.Perewostchikow@itu.int) via fax and e-mail, and Q.20/16 Rapporteur (rosario.drogodeiacovo@cselt.it) and WP3/16 Chairman (simao.campos@labs.comsat.com) via e-mail not later than close of business for the recipients on 15 January 2001. |
| 19-23 February 2001 (SG12 meeting) | - Pricing and work allocation defined. - Final version of MoU (A), MoU (B), and MoU (C) prepared and uploaded to IFA. |
| 2 March 2001 | - Two signed copies of MoU(A) received by ITU-T TSB from Candidate Proponents (via fax, original mailed on the same date). - Two signed copies of NDA received by Host Lab and Cross-checking Lab from each Candidate Proponent (if required). |
| 9 March 2001 | Completed copies of NDA received by Candidate Proponents from Host Lab and Cross-checking Lab (if required). |
| 12 March 2001 | Two signed copies of MoU(B) and MoU (C) received by ITU-T TSB from Host Laboratory and Listening Laboratory, respectively. |
| 19 March 2001 | Selection costs placed in ITU-T bank account by close of business (proof of transfer may be requested). |

- Annex Q21.A
Summary of 4-kbit/s codec floating-point selection results

In the following Tables CuT denotes Codec under Test.

Table Q21.A.1 
Codec overview.

| Proponent | CuT | Algorithm | Frame- size/ms | Look-ahead/ms | Total delay/ms |
| --- | --- | --- | --- | --- | --- |
| AT&T, Conexant, DT, FT, Matsushita, NTT | A | eX-CELP | 20 | 12.5 (encoder) +/- 2.5ms | 52.5 +/- 2.5ms |
| Ericsson, Nokia, TI | B | MELP/CELP | 20 | 15 (encoder) | 55 |
| LMGT, Samsung | C | HE-LPC | 20 | 10 (encoder) 5 (decoder) | 55 |

Table Q21.A.2 
Codec Complexity.

| CuT | MIPS | ROM/kword | RAM/kword | Noise Suppression |
| --- | --- | --- | --- | --- |
| A | <38 | 6 (program) 9 (table) | 7 | Yes |
| B | 70 | 20 (program) 25 (table) | 5 | Yes |
| C | 40 | 10 (program) 30 (table) | 5 | Yes |

Table Q21.A.3 
Requirements achieved.

| Requirements | Exp | Language | CuT A | CuT B | CuT C |
| --- | --- | --- | --- | --- | --- |
|  | 1 | NAE | n | Pass | - |
| Normal Level (-26 dBov) | 1 | F | n | - | - |
|  | 1 | J | Pass | Pass | - |
|  | 1 | NAE | - | Pass | - |
| High Input Level (-16 dBov) | 1 | F | n | n | - |
|  | 1 | J | Pass | Pass | - |
|  | 1 | NAE | - | n | - |
| Low input Level (-36 dBov) | 1 | F | Pass | Pass | - |
|  | 1 | J | Pass | Pass | - |
|  | 1 | NAE | Pass | Pass | - |
| Tandem | 1 | F | Pass | Pass | - |
|  | 1 | J | Pass | Pass | Pass |
|  | 2 | G | Pass | Pass | - |
| 3% Frame erasure | 2 | J | Pass | Pass | Pass |
|  | 2 | S | Pass | Pass | Pass |
| Vehicle noise at 15dB SNR | 3a | BE | Pass | Pass | - |
|  | 3a | J | Pass | Pass | - |
| Babble noise at 30dB SNR | 3b | NAE | Pass | Pass | - |
|  | 3b | F | Pass | Pass | - |
| Interfering Talker at 20dB | 3c | BE | - | - | - |
|  | 3c | S | - | - | - |

Notes to table:

- “Exp” denotes in which experiment the requirement was tested.

- Key to languages: BE - British English; F - French; G - German; J - Japanese; NAE - North American English; S - Spanish.

- “Pass” denotes a pass at the 95% confidence interval; “n” denotes a pass at the 99% confidence interval; “-” denotes a fail.

- Annex Q21.B
Proposed schedule for ITU-T 4-kbit/s Speech Coding Algorithm

The revised skeleton schedule for the development of a 4-kbit/s speech-coding algorithm is as follows. Due to the lack of a stable study group meeting calendar at SG16 in November 2001, this schedule and the use of the alternative approval procedure (AAP) are provisional.

| SG16 Nov 2000 | - | Identification of two candidates final selection |
| --- | --- | --- |
| Jan 2001 | - | Q.21/16 agreement of test conditions by correspondence |
| SG12 Feb 2001 | - | Completion of selection test plan by SQEG |
|  | - | Organisation of the host laboratory (software); |
|  | - | Organisation of listening laboratory sessions; |
| SG16 May 2001 | - | Review of draft selection test plan |
|  | - | Definition of global analysis |
|  | - | Definition of complexity evaluation rules |
| 1 August 2001 | - | Submission of fixed-point executables; |
| Oct 2001 | - | Global analysis results available two weeks prior to Rapp. Meeting |
| Nov 2001 | - | wMOPS based complexity evaluation available; |
| (Rapp. Meeting) | - | Review of selection test results and pre-selection deliverables (note 1); |
|  | - | Selection of one candidate; |
| (post selection) | - | Draft text of Recommendation available for review (note 2) |
|  | - | Fixed-point ANSI C-code available for review (note 2) |
| SG12 [tbd] | - | Completion of characterisation test plan by SQEG |
|  | - | Organisation of host laboratory session (software); |
|  | - | Organisation of listening laboratory sessions; |
|  | - | Commence characterisation phase; |
| SG16 Feb 2002 | - | Review of characterisation test results; |
|  | - | Initiation of alternative approval process (AAP) |
| April 2002 | - | G.4k approved |

Notes to schedule:

1.	Proponents must provide the following four pre-selection deliverables for their submission to be accepted as a candidate:

- A high level description comprising the following parts: description of the codec algorithm (formulas are not necessary); block diagrams of the encoder and the decoder; allocation of bits for each coded parameter; a clear description of any noise suppression techniques employed in the codec.

- Complexity figures derived using the evaluation rules agreed by Q.21/16. These figures will comprise wMOPS (worst-case and average), RAM, table ROM, and an estimate of program ROM.

- An analysis of the encoder and decoder delay.

- A verbal indication of intent regarding intellectual property with respect to the ITU-T patent policy.

2.	Prior to the selection meeting, the candidates proponents must deliver the following two deliverables to the TSB, where they will be kept in confidence. After the selection of a single codec, these deliverables will be made available for review (in the case of the C-code this may by under a software agreement until the final approval of the recommendation):

- Text for the paper component of G.4k that provides sufficient algorithmic detail to enable an implementer to produce a non-bit exact implementation of the algorithm, i.e. all low-level steps and equations, but not quantiser values. This text will be used as the first draft of the recommendation, and should be formatted according to the ITU-T drafting guidelines.

- Fixed-point ANSI C-code that provides the normative bit-exact description of the G.4k speech coding algorithm. In the event that bugs are found in the executable delivered to the host labs, the proponent must deliver two versions of the C-code: the first version should be provide bit-exact operation with the executable tested in the selection phase

- Annex Q21.C
Key Requirements and Objectives

The following list identifies those requirements and objectives that the Q.21/16 experts have identified to be of particular importance in the development of the 4-kbit/s speech coding algorithm.

Key requirements:

- Speech quality at normal input level (-26dBov)

- Speech quality at high input level (-16dBov)

- Speech quality at low input level (-36dBov)

- Talker dependency

- Performance with 2 tandem encodings

- Detected frame erasures (3% random)

- Performance with vehicle noise (15 dB SNR)

- Performance with babble noise  (30 dB SNR)

- Performance with interfering talker (20 dB SNR)

- Bit rate (4-kbit/s)

- Variable bit-rate operation needed

- Delay ( 55 ms)

- Complexity (single DSP)

- Bit-exact implementation (G.729 operators)

- Specification (bit-exact ANSI C-code)

Key objectives:

- Performance with random bit errors (2x10-4)

- Detected frame erasures (3% bursty)

- Complexity (as low as possible)

- Interoperation with floating-point needed

- Annex Q22.A
UGST Action Points as of November/2000

| Number | - Status | Description | Task Force |
| --- | --- | --- | --- |
| 9606.03 | Stalled | Verify PCM-domain multiplier and tone and noise generation tools | S.Campos/LMGT |
| 9606.04 | Stalled | General Processing Framework tool | S.Campos/LMGT |
| 9801.01 | Stalled | Investigate the P.56 problem and suggested changes by Mr.Kabal | S.Campos/LMGT |
| 9801.02 | Stalled | Investigate the G.711’s 1’s/2’s complement issue | S.Campos/LMGT |
| 9801.0 | Stalled | Solve inconsistencies in IS54 code | J.Stegmann/T-Nova S.Campos/LMGT |
| 9809.02 | Stalled | Add filters to speech voltmeter demo programs | S.Campos/LMGT |
| 0002.01 | Ongoing | Ad-hoc Group on Basic Operators: - review weights - consider alternative set of basic operators | R.Salami/Canada |
| 0011.01 | New | Correct error in fir_downsampling_kernel() [in fir-lib.c] with one-sample operation |  |
| 0011.02 | New | Correct run-time problem in gen-patt.c and ep-stats.c with Win32/cl and Win32/gcc |  |
| 0011.03 | New | Discuss a frequency-response calculation tool |  |
| 0011.04 | New | Prepare STL2000 User’s Manual | S.Campos/LMGT |

In the next Study Period, the following SG16 WP3 question renumbering will occur: Q.15 is continued in Q.6; Q.20 in Q.7; Q.21 in Q.8; Q.24 in Q.9. Q.22 and maintenance aspects of Q.19 will be dealt with in Q.10.

NOTE: This portion of the STL2000 is non-normative.

Subscription requests should be sent to <simao.campos@labs.comsat.com>