| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001 | Document VCEG-M07 Filename: VCEG-M07.doc Generated: 23 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wiegand Image Processing Department Heinrich Hertz Institute Einsteinufer 37, D-10587 Berlin Germany | Tel: Fax: Email: | +49 30 31 002 617 +49 30 392 72 00 wiegand@hhi.de |
| Title: | Report of the Ad Hoc Committee on H.26L Profiles and Applications |  |  |
| Purpose: | Information |  |  |

There was not much activity on the reflector. Only one e-mail has been sent to the reflector which is repeated below.

ITU-T Video Coding Experts Group (VCEG) [Q.6/SG16]

Email from Thomas Wiegand <wiegand@hhi.de>:

Dear experts,

we need to start a discussion about Profiles, Levels,

and Versions of H.26L. This e-mail is intended to get

the discussion going.

I would like to refer first to D. Lindbergh's and

S. Wenger's document (VCEG-L33r5) that contains

profile, level and version definitions.

In VCEG-L33r5, Profiles are defined by a particular

tradeoff between:

.	Encoder RAM

.	Encoder ROM

.	Encoder MIPS

.	Decoder RAM

.	Decoder ROM

.	Decoder MIPS

.	Latency

.	Error resilience

The document VCEG-L33r5 needs to be harmonized with

VCEG-L34d1 containing "Application profiles".

Here is what I think:

.     Profiles are defined by syntax components of the VCL.

(The role of the NAL should be discussed.)

Profiles are distinguished by the following parameters

* Delay: interactive / non-interactive

* Error resilience: normal / important

Transmission scenarios corresponding to identical syntax

but different parameter settings should be put into the

same Profile but may be distinguished by Levels.

For each level (or groups of levels) we should specify a

test scenario.

.     Levels (within a Profile) are distinguished by

* Decoder RAM

* Decoder ROM

* Decoder MIPS

In contrast to the above definition, there are no encoder parameters

in the profile spec. Furthermore, only two criteria distinguish profiles:

delay and error resilience. The decoder performance spec is a level

parameter. The reason for this different definition is that only a small

number of profiles (maximum 4) result from it.

Given the above profile definition, a specification of baseline and

3 more Profiles can be provided.

* Profile 0: Baseline

<Profile Definition>

The Baseline mode is the common mode supported

by all H.26L implementations, without exception.

Baseline supports video communication between all

ITU-T H.32x series systems Recommendations

(H.320, H.323, H.324).

(This is from D. Lindbergh's and S. Wenger's document:

[VCEG-L33r5]. I support it. However, it would be nice

to have something more generic than "ITU-T H.32x series

systems".)

Baseline enables INTERACTIVE video communication for

transport scenarios with FEW errors. It contains all

coding efficiency features that do not increase a

pre-defined end-to-end delay.

<Application to TML-6>

Baseline contains everything from TML-6 except for

- B-pictures

-

<Levels>

- ... tbd

<Applications>

- Video telephony between H.32x series systems

- H.320 Conversational (from VCEG-L34d1) (CCs: Q15-I-62)

- ...

* Profile 1: NON-INTERACTIVE communication and FEW errors

<Profile Definition>

This profile includes all baseline features plus all

features that introduce additional delay and enhance

coding efficiency.

<Application to TML-6>

This profile contains

- Baseline

- B-pictures

<Levels>

- ... tbd

<Applications>

- Entertainment Satellite/DVD (from VCEG-L34d1) (CCs: tbd)

Note: I intentionally for the moment left out the

interlaced coding issue here.

- ...

* Profile 2: INTERACTIVE communication and MANY errors

<Profile Definition>

This profile includes all baseline features plus all

features to enhance error resilience that that do not

increase a pre-defined delay.

<Application to TML-6>

This profile contains

- Baseline

- ...

<Levels>

- ... tbd

<Applications>

- 3GPP Conversational IP/RTP/SIP (from VCEG-L34d1) (CCs: tbd)

- 3GPP Conversational H.324/M (from VCEG-L34d1) (CCs: Q15-I-60)

- H.323 Conversational Internet/unmanaged/best effort IP/RTP

(from VCEG-L34d1) (CCs: Q15-I-61)

- ...

* Profile 3: NON-INTERACTIVE communication and MANY errors

<Profile Definition>

This profile includes all baseline features plus all

features that introduce additional delay and enhance

coding efficience and error resilience.

<Application to TML-6b>

This profile contains

- Baseline

- Profile 1

- Profile 2 (?)

<Levels>

- ... tbd

<Applications>

- 3GPP Streaming IP/RTP/RTSP (from VCEG-L34d1) (CCs: tbd)

- Streaming IP/RTP/RTSP (without TCP fallback) (from VCEG-L34d1)

- ...

Note: As mentioned in the beginning:

These are thoughts on the subject to start a discussion and are maybe subject to change.

--Thomas