| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (VCEG) _________________ 13th meeting, Austin, Texas, USA, April 2-4, 2001 | Document VCEG-M21 Filename: VCEG-M21.doc Generated: 27 Mar 2001 |
| --- | --- |

| Question: | Q6/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Jani Lainema, Marta Karczewicz Nokia Research Center 6000 Connection Drive Irving, TX 75039, U.S.A. | Tel: Fax: Email: | +1 972 894 4045 +1 972 894 4589 jani.lainema@nokia.com |
| Title: | Core Experiment results on Low Complexity Loop Filtering |  |  |
| Purpose: | Information / Proposal |  |  |

_____________________________

# Introduction

This document reports the results of the Core Experiment on Low Complexity Loop Filtering (defined in VCEG-L40).

# Results

Six experiments were carried out according to the common test conditions (VCEG-L38). Complexity and coding efficiency of the following were measured and are reported later in this document:

- Loop filter in TML 5.9 software

- As 1 but with software optimizations not affecting the bitstream syntax.

- As 2 but with combined filtering of the edge pixels (applying the same difference for both pixels as in TML-5).

- As 2 but with direct calculation of the differences prior to clipping (as in TML-5).

- As 2 but with removal of dependencies on 'chrominance strengths'.

- TML 5.9 with all the optimizations above.

Full simulation results and additional PSNR curves comparing Experiments 1 and 6 can be found in the accompanying spreadsheet VCEG-M21.xls.

## Experiment 1: TML 5.9 Loop Filter

Following table summarises the complexity of the TML 5.9 loop filter:

Table 1: TML 5.9 Loop Filter complexity (operations per Macroblock)

| QP | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average | Worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 2247 | 7071 | 2807 | 2985 | 3110 | 6330 | 6306 | 4408 | 28544 |
| 20 | 1455 | 6657 | 2548 | 2751 | 2754 | 5947 | 5289 | 3914 | 28544 |
| 24 | 1050 | 6098 | 2207 | 2362 | 2310 | 4902 | 4014 | 3278 | 28544 |
| 28 | 814 | 5739 | 1951 | 2059 | 1993 | 4030 | 3227 | 2830 | 28544 |
| Average | 1392 | 6391 | 2378 | 2539 | 2542 | 5302 | 4709 | 3608 | 28544 |

## Experiment 2: Software optimizations

In this experiment TML 5.9 loop filter software was optimized. Optimizations included reshuffling the source code and simplifying some calculations. This did not have any effect on the bitstreams syntax or compression performance. Results are given in the two tables below.

Table 2: Optimized TML 5.9 Loop Filter complexity relative to TML 5.9 complexity

| QP | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average | Worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 0.88 | 0.90 | 0.88 | 0.89 | 0.88 | 0.80 | 0.85 | 0.87 | 0.95 |
| 20 | 0.89 | 0.92 | 0.90 | 0.92 | 0.89 | 0.83 | 0.87 | 0.89 | 0.95 |
| 24 | 0.90 | 0.94 | 0.91 | 0.93 | 0.91 | 0.86 | 0.89 | 0.90 | 0.95 |
| 28 | 0.91 | 0.95 | 0.92 | 0.95 | 0.92 | 0.89 | 0.92 | 0.92 | 0.95 |
| Average | 0.89 | 0.93 | 0.90 | 0.92 | 0.90 | 0.85 | 0.88 | 0.90 | 0.95 |

Table 3: Summary of Experiment 2 results

| Average Complexity Reduction | 10% |
| --- | --- |
| Luma PSNR improvement (sweet spot) | 0.00 |
| Luma PSNR improvement | 0.00 |
| Chroma PSNR improvement (sweet spot) | 0.00 |
| Chroma PSNR improvement | 0.00 |

## Experiment 3: Combined filtering of the edge pixels

This experiment is exactly as the one above except instead of separately filtering the edge pixels they are jointly filtered as in TML 5. A common difference is calculated and applied to both edge pixels on the 4x4 grid boundary (C, D, E, F are used to denote neighboring pixels, the boundary being between D and E):

Difference = clip( 32*(E-D) + 10*(C-F)+32) >> 6 );

filtered D = clip0to255( D + Difference);

filtered E = clip0to255( E - Difference);

Two additional clipping operations are needed to keep the filtered values in 0 to 255 range, but as seen on the tables below the overall complexity is reduced and some gains are achieved in compression efficiency.

Table 4: Experiment 3 complexity relative to Experiment 2 complexity

| QP | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average | Worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 0.89 | 0.90 | 0.90 | 0.89 | 0.89 | 0.91 | 0.89 | 0.90 | 0.96 |
| 20 | 0.91 | 0.89 | 0.90 | 0.90 | 0.90 | 0.90 | 0.89 | 0.90 | 0.96 |
| 24 | 0.92 | 0.90 | 0.91 | 0.92 | 0.91 | 0.89 | 0.90 | 0.91 | 0.96 |
| 28 | 0.89 | 0.90 | 0.90 | 0.91 | 0.90 | 0.87 | 0.89 | 0.89 | 0.96 |
| Average | 0.90 | 0.90 | 0.90 | 0.90 | 0.90 | 0.89 | 0.89 | 0.90 | 0.96 |

Table 5: Experiment 3 PSNR differences with respect to TML 5.9

|  | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Luma Sweet Spot | -0.01 | 0.08 | -0.02 | 0.04 | 0.02 | 0.06 | 0.04 | 0.03 |
| Luma Total | -0.01 | 0.07 | -0.02 | 0.04 | 0.02 | 0.06 | 0.05 | 0.03 |
| Chroma Sweet Spot | N/A | 0.13 | N/A | 0.10 | 0.22 | 0.16 | 0.15 | 0.15 |
| Chroma Total | N/A | 0.14 | N/A | 0.11 | 0.21 | 0.15 | 0.15 | 0.15 |

Table 6: Summary of Experiment 3 results

| Average Complexity Reduction | 10% |
| --- | --- |
| Luma PSNR improvement (sweet spot) | 0.03 |
| Luma PSNR improvement | 0.03 |
| Chroma PSNR improvement (sweet spot) | 0.15 |
| Chroma PSNR improvement | 0.15 |

## Experiment 4: Direct calculation of all differences prior to clipping

In TML filter the difference applied to input values is limited by a clipping function. In the current design the difference was evaluated by first calculating an intermediate filter output and taking a difference between this and the input value. The same result can be obtained by directly calculating the differences for the pixels not on the 4x4 grid boundaries as follows (B, C, D, E, F, G are used to denote neighboring pixels, the boundary being again between D and E):

Difference1 = clip( 21*(B+D)-42*C + 32) >> 6);

filtered C  = C + Difference;

Difference2 = clip( 21*(E+G)-42*F + 32) >> 6);

filtered F  = F + Difference;

This saves 2 operations every time pixels C and F are being filtered this way.

## Experiment 5: Removal of dependencies on 'chrominance strengths'

In this experiment the dependencies on chrominance strengths were removed and luminance strengths were used in filtering process for all the components. Reasonable complexity reductions were achieved especially on color rich sequences as seen below.

Table 7: Experiment 5 complexity relative to Experiment 2 complexity

| QP | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average | Worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 1.14 | 0.97 | 0.99 | 1.00 | 0.92 | 0.95 | 0.98 | 0.99 | 1.00 |
| 20 | 1.09 | 0.95 | 0.97 | 0.99 | 0.93 | 0.88 | 0.94 | 0.96 | 1.00 |
| 24 | 1.06 | 0.94 | 0.94 | 0.98 | 0.91 | 0.76 | 0.87 | 0.92 | 1.00 |
| 28 | 1.01 | 0.93 | 0.92 | 0.96 | 0.87 | 0.67 | 0.81 | 0.88 | 1.00 |
| Average | 1.08 | 0.95 | 0.95 | 0.98 | 0.91 | 0.82 | 0.90 | 0.94 | 1.00 |

Table 8: Experiment 5 PSNR differences with respect to TML 5.9

|  | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Luma Sweet Spot | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 | 0.04 | 0.01 | 0.01 |
| Luma Total | -0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.06 | 0.01 | 0.01 |
| Chroma Sweet Spot | N/A | -0.01 | N/A | -0.03 | 0.03 | 0.10 | 0.02 | 0.02 |
| Chroma Total | N/A | -0.01 | N/A | -0.05 | 0.02 | 0.09 | 0.01 | 0.01 |

Table 9: Summary of Experiment 5 results

| Average Complexity Reduction | 6% |
| --- | --- |
| Luma PSNR improvement (sweet spot) | 0.01 |
| Luma PSNR improvement | 0.01 |
| Chroma PSNR improvement (sweet spot) | 0.02 |
| Chroma PSNR improvement | 0.01 |

## Experiment 6: All the optimizations combined

In this experiment all the optimizations mentioned above were integrated to the TML 5.9. Results are listed in the tables below. More exact simulation results and PSNR curves can be found in VCEG-M21.xls.

Table 10: Experiment 6 complexity (operations per Macroblock)

| QP | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average | Worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 2041 | 5612 | 2207 | 2389 | 2275 | 4471 | 4780 | 3396 | 25856 |
| 20 | 1287 | 5224 | 2002 | 2255 | 2072 | 3951 | 3921 | 2959 | 25856 |
| 24 | 923 | 4831 | 1711 | 1997 | 1746 | 2938 | 2852 | 2428 | 25856 |
| 28 | 676 | 4547 | 1481 | 1699 | 1458 | 2181 | 2180 | 2032 | 25856 |
| Average | 1232 | 5054 | 1850 | 2085 | 1888 | 3385 | 3433 | 2704 | 25856 |

Table 11: Experiment 6 complexity relative to TML 5.9

| QP | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average | Worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 0.91 | 0.79 | 0.79 | 0.80 | 0.73 | 0.71 | 0.76 | 0.78 | 0.91 |
| 20 | 0.88 | 0.78 | 0.79 | 0.82 | 0.75 | 0.66 | 0.74 | 0.78 | 0.91 |
| 24 | 0.88 | 0.79 | 0.78 | 0.85 | 0.76 | 0.60 | 0.71 | 0.77 | 0.91 |
| 28 | 0.83 | 0.79 | 0.76 | 0.83 | 0.73 | 0.54 | 0.68 | 0.74 | 0.91 |
| Average | 0.88 | 0.79 | 0.78 | 0.82 | 0.74 | 0.63 | 0.72 | 0.77 | 0.91 |

Table 12: Experiment 6 PSNR differences with respect to TML 5.9

|  | Cont | Fore | News | Sile | Pari | Mobi | Temp | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Luma Sweet Spot | -0.02 | 0.08 | -0.02 | 0.04 | 0.01 | 0.08 | 0.04 | 0.03 |
| Luma Total | -0.02 | 0.07 | -0.02 | 0.04 | 0.01 | 0.09 | 0.05 | 0.03 |
| Chroma Sweet Spot | N/A | 0.14 | N/A | 0.08 | 0.14 | 0.18 | 0.12 | 0.13 |
| Chroma Total | N/A | 0.15 | N/A | 0.05 | 0.14 | 0.17 | 0.11 | 0.12 |

Table 13: Summary of Experiment 6 results

| Average Complexity Reduction | 23% |
| --- | --- |
| Luma PSNR improvement (sweet spot) | 0.03 |
| Luma PSNR improvement | 0.03 |
| Chroma PSNR improvement (sweet spot) | 0.13 |
| Chroma PSNR improvement | 0.12 |

# Summary

Results obtained suggest that very reasonable complexity reductions can be achieved by optimizing the TML 5.9 loop filter. They also show that the compression efficiency can be maintained or even slightly improved with these optimizations. A visual demonstration comparing the optimized filter and the original TML 5.9 filter will be provided in the Austin meeting.

Given the promising results, authors of this contribution would like to ask Video Coding Experts Group to adopt the proposed optimizations to the next version of TML.