| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, TX, 02-04 April, 2001 | Document VCEG-M38 Filename: VCEG-M38.doc Generated: Mar ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Ragip Kurceren and Marta Karczewicz Nokia Research Center 6000 Connection Drive Irving, TX, 75063 | Tel: Fax: Email: | +1 972 894 6907 +1 972 894 4589 Ragip.Kurceren@nokia.com |
| Title: | Further Results for SP-frames |  |  |
| Purpose: | Proposal: H.26L |  |  |

# Introduction

In VCEG-L27[1] a new picture type, SP-picture, has been proposed. The proposed picture type has the property that identical SP-pictures can be obtained even when they are predicted using different reference frames. This feature provides functionalities for bitstream switching, random access, error recovery/resilience, fast forward and fast backward. Prior art solutions for some of these applications include I-frames and S-frames [2]. In [1] the results of coding efficiency of SP pictures are reported in comparison with I-frames while in this document, in response to comments and discussions on SP-frames [1] during Eibsee Meeting, we provide comparison with S-frames.

# S-frames

Farber et al.[2] introduced S-frames which are used during switching between bitstreams. Figure 1 depicts a switching example from bitstream 2 to bitstream 1 at time n using an S-frame. S-frame is generated by encoding the nth reconstructed frame in bitstream 1 using motion compensated prediction. The reference frame is the (n-1) st frame from bitstream 2.

Figure 1. S-frame architecture.

Note that S-frame and the n th reconstructed frame in bitstream 1 are using different reference frames. Therefore unlike I-pictures and SP-pictures, S-pictures introduce drift (the discrepancy between the reconstructed frames in the decoder and encoder). This discrepancy is not limited to the reconstructed frames the switch is occurring at. It may propagate to the future frames due to the use of:

- Motion-compensated predictive coding

- Loop filter

- Intra coding with prediction.

To minimize drift, the quantization parameter used for the S-frame should be kept small. In the following simulations, QP=3 is used. This choice conforms with the QP value suggested in [2] to ensure that the initial PSNR difference is below 0.2 dB.

Figure 2 Illustration of switching with S-frames, QP (S-frame) =3, QP1=19 and QP2=13.

In Figure 2 examples of switching between bitstreams using S-frames are presented. The PSNR profiles of bitstream encoded with QP=19, bitstream encoded with QP=13 and 2 bitstreams created by switching between them are given. At both times, we switch from the bitstream encoded with QP=19 to the bitstream encoded with QP=13 but at different frames, namely at 10th and 20th frame. As can be seen from Figure 2, PSNR values of the reconstructed frames after the switch diverge away from the values of the frames from the "target" bitstream – bitstream that we are switching to. Moreover, the drift becomes more pronounced when multiple switches occur, as illustrated in Figure 3 – it becomes larger than 1.5 dB after the last switch between bitstreams. The drift could be reduced by using even smaller QP values for S-pictures however that would increase S-picture sizes, which as it will be shown in the next section, are already substantial.

Figure 3 Multiple Switching between bitstreams with S-frames, QP(S-frame)=3, QP1=19 and QP2=13.

# Comparison with SP-frames

In this section we present a brief comparison of SP- and S- frames. Table 1 and Table 2 list the average frame sizes of S- and SP-frames. The average is taken over 10 S-frames (SP-frames) used to switch between 2 bitstreams at different locations. We switch from the bitstream encoded with QP1 to the bitstream encoded with QP2 where the values of QP1 and QP2 are given next to the sequence name in the first column of Table 1 and Table 2. Notice that in Table 1 the switch always takes place from the lower to higher quality bitstream and in Table 2 from the higher to lower quality bitstream. In both cases sizes of S-frames are considerably larger than that of SP-frames. The differences are larger when the switch occurs from higher to lower quality bitstream. For example, for the "news" sequence, the average size of S-frame is 3.4 times larger than that of SP-frame when QP1=25 and QP2=19 (Table 1) and 6.18 times larger when QP1=19 and QP2=25 (Table 2). Similar results can be observed for the rest of the sequences.

| Sequence (QP1-QP2) | S-frame size | SP-frame Size, S12 |
| --- | --- | --- |
| Foreman (22-16) | 68938 | 28304 |
| Hall (19-13) | 58290 | 26329 |
| Container (19-13) | 60808 | 28430 |
| News (25-19) | 69921 | 20610 |

Table 1 Comparison of average I, S and SP-frame sizes that would be used during switching from bitstream encoded with QP1 to the bitstream with QP2.

| Sequence (QP1-QP2) | S-frame size | SP-frame Size, S12 |
| --- | --- | --- |
| Foreman (16-22) | 61950 | 15563 |
| Hall (13-19) | 55409 | 13694 |
| Container (13-19) | 52973 | 14485 |
| News (19-25) | 66566 | 10766 |

Table 2 Comparison of average I, S and SP-frame sizes that would be used during switching from the better quality bitstream.

| Sequence (QP1-QP2) | Periodic I-frame |  | S-frames |  | SP-frames |  | % Differ. Btw.SP and I-frames | % Differ. Btw.SP and S-frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNR | Total Bits | PSNR | Total Bits | PSNR | Total Bits |  |  |
| Container (25-19) | 33.17 | 294561 | 32.84 | 412124 | 32.87 | 229422 | 28 | 80 |
| News (25-19) | 33.48 | 466260 | 33.28 | 586703 | 33.11 | 389091 | 20 | 51 |
| Foreman (22-16) | 35.27 | 894110 | 35.08 | 1012345 | 35.13 | 857023 | 4 | 18 |
| Hall (19-13) | 38.20 | 740551 | 37.91 | 721157 | 38.08 | 594827 | 24 | 21 |

Table 3 PSNR and Total Bits over 100 frames for the multiple switching example with S and SP-frames.

In the following we investigate the influence of S- and SP- frames sizes on the average PSNR and bitrate of the sequence when there are multiple switches between bitstreams. The bitstream for which PSNR and bitrate is measured is generated by switching 4 times between 2 different quality bitstreams. Switch occurs every other second and the first switch tales place from the higher to lower quality bitstream. Frame rate of all the sequences is 10 frames/sec. Also included in Table 3 is the performance of the periodic intra-coding approach. Here, one frame every second is encoded as an I-frame (coding efficiency of this scheme in comparison with SP-frames were reported in [1]). It can be noted that the total number of bits required with SP-frames is considerably less than both with S-frame and periodic intra-coding approaches. For example, for the "container" sequence approximately 1.8 times or 80 percent more bits are required with S-frames while PSNR of SP approach is still slightly higher. Similarly, periodic-intra approach requires 1.3 times more bits than the SP-frame approach.

In Table 4 we further illustrate the performance of each scheme when there is no switching between bitstreams. These results are previously reported in [1] and repeated here for the completeness of the current document.

| Sequence (QP) | Periodic I-frames |  | S-frames |  | SP-frames |  | % Differ. Btw.SP and I-frames | % Differ. Btw.SP and S-frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNR | Total Bits | PSNR | Total Bits | PSNR | Total Bits |  |  |
| Container (25) | 30.82 | 183651 | 30.59 | 91655 | 30.41 | 109566 | 67 | -16 |
| Container (19) | 34.78 | 371338 | 34.57 | 208643 | 34.53 | 232508 | 60 | -10 |
| News (25) | 30.88 | 305617 | 30.75 | 199771 | 30.47 | 217500 | 41 | -8 |
| News (19) | 35.23 | 578068 | 35.14 | 408082 | 34.9 | 432388 | 34 | -6 |
| Foreman (22) | 32.73 | 562057 | 32.74 | 476294 | 32.54 | 507101 | 11 | -6 |
| Foreman (16) | 36.96 | 1132016 | 36.94 | 973322 | 36.85 | 1020080 | 11 | -5 |
| Hall (19) | 35.65 | 436376 | 35.69 | 264648 | 35.5 | 289364 | 51 | -8 |
| Hall (13) | 39.91 | 953771 | 39.96 | 676407 | 39.91 | 711762 | 34 | -5 |

Table 4 PSNR and Total Bits over 100 frames when there is no switching with S and SP-frames and periodic-intra coding.

It can be seen from Table 4 that when there is no switching, the performance of periodic-intra coding is significantly lower than that of S and SP-frames.  Note that S-frame approach has the best coding efficiency that is the P-pictures performance in this case. The difference between S-picture and SP-picture coding efficiency is however quite small.

# Summary

We have illustrated the resulting performances of different schemes that are being used for switching between bitstreams. The first case considered, S-frame approach, introduces drift which becomes significant when there are multiple switches between bitstreams. May be more importantly, the switching frames used in S-frame approach are very large. As the number of switching between bitstreams increases, PSNR vs. bitrate performance significantly degrades.  Furthermore, since the S-frames are very large, it is necessary to employ large buffers on the receiver side.

SP-frames, on the other hand, provide drift-free switching between bitstreams and their sizes are considerably smaller than S-frames. In case when there is no switching between bitstreams, S-frames provide slightly better performance than SP-frames. However, with multiple switching, SP-frames provide significantly better PSNR vs. bitrate performance than S-frames. We have also included results of the periodic intra-coding approach. It is again noted that SP-frames provide better PSNR vs bitrate performance. In [3] we present a modified SP-frame encoding that provides flexibility to improve the coding efficiency of SP-frames.

# References

[1] M. Karczewicz and R. Kurceren, " A Proposal for SP-frames", document VCEG-L-27, ITU-T Video Coding Experts Group Meeting, Eibsee, Germany, 09-12 January 2001.

[2] N. Farber and B. Girod, " Robust H.263 compatible video transmission for mobile access to video services", International Conference on Image Processing, ICIP'97, Santa Barbara, CA, Oct.1997.

[3] M. Karczewicz and R. Kurceren, " Improved Coding Efficiency for SP-frames", document VCEG-M-38, ITU-T Video Coding Experts Group Meeting, Austin, TX, 02-04 April 2001.