| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Twelfth Meeting: Austin, USA, 2-5 March, 2001 | Document VCEG-M51 Filename: VCEG-M51.doc Generated: 21 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Marta Karczewicz Nokia Research Center, Irving, Texas 75039, USA | Tel: Fax: Email: | +972 894 4188 +972 894 4589 marta.karczewicz@nokia.com |
| Title: | Improved Intra coding |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

Improved method of encoding transform coefficients for Intra 4x4 mackroblocks for QP>24 is presented. It is based on idea of using additional VLC tables based on UVLC described in [1]. Unlike in TML single scan is used.

# New VLC

In addition to Universal VLC [2] two more VLCs are used. They will be called UVLC2 and UVLC3. The new VLCs are constructed as proposed in [1] by using codeword “1” (UVLC2) or codewords "1" and "11" (UVLC3) as an optional prefix to the rest of the codewords. See Table 1 for a comparison between UVLC, UVLC2 and UVLC3. The correspondence between run and level of the coefficient and the codeword number for each VLC table is given in Table 2.

Table 1: The first nine codewords of UVLC, UVLC2 and UVLC3.

| Index | UVLC | UVLC2 | UVLC3 |
| --- | --- | --- | --- |
| 0 | 1 | 11 | 111 |
| 1 | 001 | 001 | 001 |
| 2 | 011 | 011 | 011 |
| 3 | 00001 | 1001 | 1001 |
| 4 | 00011 | 1011 | 1011 |
| 5 | 01001 | 00001 | 11001 |
| 6 | 01011 | 00011 | 11011 |
| 7 | 0000001 | 01001 | 00001 |
| 8 | 0000011 | 01011 | 00011 |

# Switching between UVLC, UVLC2 and UVLC3

The decision which table should be used when decoding the coefficients with exception of the first one is done as follows. Calculate two indices r and l using the run and level information of the last decoded coefficient:

if (run>6)

r=6;

else

r=run+1;

end

if (level>5)

l=5;

else

l=level;

end

These indices are used to access table M given below. Entries of this table M(l,r) indicate which VLC should be used, i.e., entry 1 – UVLC, 2 – UVLC2 and 3 –UVLC3.

|  | r |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| l | 2 | 2 | 1 | 1 | 1 | 1 |
|  | 3 | 3 | 2 | 2 | 1 | 1 |
|  | 3 | 3 | 3 | 3 | 2 | 1 |
|  | 3 | 3 | 3 | 3 | 2 | 1 |
|  | 3 | 3 | 3 | 3 | 2 | 1 |

First coefficient in the block is also encoded using one of these 3 VLC tables. For each VLC table we maintain cumulative total of the number of bits we would have spent if we had used this table to encode all first coefficients encountered so far. Then we use the table with the smallest cumulative total to encode the next block. The cumulative totals are set to zero at the start of each GOB (intra image) or mackroblock (inter image). Initially table UVLC2 is used.

Table 2: The correspondence between run and level of the coefficient and the codeword number . The remaining level/run combinations are assigned a code number according to the following priority: 1) sign of level (+,-), 2) value (run+1)xlevel (ascending), 3) run (ascending) 3) absolute value of level (ascending).

| Code number | UVLC |  | UVLC1 |  | UVLC2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | level | run | level | run | level | run |
| 1 | EOB |  | EOB |  | EOB |  |
| 2, 3 | 1 (+, -) | 0 | 1 (+, -) | 0 | 1 (+, -) | 0 |
| 4, 5 | 2 (+, -) | 0 | 1 (+, -) | 1 | 2 (+, -) | 0 |
| 6, 7 | 1 (+, -) | 1 | 2 (+, -) | 0 | 3 (+, -) | 0 |
| 8, 9 | 3 (+, -) | 0 | 1 (+, -) | 2 | 4 (+, -) | 0 |
| 10, 11 | 1 (+, -) | 2 | 3 (+, -) | 0 | 1 (+, -) | 1 |
| 12, 13 | 1 (+, -) | 3 | 1 (+, -) | 3 | 1 (+, -) | 2 |
| 14, 15 | 1 (+, -) | 4 | 4 (+, -) | 0 | 2 (+, -) | 1 |
| 16, 17 | 4 (+, -) | 0 | 1 (+, -) | 4 | 5 (+, -) | 0 |
| 18, 19 | 5 (+, -) | 0 | 2 (+, -) | 1 | 6 (+, -) | 0 |
| 20, 21 | 1 (+, -) | 5 | 1 (+, -) | 5 | 7 (+, -) | 0 |
| 22, 23 | 1 (+, -) | 6 | 5 (+, -) | 0 | 3 (+, -) | 1 |
| 24, 25 | 2 (+, -) | 1 | 6 (+, -) | 0 | 1 (+, -) | 3 |
| 26, 27 | 3 (+, -) | 1 | 3 (+, -) | 1 | 1 (+, -) | 4 |
| 28, 29 | 2 (+, -) | 2 | 2 (+, -) | 2 | 8 (+, -) | 0 |
| 30, 31 | 2 (+, -) | 3 | 7 (+, -) | 0 | 4 (+, -) | 1 |
| 32, 33 | 6 (+, -) | 0 | 8 (+, -) | 0 | 2 (+, -) | 2 |
| 34, 35 | 7 (+, -) | 0 | 4 (+, -) | 1 | 1 (+, -) | 5 |
| 36, 37 | 8 (+, -) | 0 | 2 (+, -) | 3 | 2 (+, -) | 3 |
| 38, 39 | 4 (+, -) | 1 | 1 (+, -) | 6 | 1 (+, -) | 6 |
| 40, 41 | 1 (+, -) | 7 | 1 (+, -) | 7 | 1 (+, -) | 7 |

# Coding improvements

The coding improvements are reported in included Excel sheet. Since the described encoding does not change the PSNR only bitrate reductions are given. Bitrate reductions reported in [1] for 3 VLC tables are also included.

References:

[1] 	VCEG-L28, "VLC concatenation for Intra coding", : Eibsee, Germany, 9-12 January, 2001

[2]	ITU-T Q.15/SG16, H.26L Test Model Long Term Number 5 (TML-5) draft0, Doc. Q15-K-59, Oct. 2000.