| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001 | Document VCEG-M58 Filename: VCEG-M58.doc Generated: 27 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Rickard Sjöberg and Per Fröjdh Ericsson Radio Systems AB KI/ERA/T/VV SE-164 80 Stockholm, Sweden | Tel: Fax: Email: | +46 8 757 2373 +46 8 757 5550 rickard.sjoberg@era.ericsson.se |
| Title: | Results on VLC Concatenations |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

In Eibsee there was a discussion on whether we should keep the universal VLC of H.26L or replace it by, for instance, arithmetic coding. We presented an approach (VCEG-L28) which allows us to stay with the current UVLC, while still being able to code symbols with a different probability distribution. This is achieved by constructing new VLCs from concatenations of UVLC code words. By switching between tables it is possible to improve on intra coding.

In this document we review the design of the new VLCs, which we refer to as UVLC2 and UVLC3, and report on results from intra coding with compression gains up to about 5.5%. As an example of other usage, we report on coding the Coded Block Pattern (CBP) symbols with UVLC2 as well.

# The new VLC

The new UVLC2 is constructed by using the shortest original UVLC code word “1” as an optional prefix to the rest of the words. In order to be able to distinguish all code words from each other, the new index 0 code word must have a prefix. UVLC3 is analogously constructed by using up to two prefixes. See Table 1 for a comparison between UVLC, UVLC2 and UVLC3.

Table 1. The first nine code words of UVLC, UVLC2 and UVLC3

| Index | UVLC | UVLC2 | UVLC3 |
| --- | --- | --- | --- |
| 0 | 1 | 11 | 111 |
| 1 | 001 | 001 | 001 |
| 2 | 011 | 011 | 011 |
| 3 | 00001 | 1001 | 1001 |
| 4 | 00011 | 1011 | 1011 |
| 5 | 01001 | 00001 | 11001 |
| 6 | 01011 | 00011 | 11011 |
| 7 | 0000001 | 01001 | 00001 |
| 8 | 0000011 | 01011 | 00011 |

By using UVLC2 or UVLC3 instead of the original UVLC when coding a symbol one loses one bit for index 0, whereas for all other indices one either saves up to two bits or get the same performance.

# Switching between UVLC and UVLC2

The switching applies to Intra luma coefficients in double scan with the following rules:

| Coefficient | Condition for using UVLC2 |
| --- | --- |
| First of first scan | above or left block is unavailable or abs(dc_above)+abs(dc_left) > 2 |
| First of second scan | abs(dc) > 2 or if first scan had >3 coefficients |
| Second | abs(previous_level) > 2(1) for first (second) scan |
| Third through sixth | abs(previous_level)/(previous_run+2) > 0 |

where dc=1 is used outside the present macroblock.

# Switching between UVLC, UVLC2 and UVLC3

In addition to the rules above, add the stronger conditions:

| Coefficient | Condition for using UVLC3 |
| --- | --- |
| First of first scan | abs(dc_above)+abs(dc_left) > 4 |
| First of second scan | first scan had >5 coefficients |
| Second | abs(previous_level) > 3(2) for first (second) scan |
| Third through sixth | abs(previous_level)/(previous_run+3) > 0 |

# Coding improvements

The coding improvements are reported in the included Excel sheet VCEG-M58_intra.xls, using TML 5.91 and quantizers 12,14,16,18,20,22 and 24. The bit rate gains were computed for each rate-distortion point of the old curves by comparing with the linearly interpolated bit rates of the new curves at the same PSNR values. Note that all frames have been intra coded for each sequence. As intra double scan only applies to QP<24, we have omitted QP28 from the list of quantizers.

We also report on results using UVLC2 for all CBP symbols (no switching). The results are included in the Excel sheet VCEG-M58_CBP.xls below.

# Algorithm Resource Consumption Metrics

Algorithm Name: Intra double scan coding with VLC concatenations

Algorithm Description: By concatenating UVLC words, UVLC2 and UVLC3 are formed which have flatter probability distributions. Switching between VLC tables reduces the bit usage in intra double scan coding.

Source: Rickard Sjöberg and Per Fröjdh

Contact: Rickard Sjöberg, e-mail: rickard.sjoberg@era.ericsson.se

Encoding metrics

Computational complexity: non-optimized approx. 30ms/frame QCIF or 5-10% for intra coding using TML 5.91 on Windows NT

RAM memory required: 60 bytes

ROM (table) memory required: 128 bytes

Algorithmic delay: none

Program space: 100 lines of C

Decoding metrics

Computational complexity: same as for encoder

RAM memory required: 16 bytes

ROM (table) memory required: none

Algorithmic delay: none

Program space: 70 lines of C

Efficiency metrics

Signal/noise ratio: not affected

Compression efficiency: up to 4% for UVLC2 and 5.5% including UVLC3

Robustness to errors: not affected

Patent Statement and Licensing Declaration

(Typically one per ITU-T Recommendation)

This declaration does not represent an implied license grant

Please return to:	Director	Place des Nations

Telecommunication Standardization Bureau	CH-1211 Geneva 20, Switzerland

International Telecommunication Union	Fax: +41 22 730 5853

| Patent Holder/Organization: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Legal Name |  | Telefonaktiebolaget LM Ericsson |  |  |  |
| Contact for license application: |  |  |  |  |  |
| Name & Department |  | Måns Ekelöf, Vice President IPR & Licensing |  |  |  |
| Address |  | Telefonaktiebolaget LM Ericsson |  |  |  |
|  |  | SE-126 25 Stockholm, Sweden |  |  |  |
| Tel. |  | +46 8 719 5921 |  |  |  |
| Fax |  | +46 8 719 1112 |  |  |  |
| E-mail |  | Mans.Ekelof@lme.ericsson.se |  |  |  |
| ITU-T Recommendation: |  |  |  |  |  |
| Number |  | H.26L |  |  |  |
| Title |  |  |  |  |  |
| ### Licensing declaration |  |  |  |  |  |
| The Patent Holder believes to hold granted patents and/or pending applications, whose use would be required to implement the above ITU-T Recommendation and hereby declares, in accordance with the Statement on ITU-T Patent Policy (see ITU-T web site), that (check one box only). |  |  |  |  |  |
|  | 1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation. |  |  |  |  |
|  | 2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation. Such negotiations are left to the parties concerned and are performed outside the ITU-T. |  |  |  |  |
|  | 3 The Patent Holder is unwilling to grant licenses according to the provisions of either 1 or 2 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation are affected. - a description of the patent claims covering the Recommendation; |  |  |  |  |
| Signature |  |  |  |  |  |
| Organization |  |  |  |  |  |
| Name of authorized person |  |  |  |  |  |
| Title of authorized person |  |  |  |  |  |
| Signature |  |  |  |  |  |
| Place, Date |  |  |  |  |  |