| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001 | Document VCEG-M62 Filename: VCEG-M62.doc Generated: 27 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Mathias Wien and Achim Dahlhoff Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52074 Aachen, Germany | Tel: Fax: Email: | +49-241-80 7681 +49-241-8888 196 wien@ient.rwth-aachen.de |
| Title: | # Adaptive Block Transforms |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

In the TML seven tilings of a macroblock with a corresponding set of motion vectors are used for motion compensation. With Adaptive Block Transforms (ABT), each of the occurring blocks shall be coded with a transform of the maximum applicable size. For the transform of the 16x16, 16x8, 8x16, 8x8, 8x4, 4x8, and 4x4 blocks, horizontal and vertical transform matrices of size 4x4, 8x8, and 16x16 are needed.

The blocks are transformed by

1	2	3	4	5	6	7

Figure 1: Macroblock modes in TML-4

Cnxm = Tm x Bnxm  x  TnT,	(1)

where Bnxm denotes a block with n pixels m rows. Tn and Tm are the horizontal and vertical transform matrices  of size nxn and mxm, respectively. Cnxm denotes the transformed nxm block. The transform matrices are not normalized. Normalization is performed during the quantization and dequantization procedures.

# Motion Estimation

The varying transform block sizes of ABT have to be taken into account during motion estimation. In the TML, the Hadamard transform is performed on blocks of size 4x4. The sum of the absolute values of the transformed block differences (SATD) is used as the error criterion for motion estimation.

For ABT, the SATD is calculated using the block size given by the macroblock mode. Un-normalized Hadamard transforms of size 4x4, 8x8, and 16x16 are employed. Precise normalization of the SATD values would require a division by a factor containing sqrt(2) for the non-square block sizes. Here, the divisions are replaced by integer multiplications and shifts.

Instead of dividing by the norm given in column 2 of the table below, the SATD values are multiplied by the values in the right column. A slight increase in performance was observed using SATD * 9/16 instead of SATD / 2 for the 4x4 blocks in mode 7.

| #### Hadamard Transforms |  |  |
| --- | --- | --- |
| MB Mode | Norm2/2 | Used Normalization |
| 1 : 16x16 | 8 | 1 / 8 |
| 2 : 16x8 | 4*sqrt(2) | 181 / 1024 |
| 3 : 8x16 | 4*sqrt(2) | 181 / 1024 |
| 4 : 8x8 | 4 | 1 / 4 |
| 5 : 8x4 | 2*sqrt(2) | 181 / 512 |
| 6 : 4x8 | 2*sqrt(2) | 181 / 512 |
| 7 : 4x4 | 2 | 1 / 2 |

# Integer Transform Matrices

The transform matrices are integer approximations of the DCT of size 4x4, 8x8, and 16x16. In the following the integer transforms are denoted by ICT for Integer Cosine Transform. The 4x4 ICT is given in the TML. The 8x8 and the 16x16 ICTs were proposed in [Q15-K24, VCEG-L12].

T8 = [

17  17  17  17  17  17  17  17

24  20  12   6  -6 -12 –20 -24

23   7  -7 -23 -23  -7   7  23

20  -6 -24 –12  12  24   6 -20

17 -17 –17  17  17 -17 –17  17

12 -24   6  20 -20  -6  24 -12

7 –23  23  -7  -7  23 -23   7

6 -12  20 –24  24 -20  12  -6 ];

T16 = [

17  17  17  17  17  17  17  17  17  17  17  17  17  17  17  17

22  28  12  20  12  16   8   6  -6  -8 -16 -12 -20 -12 -28 -22

24  20  12   6  -6 -12 -20 -24 -24 -20 -12  -6   6  12  20  24

28  12   6 -16 -12 -22 -20  -8   8  20  22  12  16  -6 -12 -28

23   7  -7 -23 -23  -7   7  23  23   7  -7 -23 -23  -7   7  23

12   6 -20 -28  -8  12  22  16 -16 -22 -12   8  28  20  -6 -12

20  -6 -24 -12  12  24   6 -20 -20   6  24  12 -12 -24  -6  20

20 -16 -28   6  22   8 -12 -12  12  12  -8 -22  -6  28  16 -20

17 -17 -17  17  17 -17 -17  17  17 -17 -17  17  17 -17 -17  17

12 -12  -8  22  -6 -28  16  20 -20 -16  28   6 -22   8  12 -12

12 -24   6  20 -20  -6  24 -12 -12  24  -6 -20  20   6 -24  12

16 -22  12   8 -28  20   6 -12  12  -6 -20  28  -8 -12  22 -16

7 -23  23  -7  -7  23 -23   7   7 -23  23  -7  -7  23 -23   7

8 -20  22 -12  16   6 -12  28 -28  12  -6 -16  12 -22  20  -8

6 -12  20 -24  24 -20  12  -6  -6  12 -20  24 -24  20 -12   6

6  -8  16 -12  20 -12  28 -22  22 -28  12 -20  12 -16   8  -6 ];

# Quantization Tables

The quantization is performed as described in the TML. Here, five quantization tables are needed for the 7 MB modes. The quantization tables are listed below. They are the same as in [Q15-K24]. The quantization table for 4x4 blocks can be taken from the TML. The values in the quantization tables follow the condition

Anxm * Bnxm * N²nxm = 2^40.

where N²nxm denotes the norm of the nxm block transforms given below.

QUANTIZATION TABLES

| ### QP | 16x16 |  | 16x8 = 8x16 |  | 8x8 |  | 8x4 = 4x8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | A | B | A | B | A | B |
| 0 | 91 | 565 | 128 | 803 | 181 | 1136 | 335 | 2100 |
| 1 | 81 | 635 | 114 | 902 | 162 | 1270 | 299 | 2353 |
| 2 | 72 | 714 | 102 | 1008 | 144 | 1428 | 266 | 2645 |
| 3 | 64 | 803 | 91 | 1130 | 128 | 1607 | 237 | 2968 |
| 4 | 57 | 902 | 81 | 1270 | 114 | 1804 | 211 | 3334 |
| 5 | 51 | 1008 | 72 | 1428 | 102 | 2017 | 188 | 3742 |
| 6 | 45 | 1143 | 64 | 1607 | 91 | 2260 | 168 | 4188 |
| 7 | 40 | 1286 | 57 | 1804 | 81 | 2539 | 149 | 4721 |
| 8 | 36 | 1428 | 51 | 2017 | 72 | 2857 | 133 | 5289 |
| 9 | 32 | 1607 | 45 | 2286 | 64 | 3214 | 118 | 5962 |
| 10 | 29 | 1773 | 40 | 2571 | 57 | 3609 | 105 | 6700 |
| 11 | 25 | 2057 | 36 | 2857 | 51 | 4033 | 94 | 7484 |
| 12 | 23 | 2236 | 32 | 3214 | 45 | 4571 | 84 | 8375 |
| 13 | 20 | 2571 | 29 | 3546 | 40 | 5142 | 75 | 9380 |
| 14 | 18 | 2857 | 25 | 4114 | 36 | 5714 | 67 | 10500 |
| 15 | 16 | 3214 | 23 | 4472 | 32 | 6428 | 59 | 11924 |
| 16 | 14 | 3673 | 20 | 5142 | 29 | 7093 | 53 | 13274 |
| 17 | 13 | 3956 | 18 | 5714 | 25 | 8228 | 47 | 14968 |
| 18 | 11 | 4675 | 16 | 6428 | 23 | 8943 | 42 | 16750 |
| 19 | 10 | 5142 | 14 | 7346 | 20 | 10285 | 37 | 19014 |
| 20 | 9 | 5714 | 13 | 7911 | 18 | 11428 | 34 | 20691 |
| 21 | 8 | 6428 | 11 | 9350 | 16 | 12856 | 30 | 23450 |
| 22 | 7 | 7346 | 10 | 10285 | 14 | 14693 | 26 | 27058 |
| 23 | 6 | 8571 | 9 | 11428 | 13 | 15823 | 24 | 29313 |
| 24 | 6 | 8571 | 8 | 12856 | 11 | 18700 | 21 | 33500 |
| 25 | 5 | 10285 | 7 | 14693 | 10 | 20570 | 19 | 37026 |
| 26 | 5 | 10285 | 6 | 17141 | 9 | 22855 | 17 | 41382 |
| 27 | 4 | 12856 | 6 | 17141 | 8 | 25712 | 15 | 46900 |
| 28 | 4 | 12856 | 5 | 20570 | 7 | 29385 | 13 | 54116 |
| 29 | 3 | 17141 | 5 | 20570 | 6 | 34283 | 12 | 58625 |
| 30 | 3 | 17141 | 4 | 25712 | 6 | 34283 | 10 | 70350 |
| 31 | 2 | 25712 | 4 | 25712 | 5 | 41139 | 9 | 78167 |

| Block Size | Norm N²nxm |
| --- | --- |
| 16x16 | 16 * 17^2 |
| 16x8 | 8 * sqrt(2) * 17^2 |
| 8x16 | 8 * sqrt(2) * 17^2 |
| 8x8 | 8 * 17^2 |
| 8x4 | 4 * sqrt(2) * 13 * 17 |
| 4x8 | 4 * sqrt(2) * 13 * 17 |
| 4x4 | 4 * 13^2 |

# Scanning Method

For encoding of the quantized ABT coefficients the blocks are scanned using full scans of each subblock of the coded macroblock. The zigzag scan is applied. Figure 2 depicts the scans for 4x4, 4x8 and 8x8 blocks as examples. The scans for the other modes are constructed the same way. For blocks with directional segmentation (i.e. 16x8, 8x16, 8x4, 4x8) the scan starts in direction of the longer block edge.

4x4

4x8

8x8

Figure 2. Example: Scan for 4x4, 4x8 and 8x8 blocks. The scans of the other MB modes are constructed accordingly.

# Coded Block Pattern CBPY

The value range of the Coded Block Pattern for Luma (CBPY) is connected to the chosen MB mode for the macroblock to be coded. For modes 1-4 the size of the CBPY blocks corresponds to the block size used for motion compensation and transform. For the higher MB modes the CBPY ordering as described in the TML is used. The numbering of the CBPYs for each MB mode is given in Figure 3.

0

1

2

3

0

1

2

3

0

1

2

3

0

1

2

3

0

0

1

0

1

Figure 3. Numbering of the CBPY in case of ABT.

# VLC Encoding

As a result of the code experiment [VCEG-M63], the optimum performance in terms of entropy measurement is achieved with 2D encoding (level,run) of the coefficients in MB modes 5-7 and 3D encoding (level,run,last) of the coefficients in MB modes 1-4. In [VCEG-M68], UVLC encoding for the ABT coefficients is proposed, using the TML VLC for MB modes 5-7 and a modified UVLC for the 3D code words in MB modes 1-4.

For encoding of the CBPs the TML VLC is used. The CBPs are coded according to the following Table.

| Code No. | Mode 1 | Mode 2 | Mode 3 | Mode 4 | Mode 5 | Mode 6 | Mode 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 15 | 15 | 47 |
| 1 | 0 | 2 | 1 | 8 | 47 | 47 | 15 |
| 2 | 17 | 1 | 2 | 2 | 31 | 31 | 31 |
| 3 | 16 | 3 | 3 | 1 | 13 | 14 | 46 |
| 4 | 33 | 19 | 19 | 4 | 12 | 11 | 7 |
| 5 | 32 | 16 | 17 | 15 | 14 | 7 | 39 |
| 6 |  | 35 | 16 | 13 | 11 | 0 | 30 |
| 7 |  | 18 | 18 | 12 | 3 | 10 | 14 |
| 8 |  | 34 | 35 | 10 | 7 | 5 | 27 |
| 9 |  | 33 | 34 | 5 | 1 | 13 | 23 |
| 10 |  | 32 | 33 | 47 | 46 | 12 | 13 |
| 11 |  | 17 | 32 | 31 | 10 | 1 | 45 |
| 12 |  |  |  | 3 | 44 | 3 | 43 |
| 13 |  |  |  | 11 | 29 | 2 | 38 |
| 14 |  |  |  | 14 | 8 | 4 | 28 |
| 15 |  |  |  | 7 | 4 | 46 | 11 |
| 16 |  |  |  | 9 | 2 | 8 | 5 |
| 17 |  |  |  | 6 | 45 | 6 | 41 |
| 18 |  |  |  | 16 | 43 | 45 | 40 |
| 19 |  |  |  | 28 | 39 | 43 | 37 |
| 20 |  |  |  | 30 | 30 | 9 | 36 |
| 21 |  |  |  | 39 | 9 | 39 | 34 |
| 22 |  |  |  | 23 | 6 | 29 | 32 |
| 23 |  |  |  | 43 | 0 | 37 | 29 |
| 24 |  |  |  | 20 | 5 | 42 | 26 |
| 25 |  |  |  | 44 | 23 | 30 | 25 |
| 26 |  |  |  | 33 | 35 | 27 | 24 |
| 27 |  |  |  | 24 | 28 | 23 | 22 |
| 28 |  |  |  | 22 | 27 | 44 | 20 |
| 29 |  |  |  | 21 | 24 | 28 | 17 |
| 30 |  |  |  | 17 | 41 | 21 | 16 |
| 31 |  |  |  | 46 | 42 | 26 | 9 |
| 32 |  |  |  | 45 | 37 | 20 | 8 |
| 33 |  |  |  | 42 | 33 | 41 | 6 |
| 34 |  |  |  | 37 | 20 | 33 | 4 |
| 35 |  |  |  | 36 | 21 | 24 | 2 |
| 36 |  |  |  | 27 | 19 | 35 | 1 |
| 37 |  |  |  | 25 | 38 | 22 | 0 |
| 38 |  |  |  | 18 | 36 | 18 | 44 |
| 39 |  |  |  | 41 | 18 | 40 | 42 |
| 40 |  |  |  | 40 | 40 | 38 | 35 |
| 41 |  |  |  | 35 | 34 | 36 | 33 |
| 42 |  |  |  | 34 | 26 | 34 | 21 |
| 43 |  |  |  | 29 | 22 | 32 | 19 |
| 44 |  |  |  | 26 | 17 | 25 | 18 |
| 45 |  |  |  | 19 | 16 | 16 | 12 |
| 46 |  |  |  | 38 | 32 | 17 | 10 |
| 47 |  |  |  | 32 | 25 | 19 | 3 |

# Deblocking Filter

Some minor changes were introduced to the deblocking procedure for application with ABT blocks. Deblocking is performed at the edges of the transform blocks, i.e. the edges of the subblocks of the macroblocks. The number of pixels to be filtered depends on the MB mode. In the following, block length denotes the number of subblock pixels in the direction to be filtered.

The assignment of the strength parameters stays unchanged. The maximum value of the overall activity parameter n depends on the block size and is determined by the following rules.  denotes the gray level difference at the boundary,  = |r1-l1|, using the notation of the TML.

| #### Block Length 4 |  |  |
| --- | --- | --- |
| n | value  2 | 2  value |
| 1 |  + /2 |  |
| 2 |  |  +/2 |
| 3 | 0 |  |

| #### Block Length 8 |  |  |
| --- | --- | --- |
| n | value  2 | 2  value |
| 1 | 2* |  |
| 2 |  +/2 | 2* |
| 3 |  |  +/2 |
| 4 | 0 |  |

| #### Block Length 16 |  |  |
| --- | --- | --- |
| n | value  2 | 2  value |
| 1 | 3* |  |
| 2 | 2* + /2 | 3* |
| 3 | 2* | 2* + /2 |
| 4 |  + /2 | 2* |
| 5 |  |  + /2 |
| 6 | 0 |  |

The right filter activity parameter ar is equal to the smallest integer i>0 not larger than n for which the inequality |r1-ri+1|*i >  holds.  and  are the QP dependent thresholds specified in the test model (Table 1 in Deblocking section). The left filter activity parameter al is obtained using the same set of rules substituting pixels on the right of the boundary ri with pixels li on the left of the boundary. Once ar and al are calculated, the pixels to be updated and their filtering support windows are determined by using the table below. The number of affected pixels depends on the applicable block length. The same table applies for both right and left pixel although only right pixels are listed in the table. Information on how to filter the left pixels is obtained by swapping ar and al and substituting r1, r2 with l1, l2.

| ar (only if al>1) | pixel r1 | pixel r2 | pixel r3 | pixel r4 | pixel r5 |
| --- | --- | --- | --- | --- | --- |
| 1 | no filtering | no filtering | no filtering | no filtering | no filtering |
| 2 | filtered | no filtering | no filtering | no filtering | no filtering |
| 3 | filtered | filtered | no filtering | no filtering | no filtering |
| 4 | filtered | filtered | filtered | no filtering | no filtering |
| 5 | filtered | filtered | filtered | filtered | no filtering |
| 6 | filtered | filtered | filtered | filtered | filtered |

When filtering within P frames, the left (right) activity parameter is upper bounded by 2 if strength of the left (right) block of length 4 is no larger than 1. For blocks of length 8 or 16, the strength may not be larger than 3. The selected pixel on the right side are filtered using the TML filter:

R1 = (21*l1 + 22*r1 + 21*r2 + 32)/64

R2 = (21*R1 + 22*r2 + 21*r3 + 32)/64

R3 = (21*R2 + 22*r3 + 21*r4 + 32)/64

R4 = (21*R3 + 22*r4 + 21*r5 + 32)/64

R5 = (21*R4 + 22*r5 + 21*r6 + 32)/64

The selected pixel on the left side is filtered accordingly.

When either of pixels r1, l1 are updated by filtering then the difference between the filtered and nonfiltered value is clipped to the range [-C, C] where C = (Clip(QP,strengthleft) + Clip(QP,strengthright) + ar + al) / 2. When pixel r2 is updated by filtering the difference is clipped to the range [-Cright, Cright] with Cright = Clip(QP,strengthright). Similarly for pixel l2. The Clip values Clip4(qp,strength) for block length 4 are given in the TML description. The Clip values for block length 8 and 16 are set to

Clip8(qp,strength) = Clip4(qp,strength) + Clip4(qp,strength)/2
Clip8(qp,strength) = Clip4(qp,strength) + Clip4(qp,strength)/2

Filtering across macroblock boundaries connected to intra coding is performed the same way as in the TML.

# References

[Q15-K24]	M.Wien, C.Mayer, and J.R.Ohm, ‘Integer Transforms for H.26L using Adaptive
 	Block Transforms’, ITU-T Q15/SG16, Document Q15-K24, Portland, August 2000

[VCEG-L11]	L.Kerofsky and S.Sun, ‘Results of H.26L Core Experiment on Adaptive Block
 	Transform’, ITU-T VCEG, Document VCEG-L11, Eibsee, Germany, 
 	January 2000

[VCEG-L12]	M.Wien and S.Sun, ‘ICT Comparison for Adaptive Block Transforms’, 
 	ITU-T VCEG, Document VCEG-L12, Eibsee, Germany, January 2000

[VCEG-L15]	M.Wien and C.Mayer, ‘ABT Coding Elements’, ITU-T VCEG, 
 	Document VCEG-L12, Eibsee, Germany, January 2000

[VCEG-M63]	M.Wien, ‘ABT Core Experiement Report’, this meeting

[VCEG-M68]	L.Kerofsky, ‘ABT Core Experiement Report’, this meeting