| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001 | Document VCEG-M69 Filename: VCEG-M69.doc Generated: 28 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Chul-Woo Kim McubeWorks 14 F, SK TELECOM BLDG, Sunae-Dong, Bundang-Gu, Sungnam Seoul, Korea Sang Woo Rhie SK Telecom 99 Seorin-dong, Jongro-gu, Seoul 110-110, Korea | Tel: Fax: Email: Tel : Fax: E-mail: | +82-31-601-5875 +82-31-601-5878 charlie@mcubeworks.com +82-2-2121-3463 +82-2-2121-3980 rainism@sktelecom.com |
| Title: | Adaptive Double Scan |  |  |
| Purpose: | Proposal |  |  |

_____________________________

- Introduction

In this document is proposed a very simple idea to improve the double scan which is used for intra macroblocks with QP value of less than 24[1]. The concept behind this proposal is that the ‘runs’ can be made shorter depending on the divided region of double scanned blocks.

The proposed algorithm does not require any additional calculation of parameters resulting in almost ‘no’ extra computational load.

- Adaptive Double Scan

Current double scan defined in TML is shown 5.91 as in Fig. 1 (a). The adaptive double scan proposes one more double scan map shown in Fig. 1 (b)

0   1   2   5

0   2   3   6

1   3   4   7

4   5   6   7

0   0   1   4

1   2   3   5

2   3   4   6

5   6   7   7

(a)                                                   (b)

## Fig. 1 Double scan map of TML 6 and proposed algorithm

- Double scan map defined in TML 5.91 (D1)

- Proposed additional double scan map for adaptive double scan (D2)

The proposed double scan map is transposed version of the one defined in TML 6. And the proposed algorithm is to use the D1 and D2 alternatively depending on the prediction mode. The alternating logic is defined in Table 1. By alternating the double scan map, the distribution of quantized transform coefficients are concentrated in the way that the collected run-lengths between successive coefficients can be reduced.

## Table 1. Algorithm for alternating the double scan map

| Prediction Mode | Scan Map |
| --- | --- |
| Mode 0 (DC Prediction) | D1 |
| Mode 1 | D1 |
| Mode 2 (Vertical Prediction) | ## D2 |
| Mode 3 (Diagonal Prediction) | D2 |
| Mode 4 (Horizontal Prediction) | D1 |
| Mode 5 | D1 |

This can be achieved without any additional calculation of parameters but with only one comparison or  table indexing opreation using prediction mode.

- Simulation Results

TML Simulation results are shown in Fig.2~4. Fig. 2 shows the number of saved bits by adopting the proposed algorithm and Fig. 3 the percentage of saved bits relative to TML 5.91. These results show that we can achieve about 0.2 ~ 1.3 % of bit saving which results in about 50 ~ 400 bits. Foreman, Akiyo, Paris, Hall_Monitor and Mother_and_Daughter were used as test sequences and rate-distortion optimization routine was omitted to compare two algorithms at the same PSNR.

- Complexity and Storage analysis

- Additional Complexity due to Adaptive Double Scan

In the point of view of complexity both for decoder and encoder, it is unlikely to be necessary to explain in detail that ‘negligible’ additional complexity is required to adopt the adaptive double scan algorithm. It can be implemented using one ‘if’ and ‘branch’ operation pair or one table indexing operation for each double scanned block.

- Additional Storage and Code size

Assuming that two bytes are required to store the location in the scanning map, totally 32 bytes of running memory is required to describe the proposed double scan map (D2). And several more instructions are required to perform the adaptive double scan which results into several tens of bytes of extra code when 32 bit instruction is assumed as in the case of ARM 7 series.

- Summary

The simulation results of adaptive double scan reveal that the statistics of current VLC table for 2-D run-length coding of quantized coefficients on double scanned blocks does not match well when the prediction mode contains strong vertical factors. The proposed adaptive double scan algorthm improves this mismatch.

[1] Gisle Bjontegaard, “H.26L Test Model Long Term Number 6 (TML-6) draft 0,” VCEG-L-45d0, Eibsee, Germany, Jan. 2001.

### Fig 2. Bit saving of adaptive double scan

### Fig. 3. Percentage of bit saving relative to average bits / frame

Fig. 4. Percentage of bit saving relative to average Coeff. Y / frame