| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001 | Document VCEG-M72 Filename: VCEG-M72.doc Generated: 27 March ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Tom-Ivar Johansen TANDBERG Philip Pedersens v. 22 1324 Lysaker, Norway | Tel: Fax: Email: | (+47) 67838413 (+47)67125125 tij@tandberg.no |
| Title: | Investigation of RD-Quantization versus simple Quantization |  |  |
| Purpose: | Information |  |  |

_____________________________

# Introduction

The RD-Quantization was proposed in q15-h-10. This investigation seeks to quantify the effect of using rd-constrained quantization, versus normal quantization with tml5.91.

Simulation results will be presented at the meeting.

# RD-Quantization

The investigation was done by simply disallowing double coeff’s in rd_quant() of tml5.91.

The simulation shows that removing RD-quantization reduces the coding efficiency with a small amount, ca.  0.1dB. The most visible impact is that it moves the measure point downwards on the RD-curve. That is, the effective quantizer value is reduced with rd-quantization.

# Summary

Given the complexity of RD-quantization and the limited effect, it should not be considered as a part of a realtime implementation.