| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Fourteenth Meeting: Santa Barbara, CA, USA, 24-28 September 2001 | Document VCEG-N17 Filename: VCEG-N17.doc Generated: 14 August ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 Shawmin Lei Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Fax: Email: Tel: Fax: Email: | 360-817-7531 360-817-8436 ssun@sharplabs.com 360-817-8446 360-817-8436 shawmin@sharplabs.com |
| Title: | # Improved TML Loop Filter with Lower Complexity |  |  |
| Purpose: | Proposal |  |  |

_____________________________

1. Introduction

This document proposes a modification to the TML loop filter in order to reduce its complexity. It is a revision of the loop filter proposed in VCEG-M20 [1].

2. Loop Filter Proposal

A loop filter may reduce the image blocking artifacts, but it also may require a significant portion of computation cycles [2] and introduce extra blurring, which should be avoided whenever possible.

2.1 Skip mode

In VCEG-M20 [1], a “skip-mode” was proposed for block boundaries between inter-coded blocks. The method has been revised as the following.

Assume block j and block k are two inter-coded neighboring 44 blocks; R(j) and R(k) represent the reference frames for blocks j and k, respectively; V(j, x) and V(j, y) are the horizontal and vertical motion vectors for block j; V(k, x) and V(k, y) are the horizontal and vertical motion vectors for block k; and c(j) and c(k) are number of coefficients coded for blocks j and k, respectively.

If all the following conditions are satisfied,

R(j) = R(k)

| V(j, x) - V(k, x) | < 1 pixel

| V(j, y) - V(k, y) | < 1 pixel

c(j) = c(k) = 0

then, the filtering process for the block boundary between blocks j and k should be skipped.

2.2 Boundary strength

We propose the boundary strength, Bs, as a parameter to control the filtering process of all the block boundaries, instead of the Block Strength used in the current loop filter. This boundary strength parameter can also incorporate the skip mode nicely.

If either one of the two 44 blocks across the boundary is intra coded, then

Bs = 3;

else, assuming block j and block k are inter-coded 44 blocks on the two sides of a block boundary, If c(j) >0 or c(k) > 0, then

Bs = 2;

else, if R(j)  R(k) or | V(j, x) - V(k, x) |  1 pixel or | V(j, y) - V(k, y) |  1 pixel, then

Bs = 1;

Else, the block boundary is in skip mode, the boundary strength,

Bs = 0.

The procedure for determining boundary strength, Bs, for the block boundary between two neighboring blocks j and k is illustrated in Figure 1.

Figure 1. Flow chat for determining boundary strength, Bs, for the block boundary between two neighboring blocks j and k.

The value of boundary strength, Bs, for a specific block boundary is determined solely by characteristics of the two 44 blocks across the boundary. Therefore, the control of the filtering process for each individual block boundary is well localized. A block boundary is filtered only when it is necessary; and unneeded computation and blurring artifacts can be effectively avoided.

Because less block boundaries are to be filtered, compared to the Block Strength defined in the current TML [3], the proposed boundary strength approach is expected to give significant savings in computation.

Using boundary strength as control parameter has another advantage over the current block strength parameter. Many of us have noticed that the Block Strength is not quite friendly to MB-based loop filtering process, which has been proposed in VCEG-M48 [4]. For boundary strength, since the decision is localized, MB-based loop filtering process can be easily defined and implemented.

2.3 Filter function

In this proposal, we have adopted the filter function designed by the loop filter ad-hoc group [5] formed in the Austin meeting. All the Strength parameters in that filter have been replaced by the newly defined boundary strength. The table Clip(QP, strength) in the current TML document [3] is adopted.

Table 1. Definition of the table Clip(QP,strength) [3]

QP         0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 29 30 31

Clip(qp,1) 0 0 0 0 0 0 0 0 0 0  0  0  0  1  1  1  1  1  1  1  1  1  1  1  2  2  2  2  2  3  3

Clip(qp,2) 0 0 0 0 0 0 0 0 0 0  1  1  1  1  1  1  1  1  1  1  2  2  2  2  3  3  3  3  4  5  5

Clip(qp,3) 0 0 0 0 0 0 0 0 1 1  1  1  1  1  1  1  2  2  2  2  3  3  3  4  4  5  5  5  7  8  9

3. Experiments

Experiments are conducted following the “Simulation Conditions” specified in VCEG-M75 [6]. The proposed loop-filtering scheme is integrated into the TML-8 software to compare against the TML-8 codec [3]. The motion search range is set to 16. The RD optimization option is set on for the mode selection.

Table 2. Performance of the proposed loop-filtering scheme in TML-8

Table 2 shows the performance of the proposed loop-filtering scheme. Using the proposed method, we achieved computational saving of roughly 25% of the loop filter operations. The value for the computational saving derived from number of computation cycles is quite consistent with the value based on the number of calls to the deblocking filter function (i.e., loop() in the TML software). Average PSNR and bitrate differences [7] are also provided in Table 1. The experiments showed no significant difference in bit rate or PSNR.

Decoded video sequences will be provided in the meeting for subjective evaluation. The side-by-side comparison is prepared following the “Simulation Conditions.” There has been no significant visual difference observed in our evaluation.

RD curves for all the testing sequences are shown in the Appendix. Detailed experimental results are available in VCEG-N17.xls.

As a comment, since each filter coefficient takes at most 5 bits, it will be more efficient if SIMD operations are used for the computation.

4. Summary

We have demonstrated the advantages of the proposed loop filtering technique with the skip mode and boundary strength. This approach significantly reduces the computation requirements, which is critical for real-time implementation of decoding devices. The method showed no degradation in PSNR and visual quality. Therefore, we recommend VCEG to adopt the proposal into the new TML.

Legal statement

Sharp has intellectual property related to this contribution and has chosen to subscribe to subclause 2.2 for ITU-T patent policy under the condition of reciprocity.

# References

[1] Shijun Sun and Shawmin Lei, “Loop Filter with Skip Mode,” document VCEG-M20, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[2] Antti Hallapuro and Marta Karczewicz, “Complexity Analysis of H.26L,” document VCEG-M50, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[3] Gisle Bjontegaard, "H.26L Test Model Long Term Number 8 (TML-8) draft0," document VCEG-Nxx, ITU-T Video Coding Experts Group (VCEG) Meeting, 28 June 2001.

[4] Peter List, “Proposal for a Simplification of the H.26L Loop Filter,” document VCEG-M48, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[5] Peter List, “AHG Report: H.26L Loop Filtering,” document VCEG-N08, ITU-T Video Coding Experts Group (VCEG) Meeting, 4-7 September 2001.

[6] Gisle Bjontegaard, “Recommended Simulation Conditions for H.26L,” document VCEG-M75, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[7] Gisle Bjontegaard, "Calculation of average PSNR differences between RD curves," document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

# Appendix. R-D curves