| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Twelfth Meeting: Santa Barbara, USA, 24-27 September, 2001 | Document VCEG-N31 Filename: VCEG-N31.doc Generated: 10 September ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Marta Karczewicz, Antti Hallapuro Nokia Research Center, Irving, Texas 75039, USA | Tel: Fax: Email: | +972 894 4188 +972 894 4589 marta.karczewicz@nokia.com |
| Title: | Interpolation solution with low encoder memory requirements and low decoder complexity. |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

After the VCEG Eibsee meeting new interpolation called direct interpolation was adopted to the H.26L Test Model (TML-6 and later) [1], [2]. Compared to used in TML-5 subsequent interpolation direct interpolation reduces complexity of the decoder but increases either complexity or memory requirements of the encoder. In the current TML-8 software 1/4 precision interpolation is implemented using direct interpolation method and 1/8 precision interpolation is implemented using subsequent interpolation.

In this submission an interpolation method is presented which has same complexity and memory requirements as subsequent interpolation in the encoder and similar decoder complexity as direct interpolation.

# 1/4 Pel Resolution interpolation

Description of the proposed interpolation method is given. In the figure below the pixels labeled 'A' represent original pixels at integer positions and other symbols represent the pixels to be interpolated

A	d	b	d	A

e	h	f	h

b	g	c	g	b

e	h	f	i

A		b		A

Figure 1: Naming convention used for sub-pixels.

- 1/2 pixel labeled 'b' is obtained by first calculating intermediate value b=(A1–5A2+20A3+20A4-5A5+A6) using six nearest pixels at integer locations in horizontal or vertical direction. Final value of 'b' is calculated as (b+16)/32 and clipped to lie in the range [0, 255].

- 1/2 pixel labeled 'c' is calculated as (b1–5b2+20b3+20b4-5b5+b6+512)/1024 using intermediate values b of closest 1/2 pixels in the vertical or horizontal direction and clipped to lie in the range [0, 255]. Notice that using intermediate values b in the horizontal direction leads to the same result as using intermediate values b in the vertical direction.

- 1/4 pixels labeled 'd' are calculated as (A+b)/2 and pixels labeled ‘g’ as (b+c)/2 using two nearest pixels in horizontal direction. 1/4 pixels labeled 'e' are calculated as (A+b)/2 and pixels labeled ‘f’ as (b+c)/2 using two nearest pixels in vertical direction.

- 1/4 pixels labeled 'h' are calculated as (b1+b2)/2 using two nearest pixels in diagonal direction.

- Pixel labeled 'i' ("funny" position) is computed as (A1+A2+A3+A4+2)/4 using four nearest original pixels.

## Complexity of the Proposed Interpolation

### Encoder Complexity

In the encoder same sub-pixel values have to be calculated multiple times. To decrease its complexity, all sub-pixel values could be pre-calculated and stored in memory. This solution can however increase memory usage by a large margin. When motion vector accuracy is 1/4 pel in both horizontal and vertical dimension, storing pre-calculated sub-pixel values for the whole image requires 16 times the memory of the original, non-interpolated image.

To reduce memory usage all 1/2 pixels can be interpolated beforehand and 1/4 pixels only when they are needed. In case of the proposed method and subsequent interpolation 1/4 pixels can be calculated as bilinear interpolation of 1/2 pels. 4 times the original picture memory is required to store 1/2 pixels since only 8 bits are necessary to represent them.

When same strategy of interpolating 1/2 pixels beforehand is used in conjunction with direct interpolation the memory requirements increase to 9 time the original frame memory. In addition complexity of the sub-pixel interpolation during motion estimation is higher since scaling and clipping has to be done for every 1/2 and 1/4 pixel position.

### Decoder Complexity

In the decoder only 1 out of 15 sub-pixel positions is needed at a time. Therefore it is recommended that the interpolation of a sub-pixel is performed with minimum number of steps that results in correctly interpolated value.

Figure 2: Block based interpolation.

When comparing complexity of the proposed interpolation, subsequent and direct interpolation implementations it is assumed that each implementation is block based, i.e., intermediate values common for all the sub-pixels to be interpolated in a given NxM block are calculated just once.  Example is given in Figure 2. To calculate 4x4 block of 1/2 pixels labeled ‘c’ first 1/2 pixels labeled ‘b’ are calculated.

(a)					   (b)					    (c)

Figure 3: 1/2 pixels required to be calculated in order to obtain leftmost 1/4 pixel labeled ‘g’ and for subsequent interpolation (a) and proposed method (b). 1/2 pixels required to be calculated in order to obtain upper most 1/4 pixel labeled ‘h’ for subsequent interpolation (a) and proposed method (c)

Reasons for lower decoder complexity of the proposed method comparing to subsequent interpolation:

- Unlike in subsequent interpolation pixel 'c' can be interpolated by filtering either in vertical or horizontal direction. To minimize number of operations 'c' can be interpolated in vertical direction if value of pixel 'f' is needed, and in horizontal direction if value of pixel 'g' is needed. As an example all the 1/2 pixels required to be calculated in order to obtain 1/4 pixels labeled ‘g’ for 4x4 block using subsequent interpolation and using proposed method are indicated, respectively, in Figure 3 (a) and Figure 3 (b).

- Pixel label ‘h’ is calculated by bilinear interpolation of two closest pixels in the diagonal direction. 1/2 pixels required to be calculated in order to obtain upper and leftmost 1/4 pixels ‘h’ for 4x4 block using subsequent interpolation are tand using proposed method are depicted, respectively, in Figure 3  (a) and Figure 3 (c).

Table 1 summarizes decoder complexities of the interpolation methods. Complexity is measured in terms of number of 6-tap filter and bilinear filter operations. Interpolation of the "funny pixel" location is assumed to consist of two bilinear operations. The operations needed to interpolate one 4x4 block are listed for each of the 15 sub-pixel positions. Sub-pixel locations are numbered according to the following figure:

| 1 | 2 | 3 | 4 |
| --- | --- | --- | --- |
| 5 | 6 | 7 | 8 |
| 9 | 10 | 11 | 12 |
| 13 | 14 | 15 | 16 |

Location 1 is full-pixel location and locations 2-16 are sub-pixel locations of which location 16 is the "funny pixel" location. When computing average number of operations it is assumed that probability of motion vector pointing to each sub-pixel position is the same. The average complexity is therefore the average of the 15 sums calculated for each sub-pixel location and one full-pixel location.

Table 1 - Complexity of the 1/4-pel interpolation methods

|  | Subsequent iterp. |  | Direct interp. |  | Proposed |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location | Bilin. | 6-tap | Bilin. | 6-tap | Bilin. | 6-tap |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3,9 | 0 | 16 | 0 | 16 | 0 | 16 |
| 2,4,5,13 | 16 | 16 | 0 | 16 | 16 | 16 |
| 11 | 0 | 52 | 0 | 52 | 0 | 52 |
| 7,15 | 16 | 52 | 0 | 52 | 16 | 52 |
| 10,12 | 16 | 68 | 0 | 52 | 16 | 52 |
| 6,8,14 | 48 | 68 | 0 | 52 | 16 | 32 |
| 16 | 32 | 0 | 32 | 0 | 32 | 0 |
| Average | 19 | 37 | 2 | 32 | 13 | 28.25 |

It can be seen from the table that proposed method requires less 6-tap filter operations than direct interpolation and only some more bilinear operations. Since 6-tap operations are much more complex than bilinear operations complexity of the two methods is similar. Subsequent interpolation has by far the highest complexity.

# 1/8 Pel Resolution interpolation

The proposed method is extended to 1/8 pel resolution interpolation. In the below figure pixels labeled 'A' represent original pixels at integer positions and other symbols represent 1/2, 1/4 and 1/8 pixels to be interpolated.

A	d	b1	d	b2	d	b3	d	A

d	e	d	f	d	f	d	e

b1	d	c11	d	c12	d	c13	d

d	f	d	g	d	g	d	f

b2	d	c21	d	c22	d	c23	d

d	f	d	g	d	g	d	e

b3	d	c31	d	c32	d	c33	d

d	e	d	f	d	f	d	e

A

Proposed1/8 pel  interpolation 1:

- 1/2 and 1/4 pixels labeled 'b1', ‘b2’ and ‘b3’ are obtained by first calculating intermediate values

b1=(-3A1 +12A2-37A3+229A4+71A5-21A6+6A7-A8),

b2=(-3A1 +12A2-39A3+158A4+158A5-39A6+12A7-3A8), and

b3=(-A1 +6A2-21A3+71A4+229A5-37A6+13A7-3A8)

using eight nearest pixels at integer locations in horizontal or vertical direction. Final value of 'bi', i=1,2,3, is  calculated as (bi+128)/256 and clipped to lie in the range [0, 255].

- 1/2 and 1/4 pixels labeled 'cij’, i,j=1, 2, 3, are calculated as

cj1=(-3b1j+12b2j-37b3j+229b4j+71b5j-21b6j+6b7j-b8j+32768)/65536,

cj2=(-3b1j+12b2j-39b3j+158b4j+158b5j-39b6j+12b7j-3b8j+32768)/65536, and

cj3=(-b1j+6b2j-21b3j+71b4j+229b5j-37b6j+13b7j-3b8j+32768)/65536

using intermediate values b1, b2 and b3 of closest 1/2 and 1/4 pixels in the vertical direction. Before storing pixels labeled 'cij’ , i,j=1, 2, 3, in the frame memory they are clipped to lie in the range [0, 255]. Notice that these pixels can also be calculated using intermediate values b1, b2 and b3  in the horizontal direction.

- 1/8 pixels labeled ‘d’ are calculated as by bilinear interpolation of two integer position, 1/2 or 1/4 pixels closest in the horizontal or vertical direction, e.g., upper and left most pixel ‘d’ is calculated as (A+ b1+1)/2.

- 1/8 pixels labeled ‘e’ are calculated by bilinear interpolation of two 1/4 pixels closest in the diagonal direction, e.g., upper and left most pixel ‘e’ is calculated as (b1+ b1+1)/2.  1/8 pixels labeled ‘g’ are calculated as (A+3c22+2)/4 and 1/8 pixels marked as ‘f’ are calculated using bilinear interpolation of 1/2 pixels 'b2', i.e., as (3b1+ b1+2)/4 (pixel 'b2' closer to ‘f’ is multiplied by 3). Which diagonal direction should be chosen is indicated in Figure 4.

Figure 4: Diagonal interpolation for 1/8 pel interpolation.

### Encoder complexity

To reduce encoder memory requirements similar approach as for 1/4 pel precision interpolation can be used for 1/8 pixel precision interpolation in which case all 1/2 and 1/4 pixels should be interpolated beforehand and 1/8 pixels only when needed. When proposed method or subsequent interpolation are used 16 times the original picture memory is required to store 1/2 and 1/4 pixels. However in case of direct interpolation intermediate values of 1/2 and 1/4 pixels, represented with 32-bit precision, have to be used. Therefore it is better to interpolate the whole frame at the beginning of the motion compensated prediction and store the interpolated pixels in the buffer having size 64 times of the original frame size.

### Decoder complexity

Decoder complexity of the 1/8 interpolation is calculated the same way as complexity of the 1/4 interpolation. That is, it is assumed that minimum set of interpolation steps that result in interpolated pixel is used. Similarly to 1/4 interpolation, it is assumed that intermediate values common for all the sub-pixels to be interpolated in a given 4x4 block are calculated just once.

Table 2 summarizes complexities of the interpolation methods. Complexity is measured in terms of number of 8-tap filter and bilinear filter operations. The operations needed to interpolate one 4x4 block is listed for 63 sub-pixel positions that are numbered according to the following figure:

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 |
| 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 |
| 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |

Location 1 is full-pixel location and locations 2-64 are sub-pixel locations. When computing average number of operations it is assumed that probability of motion vector pointing to each sub-pixel position is the same. The average complexity is the average of the 63 sums calculated for each sub-pixel location and one full-pixel location.

Table 2 - Complexity of the 1/8-pel interpolation methods

|  | Subsequent |  | Direct |  | Proposed |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location | Bilin. | 8-tap | Bilin. | 8-tap | Bilin. | 8-tap |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3,5,7,17,33,49 | 0 | 16 | 0 | 16 | 0 | 16 |
| 19,21,23,35,37,39,51,53,55 | 0 | 60 | 0 | 60 | 0 | 60 |
| 2,8,9,57 | 16 | 16 | 0 | 16 | 16 | 16 |
| 4,6,25,41 | 16 | 32 | 0 | 16 | 16 | 32 |
| 10,16,58,64 | 32 | 76 | 0 | 60 | 16 | 32 |
| 11,13,15,59,61,63 | 16 | 60 | 0 | 60 | 16 | 60 |
| 18,24,34,40,50,56 | 16 | 76 | 0 | 60 | 16 | 60 |
| 12,14,60,62 | 32 | 120 | 0 | 60 | 16 | 32 |
| 26,32,42,48 | 32 | 108 | 0 | 60 | 16 | 32 |
| 20,22,36,38,52,54 | 16 | 120 | 0 | 60 | 16 | 76 |
| 27,29,31,43,45,47 | 16 | 76 | 0 | 60 | 16 | 76 |
| 28,30,44,46 | 32 | 152 | 0 | 60 | 16 | 60 |
| Average | 64 | 290.25 | 0 | 197.75 | 48 | 192.75 |

The number of 8-tap operations is 34% lower for proposed method than that of the subsequent interpolation used in TML-8. The number of bilinear operations is 25% lower, but this improvement is of relatively small importance compared to reduction in 8-tap operations. Should direct interpolation be utilized for 1/8 pel interpolation its complexity would be comparable to the complexity of proposed methods.

# Simulation Results

Performance was measured using TML-8 software and recommended encoder settings and test conditions. Simulation results included in VCEG-N31.xls show no significant difference between the direct interpolation, TML-5 interpolation and proposed method both for 1/4 and 1/8 pel precision interpolation.

To reduce precision requirements we tested using current 8–tap filters coefficients divided by 2, i.e., instead of current 8-tap filters

h1={ -3, 12, -37, 229,  71, -21,  6, -1 },

h2 = {-3, 12, -39, 158, 158, -39, 12, -3 },

h3= {-1,  6, -21,  71, 229, -37, 12, -3 },

we used:

h1 = {-1, 6, -19, 115, 36, -11, 3, -1},

h2 = {-2, 6, -19, 79, 79, -19, 6, -2},

h3 = {-1, 3, -11, 36, 115, -19, 6, -1}.

Results when these reduced precision coefficients are used together with the proposed interpolation method are also given in VCEG-N31.xls. Comparison with the current 1/8-pel interpolation shows no performance degradation.

# Conclusions

New interpolation method was presented. The coding efficiency and its decoder complexity are comparable to the direct interpolation method currently employed in the TML software. The main difference is the encoder complexity.

In case of 1/4 pel precision when encoder is implemented by pre-calculating 1/2 pixel values and interpolating 1/4 pixels on the fly direct interpolation requires memory 9x the size of the original frame to store 1/2 pixels comparing to 4x required by the proposed method. Moreover, interpolation of 1/4 pixels in case of direct interpolation is more complex since scaling and clipping is required in addition to bilinear interpolation.

For 1/8 precision interpolation when direct interpolation is used in the encoder all the sub-pixel values should be calculated before motion estimation is started which requires 64 times the memory of the original frame. Proposed method allows to pre-calculate 1/2 and 1/4 pixels which storage require 16 times the memory of the original frame and 1/8 pixels can be calculated as required by bilinear interpolation.

# References

- Thomas Wedi, " Complexity Reduced Motion Compensated Prediction with 1/8-pel Displacement Vector Resolution ", Document  VCEG-L20, Video Coding Experts Group (VCEG) 12th meeting, Eibsee, Germany, 9-12 January, 2001

- Telenor Broadband Services, "TML H.26L Codec v. 8.0," via ftp://standard.pictel.com/video-site/h26L, 9th July, 2001.

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 <fax number here>

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX-75039 |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 214 763 1314 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | 26/09/01 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Interpolation solution with low encoder memory requirements. |  |
| Contribution number |  | VCEG-N31 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]