| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Fourteenth Meeting: Santa Barbara, CA, USA, 21-24 September, 2001 | Document VCEG-N38 Filename: VCEG-N38.doc Generated: 01 September ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Stockhammer, Günther Liebl Tobias Oelbaum Institute for Communications Eng. Munich University of Technology 80290 Munich, Germany Thomas Wiegand, Detlev Marpe Image Processing Department Heinrich-Hertz-Institute Einsteinufer 37 D-10587 Berlin, Germany | Tel: Fax: Email: Tel: Fax: Email: | +49 89 28923510 +49 89 28923490 stockhammer@ei.tum.de +49 30 31 002 617 +49 30 392 72 00 wiegand@hhi.de |
| Title: | H.26L Simulation Results for Common Test Conditions for RTP/IP over 3GPP/3GPP2 |  |  |
| Purpose: | Information |  |  |

_____________________________

Intellectual Property Notice: The authors believe that there is no intellectual property associated with this proposal

# Summary

This document shows first results applying H.26L TML8.4 for the common test conditions for Video Performance Evaluation for RTP/IP over 3GPP/3GPP2 [4]. Only results without RLP/RLC retransmissions are reported. It serves to compare different data partitioning schemes, different entropy coding schemes and different transmission packet lengths. To obtain improved results the encoder was modified to perform well selected intra updates. Different intra macroblock update mechanisms in mobile RTP/IP error-prone systems are investigated. The RTP/IP signaling framework proposed in [3] was not yet applied but the results are meaningful as the size of the payload will change insignificantly.

# Simulation Conditions

We will briefly describe the simulation conditions used in this contribution [4]. We will discuss the reasonable changes in the following. Additionally we will present an adapted macroblock mode selection method.

## Common Conditions

Tests for proposed error patterns and video sequences have been carried out. In detail, the experiments 1, 2, 5, 6, 11,  and 12 have been carried out. The other experiments will be subject of further study. The simulator software proposed in VCEG-N37 [3] was applied. Any damaged RTP packets are ignored and an appropriate error indication is passed to the error tolerant TML8.4 software decoder. The bitrate was kept below the target bitrate by applying only one quantizer which is kept constant for the entire sequence including the first frame. As the frame rate is constant and no frames are skipped we compute the PSNR for all coded frames. The video sequence is encoded once and 10 decoding attempts are run with starting positions in the error files according to VCEG-M77 [4]. To convey the slice/picture header information  (Partition Number, Format, Picture Type, QP, Partition Mode, First MB, MV resolution) 4 bytes overhead per video packet have been assumed. For partition headers (Partition Number, First MB) 2 bytes overhead have been assumed. An appropriate adjustment according to the proposed signaling in VCEG-N52 [3] is easily possible without changing the results significantly.

As the TML8.4 decoder lacks an appropriate error concealment especially for intra frame we assume for the tests that the first I-frame is coded with the identical QP and the residual sequence, but the transmission is assumed to be error-free. This avoids the transmission of “green” blocks.

## Macroblock Mode Selection

The regular intra update results have been obtained using TML8.4. However, as it is of advantage to use more frequent intra macroblocks with increasing error rate especially in areas with higher motion a strategy for intra macroblock update has been investigated.  Therefore, in addition to this regular intra MB update in TML8.4 a macroblock mode selection based on the expected mean square error distortion was implemented. This allows for well selected intra macroblock updates. The decoder distortion is estimated by running in complete N decoders in the encoder in parallel which simulate the statistics of the channel. The statistical process of loosing a slice is assumed to be independent for each of the N decoders. The slice loss process for each decoder is also assumed to be iid, and a certain slice loss probability p is assumed to be known at the encoder. Obviously for large N the decoder gets a very good estimate of the average decoder distortion. However, with increasing N a linear increase of storage and decoder complexity in the encoder is obvious. Therefore, this method might not be practical in real-time encoding processes. Complexity and memory efficient algorithms are currently under investigation.

We will present the encoder configuration in more detail. The set of possible macroblock types is defined as SMB= { COPY_MB, INTER_16x16_MB, INTER_16x8_MB, INTER_8x16_MB, INTER_8x8_MB, INTER_8x4_MB,  INTER_4x8_MB, INTER_4x4_MB, INTRA_4x4, INTRA_16x16} according to TML8 Error: Reference source not found . For each macroblock the macroblock mode m’ was selected according to

m'=minm∈SMB    Dm+λRm

with Dm the distortion in the current macroblock when selecting macroblock mode m and Rm the corresponding rate, i.e. the number of bits. The distortion Dm is computed as

Dm=1Nn=1Nifi−fi,n,m2

with fi  the original pixel value at position i within the macroblock and fi,n,m the reconstructed pixel value at position i for coding macroblock mode m in the simulated decoder n. The decoder is the TML8.4 decoder, i.e. the simple error concealment which copies  macroblocks from the previous frames is used. Similarly to the rate control in TML8.4 the  parameter depends on the quantization parameter q as follows

λ=20q+534−qe0.1q.

The rate control is done according to the test conditions, i.e. one quantization parameter is used for the entire sequence such that the overall rate constraint is fulfilled. For more sophisticated rate control algorithms a frame-wise or even macroblock-wise adaptation of  and q is of advantageous.

# Results

## Reported Parameters

All results are shown in the attached excel-File VCEG-N38.xls.  Detailed simulation results are available if demanded. Due to the huge amount of data only the following parameters are reported:

- Video Seq: specifies the video sequence. foreman, hall are QCIF sequences, paris is a CIF sequence.

- Framerate: specifies the framerate of the coded video sequence in Hz.

- MB Selection: specifies the macroblock selection mode, especially how the intra updates are chosen. Regular is according to the intra update implemented in TML8.4. In each coded frame we chose that one line of macroblocks are intra updated. The update is periodically. Optimized macroblock selection is according to the description in the previous section. Each of the N decoders are assumed to operate on channels with statistically independent AL3-SDU losses with probability p. The parameters tuple (p, N) is given in the results.

- Error seq: specifies the applied error sequence 3, 4, 5, 6 with corresponding packet loss rates 5e-4, 1.6e-4, 1e-3, 1.3e-4, respectively.

- Entr. Code specifies the selected entropy coding mode UVLC or CABAC.

- DP specifies the Data Partitioning Mode with 1, 2, or 4 partitions.

- QP specifies the selected quantization parameter to meet the rate constraint.

- Total Bitrate specifes the resulting total bit rate including Audio and H.223 control and header data in kbit/s.

- Video Bitrate specifies the resulting video bitrate including the slice and partition header.

- MBs/Slice specifies the size in macroblock of one slice. We chose only multiple of macroblock lines.

- Runs specifies the number of runs. 10 different starting position in the error files according to [4] have been used.

- AvPSNR-Y shows the average PSNR of the luminance over all runs. Note that only the coded frames have been used for the computation of the PSNR.

- Max PSNR-Y shows the difference of the maximum PSNR of all runs to the average PSNR.

- Min PSNR-Y shows the difference of the minimum PSNR  of all runs to the average PSNR.

For comparison of different aspects the data can be sorted in the document. We will provide the conclusions when comparing different schemes. Not all combinations have been run. For the case of CABAC simulations for only 1 and 2 partitions have been carried out. The optimized intra mode selection was only done for a limited set of experiments due to the intensive encoding complexity.

## Simulation Results

The entire results are shown in VCEG-N38.xls. However, Error: Reference source not found only shows the best results with respect to the average PSNR by selecting the best combination of entropy coding, macroblocks per slice and data partitioning schemes. Error: Reference source not found shows results for experiments 1, 2, 5, 6, 11, 12 according to ITU-T VCEG-N37 [4]. Note that the results give some reasonable indication about the performance of different modes. However, experiments with more statistical preciseness will be necessary for real judgements.

We will first discuss the case with regular intra updates. It can be seen that in general for sequences with camera movement like Foreman and Paris smaller slices are of advantage (Exp. 5,6,11,12). 11 and 22 macroblocks per slice are the smallest applied. However, in case of little movement and static background like in Hall larger slice sizes perform better for low error rates. In general no data partitioning is preferable for the regular intra updates. CABAC in general outperforms UVLC as the compression efficiency is higher (Exp 1-6). In general CABAC is slightly better in terms of coding efficiency 2-5%. However, the significant gains reported previously are not necessarily present a small slice length are preferable for bit error-prone systems but obviously reduce the coding efficiency for CABAC.

| #### Exp | Video Seq | Frame rate | MB Selection | Err. seq | Entr. Code | DP | QP | Total Bitrate | Video Bitrate | MB p Slice | runs | AvPSNR Y | Max PSNR | Min PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | hall | 15 | regular | 3 | CABAC | 1 | 17 | 62,91 | 59,11 | 99 | 10 | 35,18 | 0,66 | 1,09 |
| 2 | hall | 15 | regular | 4 | UVLC | 2 | 19 | 58,47 | 51,67 | 33 | 10 | 33,70 | 1,17 | 4,14 |
| 5 | foreman | 7,5 | regular | 3 | UVLC | 1 | 19 | 63,15 | 57,25 | 11 | 10 | 31,81 | 0,96 | 0,72 |
| 5 | foreman | 7,5 | regular | 3 | CABAC | 1 | 19 | 60,09 | 54,19 | 11 | 10 | 31,66 | 1,94 | 1,41 |
| 6 | foreman | 7,5 | regular | 4 | CABAC | 1 | 18 | 63,47 | 59,37 | 33 | 9 | 31,32 | 2,85 | 3,04 |
| 6 | foreman | 7,5 | regular | 4 | UVLC | 1 | 19 | 59,91 | 55,81 | 33 | 9 | 31,21 | 2,44 | 1,64 |
| 11 | paris | 15 | regular | 5 | UVLC | 1 | 27 | 117,69 | 107,69 | 66 | 10 | 26,91 | 0,40 | 0,48 |
| 11 | paris | 15 | regular | 5 | CABAC | 1 | 27 | 114,15 | 104,15 | 66 | 10 | 26,89 | 0,39 | 0,39 |
| 12 | paris | 15 | regular | 6 | CABAC | 1 | 27 | 127,11 | 109,91 | 22 | 10 | 27,31 | 0,22 | 0,28 |
| 12 | paris | 15 | regular | 6 | UVLC | 1 | 27 | 117,69 | 107,69 | 66 | 10 | 27,28 | 0,29 | 0,38 |
| 1 | hall | 15 | O(0.06,30) | 3 | UVLC | 4 | 12 | 61,54 | 55,62 | 99 | 10 | 36,19 | 0,76 | 0,96 |
| 2 | hall | 15 | O(0.01,30) | 4 | UVLC | 1 | 13 | 58,46 | 50,43 | 11 | 10 | 35,93 | 0,93 | 1,81 |
| 5 | foreman | 7,5 | O(0.04,30) | 3 | UVLC | 2 | 19 | 60,83 | 56,43 | 33 | 10 | 30,94 | 1,08 | 1,41 |
| 6 | foreman | 7,5 | O(0.04,30) | 4 | UVLC | 2 | 19 | 60,83 | 56,43 | 33 | 10 | 31,25 | 1,20 | 2,83 |

Table 1 Best Results for Experiments 1, 2, 5, 6, 11, 12 according to ITU-T VCEG-N37 [4]

The optimized mode selection has been only performed the hall and foreman sequence with UVLC entropy coding. It can be seen that the best results for regular intra updates are outperformed by the MB mode selection for the hall sequence (experiment 1 and 2) as the selection MB updates for the static Hall sequence should be very carefully applied. It can also be seen that in this case Data Partitioning 4 partitions is very useful for some scenarios for Hall. Significant improvements of about for Hall (exp. 1, 2) in the Average PSNR (1.0 dB, 2.0 dB) can be observed. However, for Foreman the significant improvements for the optimized mode selection are not obvious. However, note that the variance of the results is always reduced by optimized intra updates.

# Conclusion

The presented results show good indication about how different options within H.26L perform for RTP/IP over 3GPP/3GPP2 error prone channels without RLC layer retransmission. However, the statistical preciseness could be increased by more repetitions to obtain more accurate results. From the results presented we draw the following conclusions. H.26L is suited for RTP/IP transmission over 3GPP/3GPP2 error prone channels without RLC layer retransmission. In the test model it would beneficial to have improved error concealment, appropriate rate control schemes and an error-resilient macroblock selection mode. In this contribution simple offline rate control by fixing the QP, a simple copy macroblock error concealment according to TML8.4, error-free transmission of the first I-frame and an error-resilient but complex macroblock mode selection method has been applied. With these preliminaries we can draw the following conclusions. The appropriate selection of encoding tools can improve the quality of the video significantly. CABAC in general also outperforms UVLC in the low bit rate RTP/IP error-prone case. The gains are less significant as usually short slices and block sizes are preferable for bit error prone transmission. For regular intra updates data partitioning is in general inferior as regular slices. However, optimized macroblock selection in combination with data partitioning shows good results in selected experiments.

In combination with unequal error protection or other priority methods data partitioning might improve error resilience in mobile RTP/IP environments even more. Especially important is also the selection of the slice and RTP payload size. In general shorter RTP payloads provide better performance. However, the trade-off of protocol signaling overhead and reduced coding efficiency for short slices versus the improved error resilience is obvious and was shown especially for low and intermediate error rates. Further investigations also including RLC layer retransmission will be necessary to judge the error resilience tools included inH.26L.

# References

- ITU-T VCEG-N08, H.26L Test Model Document, VCEG (SG16/Q6), Fourteenth Meeting, Santa Barbara, CA, September 2001.

- ITU-T VCEG-N37, Common Test Conditions for RTP/IP over 3GPP/3GPP2 – Software and Ammendments, VCEG (SG16/Q6), Fourteenth Meeting, Santa Barbara, CA, September 2001.

- ITU-T VCEG-N52, H.26L over IP and H.324 Framework, VCEG (SG16/Q6), Fourteenth Meeting, Santa Barbara, CA, September 2001.

- ITU-T VCEG-M77, Common Test Conditions for RTP/IP over 3GPP/3GPP2, VCEG (SG16/Q6), Thirteenth Meeting, Austin, TX, April 2001.