| ###### ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 14th Meeting: Santa Barbara, CA, USA, 24-27 Sep, 2001 | Document VCEG-N41 Filename: VCEG-N41.doc Generated: 10 Sept. ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Ragip Kurceren and Marta Karczewicz Nokia Research Center 6000 Connection Drive, 2-700 Irving, TX, 75039 | Tel: Fax: Email: | +1 972 894 6907 +1 972 894 4589 ragip.kurceren@nokia.com |
| Title: | Improvements on CABAC |  |  |
| Purpose: | Proposal |  |  |

# Introduction

- Context-based Adaptive Binary Arithmetic Coding (CABAC) technique [1],[2], adopted into high-complexity profile of H26L-TML provides improved coding efficiency and adaptivity to source statistics compared with the alternative entropy coding using universal table of variable length codes (UVLC). However, as also pointed out in D.151[3], the current design of the context models in CABAC is not optimal with regard to coding efficiency. In this document we investigate the performance of new context models for encoding of (RUN, LEVEL) pair in intra-coding. We also provide results of adaptive scan experiments for CABAC.

## New Context Models for (RUN, LEVEL) Encoding

- In the current CABAC in TML8.4, a (RUN, LEVEL) pair is encoded as follows: the runs and levels are first classified based on block/coefficient type: scanning mode, coefficient type(DC/AC), and coding mode (inter/intra or 16x16intra). Then the levels and runs are mapped onto a sequence of binary decisions, so-called bins. And each bin is assigned to a context based on its bin number which is defined as the location of the bin in the sequence representing the symbol. For example, for level encoding, there are 4 contexts. The first one is for the first bin; the second is for the second while the third context is for the rest of bins representing the magnitude of the level. The remaining context is used for the sign of the level. In summary, the context modeling of levels and runs are based on bin number and the coding/coefficient type.

In this document, we investigate new context models assigned to a bin representing levels of coefficients depending on the previously encoded (arithmetic encoder) or decoded (arithmetic decoder) coefficient level within the block. More specifically, similar to the current design, the symbols are first classified based on coefficient/coding mode, and then the following context models are applied for bins of levels and runs, respectively,

Proposed Context Model for Levels

if (bin_nr>3)

bin_nr=3;

end

if (Prev_level>MAX_LEVEL)

prev_level=MAX_LEVEL;

end

context_nr = (bin_nr-1)*MAX_LEVEL+Prev_level

- and

Proposed Context Model for Run

if (bin_nr>2)

bin_nr=2;

end

if (level>MAX_RUNL)

level=MAX_RUNL;

end

context_nr = (bin_nr-1)*MAX_RUNL+level

- where the prev_level is initialized to zero at the beginning of each block. Note that for double-scan, this variable is initialized at the beginning of each scan, i.e., twice per block.

| QP | Container | Foreman | News | Silent | Tempete | Mobile | Paris |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 3.19 | 3.92 | 3.11 | 4.74 | 4.01 | 3.63 | 3.34 |
| 10 | 3.10 | 3.39 | 2.85 | 4.32 | 3.88 | 3.73 | 3.04 |
| 16 | 2.64 | 2.67 | 2.26 | 3.17 | 3.37 | 3.37 | 2.55 |
| 20 | 2.20 | 2.14 | 1.76 | 2.38 | 2.79 | 2.90 | 2.20 |
| 24 | 1.30 | 1.51 | 1.35 | 2.28 | 1.89 | 2.01 | 1.54 |
| 28 | 1.16 | 0.95 | 0.99 | 1.76 | 1.55 | 1.57 | 1.18 |
| Ave.Bitrate Diff.* (%) | 1.79 | 1.83 | 1.58 | 2.37 | 2.49 | 2.40 | 1.87 |

Table 1 Performance improvement (bitrate savings in %) of new context models for intra-coding in CABAC.*Average Bitrate difference is calculated using results from QP=28,24,20,16.

- Table 1 illustrates the results of the proposed scheme in terms of the bitrate reduction in % from TML-8 where MAX_LEVEL=5 and MAX_RUNL=4. We have followed the test conditions from VCEG-M75[4] with the following exceptions. All the frames are encoded as I-frames in CABAC mode and additional QP results are included to illustrate further results. Results indicate that the proposed scheme provides bitrate savings from 0.95-3.37%(for common conditions[4]). Larger improvements upto 4.74% are observed at smaller QP values. In the last row in Table 1, we also provide average bitrate difference, as defined in [6]. It can be seen that on average, 1.5-2.5% bitrate savings can be realized by the proposed context models.

- Since double-scan is specifically designed for UVLC coding and, for the approach described above, results in initialization of context models twice per block, we have investigated the effect of removing double-scan for the proposed scheme in CABAC. Furthermore, in the following, we include results of alternative, adaptive scan methods.

- In Adaptive Intra Coding, Annex I of H.263+, two scans in addition to zigzag scan are employed for encoding of the transform coefficients of intra-coded blocks. The scan order is selected based on the prediction direction used for the transform coefficients. In the following we provide results of a similar approach in H.26L. The additional scan orders and the corresponding relationship between the intra-prediction types and the scan order, that are considered in this document, are illustrated in Table 2 and Table 3, respectively.

| 1 | 2 | 3 | 6 |  | 1 | 2 | 4 | 9 |  | 1 | 3 | 6 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 5 | 7 | 10 |  | 3 | 5 | 6 | 11 |  | 2 | 4 | 7 | 11 |
| 8 | 9 | 11 | 15 |  | 7 | 8 | 10 | 13 |  | 5 | 8 | 9 | 14 |
| 12 | 13 | 14 | 16 |  | 12 | 14 | 15 | 16 |  | 12 | 13 | 15 | 16 |
| Vertical scan |  |  |  |  | Secondary diagonal |  |  |  |  | Diagonal scan |  |  |  |

Table 2 Additional Scan Orders

| Prediction type | Scan |
| --- | --- |
| 0 | Normal |
| 1 | Secondary diagonal |
| 2 | Vertical |
| 3 | Diagonal |
| 4 | Vertical (transposed) |
| 5 | Secondary diagonal (transposed) |

Table 3 Intra-Prediction Modes

| QP | Container | Foreman | News | Silent | Tempete | Mobile | Paris |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 3.47 | 4.85 | 4.68 | 5.44 | 5.12 | 5.04 | 4.68 |
| 10 | 3.57 | 4.73 | 4.68 | 5.06 | 4.87 | 5.08 | 4.58 |
| 16 | 3.31 | 4.48 | 4.17 | 4.23 | 4.22 | 4.55 | 4.26 |
| 20 | 2.78 | 4.07 | 3.76 | 3.94 | 3.63 | 3.93 | 4.01 |
| 24 | 3.64 | 3.26 | 2.86 | 3.30 | 3.46 | 3.48 | 3.26 |
| 28 | 3.16 | 2.24 | 1.93 | 2.18 | 2.66 | 2.83 | 2.52 |
| Ave.Bitrate Diff.* (%) | 3.22 | 3.59 | 3.25 | 3.54 | 3.72 | 3.53 | 3.58 |

Table 4 Performance improvement (bitrate savings in %) with both adaptive scan and new context models. *Average Bitrate difference is calculated using results from QP=28,24,20,16.

Similar to results illustrated in Table 1, Table 4 illustrates the results of the proposed context models but, additionally, adaptive scan scheme described earlier is employed. As can be observed from Table 4, the improvements by new context models increase with the adaptive-scan schemes, as expected. We now observe bitrate savings from 1.93-4.55%(for common conditions). Again, improvements are larger at small QP values, upto 5.44%.

- Finally, in Table 5 we provide, as information, the results of adaptive scan alone. Performance improvement of adaptive scan is between 1-2%. The complexity related to this additional gain is quite minimal.

IPR Statement on Adaptive scan: The contributor(s) are not aware of any issued, pending, or planned patents associated with the technical content of this proposal.

| QP | Container | Foreman | News | Silent | Tempete | Mobile | Paris |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 1.09 | 1.38 | 1.72 | 1.59 | 1.84 | 1.87 | 1.61 |
| 10 | 1.17 | 1.55 | 1.90 | 1.34 | 1.81 | 1.79 | 1.71 |
| 16 | 1.28 | 1.83 | 1.89 | 1.21 | 1.60 | 1.46 | 1.78 |
| 20 | 1.04 | 1.93 | 1.85 | 1.44 | 1.30 | 1.21 | 1.90 |
| 24 | 2.31 | 1.70 | 1.42 | 0.99 | 1.36 | 1.52 | 1.66 |
| 28 | 1.96 | 1.26 | 0.86 | 0.35 | 1.24 | 1.10 | 1.34 |
| Ave.Bitrate Diff.* (%) | 1.66 | 1.75 | 1.57 | 1.11 | 1.35 | 1.34 | 1.73 |

Table 5 Performance improvement (bitrate savings in %) of adaptive scan in CABAC.*Average Bitrate difference is calculated using results from QP=28,24,20,16.

## Conclusion

- New context models are proposed for encoding of level/run pair in CABAC for intra coding. The results demonstrate bitrate savings from 0.95-3.37% ( for common conditions-and upto 4.74% at smaller QP). We have also investigated the effects of removing double-scan from CABAC and introducing adaptive scan. The resulting performance of the proposed contexts including adaptive scan scheme indicates that improvements from 1.93-4.48% (and upto 5.44% at smaller QP) can be realized.

# References

- [1]	Detlev Marpe, Gabi Blättermann, Thomas Wiegand, “Adaptive Codes for H.26L“, document VCEG-L13, , ITU-T Video Coding Experts Group, Eibsee, Germany, 09-12 January 2001.

- [2]	Detlev Marpe, Gabi Blättermann, Guido Heising, Thomas Wiegand, “Further Results for CABAC Entropy Coding Scheme", document VCEG-M59, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

- [3]	Detlev Marpe et al., "Improving coding efficiency of H.26L using CABAC", document D.151, ITU-T Q6/16, WP 3/16 ,Porto Seguro, 28 May – 8 June 2001.

[4]	Gisle Bjontegaard, " Recommended Simulation Conditions for H.26L", document VCEG-M75, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

[5]	Chul-Woo Kim and Sang Woo Rhie, " Adaptive Double Scan", document VCEG-M69, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

[6]	Gisle Bjontegaard, " Calculation of average PSNR differences between RD-curves ", document VCEG-M33, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

Patent Statement and Licensing Declaration

(Typically one per ITU-T Recommendation)

This declaration does not represent an implied license grant

Please return to:	Director	Place des Nations

Telecommunication Standardization Bureau	CH-1211 Geneva 20, Switzerland

International Telecommunication Union	Fax: +41 22 730 5853

| Patent Holder/Organization: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Legal Name |  | Nokia Inc. |  |  |  |
| Contact for license application: |  |  |  |  |  |
| Name & Department |  |  |  |  |  |
| Address |  |  |  |  |  |
| Tel. |  |  |  |  |  |
| Fax |  |  |  |  |  |
| E-mail |  |  |  |  |  |
| ITU-T Recommandation: |  |  |  |  |  |
| Number |  |  |  |  |  |
| Title |  |  |  |  |  |
| Licensing declaration |  |  |  |  |  |
| The Patent Holder believes to hold granted patents and/or pending applications, whose use would be required to implement the above ITU-T Recommendation and hereby declares, in accordance with the Statement on ITU-T Patent Policy (see ITU-T web site), that (check one box only). |  |  |  |  |  |
|  | 1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation. |  |  |  |  |
|  | 2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation. Such negotiations are left to the parties concerned and are performed outside the ITU-T. |  |  |  |  |
|  | 3 The Patent Holder is unwilling to grant licenses according to the provisions of either 1 or 2 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation are affected. - a description of the patent claims covering the Recommendation; |  |  |  |  |
| Signature |  |  |  |  |  |
| Organization |  |  |  |  |  |
| Name of authorized person |  |  |  |  |  |
| Title of authorized person |  |  |  |  |  |
| Signature |  |  |  |  |  |
| Place, Date |  |  |  |  |  |