| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 14th Meeting: Santa Barbara, CA, USA, 24-27 Sep., 2001 | Document VCEG-N49 Filename: VCEG-N49.doc Generated: 18 September ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Mathias Wien Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52074 Aachen, Germany | Tel: Fax: Email: | +49-241-80-27681 +49-241-80-22196 wien@ient.rwth-aachen.de |
| Title: | # New ABT Results using CABAC |  |  |
| Purpose: | Information |  |  |

_____________________________

# Introduction

The technique of Adaptive Block Transforms (ABT) was presented in detail at the last raporteurs meeting [1]. We refer to this document for details on the transforms, the ABT quantization and scan, as well as the adaption of the deblocking filter for ABT. The performance results given in the accomanying core experiment report [2] were based on an UVLC design presented in [3].

In this paper, we propose the application of CABAC for encoding of the ABT symbols. In distiction to the former UVLC approach, no 3D-Symbols (Last,Level,Run) are used here. The CABAC encoding technique is applied to the ABT CBP, Level, and Run symbols seperately, using Level = 0 as the EOB symbol. The CABAC encoding methods of the TML symbols are described in the Test Model Document [4].  Only slight modifications of the existing CABAC methods are necessary in order to employ CABAC for ABT coefficients.

Simulation results according to the common testing conditions are given in the last section of this paper. The ABT implementation is based on the TML80 software. It is shown that in terms of objective rate/distortion measurements, ABT performs equal or superior in comparison to TML80. The maximum RD gain of ABT is about 0.5 dB.

# Context-based Adaptive Binary Arithmetic Coding (CABAC) for ABT Symbols

## CBPY

0

1

2

3

0

1

2

3

0

1

2

3

0

1

2

3

0

0

1

0

1

Figure 0 : CBPY for ABT.

The value range of the Coded Block Pattern for Luma (CBPY) is connected to the chosen MB mode for the macroblock to be coded. For modes 1-4 the size of the CBPY blocks corresponds to the block size used for motion compensation and transform. For the higher MB modes the CBPY ordering as described in the TML is used. The numbering of the CBPYs for each MB mode is given in Figure 1 below.

The CBPY encoding method proposed in the TML is used for the ABT CBPY. The CBPY for mode 4-7 macroblocks is coded just using the TML technique. For mode 1 and modes 2,3, only one or two CBP bits are coded, respectively. In the TML, the context for coding each bit depends on the CBPY of the neighbouring 8x8 blocks in horizontal and vertical direction. Here, the larger blocks are treated as combinations of 8x8 blocks for context generation.

## Level

The Level values of mode 5-7 are coded using the TML Level coding routine. For modes 1-4, CBP = 1 directly signals the existence of at least one non-zero Level in the encoded subblock. Therefore,  the first coefficient Level in a mode 1-4 subblock is coded without the first (EOB-) Bin of the TML Level binarization.

## Run

The maximum Run value for ABT coefficients is macroblock mode dependend. E.g., the maximum Run value is 256 in mode 1, which cannot be encoded efficiently with the unary binarization used in the TML. The binarization used here for modes 1-7 is given in the following Table 1. Mode 7 runs are coded using the TML approach.

| #### RUN | Binarization |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mode |  |  | 5,6 |  |  |  |  | 4 | 3,2 | 1 |
| 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |
| 2 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |
| 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| ... | 0 | 0 | 0 | ... |  |  |  |  | . | . | . |
| Bin_no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

Table 1 : Binarization of the Run symbols for ABT MB modes 1-6. Runs > 2 are 
binarized by a fixed length binarization.

The first three Runs are binarized by means of the unary code tree. The remaining runs are coded using a fixed length binarization (bit plane binarization).  The number of bins for the binarization of the larger Run symbols is determined by the maximum possible Run in the Subblock of the corresponding macroblock mode. It variies from 8 bins for mode 5 and 6 to 11 bins for mode 1. For encoding of the bins 13 contexts are used. The assignment of the contexts is given in Table 2.

| Mode | Bin_no. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| 1 | 1 | 2 | 2 | 7 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 2 | 1 | 2 | 2 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | - |
| 3 | 1 | 2 | 2 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | - |
| 4 | 3 | 4 | 4 | 7 | 8 | 9 | 11 | 12 | 13 | - | - |
| 5 | 5 | 6 | 6 | 8 | 10 | 11 | 12 | 13 | - | - | - |
| 6 | 5 | 6 | 6 | 8 | 10 | 11 | 12 | 13 | - | - | - |

Table 2 : Contexts for the Bins depending on the MB mode.

# Simulation Results

Simulation results are given for the sequences recommended in the common conditions document VCEG-M75. The quantiszation parameters for the different frame types were set to

QUANT_Intra = QUANT_Inter-2

and, if applicable,

QUANT_BFrame = QUANT_Inter+2.

The encoder was set up with the basic configuration shown in Figure 2. Only the frame skip, the number of B-frames, and the quantization parameters were varied for the simulations. Both, ABT coder and TML coder allways used the same settings.

The plots show the average decoded PSNR over the resulting bit rate. The name of the sequence is printed in the figure title of each plot. The legend denotes the coder type, the used frame structure, and the resulting frame rate.

# New Input File Format is as follows

# <ParameterName> = <ParameterValue> # Comment

#

# See configfile.h for a list of supported ParameterNames

#

# Known bug: Empty strings, i.e. to signal a non-existent trace file must be coded as ""

#

TRModulus=256# Modulus for TR, not yet used, MUST be 256

PicIdModulus=256# Modulus for the PictureID (used for RPS), …

# PictureTypeSequence=PBB# A string that defines the picture types,…

##########################################################################################

# Encoder Control

##########################################################################################

MVResolution=1# Motion Vector Resolution: 0: 1/4-pel, 1: 1/8-pel

UseHadamard=1# Hadamard transform (0=not used, 1=used)

SearchRange=16# Max search range

NumberRefereceFrames =5# Number of previous frames used for inter motion…

MbLineIntraUpdate=0# Error robustness(extra intra macro block…

InterSearch16x16=1# Inter block search 16x16 (0=disable, 1=enable)

InterSearch16x8=1# Inter block search 16x8  (0=disable, 1=enable)

InterSearch8x16=1# Inter block search  8x16 (0=disable, 1=enable)

InterSearch8x8=1# Inter block search  8x8  (0=disable, 1=enable)

InterSearch8x4=1# Inter block search  8x4  (0=disable, 1=enable)

InterSearch4x8=1# Inter block search  4x8  (0=disable, 1=enable)

InterSearch4x4=1# Inter block search  4x4  (0=disable, 1=enable)

##########################################################################################

# Error Resilience / Slices

##########################################################################################

SliceMode=0# Slice mode (0=off 1=fixed #mb in slice 2=… SliceArgument=2# Slice argument (Arguments to modes 1 and 2 above)

##########################################################################################

# Output Control, NALs

##########################################################################################

SymbolMode=1# Symbol mode (Entropy coding method: 0=UVLC,… OutFileMode=0# Output file mode, to be described

PartitionMode=0# Partition Mode, to be described

SequenceHeaderType=0# Type of Sequence HeaderType (0:none, 1:MiniBinary)

##### added by HS #####

RestrictSearchRange=1# restriction for (0: blocks and ref, 1: ref, 2:no…

RDOptimization =1# rd-optimized mode decision (0:off, 1:on)

#LastFrameNumber=99# Last frame number that have to be coded

#ChangeQPP=23# QP (P-frame) for second part of sequence

#ChangeQPB=25# QP (B-frame) for second part of sequence

#ChangeQPStart=67# Frame no. for second part of sequence

Figure 2 : Encoder configuration used for all simulations.

# Discusison

The RD plots show that the application of Adaptive Block Transforms results in an improved or equal objective RD performance for the coding scheme under most of the tested conditions. The maximum PSNR gain is about 0.5 dB. Only Mobile CIF shows a minor PSNR loss under some conditions.

The subjective performance is improved by ABT. More detail is preserved and less blocking artifacts have to be removed. Even in the cases with a RD loss, the subjective quality of ABT is clearly superior to the TML results.

Subjective results will be presented at the meeting.

# IPR-Statement

The contributors are not aware of any issued, pending, or planned patents associated with the technical content of the proposed coding method.

# References

[1]	M. Wien and A. Dahlhoff, 'Adaptive Block Transforms', ITU-T Q6/16 VCEG, Doc. VCEG-M62r1,

Austin, TX, April 2001.

[2]	M. Wien, 'ABT Core Experiment Report', ITU-T Q6/16 VCEG, Doc. VCEG-M63,

Austin, TX, April 2001.

[3]	L. Kerofsky, 'Core Experiment on Adaptive Block Transform', ITU-T Q6/16 VCEG, Doc. VCEG-M68,

Austin, TX, April 2001.

[4]	G. Bjontegaard (editor), 'H.26L Test Model Long Term Number 8 (TML-8) draft 0', ITU-T Q6/16 VCEG, 
this meeting

|  |  | Container QCIF IP 10Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR dB |
| 14 | 16 | 5057 | 28.50 | 5126 | 29.00 |
| 18 | 20 | 8458 | 31.35 | 8489 | 31.76 |
| 22 | 24 | 13559 | 33.94 | 13630 | 34.33 |
| 26 | 28 | 23298 | 36.44 | 24058 | 36.71 |

|  |  | Foreman QCIF IP 10Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR dB |
| 14 | 16 | 17596 | 28.45 | 18558 | 28.92 |
| 18 | 20 | 28791 | 31.05 | 29213 | 31.43 |
| 22 | 24 | 45656 | 33.61 | 46053 | 33.88 |
| 26 | 28 | 71838 | 36.19 | 72626 | 36.37 |

|  |  | News QCIF IP 10Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR dB |
| 14 | 16 | 10817 | 28.65 | 11299 | 28.84 |
| 18 | 20 | 18630 | 31.61 | 19067 | 31.79 |
| 22 | 24 | 29865 | 34.49 | 30722 | 34.65 |
| 26 | 28 | 46214 | 37.34 | 47094 | 37.41 |

|  |  | Silent QCIF IP 15Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR dB |
| 14 | 16 | 12138 | 28.75 | 12778 | 29.05 |
| 18 | 20 | 22043 | 31.18 | 22799 | 31.48 |
| 22 | 24 | 37306 | 33.72 | 38207 | 33.96 |
| 26 | 28 | 60861 | 36.50 | 61644 | 36.67 |

|  |  | Paris CIF IP 15Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR dB |
| 14 | 16 | 60178 | 27.44 | 63615 | 27.67 |
| 18 | 20 | 111053 | 30.28 | 114782 | 30.45 |
| 22 | 24 | 192319 | 33.04 | 197919 | 33.27 |
| 26 | 28 | 317350 | 35.87 | 321869 | 36.08 |

|  |  | Mobile CIF IP 30Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR dB |
| 14 | 16 | 202190 | 25.04 | 227510 | 25.50 |
| 18 | 20 | 360810 | 28.03 | 400397 | 28.53 |
| 22 | 24 | 679920 | 30.99 | 744590 | 31.46 |
| 26 | 28 | 1276979 | 34.18 | 1336380 | 34.44 |

|  |  | Tempete CIF IP 30Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR dB |
| 14 | 16 | 157686 | 26.69 | 174358 | 27.33 |
| 18 | 20 | 293108 | 29.43 | 320886 | 29.95 |
| 22 | 24 | 565350 | 32.15 | 615906 | 32.61 |
| 26 | 28 | 1086611 | 35.12 | 1128078 | 35.33 |

|  |  | Mobile CIF IBP 30Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR DB |
| 14 | 16 | 143447 | 25.15 | 159551 | 25.46 |
| 18 | 20 | 262765 | 27.93 | 289619 | 28.30 |
| 22 | 24 | 509658 | 30.74 | 550764 | 31.07 |
| 26 | 28 | 981570 | 33.76 | 1022505 | 33.89 |

|  |  | Tempete CIF IBP 30Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR DB |
| 14 | 16 | 117799 | 26.80 | 128362 | 27.28 |
| 18 | 20 | 219169 | 29.37 | 235671 | 29.74 |
| 22 | 24 | 425192 | 31.89 | 454430 | 32.19 |
| 26 | 28 | 839593 | 34.61 | 871293 | 34.74 |

|  |  | Mobile CIF IBBP 30Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR DB |
| 14 | 16 | 125496 | 25.14 | 138160 | 25.38 |
| 18 | 20 | 232906 | 27.83 | 255177 | 28.11 |
| 22 | 24 | 457555 | 30.56 | 488380 | 30.81 |
| 26 | 28 | 900937 | 33.50 | 933798 | 33.59 |

|  |  | Tempete CIF IBBP 30Hz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TML |  | ABT |  |
| QPI | QPP | Rate bit/s | PSNR dB | Rate bit/s | PSNR DB |
| 14 | 16 | 103549 | 26.78 | 112238 | 27.18 |
| 18 | 20 | 195602 | 29.28 | 209155 | 29.55 |
| 22 | 24 | 383772 | 31.71 | 405256 | 31.92 |
| 26 | 28 | 772771 | 34.34 | 800869 | 34.42 |