| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O28 Filename: VCEG-O28.doc Generated: 28 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wedi Institute of Communication Theory and Signal Processing University of Hannover, Germany | Tel: Fax: Email: | +49 511 7625304 +49 511 7625333 wedi@tnt.uni-hannover.de |
| Title: | More Results on Adaptive Interpolation Filter for H.26L |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

In H.26L displacement vector resolutions of 1/4- or 1/8-pel are applied in the motion compensated prediction process. In order to estimate and compensate these fractional-pel displacements, invariant interpolation filters are used. The same filter-coefficients are used for all sequences and for all frames of a sequence. Therefore it is not possible to consider non-stationary statistical properties of video signals (e.g. aliasing, motion) in the interpolation process.

In this proposal an adaptive interpolation scheme is presented. This interpolation scheme is based on filter coefficients that are adapted once per frame to the non-stationary statistical properties of the video signal. The filter-coefficients are coded and transmitted. This technique was also presented in [1] and in VCEG-N28.

The experimental results are similar to the results of VCEG-N28. In contrast to VECEG-N28 five reference frames and the UMV mode are used. This leads to a slightly reduced gain for the adaptive interpolation filter compared to the results in VCEG-N28. Furthermore two new test-sequences with fading transitions were generated and used for the experimental results.

Due to the adaptive interpolation filter a coding gain up to 0.8 dB PSNR is obtained with common test-sequences. For the fading test-sequences a gain up to 1.5 dB is obtained.

# Adaptive Interpolation Filter

In this section the filter scheme, the estimation of the filter-coefficients and their transmission is presented. It is assumed that the displacement vector resolution is 1/4-pel.

### Filter Scheme

In H.26L the interpolation process of the TML is described as two successive interpolations with two different interpolation filters (Figure 1).

Figure 1: Interpolation process for 1/4-pel displacement vector resolution.
 (TML8: Anchor,   TML8-AIF: TML8 with Adaptive Interpolation Filter)

In TML8 the first interpolation filter is a symmetric non-adaptive 6-tap filter that interpolates the image signal on 1/2-pel positions (2:1 resolution) and the second one is a bilinear filter that interpolates the image signal on 1/4-pel positions (4:1 resolution).

In TML8-AIF a symmetric Adaptive Interpolation Filter is used for Filter 1. For Filter 2 the same bilinear filter is used than in TML8. In Table 1 the different filters of TML8 and TML-AIF are shown in detail. In this table ai denotes the filter-coefficients that are adapted once per frame.

| ## Codec | Filter 1 | Coefficients |
| --- | --- | --- |
| TML8 | Non-adaptive 6-tap filter | ( 1 –5 20 20 –5 1 )/32 |
| TML8-AIF | adaptive 6-tap filter | a1 a2 a3 a3 a2 a1 |

Table 1: Filter 1 of the non-adaptive (TML8) and the adaptive (TML8-AIF) codec.
a1,a2,a3  denote the filter-coefficients that are adapted once per frame.

Thus, only 3 filter coefficients a1,a2,a3 have to be adapted and transmitted. For a1=1/32, a2=-5/32, a3=20/32 the interpolation schemes of TML8-AIF and TML8 are identical. Therefore the coding efficiency of TML8-AIF is at least as good as the coding efficiency of TML8. Only a very small amount of bits have to be spend for the transmission of the filter coefficient.

### Estimation of Filter Coefficients

The scheme of filter-coefficient estimation is not part of this proposal. It is an encoder issue. However, the motion compensated prediction with the filter-coefficient estimation that is used in this contribution, consists of the following steps:

- Displacement vectors are estimated. For this purpose, an initial filter is applied.

- Estimation of filter-coefficients by minimizing the energy of the prediction error when performing the motion compensated prediction with the displacement vectors from step 1.

- The current frame is predicted by the motion compensated prediction. For this purpose the adapted filter-coefficients of step 2 and the displacement vectors of step 1 are applied.

In step 2 the coefficients of the adaptive interpolation filters are estimated once per frame. The image signal and the displacement vectors of the original frame are taken into account. Linear regression is employed to calculate the filter coefficients that minimize the energy of the prediction error.

Another, less complex coefficient estimation scheme would be the selection of the filter coefficients from a given set.

### Transmission of Filter Coefficients

The adaptive interpolation filter described in this proposal requires the transmission of their coefficients. For this purpose, a differential coding scheme with a uniform 12 bit quantization is applied. This means that even for a fixed length code only 3*12=36 additional bits are necessary for each frame.

# Experimental Results

For the experimental results two new test-sequences were generated. A fading transition to black was applied to the first 50 frames of the sequences Mobile & Calendar and Flower. Such fading transitions are very popular in movie trailers or music videos. Four images of this sequences are exemplarily shown in the following figure.

Figure 2: Four images of the two test-sequences with fading transitions to black.

Table 2 shows the test-sequences that are used for the experimental results.

| Test-sequence | Res. | Frame rate | No. of frames |
| --- | --- | --- | --- |
| Mobile & Calendar | CIF | 30 | 300 |
| Foreman | CIF | 30 | 300 |
| Bus | CIF | 30 | 150 |
| Container | CIF | 30 | 300 |
| Mobile & Calendar (fade to black) | CIF | 30 | 50 |
| Flower Garden (fade to black) | CIF | 30 | 50 |

Table 2: Applied test-sequences.

For the objective results, rate-distortion plots are given in the Appendix. Each rate-distortion plot contains the following curves:

- TML8: 		TML8 with non-adaptive interpolation filter (anchor)

- TML8-AIF: 	TML8 with        adaptive interpolation filter

For all results 1/4-pel displacement vector resolution, 5 reference frame and no B-frames are used. In contrast to the results of VCEG-N28, also the UMV mode is enabled.

# Summary

In this proposal an adaptive interpolation filter scheme for H.26L is presented. The adaptive interpolation filter is based on filter-coefficients that are estimated during the motion compensated prediction process for each frame. The coefficients are coded and transmitted. Due to this adaptive interpolation filter a coding gain of up to 0.8 dB PSNR is obtained with common test-sequences.

With the fading sequences a gain up to 1.5 dB is obtained. This gain is due to the fact that the coefficients of the adaptive filer are not normalized.

# Proposal

It is proposed to start a core experiment on the adaptive interpolation filtering technique, in order to verify the potential of this technique and further improve it.

# References

- T. Wedi, "Adaptive Interpolation Filter for Motion Compensated Hybrid Video Coding," Picture Coding Symposium (PCS 2001), Seoul, Korea, April 2001.
(downloadable via ftp://ftp.tnt.uni-hannover.de/pub/papers/2001/PCS2001-TW.pdf  )

- T. Wedi, “Adaptive Interpolation Filter for H.26L”, ITU-T SG16/Q6, doc. VCEG-N28, Santa Barbara, CA, USA, Sep. 2001

Appendix

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Institut für Theoretische Nachrichtentechnik und Informationsverarbeitung, University of Hannover |  |  |
| Mailing address | #### Institut für Theroretische Nachrichtentechnik und Informationsverarbeitung Universität Hannover Appelstr. 9a 30167 Hannover, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Thomas Wedi |  |  |
| Telephone | +49 511 762 5304 |  |  |
| Fax | +49 511 762 5333 |  |  |
| Email | wedi@tnt.uni-hannover.de |  |  |
| Place and date of submission | Pattaya, 4th December |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | More results on Adaptive Interpolation Filter for H.26L |  |
| Contribution number |  | VCEG-O16 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: