| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O17r3 Filename: VCEG-O17r3.doc Generated: 27 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Heiko Schwarz and Thomas Wiegand Heinrich Hertz Institute (HHI) Einsteinufer 37 10587 Berlin, Germany | Tel: Fax: Email: | +49-(0)30-31002 206 / 617 +40-(0)30-392 72 00 hschwarz@hhi.de wiegand@hhi.de |
| Title: | Tree-structured macroblock partition |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Summary

In the current TML [1], seven macroblock partitions for motion compensation are supported. The reference frame parameter is transmitted once per macroblock if a predictive coding mode is chosen. We propose a minor syntax alteration that provides tree-structured macroblock partitions in motion-compensated coding modes. The reference frame parameter is transmitted on block basis for 16x16, 16x8, 8x16, and 8x8 blocks. For smaller block sizes, a single reference frame parameter for all blocks of an 8x8 sub-partition is used. The experimental results show a general improvement in terms of coding efficiency while the complexity of the encoding strategy is not increased. The results indicate that the gains are increased for complex scenes and higher bit-rates. Although not yet completely verified, we believe that the 8x8 Intra mode provides potential for improvements in error-prone environments, where an increased Intra rate is often used to stop temporal error propagation. Moreover, the new tree-structured macroblock partition is believed to provide the possibility to speed up the motion search for 4x4, 4x8, and 8x4 blocks.

# Tree-structured Macroblock Partition

## P-frames

For P-frames, the predictive modes with block shapes of 8x8, 8x4, 4x8, and 4x4 samples are replaced by a single macroblock mode called 8x8(split). If this mode is chosen, four additional codewords (one for each 8x8 sub-partition) are transmitted. These codeword indicate if the corresponding 8x8 sub-partition of the macroblock is coded predictively with block shapes of 8x8, 8x4, 4x8, or 4x4, or if it is coded in INTRA-mode. The Tables 1 and 2 show the corresponding code words for the macroblock modes and the modes of 8x8 sub-partitions.

Table 1: Macroblock modes for P-frames

| Code number | Macroblock mode |
| --- | --- |
| 0 | SKIP |
| 1 | 16x16 |
| 2 | 16x8 |
| 3 | 8x16 |
| 4 | 8x8 (split) |
| 5 (UVLC only) | 8x8 (split, all ref=0) |
| 6 | Intra4x4 |
| 7 … | Intra16x16 |

Table 2: Modes for 8x8 sub-partitions in P-frames

| Code number | 8x8 partition mode |
| --- | --- |
| 0 | 8x8 |
| 1 | 8x4 |
| 2 | 4x8 |
| 3 | 4x4 |
| 4 | Intra |

For the predictive macroblock modes with block shapes of 16x16, 16x8, and 8x16 one reference parameter is coded for each block. However if the 8x8(split) mode is chosen, only one reference parameter is transmitted for each 8x8 sub-partition regardless what mode it is coded in. Of course, no reference parameter is transmitted if the Intra mode was chosen for a 8x8 partition. If UVLC is used for entropy coding, an additional macroblock mode is supported 8x8(split, all ref=0). It indicates the usage of the 8x8(split)-mode, however no reference frame parameter is transmitted for the whole macroblock; the reference frame parameters are set to zero. Since the indication of this mode costs just as many bits as the indication of the 8x8(split)-mode, 4 bits can always be saved if the previous reference frame was chosen for all 8x8 sub-partitions in the UVLC case. For CABAC, this additional mode is not used.

Beside the re-design of macroblock modes and the introduction of additional 8x8 block modes, a minor alteration concerning the motion vector prediction for 8x4 and 4x8 block was necessary. Since neighboring 8x8 partitions are generally not coded in the same mode, the directional prediction of motion vector components cannot be used for 8x4 and 4x8 block. It is replaced by median prediction.

### Motion Estimation and Mode Decision

For generating the simulation results and comparing the syntax alteration with the current TML-8 [1] using rate-distortion optimization, the following algorithm for motion estimation and mode decision was used:

- For each 8x8 sub-partition in coding order:

  - Perform motion estimation for 4x4, 4x8, 8x4, and 8x8 blocks and all reference frames

  - Determine the best reference frame for each of these four partitionings by minimizing

SAD +  Rate(Motion Vectors, Reference frame parameter),

where SAD is calculated between the original and the prediction signal.

  - Determine the coding mode of 8x8 block using the rate-constrained mode decision, i.e. minimize

SSD +  Rate(MV, REF, Luma-Coeff, block 8x8 mode)

Here the SSD calculation is based on the reconstructed signal after DCT, quantization, and IDCT.

- Perform motion estimation for 8x16, 16x8, and 16x16 blocks and determine the best reference parameter for each block by minimizing

SAD +  Rate(Motion Vectors, Reference frame parameter)

- Choose macroblock mode from the set (SKIP, 16x16, 16x8, 8x16, 8x8(split), Intra4x4, Intra16x16) by utilizing the rate-constrained mode decision as described in TML-8. At this point, the distortion and most parts of the rate term calculated during the mode decision for 8x8 partitions are re-used for the cost calculation of the 8x8(split) mode.

The complexity is virtually not increased in comparison with TML-8. The only additional operation is the cost calculation of the Intra mode in the mode decision for 8x8 partitions.

## B-frames

For B-frames, a similar tree-structured macroblock partitioning is proposed. As in P-frames, one reference frame parameter is transmitted for each 16x16, 16x8, and 8x16 block as well as for each 8x8 sub-partition. Additionally, for each 16x16, 16x8, 8x16 block, and each 8x8 sub-partition, the prediction direction (forward, backward, bi-directional) can be chosen separately. For avoiding a separate code word to specify the prediction direction, the indication of the prediction direction is incorporated into the codewords for macroblock modes and 8x8 partitioning modes, respectively, as shown in the table 3 and 4. The bi-directional coding mode specified in the TML-8 is removed.

Table 3: Macroblock modes for B-frames

| Code number | Macroblock mode | 1. block | 2. block |
| --- | --- | --- | --- |
| 0 | Direct |  |  |
| 1 | 16x16 | Forw. |  |
| 2 | 16x16 | Backw. |  |
| 3 | 16x16 | Bidirect. |  |
| 4 | 16x8 | Forw. | Forw. |
| 5 | 8x16 | Forw. | Forw. |
| 6 | 16x8 | Backw. | Backw. |
| 7 | 8x16 | Backw. | Backw. |
| 8 | 16x8 | Forw. | Backw. |
| 9 | 8x16 | Forw. | Backw. |
| 10 | 16x8 | Backw. | Forw. |
| 11 | 8x16 | Backw. | Forw. |
| 12 | 16x8 | Forw. | Bidirect. |
| 13 | 8x16 | Forw. | Bidirect. |
| 14 | 16x8 | Backw. | Bidirect. |
| 15 | 8x16 | Backw. | Bidirect. |
| 16 | 16x8 | Bidirect. | Forw. |
| 17 | 8x16 | Bidirect. | Forw. |
| 18 | 16x8 | Bidirect. | Backw. |
| 19 | 8x16 | Bidirect. | Backw. |
| 20 | 16x8 | Bidirect. | Bidirect. |
| 21 | 8x16 | Bidirect. | Bidirect. |
| 22 | 8x8(split) |  |  |
| 23 | Intra4x4 |  |  |
| 24 … | Intra16x16 |  |  |

Table 4: Modes for 8x8 sub-partitions in B-frames

| Code number | 8x8 partition mode | Prediction |
| --- | --- | --- |
| 0 | 8x8 | Forw. |
| 1 | 8x8 | Backw. |
| 2 | 8x8 | Bidirect. |
| 3 | 8x4 | Forw. |
| 4 | 4x8 | Forw. |
| 5 | 8x4 | Backw. |
| 6 | 4x8 | Backw. |
| 7 | 8x4 | Bidirect. |
| 8 | 4x8 | Bidirect. |
| 9 | 4x4 | Forw. |
| 10 | 4x4 | Backw. |
| 11 | 4x4 | Bidirect. |
| 12 | Intra |  |

For generating the simulation results, the prediction direction is determined by minimzing the following Lagrange cost:

SAD +  Rate (Motion vectors, Reference frames).

The SAD is calculated between the original and the prediction signal. No DCT, quantization, and IDCT is performed for this decision. The rate-constrained macroblock mode decision is only carried out for the best 16x16, 16x8, 8x16, and 8x8(split) partitions as well as for the Direct and Intra modes. Thus, with the new scheme, the complexity is even a bit decreased in comparison to the TML-8 [1].

# Remaining Coder Parts

All remaining coder parts are operated as described in the H.26L Test Model Long Term Number 8 (TML-8) draft 0. If CABAC is used as entropy coding method, the binarization of the macroblock modes is altered; for the coding of the 8x8 sub-partition modes, 3 or 4 (B-, P-frame) context models are used.

# Simulation Results

The simulations are conducted following the conditions specified in VCEG-N81 [2]. The proposed tree-structured macroblock partitioning was integrated in the TML-85 software. Comparisons are related to the original TML-85 software. The following configurations are used:

- P-frames only, ¼-pel MC, UVLC				(baseline config.)

- P-frames only, ¼-pel MC, CABAC				(baseline config., CABAC)

- 2 B-frames, ¼-pel MC (QCIF), 1/8-pel MC(CIF), UVLC		(advanced config., UVLC)

- 2 B-frames, ¼-pel MC (QCIF), 1/8-pel MC(CIF), CABAC	(advanced config.)

The rate-distortion optimization was always enabled; the motion vectors search was conducted over the range of 32 samples. For calculating average PSNR gains and bit-rate savings, the method specified in VCEG-M33 [3] is utilized with the QP values 28, 24, 20, and 16. The tables 5-8 show the average PSNR gains and corresponding bit-rate savings for the four configurations given above. Rate-distortion curves for all cases with 1 and 5 reference frames are plotted in the accompanying Excel sheet. As it can be seen from the tables and the rate-distortion curves, the coding efficiency of the proposed method is generally never worse than that of TML-8. The only exception is the container sequence coded with P-frames only, UVLC, and 10 reference frames; but the performance loss is negligible in this case.

On average, the proposed syntax alteration provides bit-rate savings of up to 3% depending on the number of reference frames used and the chosen entropy coding method. For higher bit-rates generally larger improvements are obtained. The largest PSNR gains and bit-rate savings have been obtained if multi-frame prediction is used and if CABAC is used as entropy coding method.

The tables 9 and 10 show average performance gains for the whole bit-rate range. Therefore the method specified in VCEG-M33 [3] is utilized with QP values of 28, 20, 12, and 4. In addition, the sequences Tempete (QCIF, 10Hz) and Bus (CIF, 30Hz) are added to the test set specified in VCEG-N81 [2]. For the whole bit-rate range, our proposed syntax alteration provides bit-rate savings of up to 5%.

To verify the statement, that our proposed syntax changing provides increased PSNR gains and bit-rate savings for more complex sequences, we performed an additional test with the interlaced sequences as specified in VCEG-N85 [4]. These interlaced sequences have been coded as field pictures using the following configuration:

1. 	P-pictures only, ¼-pel MC, 16x16 motion search, 1 or 5 ref. frames, CABAC

- 2 B-pictures inserted, ¼-pel MC, 16x16 motion search, 1 or 5 ref. frames, CABAC

The Tables 11 and 12 show the average performance gains for these sequences where the given PSNR values are averaged over all field pictures. On average, our proposed syntax alteration yields bit-rate savings of 3,23% to 4.97% in the low bit-rate range (QP=28,24,20,16) compared to the TML-85 software. The highest gains are observed if multi-frame prediction is in use.

Table 5: Results for configuration with P-frames only, UVLC, QP=28,24,20,16

|  |  | 1 reference frame |  | 5 reference frames |  | 10 reference frames |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Container | QCIF 10Hz | 0,04 | 0,84 | 0,02 | 0,43 | -0,02 | -0,40 |
| News | QCIF 10Hz | 0,08 | 1,30 | 0,12 | 1,98 | 0,14 | 2,44 |
| Foreman | QCIF 10Hz | 0,11 | 2,21 | 0,14 | 2,88 | 0,15 | 2,99 |
| Silent | QCIF 15Hz | 0,07 | 1,33 | 0,12 | 2,39 | 0,15 | 3,07 |
| Paris | CIF 15Hz | 0,03 | 0,65 | 0,10 | 2,02 | 0,13 | 2,63 |
| Mobile | CIF 30Hz | 0,03 | 0,57 | 0,12 | 2,57 | 0,15 | 3,26 |
| Tempete | CIF 30Hz | 0,04 | 1,01 | 0,11 | 2,61 | 0,12 | 2,78 |
| Average |  | 0,06 | 1,13 | 0,10 | 2,13 | 0,12 | 2,40 |

Table 6: Results for configuration with P-frames only, CABAC, QP=28,24,20,16

|  |  | 1 reference frame |  | 5 reference frames |  | 10 reference frames |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Container | QCIF 10Hz | 0,03 | 0,60 | 0,04 | 0,73 | 0,02 | 0,38 |
| News | QCIF 10Hz | 0,13 | 2,27 | 0,13 | 2,29 | 0,13 | 2,23 |
| Foreman | QCIF 10Hz | 0,15 | 3,04 | 0,17 | 3,56 | 0,21 | 4,20 |
| Silent | QCIF 15Hz | 0,10 | 2,01 | 0,13 | 2,59 | 0,19 | 3,74 |
| Paris | CIF 15Hz | 0,09 | 1,73 | 0,14 | 2,83 | 0,17 | 3,24 |
| Mobile | CIF 30Hz | 0,05 | 1,12 | 0,15 | 3,25 | 0,17 | 3,66 |
| Tempete | CIF 30Hz | 0,07 | 1,67 | 0,14 | 3,46 | 0,15 | 3,56 |
| Average |  | 0,09 | 1,78 | 0,13 | 2,67 | 0,15 | 3,00 |

Table 7: Results for configuration with 2 B-frames, UVLC, QP=28,24,20,16

|  |  | 1 reference frame |  | 5 reference frames |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Container | QCIF 10Hz | 0,05 | 0,93 | 0,03 | 0,69 |
| News | QCIF 10Hz | 0,10 | 1,76 | 0,14 | 2,44 |
| Foreman | QCIF 10Hz | 0,07 | 1,44 | 0,09 | 1,76 |
| Silent | QCIF 15Hz | 0,09 | 1,83 | 0,10 | 2,10 |
| Paris | CIF 15Hz | 0,13 | 2,45 | 0,17 | 3,20 |
| Mobile | CIF 30Hz | 0,06 | 1,33 | 0,10 | 2,27 |
| Tempete | CIF 30Hz | 0,06 | 1,73 | 0,09 | 2,21 |
| Average |  | 0,08 | 1,64 | 0,10 | 2,10 |

Table 8: Results for configuration with 2 B-frames, CABAC, QP=28,24,20,16

|  |  | 1 reference frame |  | 5 reference frames |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Container | QCIF 10Hz | 0,08 | 1,32 | 0,05 | 0,93 |
| News | QCIF 10Hz | 0,17 | 3,00 | 0,17 | 2,97 |
| Foreman | QCIF 10Hz | 0,17 | 3,60 | 0,16 | 3,27 |
| Silent | QCIF 15Hz | 0,12 | 2,54 | 0,14 | 2,94 |
| Paris | CIF 15Hz | 0,17 | 3,34 | 0,21 | 4,03 |
| Mobile | CIF 30Hz | 0,09 | 2,19 | 0,13 | 3,06 |
| Tempete | CIF 30Hz | 0,11 | 2,82 | 0,14 | 3,55 |
| Average |  | 0,13 | 2,69 | 0,14 | 2,96 |

Table 9: Results for configuration with P-frames only, CABAC, 5 reference frames, QP=28,20,12,4

|  |  | Average PSNR gain [dB] | Average rate saving [%] |
| --- | --- | --- | --- |
| Container | QCIF 10Hz | 0,09 | 1,89 |
| News | QCIF 10Hz | 0,16 | 2,51 |
| Foreman | QCIF 10Hz | 0,14 | 4,60 |
| Tempete | QCIF 10Hz | 0,27 | 4,74 |
| Silent | QCIF 15Hz | 0,21 | 3,66 |
| Paris | CIF 15Hz | 0,17 | 2,98 |
| Mobile | CIF 30Hz | 0,20 | 3,58 |
| Tempete | CIF 30Hz | 0,22 | 4,32 |
| Bus | CIF 30Hz | 0,26 | 4,75 |
| Average |  | 0,20 | 3,67 |

Table 10: Results for configuration with 2 B-frames, CABAC, 5 reference frames, QP=28,20,12,4

|  |  | Average PSNR gain [dB] | Average rate saving [%] |
| --- | --- | --- | --- |
| Container | QCIF 10Hz | 0,08 | 1,69 |
| News | QCIF 10Hz | 0,19 | 3,02 |
| Foreman | QCIF 10Hz | 0,24 | 4,49 |
| Tempete | QCIF 10Hz | 0,25 | 4,86 |
| Silent | QCIF 15Hz | 0,23 | 4,22 |
| Paris | CIF 15Hz | 0,24 | 4,24 |
| Mobile | CIF 30Hz | 0,20 | 4,12 |
| Tempete | CIF 30Hz | 0,20 | 4,49 |
| Bus | CIF 30Hz | 0,26 | 5,02 |
| Average |  | 0,21 | 4,02 |

Table 11: Results for interlaced sequences in half horizontal resolution coded as field pictures (QP=28,24,20,16 / CABAC enabled / P-frames only / 16x16 motion search / ¼-pel MC)

|  | 1 reference picture |  | 5 reference pictures |  |
| --- | --- | --- | --- | --- |
|  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Src15_ref_525 (mobile) | 0,09 | 1,85 | 0,27 | 5,49 |
| Src19_ref_525 (football) | 0,14 | 3,36 | 0,16 | 3,85 |
| Src5_ref_625 (canoe) | 0,16 | 3,50 | 0,16 | 3,59 |
| Src6_ref_625 (formula 1) | 0,14 | 3,68 | 0,22 | 5,70 |
| Src9_ref_625 (rugby) | 0,17 | 3,78 | 0,19 | 4,50 |
| Average | 0,14 | 3,23 | 0,20 | 4,63 |

Table 12: Results for interlaced sequences in half horizontal resolution coded as field pictures (QP=28,24,20,16 / CABAC enabled / 2 B-frames inserted / 16x16 motion search / ¼-pel MC)

|  | 1 reference picture |  | 5 reference pictures |  |
| --- | --- | --- | --- | --- |
|  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Src15_ref_525 (mobile) | 0,11 | 2,86 | 0,29 | 6,87 |
| Src19_ref_525 (football) | 0,15 | 3,84 | 0,16 | 4,01 |
| Src5_ref_625 (canoe) | 0,14 | 3,61 | 0,17 | 4,14 |
| Src6_ref_625 (formula 1) | 0,13 | 3,83 | 0,17 | 5,32 |
| Src9_ref_625 (rugby) | 0,17 | 4,05 | 0,19 | 4,50 |
| Average | 0,14 | 3,64 | 0,20 | 4,97 |

# References

[1]	G. Bjontegaard, T. Wiegand, “H.26L Test Model Long Term Number 8 (TML-8) draft 0”, June 2001.

[2]	G. Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency

Experiments on Low-Resolution Progressive-Scan Source Material”, Doc. VECG-N81, Sep. 2001.

[3]	G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, Doc. VCEG-M33, Mar. 2001.

[4]	P. Borgwardt, „Core Experiment on Interlaced Video Coding“, Doc. VCEG-N85, Oct. 2001.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Heinrich-Hertz-Institute |  |  |
| Mailing address | Einsteinufer 37 10587 Berlin, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Thomas Wiegand |  |  |
| Telephone | +49 - (0)30 - 31002 617 |  |  |
| Fax | +49 - (0)172 - 322 7972 |  |  |
| Email | wiegand@hhi.de |  |  |
| Place and date of submission | Pattaya, 4th December |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Tree-structured macroblock partition |  |
| Contribution number |  | VCEG-O17r3 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: