| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O25 Filename: VCEG-N25.doc Generated: 25 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Antti Hallapuro, Marta Karczewicz Nokia Research Center, Tampere, Finland | Tel: Fax: Email: | +358718035390 +358718035322 antti.hallapuro@nokia.com |
| Title: | Low complexity (I)DCT |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

It has been noted that the DCT and quantization currently employed in TML require 32-bit multiplications. It would be better if only 16-bit multiplications were required since most of the DSPs can't perform 32-bit multiplication with single instruction. In this document low complexity and low precision DCT implementation is described.

# Simplified DCT

4x4 forward DCT transform can be calculated as follows

X=ATXA=abacac−a−ba−c−aba−ba−cy00y01y02y03x10x11x12y13y20y21y22y23y30y31y32y33aaaabc−c−ba−a−aac−bb−c

and inverse transform (IDCT) as

Y=ATXAT

where A is a transform matrix and

a=1/2b=1/2⋅cosπ/8c=1/2⋅cos3π/8.

Factorization of the transform matrix A leads to the following representation of forward transform:

X=CTBYBC=111d1d−1−11−d−11111−da0000b0000a0000by00y01y02y03x10x11x12y13y20y21y22y23y30y31y32y33a0000b0000a0000b11111d−d−11−1−11d−11−d

where d=c/b. Since B is a diagonal matrix, one can write above equation as:

Y=CXCT⊗Einv=111d1d−1−11−d−111−11−dy00y01y02y03x10x11x12y13y20y21y22y23y30y31y32y33⊗a2aba2ababb2abb2a2aba2ababb2abb211111d−d−11−1−11d−11−d

where  denotes element-by-element multiplication instead of normal matrix multiplication. Since multiplying with elements of matrix Einv consists of simply scaling the output with constants, this scaling can be combined with the de-quantization.

The coefficient d in the matrix C is a non-rational number and therefore cannot be implemented efficiently using binary arithmetic. For this reason d is converted to fixed-point format. Exact value of d is 0.41421356… and three of the possible fixed-point approximations are 1/2, 3/8 and 7/16. The least complex value d=1/2 is used in the further description. In order to ensure existence of the inverse transform for the simplified DCT condition

ATA=I

must hold. By solving the above equation we can calculate value of b that must be used to compensate for approximation of d:

b=0.51+d2=0.4

# Forward Transform Precision Considerations

The forward transform using substitution d=1/2 can be written as

Y=1111112−12−11−1−1112−11−12x00x01x02x03x10x11x12x13x20x21x22x23x30x31x32x3311112112−1−11−12−11111−12⊗a2aba2ababb2abb2a2aba2ababb2abb2

Multiplication with coefficient 1/2 can be implemented using right shifts. Usage of the right shift operation can lead to small degradation at highest bit rates. Performance can be improved by scaling every other row of the first coefficient matrix and every other column of the second coefficient matrix with 2. Forward transform is then

Y=CXCT⊗Eforw=111121−1−21−1−111−22−1x00x01x02x03x10x11x12x13x20x21x22x23x30x31x32x33121111−1−21−1−12121−1⊗a2ab2a2ab2ab2b24ab2b24a2ab2a2ab2ab2b24ab2b24

# Quantization

Quantization coefficients used in TML-8 corresponding to different QP values are generated using formula:

A(QP)=2.5⋅216QP

Notice that A(QP) can be also written as follows:

A(QP)=2k⋅B(QP%6) where B(QP%6)=2.5⋅2(QP%6)/6 and k=QP/6.

Using these quantization coefficients, reconstructed values XR given the quantized transform coefficients YQ are equal to:

YR(i,j)=round(2m⋅B(QP%6)⋅YQ(i,j)⋅Einv(i,j))

XR=CTYRC//2m−k

where m is the number of fractional bits used for fixed-point de-quantization and by ’//’ we denote division with rounding. Notice that YR can be also calculated as

YR(i,j)=R(QP%6)⋅YQ(i,j)

where de-quatization matrix R(QP%6) is given by:

R(QP%6,i,j)=round2m⋅Einv.(i,j)⋅B(QP%6).

Similarly the quantized forward transform coefficients are calculated as

YQ(i,j)=(CXCT)Q(QP%6)+f/2n+k

where fixed-point quantization matrix Q(QP) is equal to

Q(QP%6,i,j)=round2n⋅2m⋅Eforw.(i,j)⋅Einv.(i,j)/R(QP%6,i,j).

By ’/’ we denote division with truncation. Only values of Q and R for QP%6=0,1,…,5 have to be pre-calculated and stored. Moreover elements of R or Q for a given QP%6 have only 3 different values.

# Transform Specification

Step by step description of the proposed transform and quantization are given in this chapter. In our describtion // denotes division with rounding to the nearest integer:

a//2b = sign(a)*((abs(a)+2b-1)>>b)

## 4x4 luma and chroma transform and quantization

Forward transform

Forward transform for input values X:

Y=111121−1−21−1−111−22−1x00x01x02x03x10x11x12x13x20x21x22x23x30x31x32x33121111−1−21−1−12121−1

For 11-bit and 13-bit residual post-scaling is performed. For 11-bit residual it is done as follows:

Y(i,j)=Y(i,j)2S11(i,j),		i, j = 0,…,3

where S11=0000010100000101

For 13-bit residual following equation is used:

Y(i,j)=signY(i,j)∗(abs(Y(i,j))+l(QP,S13(i,j)))>>S13(i,j),		i, j = 0,…,3

where S13=1212232312122323

and values of k are listed in Table 1.

Table 1 – Values of l

|  | k(QP,S13(i,j)) |  |
| --- | --- | --- |
| S13(i,j) | Even QP | odd QP |
| 1 | 1 | 0 |
| 2 | 3 | 1 |
| 3 | 4 | 4 |

Quantization

Quantization is performed according to the following equation

YQ(i,j)=(Y(i,j)⋅Q(QP%6,i,j)+f2Sp(i,j))217+QP6−Sp(i,j),		i,j = 0,…,3

where YC are transformed coefficients, Q are the quantization coefficients and f is equal to 217+QP/6/3 for intra and 217+QP/6/6 for inter frames and has the same sign as the coefficient that is being quantized. The quantization coefficients Q are given in the appendix at the end of this document. Which matrix Sp is used depends on the residual precision p. In what follows subscript p=9, 11, 13 will be denoting for which input residual precision given constant or matrix should be used. Note that for 9-bit residual Sp(i,j) is always 0.

Dequantization

Dequantization of coefficients is based on the equation

Y(i,j)=YQ(i,j)⋅R(QP%6,i,j),		i,j = 0,…,3

Where YQ are the quantized coefficients and R are the dequantization coefficients. Values of R are given in the appendix.

Inverse transform

4x4 inverse transform implements following equation

X=11112112−1−11−12−11111−12y00y01y02y03y10y11y12y13y20y21y22y23y30y31y32y331111112−12−11−1−1112−11−12

The implementation of inverse transform uses arithmetic bit-shifts. 4-point inverse transform is defined as follows:

M0 = Y0 + Y2;

M1 = Y0 - Y2;

M2 = (Y1>>1) - Y3;

M3 = Y1 + (Y3>>1);

X0 = M0 + M3;

X3 = M0 - M3;

X1 = M1 + M2;

X2 = M1 - M2;

Finally, inverse transformed coefficients are normalized with

X(i,j)=(X(i,j)+25−QP6)>>(6−QP6)

## 4x4 luma DC transform and quantization

Forward fransform

Luma DC transform is added on top of intra 16x16 transform. Input matrix XD is formed by picking out DC coefficients from the 16 transformed 4x4 blocks. DC coefficients are then transformed using Hadamard transform:

YD=111111−1−11−1−111−11−1xD00xD01xD02xD03xD10xD11xD12xD13xD20xD21xD22xD23xD30xD31xD32xD33111111−1−11−1−11111−1//kp

k9=2,		k11=8,	k13=16

Quantization

Quantization is performed using the following equation

YQD(i,j)=(YD(i,j)⋅Q(QP%6,0,0)+f2sp)217+QP6−sp,		i,j = 0,…,3

s9=−1,		s11=1,	s13=3

Inverse transform

Hadamard inverse transform is used. If YQD are the quantized luma DC coefficients then the inverse transform is

XQD=111111−1−11−1−11111−1yQD00yQD01yQD02yQD03yQD10yQD11yQD12yQD13yQD20yQD21yQD22yQD23yQD30yQD31yQD32yQD33111111−1−11−1−111−11−1

Dequantization

Dequantization of coefficients is based on the equation

XD(i,j)=(XQD(i,j)⋅R(QP%6,0,0))4,		i,j = 0,…,3

Note that the order of inverse transform and de-quantization of DC coefficients is changed. It does not influence the results as long as scaling is done after both of them, i.e. Y=(AT(X*k)A)/4 is equal to Y=((ATXA)*k)/4. However, it is better to do inverse transform first for lowest possible dynamic precision during inverse transform computations.

# 2x2 chroma DC transform and quantization

Forward transform

Chroma DC transform is added on top of chroma transform. Input matrix is formed by picking out DC coefficients from the 4 transformed 4x4 blocks. If these coefficients are denoted as XD then transform is computed as

YD=111−1xD00xD01xD10xD11111−1//kp

k9=1,		k11=2,	k13=4

Quantization

Quantization is performed using the following equation

YQD(i,j)=(YD(i,j)⋅Q(QP%6,0,0)+f2sp)//217+QP6−sp,		i,j = 0,…,1

s9=−1,		s11=0,	s13=2

Inverse transform

Chrominance DC inverse transform is computed same way as in TML-8 except that scaling with ½ is moved to dequantization:

XQD=111−1YQD00YQD01YQD10YQD11111−1

Dequantization

Dequantization of chroma DC coefficients is based on the equation

XD(i,j)=(XQD(i,j)⋅R(QP%6,0,0))2,		i,j = 0,…,1

Also for chroma the order of inverse transform and dequantization of DC coefficients is changed.

# Complexity

Table 2 summarized the precision requirements for each stage. There are three numbers in each cell corresponding to 9, 11 and 13-bit input residuals.

Table 2 – Precision

| Luma and chroma transform and quantization |  |  |
| --- | --- | --- |
|  | Input bits | Output bits |
| Forward transform | 9, 11, 13 | 15, 16, 16 |
| Quantization | 15, 16, 16 | 10, 12, 14 |
| Dequantization | 10, 12, 14 | 15, 17, 19 |
| Inverse transform | 15, 17, 19 | 9, 11, 13 |
| Luma DC transform and quantization |  |  |
|  | Input bits | Output bits |
| Transform | 13, 15, 16 | 16, 16, 16 |
| Quantization | 16, 16, 16 | 12, 14, 16 |
| Inverse transform | 12, 14, 16 | 12, 14, 16 |
| Dequantization | 12, 14, 16 | 15, 17, 19 |
| Chroma DC transform and quantization |  |  |
|  | Input bits | Output bits |
| Transform | 13, 15, 16 | 15, 16, 16 |
| Quantization | 15, 16, 16 | 11, 13, 15 |
| Inverse transform | 11, 13, 15 | 11, 13, 15 |
| Dequantization | 11, 13, 15 | 15, 17, 19 |

Since the propose transform can be implemented using only additions and shifts the multiplications are only used during quantization and de-quantization. It can be seen from the table that all the quantization and de-quantization can be computed with 16-bit multiplication since input values for these operations never exceed 16 bits for 9, 11 and 13 bit residual data. The proposed method can be implemented fully with 16-bit arithmetic when 9-bit residual is used.

Table 3 and Table 4 summarize the number of operations required by the encoder and the decoder, respectively. The number of counted are computed for the fastest implementation of the transform and inverse transform that are possible using formulas and coefficients presented in this document.

Table 3 – Encoder operations

| 4x4 transform for 24 4x4 luminace and chrominance blocks |  |  |
| --- | --- | --- |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 64 |
|  | << | 16 |
|  | // | 0,4 or 16 |
| 4x4 transform for one 4x4 luminace DC block |  |  |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 64 |
|  | // | 16 |
| 2x2 transform for two 2x2 Chrominace DC blocks |  |  |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 8 |
|  | // | 0 or 4 |
| quantization for coefficient |  |  |
| Process | Operation | Number per coefficient |
| Quantization | * | 1 |
|  | // | 1 |

Table 4 – Decoder operations

| 4x4 inverse transform and dequantization for 24 4x4 luminace and chrominance blocks |  |  |
| --- | --- | --- |
| Process | Operation | Number per block |
| Dequantization | * | 16 |
| 4x4 inverse transform | + | 64 |
|  | >> | 16 |
| Normalization | + | 16 |
|  | >> |  |
| 4x4 inverse transform and dequantization for one 4x4 luminace DC block |  |  |
| Process | Operation | Number per block |
| 4x4 inverse transform | + | 64 |
| Dequantization | * | 16 |
|  | // | 16 |
| 2x2 inverse transform and dequantization for two 2x2 Chrominace DC blocks |  |  |
| Process | Operation | Number per block |
| 2x2 inverse transform | + | 8 |
| Dequantization | * | 4 |
|  | // | 4 |

# Results

The attached document contains results when the encoder is implemented to ensure only 16 bit multiplications for 9, 11 and 13 bit residual data. Decoder structure is independent of residual precision.

Note that in the test only 9 bit residual data was used. The reasons were lack of the testing material and the current TML software, which is not capable of encoding such material. Such test are however already good indication how the transform would perform for 11 and 13 bit residual data. There is no performance degradation for encoder implemented for 9 and 11 bit residual data and minimal for the one implemented for 13 bit residual data.

Notice as well that if for 11-bit and 13-bit residual you want to keep the maximum precision in the encoder you can use in both cases encoder implementation as specified above for 9-bit residual. In this case encoder implementation will require 32 bit multiplications however decoder can still be implemented using only 16 bit multiplications.

# Summary

Coding efficiency of the DCT currently used in TML and the proposed implementation are almost identical. Currently inverse transform, quantization and de-quantization require 32-bit multiplications. Proposed (I)DCT does not require any multiplication and proposed (de)quantization require only 16-bit multiplications even with 13-bit residual. Inverse transform with 9-bit residual can be implemented fully with 16-bit arithmetic.

# Appendix

Quantization coefficients

Q[QP%6][i][j]=quantMat[QP%6][0] for (i,j)={(0,0),(0,2),(2,0),(2,2)},

Q[QP%6][i][j]=quantMat[QP%6][1] for (i,j)={(1,1),(1,3),(3,1),(3,3)},

Q[QP%6][i][j]=quantMat[QP%6][2] otherwise.

R[QP%6][i][j]=dequantMat[QP%6][0] for (i,j)={(0,0),(0,2),(2,0),(2,2)},

R[QP%6][i][j]=dequantMat[QP%6][1] for (i,j)={(1,1),(1,3),(3,1),(3,3)},

R[QP%6][i][j]=dequantMat[QP%6][2] otherwise.

quantMat[6][3] = {

{13107, 5243, 8224}, {11651, 4660, 7358}, {10486, 4143, 6554}, {9198, 3687, 5825}, {8322, 3290, 5243}, {7384, 2943, 4660}};

dequantMat[6][3] = {

{40, 64, 51}, {45, 72, 57}, {50, 81, 64}, {57, 91, 72}, {63, 102, 80}, {71, 114, 90}};

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 <fax number here>

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX-75039 |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 214 763 1314 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | 11/25/01 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Low complexity (I)DCT |  |
| Contribution number |  | VCEG-O25 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]