| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O26 Filename: VCEG-O26.doc Generated: 26 November 2001 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Satoshi Kondo, Shinya Kadono and Martin Schlockermann Matsushita Electric Industrial Co., Ltd. 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Fax: Email: | +81-6-6900-9689 +81-6-6900-9674 kondo@drl.mei.co.jp |
| Title: | New Prediction Method to Improve B-picture Coding Efficiency |  |  |
| Purpose: | Proposal |  |  |

_____________________________

1. Introduction

Bi-directional prediction is widely adopted in various video coding standards such as MPEG-4 video and H.26L. In general, higher coding efficiency gains by using bi-directional prediction. In this document, a new prediction method for B-picture to improve coding efficiency is proposed. In the new prediction method, blocks in B-picture can be encoded by referring previous B-pictures in addition to I- or P-pictures as forward reference picture. The details of the proposed method are presented, followed by the simulation results. According to the simulation results, the proposed method improves coding efficiency up to 6 %.

2. Proposed Method

2. 1. Basic Idea

In the proposed method, a block in B-picture can be encoded by referring not only previous I- or P-pictures but also the most recent previous B-picture as forward reference picture. Backward reference picture is the most recent subsequent I- or P-picture as well as the conventional methods. Note that P-pictures refer only previous P-pictures, therefore encoding procedure for P-pictures is same as the conventional method.

Figure 1 shows the typical picture structure including B-pictures and prediction relationship between each picture. In Figure 1, arrows indicate the relationship of prediction, and the prediction relationships for picture B6, B7, and P8 are shown. The solid line arrows show the conventional prediction mode, and the dotted line arrows show additional new prediction mode. The end point of the arrow indicates the coding target picture, and the start point of the arrow indicates the picture referred by the coding picture. For example, picture B7 is encoded by using picture P5 and B6 as forward reference picture and using picture P8 as backward reference picture.

Figure 1: New prediction mode for B-picture.

2. 2. Reference Frame Number

If the multiple reference method for forward reference is used, reference frame number is indicated in macroblock header. In the proposed method, the reference frame number is assigned from the most recent reference picture in display order, and it doesn’t depend on its picture type. Figure 2 shows an example of the numbering method of reference frame number. In Figure 2, coding target picture is picture B7, and picture B7 refers picture P2, P5, and B6 as forward reference picture. Because the assigned reference frame number increases from the most resent reference picture, the reference frame number of picture B6 is ‘0’, the one of picture P5 is ‘1’, and the one of picture P2 is ‘2’.

Figure 2: Numbering method of reference frame number in proposed method.

2. 3. Direct Mode

Motion vector in direct mode is derived by scaling the motion vector of the block at same position as the current block in the subsequent reference picture. Forward reference picture in direct mode is the most recent previous picture, and it doesn’t depend on the type of the referenced picture, and doesn’t depend on the type and the position of the picture referenced by the corresponding block in the subsequent reference picture.

Figure 3 shows an example of motion compensation method in direct mode in the proposed method. In Figure 3, block ‘a’ shows the target block to be coded, block ‘b’ shows the corresponding block of the block ‘a’ in the subsequent reference P-picture, and block ‘c’ shows the blocks referenced by block ‘a’ in direct mode.

Figure 3: Motion compensation method in direct mode in proposed method.

- The forward motion vector MVF and backward motion vector MVB for current block ‘a’ for direct mode are calculated as follows.

MVF=MV×TRFTRD							(Eq. 1)

MVB=(TRB−TRD)×MVTRD						(Eq. 2)

- Where the vector component MV represents the motion vector of block ‘b’. TRD is the temporal distance between the subsequent reference picture and the picture referenced by the subsequent reference picture, TRB is the temporal distance between the current picture and the picture referenced by the subsequent reference picture, and TRF is the temporal distance between the current picture and the most recent previous picture.

2. 4. Other Items

- Mode selection method is same as the one using multiple reference mode in H.26L TML8 [1].

- Definition of skipped macroblock in B-picture is same as H.26L TML8.4 software, namely, ‘not coded in direct mode.’

- Flag that shows B-pictures are coded by using other B-pictures as reference picture may be needed as sequence header information.

- Total amount of reference frame memory is same as the conventional method by reducing the number of previous P-pictures to be referred. For example, if 5 P-pictures are used as forward reference picture in the conventional method, 4 P-pictures are used in P-picture encoding and 4 P-pictures and 1 B-picture are used as forward reference picture in the proposed method.

3. Simulation

3. 1. Conditions of Simulation

In order to confirm the effectiveness of the proposed method, we adopted the proposed method to the H.26L TML8.4 software. And we have compared the coding efficiencies of the proposed method and the H.26L TML8.4. The conditions of simulation are as follows:

- Software version: H.26L TML8.4.

- Motion estimation and mode selection: High complexity mode.

- QP: 16, 20, 24, 28, and fixed for all over sequence.

- Sequences: Foreman, Silent, Container, News (QCIF), Mobile and calendar, Tempete, Paris (CIF).

- Frame rate: 10 fps, 15 fps, 30 fps. And it depends on the sequence.

- Number of B-pictures: 2 (M=3, IBBPBBPBBPBB….)

- Number of reference frames: 5 frames for the conventional method. For the proposed method, 4 P-pictures are used as reference picture in the P-picture coding, and 4 P-pictures and 1 B-picture are used as reference picture in B-picture coding. Therefore, Amount of frame memory is same in the TML8.4 and the proposed method.

3. 2. Simulation Results

Table 1 shows quantitative values on the R-D performance difference obtained by avsnr4 [2]. And R-D curves for several test sequences are shown from Figure 4 to Figure 11. These results show that the proposed method outperforms over whole range of the bit rates for various test sequences. Note that the proposed method shows better performance for higher frame rate in the same test sequence.

Table 1: Quantitative difference of R-D performance.

| Sequence | Resolution / Frame rate / No. of frames | Total |  | B-picture |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average bitrate (%) | Average PSNR (dB) | Average bitrate (%) | Average PSNR (dB) |
| Foreman | QCIF / 10 / 100 | -1.60 | +0.09 | -4.35 | +0.22 |
| Foreman | QCIF / 15 / 150 | -1.76 | +0.09 | -6.02 | +0.30 |
| Silent | QCIF / 15 / 150 | -2.89 | +0.14 | -7.65 | +0.35 |
| Container | QCIF / 10 / 100 | +1.90 | -0.10 | +6.12 | -0.18 |
| Container | QCIF / 15 / 150 | -1.53 | +0.11 | -7.10 | +1.41 |
| News | QCIF / 10 / 100 | -1.87 | +0.11 | -2.93 | +0.27 |
| News | QCIF / 15 / 150 | -2.92 | +0.17 | -7.57 | +0.43 |
| Mobile and calendar | CIF / 30 / 300 | -2.93 | +0.13 | -7.95 | +0.29 |
| Tempete | CIF / 30 / 260 | -2.57 | +0.10 | -6.07 | +0.20 |
| Paris | CIF / 15 / 150 | -3.11 | +0.16 | -8.36 | +0.43 |
| Paris | CIF / 30 / 300 | -6.03 | +0.32 | -15.06 | +0.82 |

4. Conclusion and Remarks

A new prediction method for B-picture to improve B-picture coding efficiency was proposed. By simulation results, it has shown that the proposed method improves coding efficiency up to 6 %. And amount of frame memory for the proposed method is same as TML8.4 codec in multiple reference mode. Consequently, it is obtained higher compression efficiency by using the proposed method.

# References

- G.Bjontegaad, “H.26L Test Model Long Term Number 8 (TML-8) draft 0”, VCEG-N10, June 2001.

- G.Bjontegaad, “Calculation of average PSNR differences between RD-curves,” VCEG-M33, Apr. 2001.

Figure 4: R-D curves for total sequence of ‘Foreman’.

Figure 5: R-D curves for B-pictures of ‘Foreman’.

Figure 6: R-D curves for entire sequence of ‘Slient’.

Figure 7: R-D curves for B-pictures of ‘Slient’.

Figure 8: R-D curves for entire sequence of ‘Mobile and calendar’.

Figure 9: R-D curves for B-pictures of ‘Mobile and calendar’.

Figure 10: R-D curves for entire sequence of ‘Paris’.

Figure 11: R-D curves for B-pictures of ‘Paris’.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma-City, Osaka, 571-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | Pattaya, Thailand, 3 December 2001 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ## JVT |  |
| Title |  | ## New Prediction Method to Improve B-picture Coding Efficiency |  |
| Contribution number |  | ## VCEG-O26r1 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: