| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O30 Filename: VCEG-O30.doc Generated: 26 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Mathias Wien and Jens-Rainer Ohm Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52074 Aachen, Germany | Tel: Fax: Email: | +49-241-80-276-{81,71} +49-241-80-22196 wien@ient.rwth-aachen.de ohm@ient.rwth-aachen.de |
| Title: | Simplified Adaptive Block Transforms |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

At the last meeting, Adaptive Block Transforms (ABT) were presented for adoption into H.26L with a full integration into the TML-8.0 software [VCEG-N49]. The rate/distortion performance of the coding scheme was shown to be equal or better than the performance of the TML coder. The subjective quality was rated better at high rates and not better at lower rates. This negative vote was primarily due to the fact that besides the preservation of more detail increased ringing artifacts were induced through the larger transforms.

Here, we present a simplified ABT concept that employs only the TML 4x4 transform and the proposed 8x8 transform. In terms of RD gain SABT performs comparable or slightly better than full ABT on the testing set. The ringing artifacts are reduced since the maximum transform block size is only half of the maximum size in  the previous proposal.

# Simplified Adaptive Block Transforms

With SABT, the block size of the transform is connected to the block size used for motion compensation up to a block size of 8x8 pixels. The larger blocks (like 16x8, 8x16, and 16x16) are encoded using the 8x8 block transform. The block tilings and the transform blocks are given in Figure 1.

Figure 1: Macroblock modes and transform block sizes as used with SABT. The transform block size is marked blue if applicable. For block sizes smaller than 8x8, the transform block size is equal to the MC block size.

The ABT coding techniques can be applied directly for SABT. The coded block pattern CBPY is coded like in the TML. The Level values of mode 5-7 are coded using the TML Level coding routine.

For modes 1-4, CBP = 1 directly signals the existence of at least one non-zero Level in the encoded 8x8 blocks. Therefore,  the first coefficient Level in 8x8 subblock is coded without the first (EOB-) Bin of the TML Level binarization.

The ABT Run binarization is reduced to the maximum block size of 8x8. The resulting binarization is given in Table 1 below. Currently, the ABT contexts are used for context modeling of the coefficients.

| RUN | Binarization |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mode |  |  | 5,6 |  |  |  |  | 4 |
| 0 | 1 |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 |  |  |  |  |  |  |  |
| 2 | 0 | 0 | 1 |  |  |  |  |  |  |
| 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 5 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| … | 0 | 0 | 0 | ... |  |  |  |  | . |
| Bin_no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |

Table 1: Run Binarization from ABT Document VCEG-N49, reduced to maximum 8x8 transform blocks.

For motion estimation with ABT and SABT, the block size of the Hadamard transform used for SATD calculation is connected to the transform block size applied for encoding. I.e., here the maximum block size for the Hadamard transform is 8x8.

# Complexity Analysis

## SATD calculation

The SATD is used for motion estimation on subpel positions. The Hadamard Transform of a length-N vector can be implemented with  Nlog2N additions (including subtractions). Using this, the number of additions needed for SATD calculation can be given for each block size:

| Block Size | Adds / Block | Adds / MB |
| --- | --- | --- |
| 4x4 | 64 | 1024 |
| 4x8,8x4 | 160 | 1280 |
| 8x8 | 384 | 1536 |

## Transform Complexity

The integer 8x8 transform proposed for ABT in [VCEG-L15] and the TML 4x4 transform can be efficiently implemented by exploiting their symmetry properties. The efficiency of these implementations is similar to a fast implementation of the DCT. The number of additions and multiplications needed for transforming a vector of length 4 and 8 using the appropriate transform is given below:

| Transform | Adds | Multiplications |
| --- | --- | --- |
| T4 | 8 | 6 |
| T8 | 28 | 22 |

For transforming a MB of 16x16 pixel according to the MB modes the number of adds and multiplications is given in the following table:

| Block Size | Adds / MB | Mult. / MB |
| --- | --- | --- |
| 4x4 | 1024 | 768 |
| 4x8,8x4 | 1408 | 1088 |
| 8x8 | 1792 | 1408 |

At the Santa Barbara Meeting strong interest was expressed in a full 16bit implementation of the coding scheme. We are currently working on a 16bit implementation of the 8x8 transform and quantization process.

# Simulation Results

The proposed technique is integrated into the TML-8.0 software. Experiments were conducted according to the guideline of the common conditions document [VCEG-N81]. Three coder configurations were tested including the original TML-8.0 coder (TML), the ABT coder as presented at the last meeting (ABT), and the proposed SABT coder (SABT). The same QP was used for I-, P-, and B-Frames. The basic encoder configuration for all tests is given below, all MB modes are enabled:

| MV resolution | 1/8 pel |
| --- | --- |
| UseHadamard | ON |
| Reference Frames | 5 |
| Search Range | 32 |
| Symbol Mode | CABAC |
| Restrict Search Range | 2 |
| RD optimization | ON |
| QP | 16,20,24,28 |

The resulting RD plots of all three coders are given in the Appendix of this document. The common test sequences were used for the experiments.

Table 2 gives the difference in terms of rate and PSNR as calculated using the Bjontegaard measurement method [VCEG-M33]. Both, ABT and SABT are compared to the performance of the original TML coder.

| Test Sequences | SABT average % bit savings (avsnr) | SABT average dB PSNR gain (avsnr) | ABT average % bit savings (avsnr) | ABT average dB PSNR gain (avsnr) |
| --- | --- | --- | --- | --- |
| Container QCIF | 2.78 | 0.152 | 6.11 | 0.340 |
| Foreman QCIF | 6.60 | 0.374 | 4.29 | 0.240 |
| News QCIF | 0.26 | 0.018 | 0.22 | 0.017 |
| Silent QCIF | 3.42 | 0.169 | 3.06 | 0.153 |
| Paris CIF | -0.18 | -0.008 | 1.43 | 0.077 |
| Mobile CIF | 2.15 | 0.107 | -0.43 | -0.021 |
| Tempete CIF | 3.82 | 0.171 | 2.80 | 0.123 |
| average | 2.69 | 0.140 | 1.70 | 0.132 |

Table 2: Objective comparison for SABT and ABT with the TML coder.

## Discussion

The curves and the results in Table 2 reveal a comparable performance of both ABT and SABT in terms of objective RD measurement. The results show that the scheme using variable block size transforms works sufficiently well even with a reduced number of block sizes. Ringing artifacts in the reconstructed signal are reduced since the maximum transform size is restricted to 8x8.
For some sequences (e.g. Container QCIF), the performance of SABT is a bit lower than the performance of ABT. Here, the distribution of MB modes is comparable for both coders but the SABT coder has to spend more bits for update coding.
Some sequences show an improved performance of SABT over ABT. E.g. on Foreman QCIF the performances increases by 6.6 % rate savings or 0.37 dB of PSNR relative to the TML coder. The statistics for Foreman show the frequency of Mode 1 increased by about 9% for SABT at QP = 20.

# Conclusion

A simplified ABT concept is proposed using only transforms of size 4x4 and 8x8. The results reveal a similar performance of the simplified scheme in comparison to the full ABT scheme. Ringing is reduced by elimination of the 16x16 transform. In some experiments even a slight gain could be observed for the SABT scheme. The elimination of the 16x16 transform reduces the complexity of the ABT scheme for motion estimation as well as for the transform.

We propose to consider the application of the ABT/SABT scheme for medium to high complexity profiles. In [VCEG-O-31] intra coding using variable block size transforms is proposed. It is shown that the intra performance is improved by the application of variable block size transforms. As reported in [VCEG-O32] the application of ABT also shows promising results for interlaced coding in H.26L.

# References

[VCEG-N49]	M. Wien and A. Dahlhoff, 'New ABT Results using CABAC', ITU-T Q6/16 VCEG, 
Doc. VCEG-N49r1, Santa Barbara, CA, Sept. 2001.

[VCEG-M62]	M. Wien and A. Dahlhoff, 'Adaptive Block Transforms', ITU-T Q6/16 VCEG, 
Doc. VCEG-M62r1, Austin, TX, April 2001.

[VCEG-N81]	G. Bjontegaard and G. Sullivan, ' Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material', ITU-T Q6/16 VCEG, Doc. VCEG-N81, Santa Barbara, CA, Sept. 2001.

[VCEG-M33]	Gisle Bjontegaard, 'Calculation of average PSNR differences between RD 
curves,' document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

### IPR Statement

The patent holder has chosen to subscribe to subclause 2.2 for ITU-T patent policy under the condition of reciprocity.

# Appendix – Rate/Distortion Plots

container qcif TML 80 10Hz IP

QP     Rate	PSNR

28     4885	27.83

24     8187	30.82

20    13210	33.59

16    22616	36.17

container qcif ABT 10Hz IP

QP     Rate	PSNR

28     5028	28.41

24     8300	31.32

20    13380	33.96

16    23804	36.52

container qcif SABT 10Hz IP

QP     Rate	PSNR

28     5181	28.48

24     8727	31.41

20    14574	34.17

16    25778	36.73

foreman qcif TML 80 10Hz IP

QP     Rate	PSNR

28    17309	28.27

24    28269	30.94

20    45204	33.46

16    70992	36.09

foreman qcif ABT 10Hz IP

QP     Rate	PSNR

28    18306	28.75

24    28748	31.30

20    45750	33.78

16    71235	36.29

foreman qcif SABT 10Hz IP

QP     Rate	PSNR

28    18626	28.99

24    29329	31.53

20    47579	34.14

16    75434	36.75

news qcif TML 80 10Hz IP

QP     Rate	PSNR

28    10674	28.08

24    18194	30.95

20    29448	33.94

16    45788	36.83

news qcif ABT 10Hz IP

QP     Rate	PSNR

28    11018	28.24

24    18785	31.20

20    30398	34.11

16    46933	37.01

news qcif SABT 10Hz IP

QP     Rate	PSNR

28    11279	28.40

24    19124	31.28

20    31370	34.33

16    48719	37.26

silent qcif TML 80 15Hz IP

QP     Rate	PSNR

28    11778	28.07

24    21658	30.60

20    36916	33.21

16    60346	35.90

silent qcif ABT 15Hz IP

QP     Rate	PSNR

28    12656	28.44

24    22260	30.91

20    37901	33.49

16    61115	36.10

silent qcif SABT 15Hz IP

QP     Rate	PSNR

28    12672	28.48

24    22400	30.95

20    38542	33.60

16    63018	36.26

paris cif TML 80 15Hz IP

QP     Rate	PSNR

28    59403	26.91

24   109649	29.86

20   190247	32.67

16   315739	35.60

paris cif ABT 15Hz IP

QP     Rate	PSNR

28    62254	27.11

24   113157	30.08

20   196494	32.95

16   320782	35.86

paris cif SABT 15Hz IP

QP     Rate	PSNR

28    65259	27.23

24   118738	30.23

20   205854	33.13

16   332890	36.04

mobile cif TML 80 30Hz IP

QP     Rate	PSNR

28   200598	24.91

24   358847	27.97

20   680096	30.96

16  1276983	34.16

mobile cif ABT 30Hz IP

QP     Rate	PSNR

28   227246	25.41

24   400757	28.48

20   747514	31.44

16  1340778	34.42

mobile cif SABT 30Hz IP

QP     Rate	PSNR

28   241146	26.07

24   431209	28.95

20   807203	31.89

16  1431550	34.86

tempete cif TML 80 30Hz IP

QP     Rate	PSNR

28   156189	26.56

24   291683	29.36

20   564597	32.11

16  1084078	35.08

tempete cif ABT 30Hz IP

QP     Rate	PSNR

28   173757	27.25

24   320390	29.90

20   614775	32.58

16  1128426	35.31

tempete cif SABT 30Hz IP

QP     Rate	PSNR

28   179478	27.53

24   333223	30.12

20   653514	32.85

16  1200817	35.66

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Institut und Lehrstuhl für Nachrichtentechnik, RWTH Aachen |  |  |
| Mailing address | Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 53074 Aachen, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Mathias Wien |  |  |
| Telephone | +49-241-8027681 |  |  |
| Fax | +49-241-8888196 |  |  |
| Email | wien@ient.rwth-aachen.de |  |  |
| Place and date of submission | Pattaya, December 4th 2001 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Simplified Adaptive Block Transforms |  |
| Contribution number |  | VCEG-O30 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: