| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O33 Filename: VCEG-O33.doc Generated: 26 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Mathias Wien Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52074 Aachen, Germany | Tel: Fax: Email: | +49-241-80-27681 +49-241-80-22196 wien@ient.rwth-aachen.de |
| Title: | # Core Experiment Report on Global Motion Vector Coding |  |  |
| Purpose: | Information |  |  |

_____________________________

# Introduction

In this document the results of the Core Experiment on Global Motion Vector Coding (GMVC) are reported. The Core Experiment was defined in [VCEG-N77]. The concept of GMVC was proposed at the Santa Barbara meeting in document [VCEG-N16].

According to the tasks defined in the CE description, the GMVC was integrated into the TML-8.5 software. The implementation was verified by cross-checking the decodability between our implementation and the version of Shijun Sun from Sharp Labs. The results reported here are consistent with the results presented by Shijun in [VCEG-O21].

# Testing Conditions

The sequences specified in the common conditions document [VCEG-N81] were used for the experiments. Additionally, the CIF sequences Coastguard and Stefan were employed to demonstrate the performance of the scheme for sequences where global motion is present. These sequences were encoded at 15Hz and 30Hz.

Two codecs were compared in this Core Experiment:

TML	: the original TML-8.5 coder

GMVC : the proposed GMVC scheme integrated into TML-8.5

The following encoder configuration was used for all simulations. The GMV search range was the same as for local motion search.

| MV resolution | ¼ pel |
| --- | --- |
| UseHadamard | ON |
| Reference Frames | 5 |
| Search Range | 32 |
| Symbol Mode | UVLC |
| Restrict Search Range | 2 |
| RD optimization | ON |
| QP | 16,20,24,28 |

# P-frame Coding : IPPP….

The test sequences were encoded using a IPPP-configuration of the coder. Table 1 gives the difference between TML and GMVC in terms of rate and PSNR as calculated using the Bjontegaard measurement method [VCEG-M33]. The corresponding rate/distortion plots are given in Appendix A.

| Test Sequences | GMVC average % bit savings (avsnr) | GMVC average dB PSNR gain (avsnr) |
| --- | --- | --- |
| Container QCIF | -4.39 | -0.211 |
| Foreman QCIF | 0.36 | 0.019 |
| News QCIF | -1.25 | -0.071 |
| Silent QCIF | -2.10 | -0.101 |
| Paris CIF | -0.88 | -0.044 |
| Mobile CIF | 0.72 | 0.032 |
| Tempete CIF | -1.31 | -0.053 |
| Coastguard CIF 15Hz | 1.41 | 0.053 |
| Stefan CIF 15Hz | 3.75 | 0.129 |
| Coastguard CIF 30Hz | -0.74 | -0.039 |
| Stefan CIF 30Hz | -0.59 | -0.030 |
| average | -0.46 | -0.029 |

Table 1: Objective comparison for GMVC with the TML coder for IPPP-configuration.

# B-frame Coding: IBPBP…

The test sequences were encoded using a IBP-configuration of the coder. Table 2 gives the difference between TML and GMVC in terms of rate and PSNR as calculated using the Bjontegaard measurement method [VCEG-M33]. The corresponding rate/distortion plots are given in Appendix B. The 10Hz sequences were not used in this test.

| Test Sequences | GMVC average % bit savings (avsnr) | GMVC average dB PSNR gain (avsnr) |
| --- | --- | --- |
| Silent QCIF | -0.46 | -0.022 |
| Paris CIF | 0.23 | 0.012 |
| Mobile CIF | -1.03 | -0.048 |
| Tempete CIF | -1.47 | -0.057 |
| Coastguard CIF 15Hz | 0.58 | 0.020 |
| Stefan CIF 15Hz | 6.72 | 0.363 |
| Coastguard CIF 30Hz | 1.45 | 0.046 |
| Stefan CIF 30Hz | 2.55 | 0.127 |
| average | 1.07 | 0.055 |

Table 2: Objective comparison for GMVC with the TML coder for IBP-configuration.

# B-frame Coding: IBBPBBP…

The test sequences were encoded using a IBBP-configuration of the coder. Table 2 gives the difference between TML and GMVC in terms of rate and PSNR as calculated using the Bjontegaard measurement method [VCEG-M33]. The corresponding rate/distortion plots are given in Appendix C. The 10Hz sequences were not used in this test.

| Test Sequences | GMVC average % bit savings (avsnr) | GMVC average dB PSNR gain (avsnr) |
| --- | --- | --- |
| Silent QCIF | -0.32 | -0.017 |
| Paris CIF | 0.95 | 0.048 |
| Mobile CIF | -1.72 | -0.072 |
| Tempete CIF | -1.21 | -0.045 |
| Coastguard CIF 15Hz | 0.16 | 0.002 |
| Stefan CIF 15Hz | 0.57 | 0.030 |
| Coastguard CIF 30Hz | 0.84 | 0.024 |
| Stefan CIF 30Hz | 4.62 | 0.234 |
| average | 0.49 | 0.026 |

Table 3: Objective comparison for GMVC with the TML coder for IBBP-configuration.

# Summary and Discussion

The decodability between our implementation and the implementation of Shijun Sun from Sharp Labs was cross checked and verified.

The RD performance of GMVC is comparable to the performance of the original TML for all sequences without global motion. The scheme performs superior to the TML coder in the case where global motion is present. In particular, gains are high for the sequence Stefan coded at 15Hz IBP and 30Hz IBBP. The subjective quality of coded sequences with global motion is cleary improved, especially at lower bit rates.

Global motion estimation is performed once for a whole frame and has very little impact on the overall encoding time. When GMVC is switched off, e.g. if no global motion is presesent in a frame, only a single additional bit has to be encoded in the header, which has negliglible impact on the coding efficienty.

# References

[VCEG-N10]	Gisle Bjontegaard, "H.26L Test Model Long Term Number 8 (TML-8) draft0," document VCEG-N10, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[VCEG-N16]	Shijun Sun and Shawmin Lei, "Motion Vector Coding with Global Motion Parameters," document VCEG-N16, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[VCEG-N77]	Shijun Sun and Shawmin Lei, "Core Experiment Description: Motion Vector Coding with Global Motion Parameters," document VCEG-N16, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[VCEG-O21]	Shijun Sun and Shawmin Lei, "GMVC: Core Experiment Report," document VCEG-O21, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, Thailand, 4-7. December 2001.

[VCEG-N81]	Gary Sullivan and Gisle Bjontegaard, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan source Material,” document VCEG-N81, ITU- T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[VCEG-M33]	Gisle Bjontegaard, "Calculation of average PSNR differences between RD curves," document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

# Appendix A :  IPPP Rate/Distortion Plots

container qcif TML85 10Hz IP

QP     Rate	PSNR

28     4964	27.96

24     8612	30.77

20    14327	33.55

16    25513	36.10

container qcif GMVC  10Hz IP

QP     Rate	PSNR

28     5458	27.99

24     9096	30.82

20    14764	33.55

16    26182	36.12

foreman qcif TML85 10Hz IP

QP     Rate	PSNR

28    18630	28.45

24    29942	31.08

20    47222	33.61

16    75166	36.17

foreman qcif GMVC  10Hz IP

QP     Rate	PSNR

28    17477	28.21

24    29116	30.95

20    46775	33.57

16    75388	36.14

news qcif TML85 10Hz IP

QP     Rate	PSNR

28    10746	28.15

24    18446	31.05

20    30320	33.94

16    48652	36.86

news qcif GMVC  10Hz IP

QP     Rate	PSNR

28    11098	28.13

24    18881	31.10

20    30607	33.96

16    48896	36.88

silent qcif TML85 15Hz IP

QP     Rate	PSNR

28    12276	28.21

24    21930	30.75

20    37179	33.30

16    61128	35.94

silent qcif GMVC  15Hz IP

QP     Rate	PSNR

28    12845	28.21

24    22530	30.74

20    37562	33.30

16    61566	35.95

paris cif TML85 15Hz IP

QP     Rate	PSNR

28    60017	26.85

24   112632	29.79

20   200025	32.67

16   339621	35.59

paris cif GMVC  15Hz IP

QP     Rate	PSNR

28    61907	26.87

24   114593	29.82

20   201954	32.70

16   341275	35.63

mobile cif TML85 30Hz IP

QP     Rate	PSNR

28   236874	25.22

24   421096	28.03

20   836478	30.92

16  1638024	34.09

mobile cif GMVC  30Hz IP

QP     Rate	PSNR

28   215988	24.99

24   410026	27.96

20   832146	30.88

16  1637827	34.07

tempete cif TML85 30Hz IP

QP     Rate	PSNR

28   168274	26.85

24   312940	29.53

20   622350	32.17

16  1245024	35.07

tempete cif GMVC  30Hz IP

QP     Rate	PSNR

28   164682	26.70

24   313578	29.47

20   625714	32.14

16  1248842	35.06

coastguard cif TML85 15Hz IP

QP     Rate	PSNR

28    85969	27.07

24   196970	29.49

20   404090	32.03

16   757970	34.74

coastguard cif GMVC  15Hz IP

QP     Rate	PSNR

28    76325	26.98

24   188476	29.43

20   398076	31.98

16   753914	34.72

stefan cif TML85 15Hz IP

QP     Rate	PSNR

28   170221	26.64

24   293600	29.66

20   545805	32.51

16   951753	35.50

stefan cif GMVC  15Hz IP

QP     Rate	PSNR

28   164396	26.62

24   288622	29.64

20   516740	32.52

16   904309	35.52

# Appendix B :  IBP Rate/Distortion Plots

silent qcif TML85 15Hz IBP

QP     Rate	PSNR

28    10439	28.20

24    19037	30.69

20    32725	33.26

16    54868	35.91

silent qcif GMVC  15Hz IBP

QP     Rate	PSNR

28    10708	28.24

24    19204	30.70

20    32880	33.28

16    55142	35.92

paris cif TML85 15Hz IBP

QP     Rate	PSNR

28    55280	26.85

24   101221	29.76

20   182650	32.64

16   316050	35.58

paris cif GMVC  15Hz IBP

QP     Rate	PSNR

28    56156	26.85

24   101180	29.77

20   181444	32.64

16   313974	35.58

mobile cif TML85 30Hz IBP

QP     Rate	PSNR

28   179527	25.21

24   338301	27.98

20   714058	30.84

16  1464768	34.03

mobile cif GMVC  30Hz IBP

QP     Rate	PSNR

28   183692	25.05

24   337831	27.94

20   712226	30.82

16  1462647	34.01

tempete cif TML85 30Hz IBP

QP     Rate	PSNR

28   136123	26.92

24   257795	29.50

20   535830	32.08

16  1130268	34.97

tempete cif GMVC  30Hz IBP

QP     Rate	PSNR

28   139480	26.85

24   261297	29.50

20   539273	32.06

16  1133990	34.97

coastguard cif TML85 15Hz IBP

QP     Rate	PSNR

28    67289	26.94

24   162435	29.30

20   350786	31.84

16   679080	34.61

coastguard cif GMVC  15Hz IBP

QP     Rate	PSNR

28    63325	26.85

24   159637	29.28

20   349979	31.83

16   677992	34.59

stefan cif TML85 15Hz IBP

QP     Rate	PSNR

28   174849	26.40

24   293835	29.37

20   535087	32.25

16   942880	35.30

stefan cif GMVC  15Hz IBP

QP     Rate	PSNR

28   159870	26.40

24   279731	29.39

20   496821	32.27

16   881116	35.32

coastguard cif TML85 30Hz IBP

QP     Rate	PSNR

28    98198	27.05

24   228747	29.43

20   496279	31.95

16  1010763	34.69

coastguard cif GMVC  30Hz IBP

QP     Rate	PSNR

28    88894	26.96

24   220509	29.38

20   490188	31.91

16  1005650	34.67

stefan cif TML85 30Hz IBP

QP     Rate	PSNR

28   227225	26.53

24   399823	29.55

20   766129	32.40

16  1426942	35.44

stefan cif GMVC  30Hz IBP

QP     Rate	PSNR

28   222189	26.52

24   395734	29.54

20   735534	32.40

16  1376076	35.45

# Appendix C :  IBBP Rate/Distortion Plots

silent qcif TML85 15Hz IBBP

QP     Rate	PSNR

28    11141	28.19

24    20656	30.69

20    35880	33.23

16    60158	35.88

silent qcif GMVC  15Hz IBBP

QP     Rate	PSNR

28    11347	28.22

24    20886	30.69

20    35870	33.25

16    60116	35.89

paris cif TML85 15Hz IBBP

QP     Rate	PSNR

28    60958	26.83

24   110877	29.72

20   200071	32.62

16   347910	35.56

paris cif GMVC  15Hz IBBP

QP     Rate	PSNR

28    61036	26.85

24   110038	29.72

20   198326	32.64

16   344738	35.58

mobile cif TML85 30Hz IBBP

QP     Rate	PSNR

28   165372	25.12

24   324800	27.88

20   704475	30.76

16  1473281	33.96

mobile cif GMVC  30Hz IBBP

QP     Rate	PSNR

28   171778	25.00

24   325931	27.84

20   706511	30.73

16  1474778	33.94

tempete cif TML85 30Hz IBBP

QP     Rate	PSNR

28   126415	26.90

24   246780	29.43

20   528243	31.99

16  1141881	34.89

tempete cif GMVC  30Hz IBBP

QP     Rate	PSNR

28   128889	26.83

24   248374	29.41

20   530507	31.98

16  1145448	34.88

coastguard cif TML85 15Hz IBBP

QP     Rate	PSNR

28    70544	26.82

24   173575	29.21

20   385666	31.76

16   751293	34.52

coastguard cif GMVC  15Hz IBBP

QP     Rate	PSNR

28    68024	26.77

24   171914	29.17

20   383404	31.75

16   750249	34.51

stefan cif TML85 15Hz IBBP

QP     Rate	PSNR

28   193911	26.29

24   332093	29.21

20   595932	32.11

16  1043750	35.21

stefan cif GMVC  15Hz IBBP

QP     Rate	PSNR

28   193383	26.29

24   331020	29.22

20   592585	32.12

16  1037333	35.20

coastguard cif TML85 30Hz IBBP

QP     Rate	PSNR

28    93183	26.98

24   225242	29.35

20   510117	31.85

16  1067284	34.60

coastguard cif GMVC  30Hz IBBP

QP     Rate	PSNR

28    86686	26.92

24   220228	29.31

20   506785	31.83

16  1066397	34.59

stefan cif TML85 30Hz IBBP

QP     Rate	PSNR

28   248688	26.43

24   434005	29.41

20   808926	32.27

16  1500059	35.34

stefan cif GMVC  30Hz IBBP

QP     Rate	PSNR

28   239090	26.41

24   416536	29.42

20   768358	32.29

16  1439887	35.36