| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O36 Filename: VCEG-O36.doc Generated: 24 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Chul-Woo Kim McubeWorks 14 F, SK Telelcom Bldg, Sunae-Dong, Bundang-Gu, Sungnam Seoul, 463-784, Korea Sang Woo Rhie SK Telecom 99 Seorin-dong, Jongro-gu, Seoul, 110-110, Korea | Tel: Fax: Email: Tel: Fax: Email: | +82-31-601-5875 +82-31-601-5878 charlie@mcubeworks.com +82-2-2121-3463 +82-2-2121-3980 rainism@sktelecom.com |
| Title: | Verification of Adaptive Double Scan on Common Conditions |  |  |
| Purpose: | Proposal |  |  |

_____________________________

- Introduction

This contribution originated from VCEG-M69 that proposed the adaptive double scan to improve the compression efficiency of intra macroblocks. In Austin meeting (VCEG-M, April, 2001), there have been indications that the contribution did not provide enough experimental results under the common conditions. In this contribution are provided experimental results for the adaptive double scan algorithm which proposes alternative splitting of 4x4 block into two regions. Experimental results are given in VCEG-O36.xls and the results show that the overall bit saving of 0.1 ~ 1.3 % can be achieved with negligible complexity and code-size increase.

- Adaptive Double Scan

Current double scan defined in TML 8.0 is shown as in Fig. 1 (a). This double scan algorithm has been existing since the first release of TML to preserve the efficiency of 1 bit length of EOB which is a given factor in the current UVLC table. The adaptive double scan proposes one more double scan map shown in Fig. 1 (b)

0   1   2   5

0   2   3   6

1   3   4   7

4   5   6   7

0   0   1   4

1   2   3   5

2   3   4   6

5   6   7   7

(a)                        (b)

## Fig. 1 Double scan map of TML 6 and proposed algorithm

- Double scan map defined in TML 8.0 (D1)

- Proposed additional double scan map for adaptive double scan (D2)

The proposed double scan map is the transposed version of the one defined in TML 8.0. And the proposed algorithm is to use the D1 and D2 alternatively depending on the intra prediction mode. The alternating logic is defined in Table 1. By alternating the double scan map, the distribution of quantized transform coefficients are concentrated in the way that the collected run-lengths between successive coefficients can be reduced.

## Table 1. Algorithm for alternating the double scan map

| Prediction Mode | Scan Map |
| --- | --- |
| Mode 0 (DC Prediction) | D1 |
| Mode 1 (RL Prediction) | ## D2 |
| Mode 2 (Vertical Prediction) | ## D2 |
| Mode 3 (Diagonal Prediction) | D1 |
| Mode 4 (Horizontal Prediction) | D1 |
| Mode 5 (LR Prediction) | D1 |

This can be achieved without any additional calculation of parameters but with only one comparison or  table indexing operation using intra prediction mode.

- Simulation Results

TML 8.7a simulation results are shown in VCEG-O36.xls and Fig. 2. The percentage of bit saving relative to TML 8.7a is shown in the result. All the frames in the sequences are intra coded and QP set of (8, 12, 16, 20, 23) was used for the test. This QP set includes two QP values of 16 and 20, which are specified in common conditions. Akiyo, Mother and Daughter and Hall Monitor results are shown in addition to the results of sequences specified in VCEG-M75. In all the sequences, adaptive double scan shows better performance compared with the current double scan. Overall bit saving of from 0.1 to 1.3 % could be achieved through adaptive double scan. Though at sequences such as Tempete, Foreman, Container, Silent and Mobile, the bit saving is rather low (below 0.5%), results of Paris, News, Hall, Mother and Daughter and Akiyo show 0.5 ~ 1.3 % of bit saving.

- Complexity and Storage Analysis

- Additional Complexity due to Adaptive Double Scan

In the point of view of complexity both for decoder and encoder, ‘negligible’ additional complexity is required to adopt the adaptive double scan algorithm. It can be implemented using one ‘if’ and ‘branch’ operation pair or one table indexing operation for each double scanned block.

- Additional Memory and Code size

Assuming that two bytes are required to store the location in the scanning map, totally 32 bytes of memory is required to describe the proposed double scan map (D2) with several more instructions to perform branch operation.

- Summary

The simulation results reveal that the statistics of UVLC table becomes more efficient by adaptive double scan for 2-D run-length coding of quantized coefficients on double scanned blocks when the prediction mode contains strong vertical factors. And it improves the performance of TML when adaptive double scanning is adopted.

- IPR Strategy

McubeWorks and SK Telecom agreed and decided to have intellectual property related to this contribution and have chosen to subscribe to sub clause 2.2.1 of the ITU-T patent policy under the condition of reciprocity.

7.  Reference

[1] Gisle Bjontegaard, “H.26L Test Model Long Term Number 8 (TML-8) draft 0,” June. 2001.

Fig. 2. Performance of Adaptive Double Scan relative to TML 8.0

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | McubeWorks Inc. & SK Telecom |  |  |
| Mailing address | 14F, SK Telecom Bldg. 9-1 Sunae-Dong, Bundang-gu, Sungnam City, Kyungki-do 463-784 |  |  |
| Country | Korea |  |  |
| Contact person | Chul-Woo Kim |  |  |
| Telephone | +82 - (0)31 – 601 5875 |  |  |
| Fax | +82 - (0)31 – 601 5878 |  |  |
| Email | charlie@mcubeworks.com |  |  |
| Place and date of submission | Pattaya, 4th December |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Verification of Adaptive Double Scan |  |
| Contribution number |  | VCEG-O36r1 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: