| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th meeting, Pattaya, Thailand, December 4-6, 2001 | Document VCEG-O38 Filename: VCEG-O38.doc Generated: 27 Nov 2001 |
| --- | --- |

| Question: | Q6/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Anthony Joch, Faouzi Kossentini UB Video Suite 400 1788 West 5th Avenue Vancouver, BC, Canada V6J 1P2 | Tel: Fax: Email: | +1 604 737 2426 +1 604 737-1514 anthony@ubvideo.com |
| Title: | Initial Results From a Near-Real-Time H.26L Encoder |  |  |
| Purpose: | Information |  |  |

_____________________________

# Overview

We present results from an H.26L encoder implemented in C that achieves encoding of CIF resolution video at an average speed of 13 frames per second on a Pentium III 800 MHz laptop.  We compare the RD-performance of our encoder with the that of the TML 8.5 software, and also against an RD-optimized implementation of an H.263 encoder using Annexes D, F, I, J and T.  Our results illustrate that it is possible to achieve near-real-time H.26L encoding at CIF resolution without a large drop in coding performance.

# Description of Software

UB Video’s H.26L encoder is a highly-optimized C-language implementation that has been developed from scratch.  The main sources of the speed improvement relative to the TML software are the highly efficient design and an efficient motion estimation algorithm that is based on a floating-center diamond search.  The encoder is currently in the early stages of development and does not yet provide support for multiple reference frames, B-frames or eighth-pel motion vector accuracy. However, the CABAC entropy coding is supported by our encoder and is used for the results generated in this contribution.

The syntax of the encoder is compliant with TML 8.5, with the addition of two features that were proposed at the Santa Barbara meeting and accepted: Nokia’s low-complexity interpolation method [1] and Sharp’s proposal on interpolating sub-pixel positions that lie outside of the frame boundaries [2].   Since we have not yet implemented our own decoder, and have not modified the TML software to reflect this second item, our decoded bit stream will have some mismatch near the  frame edges.  Thus, results are presented by using the encoder’s reconstructed frames, which in practice would be identical to the decoded output.

# Simulations

Simulations were generated using the following system:

CPU: Pentium III, 800 MHz

Memory: 128 MB SDRAM

Compiler: Microsoft Visual C 6.0

All of the H.26L results presented use CABAC entropy coding and just one reference frame.  The TML 8.5 encoder uses the low-complexity mode decision and a search range of 16 pixels from the predictor.  The UBC H.263 implementation is used in RD-optimized mode and uses Annexes D, F, I, J and T with a full search range of 32 pixels.

RD-Performance

The RD-curves shown below illustrate that near-real-time encoding of CIF resolution content can be achieved in H.26L while experiencing minimal reductions in coding performance.  The drop in coding performance due to the algorithmic simplifications used in the UB Video encoder is less than 0.5 dB in most cases, with the largest drop occurring at high bitrates.  Furthermore, the results show that the performance of UB Video’s fast H.26L encoder substantially outperforms the fully RD-optimized H.263 encoder in terms of coding efficiency.

Encoding Speed

The following figures illustrate the average encoding speed in frames per second corresponding to the points plotted in the RD-curves above. The average encoding speed of the UB Video H.26L encoder is approximately 13 frames per second for CIF content on this test platform. Although we recognize that the TML software is not  designed or implemented with encoding speed in mind, we will note that encoding speeds for the UB Video encoder are roughly 50 times faster than those of the public encoder, as configured in this experiment.

# Acknowledgement

The authors would like to thank Todd Gosselin and Yon Yu for their work in developing the H.26L encoding software discussed in this contribution.

# References

- Marta Karczewicz, Antti Hallapuro, " Interpolation solution with low encoder memory requirements and low decoder complexity ", Document  VCEG-N31, Video Coding Experts Group (VCEG) 14th meeting, Santa Barbara, CA, USA, 24-27 September, 2001.

- Tianli Chu, Shijun Sun, Louis Kerofsky and Shawmin Lei, " Discussion on Efficient 16-bit Implementation of the Interpolation Routine", Document  VCEG-N19, Video Coding Experts Group (VCEG) 14th meeting, Santa Barbara, CA, USA, 24-27 September, 2001.