| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O39 Filename: VCEG-O39.doc Generated: 27 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Guy Côté, Lowell Winger, Michael Gallant VideoLocus Inc 109 Randall Dr, Waterloo, On. Canada, N2V 1C5 | Tel: Fax: Email: | +1.519.725.9797 +1.519.725.5345 gcote@videolocus.com, lwinger@videolocus.com, mgallant@videolocus.com |
| Title: | Lower complexity deblocking filter with in-place filtering |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

The current description of the deblocking filter requires that the macroblock pixels be copied to a temporary buffer before the vertical and horizontal filtering operations. Performing the filtering operations in-place can significantly reduce the complexity of the deblocking filter. This document compares these two methods.

# Modification to the deblocking filter

The only change to the deblocking filter is that the filtering operations are done in-place. Therefore, for filtering block boundaries within a macroblock, the input samples for the current block are the output (filtered) samples from the previous block.

# Results

Results are presented according to document VCEG-N81 for the baseline profile.

### PSNR Plots

### Tabulated Results

| ## Container, QCIF, frameskip=2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/ Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 37.301 | 40.6879 | 40.5973 | 27880 | 37.3018 | 40.7893 | 40.6021 | 27840 |
|  | Avg | 36.0825 | 40.486 | 40.0859 | 25.46 | 36.0863 | 40.4685 | 40.0773 | 25.41 |
| QP 20 | I-frame | 34.6138 | 38.9466 | 38.4937 | 19016 | 34.5921 | 38.6668 | 38.4296 | 19008 |
|  | Avg | 33.5382 | 38.9472 | 38.4862 | 14.2 | 33.5399 | 38.7009 | 38.7009 | 14.25 |
| QP 24 | I-frame | 31.5782 | 37.1607 | 36.4535 | 12912 | 31.6273 | 36.6039 | 36.1021 | 12976 |
|  | Avg | 30.7587 | 37.3783 | 36.839 | 8.53 | 30.7839 | 37.0162 | 36.5633 | 8.61 |
| QP 28 | I-frame | 28.7691 | 36.4662 | 35.2373 | 8112 | 28.815 | 36.1135 | 35.2082 | 8168 |
|  | Avg | 27.9597 | 36.636 | 35.7129 | 4.98 | 28.0359 | 36.5078 | 35.687 | 5.01 |

| ## News, QCIF, frameskip=2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 37.6877 | 39.3541 | 40.1694 | 31264 | 37.6734 | 39.4223 | 40.0482 | 31208 |
|  | Avg | 36.8549 | 39.1271 | 39.5972 | 48.63 | 36.8337 | 39.1302 | 39.5327 | 48.69 |
| QP 20 | I-frame | 34.5173 | 37.6112 | 38.3794 | 22120 | 34.5506 | 37.4546 | 38.1027 | 22144 |
|  | Avg | 33.9361 | 37.4989 | 38.2783 | 30.35 | 33.9655 | 37.3377 | 38.1379 | 30.32 |
| QP 24 | I-frame | 31.536 | 36.0745 | 36.3593 | 15728 | 31.5268 | 35.7455 | 36.0926 | 15576 |
|  | Avg | 31.0608 | 35.9697 | 36.7315 | 18.56 | 31.0577 | 35.8763 | 36.7132 | 18.44 |
| QP 28 | I-frame | 28.4593 | 34.3049 | 35.3465 | 9944 | 28.4189 | 34.2065 | 34.9749 | 9864 |
|  | Avg | 28.151 | 34.7196 | 35.6483 | 10.74 | 28.1351 | 34.452 | 35.4693 | 10.76 |

| ## Foreman, QCIF, frameskip=2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Anchor |  |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y |  | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 36.9257 | 39.4116 | 41.5349 | 27024 | 36.9514 |  | 39.3644 | 41.4199 | 27064 |
|  | Avg | 36.0656 | 39.0969 | 40.5595 | 79.48 | 36.0627 |  | 39.0787 | 40.5767 | 79.62 |
| QP 20 | I-frame | 34.258 | 38.1711 | 40.0595 | 18272 | 34.2199 |  | 38.0573 | 40.1447 | 18152 |
|  | Avg | 33.6546 | 37.8852 | 38.9084 | 47.88 | 33.6536 |  | 37.7999 | 38.9304 | 48.06 |
| QP 24 | I-frame | 31.4613 | 37.1124 | 38.742 | 11736 | 31.3563 |  | 37.1977 | 38.4101 | 11592 |
|  | Avg | 31.4258 | 36.6732 | 37.4283 | 29.59 | 31.3615 |  | 36.5944 | 37.4303 | 29.78 |
| QP 28 | I-frame | 28.6142 | 36.5068 | 37.8457 | 7304 | 28.7407 |  | 36.4712 | 37.6295 | 7344 |
|  | Avg | 28.9072 | 35.9455 | 36.4877 | 18.87 | 28.7785 |  | 35.9514 | 36.4759 | 18.69 |

| ## Paris, CIF, frameskip=1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 36.7686 | 39.2431 | 39.1374 | 155944 | 36.7707 | 39.1893 | 39.2013 | 155784 |
|  | Avg | 35.6019 | 38.4511 | 38.4297 | 339.7 | 35.5904 | 38.3835 | 38.3893 | 339.83 |
| QP 20 | I-frame | 33.7542 | 36.995 | 37.1687 | 108776 | 33.7548 | 36.7535 | 37.0253 | 108664 |
|  | Avg | 32.6669 | 36.4213 | 36.6288 | 200.68 | 32.6721 | 36.318 | 36.6747 | 200.8 |
| QP 24 | I-frame | 30.6751 | 35.0343 | 35.4423 | 75016 | 30.7054 | 34.8327 | 35.1501 | 75248 |
|  | Avg | 29.7867 | 34.6027 | 34.9753 | 112.83 | 29.7988 | 34.5104 | 34.8704 | 113.39 |
| QP 28 | I-frame | 27.6111 | 33.7908 | 34.0853 | 47432 | 27.6231 | 33.6842 | 33.8369 | 47440 |
|  | Avg | 26.8439 | 33.1125 | 33.6057 | 60.48 | 26.8384 | 33.0156 | 33.4665 | 60.4 |

| ## Silent, QCIF, frameskip=1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 36.4086 | 38.6776 | 39.5074 | 29040 | 36.4238 | 38.6095 | 39.4017 | 29016 |
|  | Avg | 35.9594 | 38.3366 | 39.2011 | 61.18 | 35.967 | 38.3048 | 39.1234 | 61.24 |
| QP 20 | I-frame | 33.754 | 36.6913 | 38.0421 | 18544 | 33.6843 | 36.438 | 37.6762 | 18480 |
|  | Avg | 33.2975 | 36.5403 | 37.9809 | 37.41 | 33.2601 | 36.3953 | 37.7958 | 37.29 |
| QP 24 | I-frame | 31.2181 | 34.9919 | 36.9644 | 11224 | 31.1111 | 34.8064 | 36.8384 | 11184 |
|  | Avg | 30.7166 | 35.0528 | 36.8011 | 21.98 | 30.6871 | 34.8315 | 36.7689 | 22.09 |
| QP 28 | I-frame | 28.7226 | 34.435 | 36.3048 | 6704 | 28.5532 | 34.2784 | 36.3138 | 6672 |
|  | Avg | 28.2395 | 34.3549 | 36.0949 | 12.3 | 28.1011 | 34.2388 | 36.0845 | 12.27 |

| ## Mobile, CIF, frameskip=0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 35.5323 | 37.3703 | 37.1698 | 255872 | 35.5083 | 37.3618 | 37.1538 | 255616 |
|  | Avg | 34.0906 | 35.2332 | 35.1409 | 1635.18 | 34.0975 | 35.2323 | 35.1247 | 1636.97 |
| QP 20 | I-frame | 32.0934 | 35.3375 | 35.0727 | 181288 | 32.0954 | 35.2565 | 35.0031 | 181464 |
|  | Avg | 30.9221 | 33.1654 | 32.9852 | 836.24 | 30.922 | 33.1524 | 32.9784 | 836.15 |
| QP 24 | I-frame | 28.7015 | 33.4622 | 33.1113 | 127424 | 28.6901 | 33.2529 | 32.9546 | 127336 |
|  | Avg | 28.0346 | 31.2946 | 31.0034 | 422.11 | 28.0308 | 31.2399 | 30.9884 | 422.53 |
| QP 28 | I-frame | 25.5725 | 32.2978 | 32.0856 | 78280 | 25.5579 | 32.0937 | 31.942 | 78392 |
|  | Avg | 25.1876 | 30.0598 | 29.6597 | 236.73 | 25.187 | 29.9562 | 29.6165 | 238.04 |

| ## Tempete, CIF, frameskip=0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 36.014 | 37.5817 | 39.4985 | 176640 | 36.0069 | 37.5633 | 39.5413 | 176792 |
|  | Avg | 35.0735 | 36.7043 | 38.5653 | 1246.72 | 35.0713 | 36.6588 | 38.53 | 1246.61 |
| QP 20 | I-frame | 32.8855 | 35.7753 | 37.983 | 120936 | 32.8421 | 35.5956 | 37.9288 | 120712 |
|  | Avg | 32.1636 | 35.1613 | 37.2134 | 623 | 32.1619 | 35.121 | 37.1951 | 623.4 |
| QP 24 | I-frame | 29.7499 | 34.1852 | 36.6521 | 78608 | 29.6837 | 34.0916 | 36.4471 | 78288 |
|  | Avg | 29.5185 | 33.637 | 35.89 | 313.67 | 29.5024 | 33.5779 | 35.8194 | 313.64 |
| QP 28 | I-frame | 26.8379 | 33.3169 | 35.7112 | 45800 | 26.8073 | 33.1642 | 35.6404 | 45952 |
|  | Avg | 26.8395 | 32.5541 | 34.9178 | 168.56 | 26.8011 | 32.4716 | 34.8162 | 168.56 |

### Method of VCEG-M33

Performance difference between the anchor and the proposed method was computed with the avsbr4 software.

| Sequence | Bitrate Increase (%) | PSNR Gain (Db) |
| --- | --- | --- |
| Container | 0.13 | -0.008 |
| New | -0.32 | 0.019 |
| Foreman | 1.05 | -0.051 |
| Paris | 0.10 | -0.006 |
| Silent | 0.98 | -0.046 |
| Mobile | 0.13 | -0.006 |
| Tempete | 0.28 | -0.012 |

# Conclusions

Performing the filter operations in-place significantly reduces the complexity of the deblocking filter. Results indicate that the compression performance is almost the same.  We propose that the deblocking filter description be changed to adopt in-place filtering.