| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O40 Filename: VCEG-O40.doc Generated: 3 December ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Michael Gallant, Lowell Winger, Guy Côté VideoLocus Inc 109 Randall Dr, Waterloo, On. Canada, N2V 1C5 | Tel: Fax: Email: | +1.519.725.9797 +1.519.725.5345 mgallant@videolocus.com, lwinger@videolocus.com, gcote@videolocus.com |
| Title: | Interlaced Field Coding Core Experiment |  |  |
| Purpose: | Core Experiment |  |  |

_____________________________

# 1.0 Introduction:

This document discusses progress on the portion of the interlaced coding core experiment, defined in VCEG-N85, being conducted by VideoLocus. Specifically, we are conducting the picture level experiments for structure combinations A and B outlined in the common conditions of VCEG-N85. The functionality for this core experiment has been incorporated in the Test Model software, intermediate release TML8.7a, except for one item (discussed below). Results are currently being generated but will not be available in time for the meeting. In the process of generating the results, we have identified several items to be addressed.

# 2.0 Issues

## 2.1 Direct Mode Prediction in B Field Pictures

VCEG-N85 specifies that, for direct mode in B field pictures, backward prediction must be from the closest subsequent reference field of the same parity. This poses a problem for direct mode prediction in every second B field picture. The problem arises because the second subsequent reference field picture can, and often does, predict from the first subsequent reference field picture. VCEG-N85 does not specify what action to take for direct mode prediction in B field pictures in this case. Therefore, we have maintained the approach taken in VCEG-N84, i.e. direct mode prediction in B field pictures is from the closest subsequent reference field picture. We propose that the core experiment define the action to be taken in this case.

## 2.2 Number of Reference Frames

We propose to modify the core experiment to use, at a minimum, the same number of forward reference fields for P and B pictures and, ideally, only two forward and two backward reference field pictures. In particular, for the experiments where B frames are enabled, the use of 6 reference fields (or 3 reference frames) provides little benefit.

## 2.3 Adaptive Frame/Field Pictures

If structure combination E of the common conditions defined in VCEG-N85, adaptive frame/field pictures, is to be tested, we propose that the core experiment define how frame or field coding is to be selected at the picture level. One simple, though slow, approach would be to code a frame once as a frame picture and once as a field picture and select the method that requires fewer bits.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | VideoLocus, Inc. |  |  |
| Mailing address | 109 Randall Drive, Waterloo, Ontario, Canada N2V 1C5 |  |  |
| Country | Canada |  |  |
| Contact person | Lowell Winger |  |  |
| Telephone | +01-519-725-9797 |  |  |
| Fax | +01-519-725-5345 |  |  |
| Email | lwinger@videolocus.com |  |  |
| Place and date of submission | Pattaya, 4-6 Dec 2001 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Lower complexity deblocking filter with in-place filtering |  |
| Contribution number |  | VCEG-O39 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)