| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-Oxx Filename: VCEG-Oxx.doc Generated: 27 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Lowell Winger,Guy Cote,Michael Gallant VideoLocus Inc. 109 Randall Drive Waterloo, Ontario, Canada N2V 1C5 | Tel: Fax: Email: | +1 (519) 725-9797 +1 (519) 725-5345 lwinger@videolocus.com |
| Title: | 16x16 planar intra prediction mode (imode 3) improvement |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

In section 5.9.1 of TML8, the 16x16 intra plane prediction mode (imode 3),

Pred(i,j) can go negative. We propose that it be clipped to [0-255] to improve the coding efficiency of this mode, and permit 8-bits to be used in this mode and as the common input format to the subsequent coding stage.

The concern is that, otherwise, 16-bits of precision is required in software for the prediction values, and that this is the only prediction mode where it would be needed.

# Conclusions

Clipping the prediction always improves the coding efficiency. Complexity, simplicity, and coding performance are improved with this modification.

We propose that this change also be incorporated into the test model.