| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O42 Filename: VCEG-O42.doc Generated: 27 Nov 2001 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Anthony Joch, Faouzi Kossentini University of British Columbia 2356 Main Mall, Vancouver, BC, Canada, V6T 1Z4 | Tel: Fax: Email: | +1 604 808-4284 +1 604 822-5949 anthonyj@ece.ubc.ca |
| Title: | Performance analysis of H.26L coding features |  |  |
| Purpose: | Information |  |  |

_____________________________

# Overview

This document provides extensive information about the coding gain provided by four key features of the current TML: CABAC, multi-frame prediction (using 5 reference frames), 1/8th-pel motion compensation and the use of numerous motion compensation block sizes. The coding gain from multi-frame prediction in H.26L is also compared to the gain provided by enabling similar functionality in H.263 using Annex U.  Experiments were conducted using a much larger set of test sequences than in the common conditions document [1] with the hope of obtaining results that are representative of general video content at CIF resolution and lower. Results are presented in terms of RD-curves and Bjontegaard Delta SNR values.

The purpose of these experiments is to provide valuable information that can be used in conjunction with complexity analysis to define the profiles of H.26L, and also to assist in the design of efficient encoding algorithms through a better understanding of the gains provided by these features. To this end, our results highlight some dependencies on content, resolution and bit rate that exist with these features. The results yield the following conclusions:

1. The CABAC mode provides a fairly consistent improvement in coding efficiency of between 5 and 10%, with the higher gains being achieved (as expected) for the higher resolution sequences and the very low bit rates,

2. Coding gains using multi-frame prediction with 5 reference frames in a sliding window buffer appear to be highly dependent on source content; for most of the sequences, average bit savings are less than 5%, with the exception being the Mobile and Tempete sequences, which show savings around 20%, and

3. Eighth-pel accuracy is only beneficial at high resolutions and high bit rates, and it performs best on sequences that are high in spatial detail.

4. As expected, smaller block sizes become more beneficial as the bit rate is increased.  However, our results also suggest that small block sizes have less benefit as the frame resolution is increased.

Another interesting conclusion is that the current set of CIF sequences in the common conditions document is probably not representative of general video content, and that the group may want to consider amending this set to include a more representative sample of video content.

# Test Conditions

The TML-8.5 software was used in the RD-optimized mode.  The default configuration used 5 reference frames, UVLC coding, quarter-pel motion compensation, and all 7 motion compensation block types.  B-frames were not used in any part of these experiments.  A full search range of 32 pixels from the predictor with no restrictions was only used for the experiments involving multi-frame prediction, where a smaller search range might limit the use of temporally distant frames.  In the other experiments, a search range of only 16 pixels was used, since the smaller search range should not provide any significant advantage or disadvantage to the features being tested. QUANT values of 16, 20, 24 and 28 were tested for all sequences, as specified in the common conditions.

For the comparison of multi-frame prediction in H.26L vs. H.263, the UBC H.263 encoder version 0.3 was used to generate H.263 bitstreams. Annexes D, F, I, J and T were used in all cases and   Annex U with 5 reference frames was optionally turned on. These settings correspond to Profiles 3 and 5 of Annex X.  Quantizer values of 5, 8, 13 and 21 were used.

The video content used in the test includes all of the sequences in the common conditions for coding efficiency tests, plus several other well-known CIF resolution sequences and one internally generated 320x240 sequence.  See Table 1 for details.

| Sequence | Format | Frame Rate | No. Frames Coded |
| --- | --- | --- | --- |
| Container Ship | QCIF | 10 fps | 100 |
| Foreman | QCIF | 10 fps | 100 |
| News | QCIF | 10 fps | 100 |
| Silent | QCIF | 15 fps | 150 |
| PI | 320x240 | 15fps | 400 |
| Paris | CIF | 15 fps | 150 |
| Silent | CIF | 15 fps | 150 |
| Bus | CIF | 30 fps | 150 |
| Coastguard | CIF | 30 fps | 300 |
| Flower garden | CIF | 30 fps | 250 |
| Foreman | CIF | 30 fps | 300 |
| Mobile | CIF | 30 fps | 300 |
| Tempete | CIF | 30 fps | 260 |
| Trailblazers | CIF | 30 fps | 300 |

Table 1: Input video sequences used in experiments

PI is a 400 frame sequence captured at 15 fps using a fixed low-end desktop camera.  It consists of two people talking and gesticulating while holding various objects (similar to Paris). The sequence is somewhat difficult because it is bright, has lots of contrast and the subjects are relatively close to the camera, filling approximately two-thirds of the field of view. This is used to represent some typical low-end video-conferencing or web-cam content. The first frame of the sequence is shown below. Trailblazers is a CIF version of the basketball sequence that was presented by Peter Borgwardt at the Santa Barbara meeting.

# Objective Results

Three experiments were carried out as described below.

Experiment 1: Entropy Coding

In this first experiment, the CABAC entropy coding mode was turned on and compared to the UVLC coding mode.  Quarter-pixel motion compensation and 5 reference frames were used.  The search range was 16 pixels from the predicted motion vector. Full results and RD-curves are presented in the Excel spreadsheet VCEG-O42_Entropy.xls. Summary BD-SNR results are presented in Table 2.

| Res. | Hz | Sequence | % Bits Savings | PSNR Gain |
| --- | --- | --- | --- | --- |
| QCIF | 10 | Container Ship | 4.96 | 0.251 |
|  |  | Foreman | 7.56 | 0.436 |
|  |  | News | 4.98 | 0.295 |
|  | 15 | Silent | 4.81 | 0.238 |
| 320x240 | 15 | PI | 6.35 | 0.355 |
| CIF | 15 | Paris | 5.47 | 0.288 |
|  |  | Silent | 6.94 | 0.298 |
|  | 30 | Bus | 9.49 | 0.485 |
|  |  | Coastguard | 9.50 | 0.376 |
|  |  | Flowergarden | 9.98 | 0.536 |
|  |  | Foreman | 13.07 | 0.686 |
|  |  | Mobile | 9.39 | 0.467 |
|  |  | Tempete | 7.54 | 0.329 |
|  |  | Trailblazers | 7.48 | 0.349 |
|  |  | AVERAGE | 7.68 | 0.385 |
|  |  | MIN | 4.81 | 0.238 |
|  |  | MAX | 13.07 | 0.686 |

Table 2: BD-SNR summary for CABAC  entropy coding

These results show that CABAC provides consistent gains in coding efficiency, typically between 5 and 10 percent, over a wide variety of content. CABAC seems to provide larger gains for the higher resolution sequences.  From the RD-curves, we note that the coding efficiency gains are present at all bit rates, and tend to be largest at very low and very high bit rates.

Experiment 2: Multiple Reference Frames

In this experiment, the sequences were coded with a single reference frame and with five reference frames.  UVLC coding and quarter-pel motion compensation were used and the search range was set to 32 pixels from the predictor.  In an attempt to capture background uncovering, we performed some experiments in which the (0,0) motion vector was always included in the search area.  However, with a large search range of 32 pixels, we found that this provided no significant benefit for this content.

This experiment also provides an opportunity to compare the benefit of using multiple reference frames in H.26L and H.263. The RD-optimized H.263 encoder was used with Annexes D, F, I, J, T, and optionally U. A full search range of 32 pixels was used with quantizer values of 5, 8, 13 and 21. Full results and RD-curves are presented in the Excel spreadsheet VCEG-O42_MultiFrame.xls.  The summary BD-SNR results are presented in Table 3.

|  |  |  | H.26L |  | H.263 |  | H.26L/H.263 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res. | Hz | Sequence | %Bits | PSNR | %Bits | PSNR | %Bits | PSNR |
| QCIF | 10 | Container | 2.73 | 0.135 | 16.83 | 0.841 | 0.162 | 0.161 |
|  |  | Foreman | 4.34 | 0.246 | 16.31 | 0.932 | 0.266 | 0.264 |
|  |  | News | 0.95 | 0.055 | 2.06 | 0.107 | 0.461 | 0.514 |
|  | 15 | Silent | 4.07 | 0.199 | 7.58 | 0.362 | 0.537 | 0.550 |
| 320x240 | 15 | PI | 1.65 | 0.090 | 2.85 | 0.146 | 0.579 | 0.616 |
| CIF | 15 | Paris | 3.36 | 0.173 | 7.29 | 0.372 | 0.461 | 0.465 |
|  |  | Silent | 5.69 | 0.240 | 6.36 | 0.331 | 0.895 | 0.725 |
|  | 30 | Bus | 6.76 | 0.336 | 11.82 | 0.628 | 0.572 | 0.535 |
|  |  | Coastguard | 0.53 | 0.019 | 2.29 | 0.090 | 0.231 | 0.211 |
|  |  | Flowergarden | 6.32 | 0.329 | 10.10 | 0.610 | 0.626 | 0.539 |
|  |  | Foreman | 3.19 | 0.153 | 11.82 | 0.541 | 0.270 | 0.283 |
|  |  | Mobile | 20.05 | 1.027 | 30.78 | 1.935 | 0.651 | 0.531 |
|  |  | Tempete | 18.64 | 0.845 | 28.9 | 1.360 | 0.645 | 0.621 |
|  |  | Trailblazers | 1.15 | 0.050 | 1.55 | 0.070 | 0.742 | 0.714 |
|  |  | MEAN | 5.67 | 0.278 | 11.18 | 0.595 | 0.507 | 0.481 |
|  |  | MIN | 0.53 | 0.019 | 1.55 | 0.070 | 0.162 | 0.161 |
|  |  | MAX | 20.05 | 1.027 | 30.78 | 1.935 | 0.895 | 0.725 |

Table 3: BD-SNR summary for multi-frame prediction in H.26L and H.263.  The two right-most columns show the relative coding gain provided in H.26L compared to H.263.

These results illustrate that the performance of multi-frame prediction is highly dependent on the source content, much more so than in the case of CABAC. Moreover, searching 5 frames in a sliding window buffer in H.26L provides approximately half of the gain that can be provided by searching a similar buffer using Annex U in an H.263 encoder. We believe that the main reason for this difference is that other features of H.26L that are not in H.263, such as quarter-pel motion compensation and the availability of more block sizes, already capture some of the coding gain that Annex U can capture in H.263.  Additionally, the macroblock level syntax for multi-frame prediction allows for less flexibility in H.26L, since the entire macroblock must be predicted from the same reference frame, whereas Annex U allows each 8x8 block to be predicted from a different frame.

The strong content-dependency that is present in these results lead to some observations on the current set of sequences included in the common conditions for coding efficiency.  Using H.26L, two sequences – Mobile and Tempete – show gains from multi-frame prediction that are nearly 3 times greater than the gains shown for any of the other 12 sequences. These two sequences seem to be unique in terms the benefit that multi-frame prediction can provide, and may not be representative of typical video content.  However, these are the only two 30Hz CIF sequences included in the common conditions, which are intended to represent typical content and conditions that are to be encountered by future H.26L encoders. Therefore, we believe that the group should consider revisiting the set of sequences included in the common conditions to include a larger number of sequences at CIF (and possibly higher) resolution.  We suggest that some of the sequences used in this comparison or some of the sequences from VQEG be used as additional test sequences.

One other interesting observation that can be drawn from the RD-curves is that H.26L outperforms H.263 significantly for all sequences in this set, with the notable exception of Silent, at both CIF and QCIF resolution.  For this sequence, the RD-curves for the two standards almost overlap. Informal subjective testing has shown that H.26L offers a small subjective improvement over H.263 on this sequence, however. The main difference that is observed is that H.26L introduces less blurring, likely due to its improved deblocking filter. Finally, we note that since the CABAC mode was not used for the H.26L results, further improvement in coding performance is achievable using H.26L.

Experiment 3: Eighth-pel Motion Compensation

This experiment assesses the benefit of increasing the spatial accuracy of motion vectors from quarter-pel to eighth-pel. Five reference frames, UVLC entropy coding, and a search range of 16 pixels were used. Complete results and RD-curves are available in the Excel spreadsheet VCEG-O42_MVAccuracy.xls.  BD-SNR results are presented in Table 4.

| Res. | Hz | Sequence | % Bits Savings | PSNR Gain |
| --- | --- | --- | --- | --- |
| QCIF | 10 | Container Ship | 0.57 | 0.032 |
|  |  | Foreman | -5.6 | -0.314 |
|  |  | News | -3.47 | -0.205 |
|  | 15 | Silent | -6.05 | -0.285 |
| 320x240 | 15 | PI | -4.66 | -0.252 |
| CIF | 15 | Paris | -1.85 | -0.097 |
|  |  | Silent | -5.28 | -0.217 |
|  | 30 | Bus | 2.5 | 0.129 |
|  |  | Coastguard | -2.5 | 0.089 |
|  |  | Flowergarden | 6.8 | 0.38 |
|  |  | Foreman | -5.22 | -0.254 |
|  |  | Mobile | 10.78 | 0.574 |
|  |  | Tempete | 1.88 | 0.082 |
|  |  | Trailblazers | -5.99 | -0.265 |
|  |  | AVERAGE | -1.29 | -0.043 |
|  |  | MIN | -6.05 | -0.314 |
|  |  | MAX | 10.78 | 0.574 |

Table 4: BD-SNR summary for eighth-pel motion vector accuracy

Initially, it seems that eighth-pel accuracy provides little benefit, especially considering its added complexity. However, by looking at the RD-curves, it is clear that the benefit provided by using eighth-pel accuracy increases as the fidelity of the coded video is increased, and also as the resolution is increased.  Therefore, eighth-pel would likely be beneficial in profiles meant to target applications that require high-quality video, digital cinema for example.

Experiment 4: Motion Compensation Block Types

Our final experiment evaluates the effect of enabling various logical combinations of the 7 motion compensation block types that are available in TML 8.5. Five reference frames, UVLC entropy coding, quarter-pel accuracy and a search range of 16 pixels were used.  Complete results and RD-curves are available in the Excel spreadsheet VCEG-O42_BlockSize.xls. The combinations of block types that were tested are listed in Table 5.  BD-SNR results comparing the benefit of the various combinations of block types versus using the 16x16 mode only are presented in Table 6.

| 1 | 16x16 |
| --- | --- |
| 1, 4 | 16x16, 8x8 |
| 1 to 3 | 16x16, 16x8, 8x16 |
| 1 to 4 | 16x16, 16x8, 8x16, 8x8 |
| 1 to 6 | 16x16, 16x8, 8x16, 8x8, 8x4, 4x8 |
| 1 to 7 | 16x16, 16x8, 8x16, 8x8, 8x4, 4x8, 4x4 |

Table 5: Key to Combinations of Block Types

|  |  |  | 1, 4 |  | 1 to 3 |  | 1 to 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Res. | Hz | Sequence | %Bits | PSNR | %Bits | PSNR | %Bits | PSNR |
| QCIF | 10 | Container | 13.60 | 0.699 | 15.76 | 0.807 | 16.79 | 0.872 |
|  |  | Foreman | 8.94 | 0.505 | 12.22 | 0.682 | 13.49 | 0.774 |
|  |  | News | 10.05 | 0.586 | 10.98 | 0.631 | 13.33 | 0.786 |
|  | 15 | Silent | 7.19 | 0.347 | 9.48 | 0.454 | 11.38 | 0.561 |
| 320x240 | 15 | PI | 12.44 | 0.703 | 13.61 | 0.758 | 16.27 | 0.925 |
| CIF | 15 | Paris | 12.75 | 0.677 | 13.44 | 0.700 | 15.61 | 0.831 |
|  |  | Silent | 4.56 | 0.191 | 7.73 | 0.325 | 8.34 | 0.354 |
|  | 30 | Bus | 12.11 | 0.622 | 15.56 | 0.800 | 17.05 | 0.886 |
|  |  | Coastguard | 6.72 | 0.243 | 9.67 | 0.351 | 10.67 | 0.391 |
|  |  | Flowergarden | 14.39 | 0.804 | 16.26 | 0.906 | 17.43 | 0.980 |
|  |  | Foreman | 7.69 | 0.374 | 12.00 | 0.589 | 12.50 | 0.620 |
|  |  | Mobile | 10.28 | 0.500 | 11.55 | 0.561 | 13.04 | 0.641 |
|  |  | Tempete | 9.20 | 0.395 | 12.19 | 0.529 | 12.91 | 0.565 |
|  |  | Trailblazers | 8.39 | 0.400 | 11.13 | 0.522 | 12.75 | 0.605 |
|  |  | MEAN | 9.88 | 0.503 | 12.26 | 0.615 | 13.68 | 0.699 |
|  |  | MIN | 4.56 | 0.191 | 7.73 | 0.325 | 8.34 | 0.354 |
|  |  | MAX | 14.39 | 0.804 | 16.26 | 0.906 | 17.43 | 0.980 |

|  |  |  | 1 to 6 |  | 1 to 7 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Res. | Hz | Sequence | %Bits | PSNR | %Bits | PSNR |
| QCIF | 10 | Container | 19.19 | 1.017 | 19.59 | 1.039 |
|  |  | Foreman | 16.08 | 0.950 | 16.38 | 0.970 |
|  |  | News | 16.12 | 0.984 | 16.42 | 1.013 |
|  | 15 | Silent | 12.71 | 0.641 | 12.74 | 0.645 |
| 320x240 | 15 | PI | 19.21 | 1.130 | 19.51 | 1.153 |
| CIF | 15 | Paris | 19.02 | 1.048 | 19.21 | 1.069 |
|  |  | Silent | 9.28 | 0.402 | 9.12 | 0.396 |
|  | 30 | Bus | 19.80 | 1.056 | 20.00 | 1.072 |
|  |  | Coastguard | 11.67 | 0.434 | 11.90 | 0.443 |
|  |  | Flowergarden | 23.31 | 1.359 | 23.59 | 1.381 |
|  |  | Foreman | 14.89 | 0.756 | 14.85 | 0.755 |
|  |  | Mobile | 16.13 | 0.814 | 16.38 | 0.830 |
|  |  | Tempete | 15.07 | 0.673 | 15.26 | 0.683 |
|  |  | Trailblazers | 14.69 | 0.710 | 14.88 | 0.722 |
|  |  | MEAN | 16.23 | 0.855 | 16.42 | 0.869 |
|  |  | MIN | 9.28 | 0.402 | 9.12 | 0.396 |
|  |  | MAX | 23.31 | 1.359 | 23.59 | 1.381 |

Table 6: BD-SNR summary for motion compensation block sizes

The results of this experiment yield the following key points:

- Allowing either 8x8 blocks (1 and 4) as in H.263 and MPEG-4, or 16x8 and 8x16 block (1 to 3) provides a substantial gain in coding efficiency for all content and bitrates tested.  However, the larger blocks generally provide a greater improvement, particularly at low bitrates.

- Allowing block types 1 to 4 provides a small gain in coding efficiency over either of the previous two cases, particularly at higher bit rates.

- Block sizes smaller than 8x8 tend to be beneficial only at higher bitrates.  The benefit increases as the bitrate is increased.

- Our results may suggest that smaller block sizes provide less benefit for higher resolution sequences.  In particular, the benefit of adding more motion compensation block types is for the Silent sequences is less at CIF resolution than at QCIF.  Foreman exhibits a similar, but smaller effect, although the two Foreman sequences are also at different frame rates.  Further experiments in this area including resolutions higher than CIF would be beneficial.

- The 4x4 block type provides minimal improvement in PSNR performance on all of this content. There may be some subjective improvement, but we have not conducted subjective testing in this area.

# Summary

We presented a large set of data generated with the RD-optimized TML 8.5 software in order to test the performance of several features of the current H.26L draft standard. The results indicate that the CABAC mode provides a fairly consistent improvement in coding efficiency of between 5 and 10%.  This seems to be largest for higher resolution sequences and at very low bit rates.  Coding gains using multi-frame prediction seem to be highly dependent on source content. For the majority of sequences, average bit savings are less than 5%, yet two sequences show savings around 20%.   Since these two sequences are currently the only two 30Hz CIF sequences in the common conditions for coding efficiency, this result suggests that the conditions may need to be amended to provide a broader representation of video content. Our observations with regards to the benefit of eighth-pel accuracy indicate that such is only beneficial at high resolutions and high bit rates.  Eighth-pel accuracy performs best on sequences that are high in spatial detail.  Finally, the use of many different block sizes provides a consistent improvement, averaging 16% bit savings if all block types are used versus using 16x16 mode only.   Most of the benefit can be captured by using 8x8 and larger blocks, although the smaller blocks are more beneficial as the bitrate is increased.

# References

- Gary Sullivan, " Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material ", Document  VCEG-N81, Video Coding Experts Group (VCEG) 14th meeting, Santa Barbara, CA, USA, 24-27 September, 2001.