| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O43 Filename: VCEG-O43.doc Generated: 26 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Chul-Woo Kim McubeWorks 14 F, SK Telecom Bldg, Sunae-Dong, Bundang-Gu, Sungnam Seoul, 463-784, Korea Dong-Wook Kang Kookmin University 861-1, Chongnung-dong Songbuk-gu, Seoul 136-702, Korea Sang Woo Rhie SK Telecom 99 Seorin-dong, Jongro-gu, Seoul, 110-110, Korea | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: | +82-31-601-5875 +82-31-601-5878 charlie@mcubeworks.com +82-2-910-4641 +82-2-910-4641 dwkang@kookmin.ac.kr +82-2-2121-3463 +82-2-2121-3980 rainism@sktelecom.com |
| Title: | High Complexity Mode Decision for Error Prone Environment |  |  |
| Purpose: | Test Model |  |  |

_____________________________

- Introduction

It is known that the drift noise, which is defined as the encoder-decoder mismatch of reference frame, is the major factor that causes degradation of video quality when transmitting compressed video bitstream through error prone channel. Video quality degradation due to the existence of drift noise propagates through successive frames. Though intra-refreshment is known to be the most effective method of reducing the drift noise, it generally requires sacrifice of compression efficiency. This contribution proposes an R-D optimization method considering estimated drift noise to perform optimized intra-refreshment.

- High Complexity Mode Decision

# Distortion Model with Error-Prone Channel

Video transmission system through error prone channel can be modeled as in Fig. 1. Encoder may or may not exploit closed-loop temporal prediction according to encoding mode:

sn=fxn,xn−1=frxn,xn−1INTERfaxnINTRA

We assume that the only noise incurred by encoding process is quantization noise:

xn=xn+qn

Channel error may be added during transmission of bitstream:

rn=sn+en

Fig. 1. Model of video transmission through error-prone channel

Decoding process is basically the inverse of encoding process. But if errors are detected, decoder operation changes into error concealment mode (temporal error concealment is assumed).

yn=yn−1errorconcealmentf−1rn,yn−1normaldecoding

There may exist drift noise between reference frames in encoder and decoder due to channel errors:

yn=xn+dn

Considering the possibility of channel error and its corresponding error concealment algorithm,

For INTER macroblocks

dn=dn−1+xn−1−xnerrorconcealmentdn−1mv+xn−1mv+xn−xn−1mv+qn−xnnormalINTERdecodingdn−1+xn−1−xnerrorconcealmentdn−1mvnormalINTERdecoding

For INTRA macroblocks

dn=dn−1+xn−1−xnerrorconcealmentxn+qn−xnnormalINTRAdecodingdn−1+xn−1−xnerrorconcealment0normalINTRAdecoding

The expected energy of drift noise can be updated from-frame-to-frame:

For INTER mode

dn2=PEC⋅dn−12+xn−1−xn2+1−PEC⋅dn−1mv2

For INTRA mode

dn2=PEC⋅dn−12+xn−1−xn2

where PEC represents the probability of temporal error concealment.

The expected total distortion may be estimated as following:

Dtotal=Ddrift+Dquantdn2+qn2

# 2.2  Mode Decision Algorithm

It is assumed that PEC can be obtained during channel negotiation. Mode decision is performed by selecting INTRA or INTER mode based on the total expected distortion shown above and expected length of bitstream:

Minimize Dari+λ⋅ℓsni.

- Simulation Results

The proposed mode decision algorithm was tested according to the conditions in VCEG-M77. The overall simulation results are shown in VCEG-O43.xls. Anchor 1,2,5,6,11 and 12 were used in the test regarding the conversational service. The performance of proposed mode decision method was compared with the forced-intra refreshment using MbLineIntraUpdate, which is implemented in TML 8.7a. Optimal MbLineIntraUpdate values were searched depending on the channel characteristics to be used for the forced-intra refreshment. First frame was intra-coded at the same bit rate for both cases and the result from the I-frame was excluded in the comparison for it was assumed that no channel error occurred in the transmission of I-frame. Remaining 99 frames were inter-coded and the results from these P-frames were included in the simulation results. The proposed mode decision method shows better performance above 1 dB over the forced-intra refreshment.

- Summary

The proposed algorithm shows effective R-D optimized mode decision performance for error prone channel. This mode decision method can be refined when it is combined with data partitioning. The proposed mode decision method applied to MPEG-4 also shows highly better performance when compared with high complexity mode decision method in TMN 11 of H.263.

- IPR Strategy

McubeWorks and SK Telecom have agreed and decided to have intellectual property related to this contribution and have chosen to subscribe to subclause 2.2 for ITU-T patent policy under the condition of reciprocity.

- Reference

[1] Gisle Bjontegaard, “H.26L Test Model Long Term Number 8 (TML-8) draft 0,” June. 2001.

[2] Y. Wang, et. al., "Error resilient video coding techniques," IEEE Signal Processing Magazine, pp.61-82, July 2000.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | McubeWorks Inc. & SK Telecom |  |  |
| Mailing address | 14F, SK Telecom Bldg. 9-1 Sunae-Dong, Bundang-gu, Sungnam City, Kyungki-do 463-784 |  |  |
| Country | Korea |  |  |
| Contact person | Chul-Woo Kim |  |  |
| Telephone | +82 - (0)31 – 601 5875 |  |  |
| Fax | +82 - (0)31 – 601 5878 |  |  |
| Email | charlie@mcubeworks.com |  |  |
| Place and date of submission | Pattaya, 4th December |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | High Complexity Mode Decision for Error Prone Environment |  |
| Contribution number |  | VCEG-O43r1 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: