| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O51 Filename: VCEG-O51.doc Generated: 27 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wiegand, Karsten Müller, and Heiko Schwarz Image Processing Department Heinrich-Hertz-Institute Einsteinufer 37 D-10587 Berlin, Germany | Tel: Fax: Email: | +49 30 31 002 619/621/617 +49 30 392 72 00 wiegand@hhi.de kmueller@hhi.de, hschwarz@hhi.de |
| Title: | H.26L TML 8 for Digital Cinema Applications |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Summary

This contribution provides an overview of the coding efficiency of H.26L TML 8 ver. 8.50 in comparison to the new anchor sequences of MPEG-4 digital cinema. For most sequences, H.26L provides a bit-rate saving of 30 % at 16 Mbit/s indicating a performance benefit but also further space for optimization of H.26L for this application.

# H.26L TML-8 Settings

Freq. for encoded bitstream		: 24

Hadamard transform			: Used

Image format				: 1920x1080

Error robustness			: Off

No of ref. frames used in P pred		: 5

Search range				: 16x16 for previous and 8x8 for older images

MV resolution				: 1/4-pel

Sequence type				: IPPP (QP: I 16, P 16)

Entropy coding method			: CABAC

Search range restrictions		: older reference frames

RD-optimized mode decision		: used

Data Partitioning Mode			: 1 partition

Output File Format			: H.26L File Format

# Test sequences

As good as it gets		1920x1080	251 frames

Beavis and Butthead		1920x1080	279 frames

Book				1920x1080	221 frames

Budweiser			1920x1080	240 frames

Car through Landscape		1920x1080	210 frames

Men in Black			1920x1080	290 frames

# Results

As good as it gets

|  | TML 8 ver8.50 | (H.26L) |  |  | New Anchor | (MPEG-4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) |
| 5 | 17467 | 47,08 | 46,24 | 47,83 | 16000 | 46,2 | 44,7 | 46,9 |
| 3 | 26963 | 48,20 | 47,20 | 48,45 | 28000 | 47,6 | 45,7 | 47,6 |
| 0 | 47966 | 49,93 | 49,00 | 49,63 | 48000 | 49,9 | 47,5 | 48,7 |

Beavis and Butthead

|  | TML 8 ver8.50 | (H.26L) |  |  | New Anchor | (MPEG-4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) |
| 16 | 16641 | 37,78 | 38,25 | 38,39 | 16000 | 36,1 | 36,5 | 36,6 |
| 12 | 29138 | 40 | 40,56 | 40,82 | 28000 | 38,5 | 39 | 39,1 |
| 9 | 44514 | 41,95 | 42,36 | 42,66 | 48000 | 41,5 | 41,5 | 41,6 |
| 5 | 75319 | 44,93 | 45,04 | 45,14 | 80000 | n/a | n/a | n/a |

Book

|  | TML 8 ver8.50 | (H.26L) |  |  | New Anchor | (MPEG-4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) |
| 8 | 15669 | 43,45 | 47,21 | 46,42 | 16000 | 42,5 | 47 | 46 |
| 6 | 25886 | 44,82 | 47,45 | 46,99 | 28000 | 44,3 | 47,7 | 47 |
| 3 | 45283 | 46,93 | 48,24 | 48,07 | 48000 | 46,5 | 48,6 | 48,2 |
| 0 | 72683 | 49,18 | 49,61 | 49,59 | 80000 | n/a | n/a | n/a |

Budweiser

|  | TML 8 ver8.50 | (H.26L) |  |  | New Anchor | (MPEG-4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) |
| 9 | 14617 | 42,06 | 43,09 | 45,22 | 16000 | 41,7 | 40,6 | 43,1 |
| 7 | 25511 | 43,1 | 43,86 | 46,07 | 28000 | 42,5 | 41,1 | 43,7 |
| 5 | 44100 | 44,65 | 45,13 | 47,08 | 48000 | 43,5 | 41,06 | 44,2 |
| 2 | 77518 | 47,11 | 47,43 | 48,65 | 80000 | n/a | n/a | n/a |

Car through Landscape

|  | TML 8 ver8.50 | (H.26L) |  |  | New Anchor | (MPEG-4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) |
| 10 | 14155 | 42,34 | 44,64 | 44,52 | 16000 | 41,9 | 44 | 43,9 |
| 7 | 28229 | 44,3 | 45,61 | 45,53 | 28000 | 43,3 | 44,7 | 44,6 |
| 4 | 46890 | 46,38 | 46,62 | 46,64 | 48000 | 45,8 | 46,1 | 45,9 |
| 1 | 74820 | 48,75 | 48,43 | 48,27 | 80000 | n/a | n/a | n/a |

Men in Black

|  | TML 8 ver8.50 | (H.26L) |  |  | New Anchor | (MPEG-4) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) | Bitrate (kBits/s) | Y (dB) | Cb (dB) | Cr (dB) |
| 6 | 15877 | 46,28 | 46,42 | 49,02 | 16000 | 46,4 | 45,4 | 48,5 |
| 4 | 22917 | 47,23 | 46,69 | 49,13 | 28000 | 47,9 | 46,2 | 49 |
| 1 | 42307 | 48,91 | 48,16 | 49,94 | 48000 | 49,4 | 47,2 | 49,5 |
| 0 | 52031 | 49,57 | 48,83 | 50,24 | 80000 | n/a | n/a | n/a |

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Heinrich-Hertz-Institute |  |  |
| Mailing address | Einsteinufer 37 10587 Berlin, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Thomas Wiegand |  |  |
| Telephone | +49 - (0)30 - 31002 617 |  |  |
| Fax | +49 - (0)172 - 322 7972 |  |  |
| Email | wiegand@hhi.de |  |  |
| Place and date of submission | Pattaya, 4th December |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | H.26L |  |
| Title |  | H.26L TML 8 for Digital Cinema Applications |  |
| Contribution number |  | VCEG-O51 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: