| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O53 Filename: VCEG-O53.doc Generated: 02 December ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Teruhiko Suzuki and Yoichi Yagasaki Sony Corp. Gate City Osaki East Tower 1-11-1, Osaki, Shinagawa-ku, Tokyo 141-0032, Japan | Tel: Fax: Email: | +81-3-5435-3669 +81-3-5435-3891 teruhiko@av.crl.sony.co.jp |
| Title: | ### Tool to support random access |  |  |
| Purpose: | Proposal |  |  |

# Introduction

H.26L TML 8 does not fulfill Random Access functionality. For this functionality, I would like to propose MPEG1/2 GOP(Group of Picture) structure/layer into H.26L. In this contribution, this layer/structure is referred as Group of Video Object Plane(GOP) layer.

The advantages and disadvantages of the GOP structure/layer are as follows.

Advantages:

- Random Access Functionality

- The GOP layer is not mandatory layer, but an optional layer.

In other words, some applications which do not require Random Access functionality do not have to use GOP structure. So introducing GOP structure in JM will not sacrifice any functionalities which are currently supported in JM.

- Re-synchronization Tool for Error Recovery

The decoder can synchronize at GOP layer after encountering error. So the GOP structure can be seen as a tool for error recovery.

- Confirmed by MPEG1&2&4

- Many MPEG1&2 applications, such as DVD, Video CD use the GOP structure for Random Access functionality. So I think the GOP structure is confirmed as a useful tool in real applications, and this technology is very matured.

Disadvantage:

- Coding Efficiency

The GOP layer needs 59 bits in MPEG2 standards. It consists 32bit for start code, 25bits for time_code and 2bits for flags. The frequency of GOP layer is depend on encoding. And this is a trade off between coding efficiency and random access functionality.

The key technologies of GOP layer to support random access are as follows.

- Unique start code: In order to find random access point efficiently, unique start code must be inserted at all random access point. Using the unique code, the decoder can find random access point without parsing a bitstream.

- Time code: Absolute display time is necessary at random access point to identify the target frame.

- Dependency between GOP: The flag to identify whether the GOP is dependent to the other GOP. If one of the frame in the GOP refers to the frame in other GOP, the frame will not be decoded correctly. The flag is necessary for decoder to identify whether the reconstructed frame is correct or not.

# Syntax and Semantics of GOP layer

Now I would like to introduce GOP structure/layer for random access purpose. For random access, absolute display time is one of essential key to search the target frame. However in the Picture header, we have temporal_reference on local time base by indication the display order of each frame. In other word, display time for each frame is set with relative value and there is no way to set absolute display time. So we need time_code which indicates absolute display time in the GOP layer. The time_code refers to the first frame (in display order) after the GOP header. The syntax of GOP layer is as follows, and it is the same as one in MPEG2 standards.

## Syntax of Group of Plane

| group_of_plane() { | No. of bits | Mnemonic |
| --- | --- | --- |
| group_start_code | 32 | bslbf |
| time_code | 25 | bslbf |
| closed_gop | 1 | uimsbf |
| broken_link | 1 | uimsbf |
| next_start_code() |  |  |
| } |  |  |

## Semantics of Group of Plane

group_start_code -- The group_start_code is the bit string ‘000001B8’ in hexadecimal. It identifies the beginning of a group of plane header.

time_code -- This is a 25-bit integer containing the following: drop_frame_flag, time_code_hours, time_code_minutes, marker_bit, time_code_seconds and time_code_pictures as shown in Table1. The parameters correspond to those defined in the IEC standard publication 461 for “time and control codes for video tape recorders”. The time code refers to the first frame (in display order) after the GOP header.

The drop_frame_flag can be set to either ‘0’ or ‘1’. It may be set to ‘1’ only if the frame rate is 29,97Hz. If it is ‘0’ then pictures are counted assuming rounding to the nearest integral number of pictures per second, for example 29,97Hz would be rounded to and counted as 30Hz. If it is ‘1’ then picture numbers 0 and 1 at the start of each minute, except minutes 0, 10, 20, 30, 40, 50 are omitted from the count.

The rest of parameter in tim_code are time_code_hours, time_code_minutes, marker_bit, time_code_seconds and time_code_pictures. The time_code_pictures specifies time based on frame rate. Since the display time of frame is independent from frame rate, we may not use time_code_pictures and fix to ‘0’. So the precision of time_code is second.

NOTE -	The information carried by time_code plays no part in the decoding process.

Table 1 — time_code

| time_code | range of value | No. of bits | Mnemonic |
| --- | --- | --- | --- |
| drop_frame_flag |  | 1 | uimsbf |
| time_code_hours | 0 - 23 | 5 | uimsbf |
| time_code_minutes | 0 - 59 | 6 | uimsbf |
| marker_bit | 1 | 1 | bslbf |
| time_code_seconds | 0 - 59 | 6 | uimsbf |
| time_code_pictures | 0 - 59 | 6 | uimsbf |

closed_gop -- This is a one-bit flag which indicates the nature of the predictions used in any frames in the current GOP. The closed_gop is set to ‘1’ to indicate that all of frames following the GOP header have been encoded without predicting the frames previous to GOP header or intra coding. This bit is provided for use during any editing which occurs after encoding. If the previous pictures have been removed by editing, broken_link may be set to ‘1’ so that a decoder may avoid displaying these frames predicting frames in other GOPs. However if the closed_gop bit is set to ‘1’, then the editor may choose not to set the broken_link bit as all frames in the GOP can be correctly decoded.

broken_link -- This is a one-bit flag which shall be set to ‘0’ during encoding. It is set to ‘1’ to indicate that all frames following the group of picture header may not be correctly decoded because the reference frame which is used for prediction is not available (because of the action of editing). A decoder may use this flag to avoid displaying frames that cannot be correctly decoded.

# Conclusion

H.26L TML 8 does not fulfill Random Access functionality. For this functionality, I propose Group of Picture layer. The GOP layer is well tested and is confirmed to be useful in many applications. Therefore it should be included in the new version of H.26L.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Tool to support random access |  |
| Contribution number |  | VCEG-O53 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: