| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B019 Filename: JVT-B019.doc Generated: 2002-01-7 |
| --- | --- |

| Title: | Global Motion Vector Coding (GMVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 Shawmin Lei Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607Name | Tel: Email: | 360-817-7531 ssun@sharplabs.com 360-817-8446 shawmin@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

Global Motion Vector Coding (GMVC) has been proposed in the previous meetings [1]. This contribution is a revision of the previous GMVC proposal.

1. Bit stream syntax

For each P frame, a GMVC_flag symbol is added right after the slice header to signal if GMVC is on or off for the current slice. If GMVC is off, then all following bit stream will be exactly same as that of current TML [2]; comparing to the original TML syntax, only 1 extra bit per inter frame is needed for the GMVC_flag. If GMVC is on, the global motion vectors (GMV’s) are to be coded after the GMVC_flag; then, for each image MB, if the MB mode is determined to be one of the GMVC modes, motion vector coding can be simply skipped; the motion vector values for all 4×4 image blocks in that specific MB can be derived from GMV’s through a bilinear interpolation. The motion vector interpolation is as same as the previous proposal [1]. The motion vector accuracy can vary. For example, for the future TML baseline profile, we have chosen to represent the motion vectors in ¼ pixels.

Table 1. Codeword number of MB modes

The code number of a GMVC MB mode, GMVC_4x4 (previously called GMVC_16 [1]), has been rearranged to increase the bit saving. Table 1 lists the revised codeword numbers of the MB modes for both GMVC MB modes.

Comparing to the current TML syntax, the “copy” mode is replaced by the “GMVC_COPY” mode in a GMVC frame; a new mode “GMVC_44” is added to the GMVC frame. For all the non-GMVC modes in the GMVC frame, motion vectors are coded explicitly into the bitstream.

2. Experiment

2.1 P frame coding

The coding efficiency experiments are conducted mainly following the “Simulation Common Conditions” specified in VCEG-N81 [3] and the specification used in the previous proposal [1]. The proposed GMVC method is integrated into the TML8.5 software to compare against the TML8.5 performance. The motion search range parameter is set to 32; and the RD optimization option is turned on for the experiments. The global motion search algorithm is as same as we presented previously [1], but further optimized.

RD curves for all the testing sequences are shown in the Appendix 1. Detailed experimental results are available in JVT-B019.xls. There is almost no performance difference for sequences without global motion, such as “containet_qcif,” “news_qcif,” “silent_qcif,” and “paris_cif.” Table 2 summarizes the results of the sequences, where global motions are present. Bit savings of up to 20% can be observed at very low bit rate (or when QP is high).

Table 2. Comparison of TML8.5 and the proposed GMVC (IPPP)

Side-by-side comparison is arranged for the meeting. QP’s in bold in Table 2 are available for the subjective evaluation. In summary, the GMVC improves visual quality in many background areas when global motions are present, for example, in “coast” and “foreman” sequences.

2.2 B frame coding

Per some JVT members’ request, experiments are conducted to analyze the coding efficiency when B frame coding is enabled; even though GMVC is proposed for H.26L baseline and B frame syntax has not been changed.

Subset of the sequences used in Section 2.1 is used for B frame coding, with two B frames inserted into each pair of P frames. The results are summarized in Table 3. RD curves are available in the Appendix 2 and detailed results are available in JVT-B019.xls.

For “foreman” and “coast_guard” sequences, the average bit savings become less because of the B frame coding. However, it is interesting to see that GMVC gives some bit savings for “Stefan” sequences even at high bit rate.

Side-by-side comparison is arranged for the meeting. QP’s in bold in Table 3 are chosen for the subjective evaluation. The visual quality improvement similar to that of P frame coding can be observed.

Table 3. Comparison of TML8.5 and the proposed GMVC (IBBP)

2.3 Complexity

As we presented previously [1], there is no significant increase of complexity on the decoder side for GMVC. On the encoder side, a simple global motion estimation method is developed, which takes as low as 1-2% of the computation. Currently, the global motion estimation and the TML local motion search are both executed. It appears that the computation of local motion search potentially can be significantly reduced when the global motion information is considered. Therefore, our GMVC method will not introduce any significant extra requirement for the total computation in that case.

3. Summary

We have demonstrated the advantages of the proposed Global Motion Vector Coding (GMVC) technique. This coding method gives certain benefit for low-bit-rate video coding by reducing bit rate and enhancing visual quality; it is especially useful for video applications where the global or camera motion is highly expected. It takes only one bit per frame to signal off the GMVC option if the GMVC is not to be used; the overhead for coding efficiency is negligible. As we discussed in Section 2.3, there is no significant increase in complexity for GMVC. Therefore, we recommend JVT to adopt the proposal into the new TML.

# References

[1] Shijun Sun and Shawmin Lei, " Global Motion Vector Coding (GMVC)," document VCEG-O20, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, Thailand, 4-7 December 2001.

[2] Gisle Bjontegaard, "H.26L Test Model Long Term Number 8 (TML-8) draft0," document VCEG-N10, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[3] Gary Sullivan and Gisle Bjontegaard, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan source Material,” document VCEG-N81, ITU- T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

# Appendix

# 1. R-D curves for P-frame coding (Section 2.1)

# 2. R-D curves for B-frame coding (Section 2.2)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp Labs of America |  |  |
| Mailing address | 5750 NW Pacific Rim Blvd, Camas, WA 98607 |  |  |
| Country | USA |  |  |
| Contact person | Shijun Sun |  |  |
| Telephone | 360 817 7531 |  |  |
| Fax | 360 817 8436 |  |  |
| Email | ssun@sharplabs.com |  |  |
| Place and date of submission | Geneva, Jan 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Global Motion Vector Coding |  |
| Contribution number |  | JVT-B019 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Pending |  |
| Inventor(s)/Assignee(s) |  | Shijun Sun and Shawmin Lei |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: To support a royalty-free baseline we follow ITU-T 2.2.1 for Baseline Profile decoder applications agreeing to a free license on condition that all other patent holders do the same.