| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B029 Filename: JVT-B029.doc Generated: 2002-01-11 |
| --- | --- |

| Title: | Reduced Complexity VLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Louis Kerofsky 5750 NW Pacific Rim Blvd. Camas, WA 98607 Minhua Zhou Texas Instruments Incorporated 12500 TI Boulevard, MS 8649 Dallas, Texas 75243, USA | Tel: Email: Tel: Email: | (360)-817-7644 lkerofsky@sharplabs.com +1 214 480-3816 zhou@ti.com |
| Source: | Sharp Labs of America, Texas Instruments |  |  |

_____________________________

# Introduction

Currently, H.26L supports two entropy encoding tools CABAC and the UVLC [1.].  CABAC uses adaptive arithmetic coding to achieve maximal coding performance at the cost of complexity.  The UVLC is based on variable length code words.  The UVLC is an example of the “back to basics” philosophy of H.26L.  A single VLC table is used for all syntax elements rather than designing distinct VLC tables for each syntax element as has been commonly seen in the past.  This design gives good coding performance without the complexity of arithmetic coding.

# UVLC coding model

The UVLC is based on a logarithmic codeword length distribution, the length of the ith codeword is given by L(i)=2⋅log2(i+1)+1. This distribution determines the coding performance of the UVLC.  The success of the UVLC has been the ability to design the syntax so that this logarithmic codeword distribution approximates the entropy code of each syntax element.  Good examples of modifying the syntax to match the UVLC coding model can be seen in the use of the double scan for coding coefficients [1.] and the run-length coding of skipped macroblocks [2.].

# UVLC code words

As described in TML, the UVLC codewords have a regular structure consisting of data bits interleaved with synchronization bits.  The codeword of length L=2N+1 consists of N data bits interleaved with the N+1 bit synchronization codeword consisting of N zeros followed by a 1.

Table 1 UVLC Codewords

| Data bits | Synchronization bits | UVLC codeword |
| --- | --- | --- |
| X1X2…XN | 0102…0N1 | 01X102X2…0NXN1 |

1

0x1

0x0x1

0x0x0x1

…..

This interleaved structure of the codewords provides a resynchronization property designed to mimic the functionality of reversible VLC codes as used in MPEG-4 for example.  This resynchronization property comes at the cost of additional complexity dealing with the interleaved codewords.

# Resynchronization property

As designed the UVLC resynchronizes to VLC codeword boundaries following a bit error.  This property provides functionality similar to reversible VLCs allowing symbols following a bit error to be decoded.  The UVLC differs in its ability to detect and localize a bit error.  The impact of bit errors on the UVLC is analyzed in [3.].  Synchronization bit errors cause symbol insertions or deletions at the decoder which are difficult to localize.  The problem of error detection becomes very difficult.  Illegal VLC codewords are not present.  Errors must be detected at the syntax level where symbols may have been damage, inserted or deleted.

In using reversible decoding, data partitioning is used along with synchronization points within the data partition.  When an error is detected, the decoder advances to the next synch point and decodes in the reverse direction until another error is detected.  Thus valid data can be decoded up to the first error and following the last error.  This technique depends upon error localization and syntax supporting reversible decoding.  For instance motion vector prediction must be modified to support reversible decoding.  Currently, the use of the UVLC resynchronization property is in unclear.  Error detection and localization need to be understood.  How does the decoder handle symbol insertions or deletions?  The VLC resynchronization property is not supported by the encoding syntax.  For instance the motion vector data is coded differentially.  If the VLC can resynchronize and correctly decode motion vector differences following a loss of synchronization they are useless since the motion vector prediction will be incorrect.  Unequal error protection combined with data partitioning may give comparable or better performance.  Unequal error protection has the advantage of being a function of the Network Adaptation Layer rather than the video encoding layer.  Thus precoded data not originally coded for a bit error prone network can easily be modified to increase the resilience to bit errors.

# Low complexity VLC codewords.

Currently, the interleaved structure of the UVLC codewords provides no advantage but increases the complexity due to the bit interleaving operation.  This complexity was examined in [4.] and [5.].  To eliminate the complexity of encoding and decoding the UVLC codewords, a VLC table with identical coding performance but alternate codewords was proposed in [5.].  The distribution of codeword lengths is the same as that of the UVLC.  The interleaved structure of the UVLC is replaced by a partitioning into a prefix and suffix.  The resynchronization property of the UVLC is sacrificed in exchange for simpler codewords.

Table 2 Low Complexity Codewords

| Data bits | Synchronization bits | Low Complexity codeword |
| --- | --- | --- |
| X1X2…XN | 0102…0N1 | 0102…0N1 X1X2…XN |

1

01x

001xx

0001xxx

…..

The UVLC and low complexity codewords have a simple correspondence consisting of moving all data bits of a UVLC codeword to the end.  Thus both codes have identical coding performance differing only in the form of the codewords..

Table 3 Equivalence of codewords

| UVLC | Low Complexity |
| --- | --- |
| 0X10X2…0XN1 | 00…01 X1X2…XN |

# IPR Statement

Texas Instruments Inc. has intellectual property related to this contribution as stated in the prior contribution [5.] and has chosen to subscribe to sub clause 2.2.1 of the ITU-T patent policy under the condition of reciprocity.  Sharp Labs of America is not aware of additional IP related to this proposal.

# References

- H.26L Test Model Long Term Number 8 (TML-8) draft0, Q.6/SG16(VCEG), 6/28/01

- Use of run-length coding to identify coded macroblocks, Gisle Bjontegaard, VCEG-M29

- Bit errors within the UVLC, Louis Kerofsky, VCEG-L23

- Benchmark Analysis of H.26L Decoder Functional Blocks, Minhua Zhou, VCEG-N23

- Modified Universal Variable Length Coding, Minhua Zhou, VCEG-N36

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 <fax number here>

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Teaxs Instruments Inc. |  |  |
| Mailing address | 12500 TI Blvd., TX-75093 |  |  |
| Country | USA |  |  |
| Contact person | Minhua ZHOU |  |  |
| Telephone | +1 214 480 3816 |  |  |
| Fax |  |  |  |
| Email | zhou@ti.com |  |  |
| Place and date of submission | 01/18/2002 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Reduced Complexity VLC |  |
| Contribution number |  | JVT-B029 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]